3GPP TSG-RAN WG4 # 117	R4-2520231

Dallas, USA, 17th November 2025 – 21st November 2025

Agenda item:	7.5.3

Source:	MediaTek Inc.

Title:	Discussion on performance requirements for FR1 SU-MIMO with spatial channel model

Document for:	Discussion

# Introduction

The Rel-19 study item [1] investigated practical spatial channel modelling methodologies for MIMO demodulation and CSI reporting. The findings of the SI were reported in TR 38.753 [2]. In the Rel-20 work item [3], the target for RAN4 is to specify new performance requirements as follows:

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1

  - Using rCDL-C1 channel model detailed in TR 38.753 as starting point

    - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case.

    - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding.

  - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias

  - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

RAN4 started its work in RAN4#116-bis in October 2025. The agreements and way forward were documented in [4].

# Discussion

# 2.1 Angular scaling update

Angle scaling for rCDL

- Agreement:

  - Update the rCDL channel model based on the latest angular scaling procedure as in TR 38.901 v19.1.0 and further discuss the details.

- Agreement on the details:

  - Use the ‘s’ values for tabulated desired AS values rather than the desired AS values we used in 38.753 to save us the effort of obtaining ‘s’ and aligning among companies.

  - Obtain ‘s’ from the tabulated example AS desired values and use the following desired AS: ASD – 25; ASA – 60; ZSD – 5, ZSA – 15 as baseline to configure rCDL channel model

  - Additional rCDL Channel Models based on different AS are not precluded

In the latest version of TR 38.901 V19.1.0 [5], Section 7.7.5.1 and the related definitions (Appendix A) have significantly changed. As a result, the scaling process has changed from the earlier versions of TR 38.901.

We used the updated AS scaling process to derive a new version of CDL-C. We took the original CDL-C channel model from TR38.901 and applied the scaling process described in TR 38.901 V19.1.0. The desired angular spread values are as agreed in WF, and the corresponding scaling factors are selected from Table 7.7.5.1-1 of TR 38.901 V19.1.0 and shown below in Table 1. Using the angular spread definition in Appendix A.3 of TR 38.901 V19.1.0, our derived model achieves all four angular spread targets very closely.

Our result (CDL-C2) of the AS scaling is provided in Table 2 without cluster truncation or sub-cluster merging. However, side information for the truncation such as antenna element radiation pattern (AEP) attenuation and total cluster power = power + AEP is shown. AEP is calculated as the attenuation for the middle AOD/ZOD of each cluster (as specified in TR38.753 Appendix B.3.1), i.e., not calculated as sum of the per ray AEPs. It is good to note that clusters 16 and 17 are almost equally weak so that the selection between them for rCDL-C2 is a delicate choice.

Table 1: Desired AS and scaling factors

| CDL Type | Desired AOD Spread（°） | Scale Factor (AOD) | Desired AOA Spread（°） | Scale Factor (AOA) | Desired ZOA Spread（°） | Scale Factor (ZOA) | Desired ZOD Spread（°） | Scale Factor (ZOD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDL-C | 25 | 0.6513 | 60 | 0.8585 | 15 | 1.9504 | 5 | 1.8219 |

Table 2: Untruncated CDL-C2 with updated angular scaling (with side info for truncation)

| Cluster # | Cluster delay [ns] |  | Power in [dB] | AEP [dB] | Total power [dB] |  | AOD in [°] | AOA in [°] |  | ZOD in [°] | ZOA in [°] | rCDL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 |  | -4.4 | -4.24 | -8.64 |  | -38.4 | -117.2 |  | 95.4 | 101.1 | yes |
| 2 | 76 |  | -1.2 | -1.49 | -2.69 |  | -22.8 | 123.5 |  | 98.0 | 70.9 | yes |
| 3 | 80 |  | -3.5 | -1.49 | -4.99 |  | -22.8 | 123.5 |  | 98.0 | 70.9 | merge |
| 4 | 86 |  | -5.2 | -1.49 | -6.69 |  | -22.8 | 123.5 |  | 98.0 | 70.9 | merge |
| 5 | 80 |  | -2.5 | -3.39 | -5.89 |  | -34.5 | -140.0 |  | 101.6 | 66.9 | yes |
| 6 | 232 |  | 0 | -0.18 | -0.18 |  | -7.8 | 166.7 |  | 99.1 | 77.1 | yes |
| 7 | 236 |  | -2.2 | -0.18 | -2.38 |  | -7.8 | 166.7 |  | 99.1 | 77.1 | merge |
| 8 | 240 |  | -3.9 | -0.18 | -4.08 |  | -7.8 | 166.7 |  | 99.1 | 77.1 | merge |
| 9 | 240 |  | -7.4 | -4.74 | -12.14 |  | 39.6 | 68.0 |  | 110.0 | 61.7 | yes |
| 10 | 290 |  | -7.1 | -7.29 | -14.39 |  | -50.0 | 77.5 |  | 92.0 | 54.7 | yes |
| 11 | 300 |  | -10.7 | -5.94 | -16.64 |  | 44.2 | -71.8 |  | 111.7 | 69.5 | yes |
| 12 | 340 |  | -11.1 | -14.78 | -25.88 |  | -71.2 | 60.7 |  | 88.7 | 48.2 | trunc |
| 13 | 448 |  | -5.1 | -5.56 | -10.75 |  | -44.0 | 78.9 |  | 107.3 | 106.9 | yes |
| 14 | 478 |  | -6.8 | -7.26 | -14.06 |  | -49.9 | -89.5 |  | 108.2 | 47.4 | yes |
| 15 | 792 |  | -8.7 | -10.35 | -19.05 |  | -59.1 | 90.4 |  | 87.8 | 49.2 | yes |
| 16 | 990 |  | -13.2 | -10.04 | -23.24 |  | 58.9 | 46.8 |  | 108.2 | 126.6 | trunc |
| 17 | 1554 |  | -13.9 | -9.32 | -23.22 |  | 56.6 | 6.4 |  | 91.3 | 51.7 | yes |
| 18 | 1680 |  | -13.9 | -7.46 | -21.36 |  | 49.8 | 23.7 |  | 88.0 | 60.3 | yes |
| 19 | 2004 |  | -15.8 | -16.24 | -32.04 |  | -74.4 | 8.7 |  | 86.3 | 33.4 | trunc |
| 20 | 2046 |  | -17.1 | -8.16 | -25.26 |  | 52.1 | 28.8 |  | 112.8 | 50.8 | trunc |
| 21 | 2302 |  | -16 | -8.33 | -24.33 |  | 52.8 | 25.3 |  | 87.8 | 31.5 | trunc |
| 22 | 2422 |  | -15.7 | -11.15 | -26.85 |  | 61.4 | 21.0 |  | 87.6 | 50.6 | trunc |
| 23 | 2570 |  | -21.6 | -14.14 | -35.74 |  | 69.8 | 1.6 |  | 110.0 | 43.4 | trunc |
| 24 | 3158 |  | -22.8 | -22.93 | -45.73 |  | -88.6 | 49.3 |  | 114.8 | 41.4 | trunc |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  |  | cZSA in [°] |  | XPR in [dB] |  |
| Value |  | 2 |  | 15 |  | 3 |  |  | 7 |  | 7 |  |

Observation #1: We applied the updated AS scaling process, and the resulting channel achieves the desired ASD, ASA, ZSA, and ZSD values.

Proposal #1: RAN4 companies to do angular scaling using the updated process in TR 38.901 V19.1.0 and cross-check the results.

# 2.2 Delay scaling after cluster truncation

Delay scaling after cluster truncation

- Candidate options:

  - Option 1: Consider omitting the post-truncation delay scaling step in rCDL model derivation. Methods other than linear scaling and/or metrics other than RMS delay spread could be investigated.

  - Option 2: Reuse the Rel-19 SI approach captured in TR38.753

In TR 38.753, the derivation process of rCDL-C1 consists of angular scaling, sub-cluster merging + cluster truncation down to 12 clusters, and post-truncation delay scaling. The rationale of the delay scaling step is to retain the RMS delay spread of the original untruncated channel model. The applied scaling factor for rCDL-C1 was approximately 1.7. The truncation and scaling processes are described in TR38.753 Appendix B.3.1.

We applied sub-cluster merging and cluster truncation on CLD-C2, and the result is shown in Table 3. For cluster delays, both unscaled and delay-scaled values are tabulated. The applied delay scaling factor was 1.5955.

Table 3: Truncated rCDL-C2 before and after post-truncation delay scaling (DS)

| Cluster # | Cluster delay [ns] before DS | Cluster delay [ns] after DS | Power in [dB] |  | AOD in [°] |  | AOA in [°] |  | ZOD in [°] |  | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | -7.4 |  | -38.4 |  | -117.2 |  | 95.4 |  | 101.1 |
| 2 | 76 | 122 | -1.2 |  | -22.8 |  | 123.5 |  | 98.0 |  | 70.9 |
| 3 | 80 | 128 | -5.5 |  | -34.5 |  | -140.0 |  | 101.6 |  | 66.9 |
| 4 | 232 | 370 | 0 |  | -7.8 |  | 166.7 |  | 99.1 |  | 77.1 |
| 5 | 240 | 382 | -10.4 |  | 39.6 |  | 68.0 |  | 110.0 |  | 61.7 |
| 6 | 290 | 462 | -10.1 |  | -50.0 |  | 77.5 |  | 92.0 |  | 54.7 |
| 7 | 300 | 478 | -13.7 |  | 44.2 |  | -71.8 |  | 111.7 |  | 69.5 |
| 8 | 448 | 714 | -8.1 |  | -44.0 |  | 78.9 |  | 107.3 |  | 106.9 |
| 9 | 478 | 762 | -9.8 |  | -49.9 |  | -89.5 |  | 108.2 |  | 47.4 |
| 10 | 792 | 1264 | -11.7 |  | -59.1 |  | 90.4 |  | 87.8 |  | 49.2 |
| 11 | 1554 | 2480 | -16.9 |  | 56.6 |  | 6.4 |  | 91.3 |  | 51.7 |
| 12 | 1680 | 2680 | -16.9 |  | 49.8 |  | 23.7 |  | 88.0 |  | 60.3 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Parameter Value |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  | cZSA in [°] |  | XPR in [dB] |  |
|  |  | 2 |  | 15 |  | 3 |  | 7 |  | 7 |  |

We believe that delay scaling more alters than retains the original channel properties. Cluster truncation effectively cuts the weak tail off from the original PDP, and delay scaling overcompensates by significantly increasing the delays of all remaining clusters. Moving the delays of the strong clusters has a significant impact on channel frequency selectivity properties.

One aspect of the issue is that in simulations and in the cluster truncation phase, BS antenna element radiation pattern (AEP) is applied, while in the delay scaling phase (RMS delay spread calculation) the effect of AEP is omitted. From the testing and simulation point of view, the more significant measure is RMS delay spread with AEP included (AEP-RMS-DS). The impact of the delay scaling is that while RMS-DS is retained at 365 ns, AEP-RMS-DS is actually increased. For the untruncated CDL-C2 channel, AEP-RMS-DS is 171.6 ns. After truncation, for rCDL-C2 the value is reduced to 132.6 ns, and after delay scaling it is increased to 211.3 ns.

Observation #2: Current post-truncation delay scaling strategy increases the antenna element radiation pattern weighted delay spread (AEP-RMS-DS) compared to the original 24-cluster model.

In Figure 1 we illustrate the power delay profile (PDP) of three model variants: the untruncated 24-cluster model CDL-C2, the truncated 12-cluster model rCDL-C2 without delay scaling, and the delay-scaled 12-cluster model rCDL-C2b. In Figure 2, for the same three models, we show the measured frequency coherence (FC) as defined in TR 38.753 Section 6.4.1. In addition, FC for a 16-cluster model variant rCDL-C2c without delay scaling is shown. As can be seen, without delay scaling, the 12-cluster and 16-cluster models are almost identical in terms of FC. This suggests that the clusters weaker than the 12 strongest ones have a marginal impact on the effective channel.

Observation #3: Without delay scaling, the 12-cluster and 16-cluster variants of rCDL-C2 are almost identical in terms of frequency coherence.

As can be seen in Figure 1, the PDP of the 24-cluster model includes a long tail consisting of very weak delay taps that have relatively small impact on the channel response or receiver performance. However, the impact of the tail to the calculated RMS delay spread is so significant that when truncating the tail, noticeable delay scaling is needed to maintain the same level of RMS delay spread.

As can be seen in Figure 2, the post-truncation delay scaling significantly alters the frequency coherence and selectivity properties of the channel. In our view, the truncated model without delay scaling retains the FC properties of the original channel better than the delay-scaled model. Therefore, we propose to omit the additional delay scaling step in channel derivation.

Observation #4: Post-truncation delay scaling alters the frequency selectivity of the truncated rCDL-C2 model compared to the original untruncated model.

Proposal #2: Omit the post-truncation delay scaling step in rCDL-C2 model derivation.

Figure 1: PDP in rCDL-C2 derivation

Figure 2: FC in rCDL-C2 derivation with 30kHz SCS

Finally, from the simulated channel, we measured the Spatial Domain Power Density (SDPD) as defined in TR38.753 Section 6.4.1. The results are depicted in Figure 3. As can be see, SDPD is similar but not identical for CDL-C1 and CDL-C2, while cluster truncation has a marginal effect on both. We believe that the difference between rCDL-C1 and rCDL-C2 is acceptable so that the conclusions of Rel-19 SCM SI remain valid.

Figure 3: SDPD comparison at TX side (left) and at RX side (right).

# 2.3 UE antenna modelling

UE antenna modelling

- <For INFORMATION> It is RAN4 common understanding that the following were added in Rel-19 version of TR 38.901:

  - #1: Candiadate location for UE antenna placements

  - #2: Directional antenna assumption for UE

- Further discuss the two options in the next meeting:

  - Option 1: Update the UE antenna placement and assumption of rCDL based on v19.0.0 of 38.901.

  - Option 2: Use CDL parameters listed in Table 6.1-2 of TR 38.753 as baseline

  - The moderator requests proponents of option 1 to provide detailed simulation assumptions in the next meeting.

We support option 2 to keep the UE antenna array assumptions as in TR38.753. The new antenna placement models defined in TR38.901 v19.1.0 may become useful in 6G. However, in the scope of this work item, there is not enough time to implement and align the changes.

Proposal #3: Keep the UE antenna array assumptions as in TR38.753 due to time limitations (option 2).

# 2.4 CDL model alignment

Further rCDL-C1 channel alignment

- Candidate options:

  - Option 1: Measure and align channel statistics such as Spatial Domain Power Density (SDPD), Time Coherence (TC), and Frequency Coherence (FC) as described in TR 38.753 Section 6.4.

  - Option 2: Not to do further rCDL-C1 channel alignment

During the Rel-19 SCM study item it became clear that aligning the link-level CDL models between companies is particularly difficult. This is partly due to the lengthy documentation in TR 38.901 that focuses on system level and geometric model descriptions so that it takes lot of effort to clarify the intended implementation details and parameters of the link level model.

Demodulation and CSI performance simulations are complicated and challenging to align between companies even with the simple TDL channel models. When introducing CDL models into RAN4 tests, it is desirable to avoid any unnecessary additional uncertainty/ambiguity in the simulation alignment. As RAN4 channel models, CDL models will have to be implemented and simulated by a wider range of companies and institutions than before.

To enable CDL channel model alignment and to minimise model ambiguity, we propose RAN4 to consider the following approach: measure and align channel statistics such as Spatial Domain Power Density (SDPD), Time Coherence (TC), and Frequency Coherence (FC) as described in TR 38.753 Section 6.4. For easy and repeatable alignment, reference statistics for each specified channel model variant should be provided by RAN4.

Observation #5: As observed during the Rel-19 SCM study item, aligning the link-level CDL models between companies is particularly difficult.

Proposal #4: To enable CDL model alignment, we propose to measure and align channel statistics such as Spatial Domain Power Density (SDPD), Time Coherence (TC), and Frequency Coherence (FC) as described in TR 38.753 Section 6.4.

# 2.5 PMI bias

PMI bias

- Candidate options:

  - Option 1: RAN4 shall exploit approaches to address the PMI bias issue

- Option 1a: Apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3

- Option 1b: time variant mean angles (AOA/AOD/ZOA/ZOD)

- Other options are not precluded

  - Option 2: No need to consider time-wise evolution for PMI reporting tests

  - Option 3: Defer a consideration of a comprehensive long-term solutions to the 6G Radio Study Item

The spatial properties of the existing link-level CDL models are constant since the cluster directions (AOA, AOD, ZOA, ZOD) are fixed (tabulated) to certain angles that do not change over time or from one simulation realization to another. As a result, the optimal PMI may remain almost constant so that a UE could in the worst case pass a PMI reporting test just by reporting a single fixed PMI.

There are two alternative approaches to improve the PMI test coverage with CDL channel models:

- Option 1a: The simple approach to improve PMI test coverage is to directly apply the existing single-cluster beam steering approach specified in TS 38.101-4 Annex B (B.2.3.2.3). In beam steering, the MIMO channel matrix is phase-rotated column-wise in time-variant manner so that the optimal transmit precoder beam directions vary. This approach is readily available and can be applied on top of any MIMO channel model.

- Option 1b: Apply time-variant mean angles (AOA, AOD, ZOA, ZOD) to the model. As a result, the angles of all clusters and all rays would be varying over time. This approach would mean that the ray-specific parameters such as antenna radiation pattern (power) responses, directional array responses, and Doppler shifts would have to be dynamically updated during the simulation. This approach requires major changes to the existing CDL model implementations and would be very challenging to align between companies.

Proposal #5: To reduce PMI bias, choose option 1a: apply the existing simple beam steering approach specified in TS 38.101-4 Annex B (B.2.3.2.3).

# 2.6 PDSCH demodulation test cases

Precoding, MCS and test metric

- Agreement and candidate options:

  - For 4T4R 4L test, it is agreed to use the test metric: SNR @ 70% max TP.

  - For 8T8R 8L test, it is agreed to use test metric: Define single SNR and further discuss the SNR is based on:

    - Option a: Based on per CW thput with Different MCS per CW

    - Option b: Based on total thput with different MCS per CW

    - Option c: Based on total thput with same MCS per CW

    - Further discuss: How to define the SNR for option a. How to select the MCS for different CW.

We simulated MCS13 tput for 4TX-4RX-4L and 8TX-8RX-8L with random precoding in both CDL-C1 and CDL-C2 based model variants. The results are shown in Figure 4. As can be seen, the old and the new channel models have similar performance curves. For 8TX-8RX-8L, the quality of CW0 is clearly better than the quality of CW1.

Observation #6: 8TX-8RX-8L simulation results in rCDL-C2 with random precoding show clear performance difference between CW0 and CW1.

For 8TX-8RX-8L, the BLER performance difference between the codewords can be reduced by assigning different MCSs to them. This strategy further improves the test coverage and makes the use case more realistic.

We simulated MCS options 10, 11, 12, and 13 for CW1 while keeping MCS 13 for CW0. The results are shown in Figure 5. It is worth noting that here the performance of CW0 is not affected by changing the MCS of CW1.

As can be seen from the per-CW relative tput curves in Figure 5 (right), when MCS 10 is assigned to CW1, the two codewords perform almost identically. Thus, a common SNR requirement can be set to the two codewords. For the test metric, we support Option a: per CW tput with different MCS per CW. We believe it is crucial for the test coverage that both codewords are performing adequately. If the requirement is set for the total tput only, CW0 with its higher tput will dominate the test metric. In the worst case, test could be passed even if CW1 is performing poorly.

Proposal #6: For 8TX-8RX-8L demod, use random precoding, assign different MCS levels (13, 10) for the two codewords, and set single SNR requirement for per-CW 70% relative tput.

Figure 4: MCS13 PDSCH demod tput for 4TX-4RX-4L (left) and at 8TX-8RX-8L (right).

Figure 5: 8TX-8RX-8L demod in rCDL-C2 with different MCS for CW1: absolute tput (left) and normalized tput (right).

# 2.7 PMI test cases

Whether to define eType II PMI reporting requirements based on SCM

- Candidate options:

  - Option 1: Specify requirements for PMI performance with eType II precoding

- Option 1a: Select the lowest g  value among the ones companies provided.

- Option 1b: Outlier removal to be applied to form requirements, if outliers are not aligned in second simulation results delivery round.

  - Option 2: Further simulation result alignment is required

- Option 2a: Collect both TP gain and SNR @ 90% Max TP to evaluate alignment

- Option 2b: RAN4 can take 8Tx 4 layers Type-I PMI case for initial alignment if rCDL model is updated.

We support option 2. We believe simulation alignment with the updated rCDL model must be achieved before new requirements can be specified.

Test scope for eType II PMI reporting (if introduced)

- Candidate options:

  - Option 1: RAN4 needs to define requirements for both 8T4R and 4T4R cases as captured in the WID

During Rel-19 SCM SI, no PMI cases in 4TX-4RX setup were considered. However, this case is now listed in the WID as a new scenario. It is difficult to see why this case would be of additional interest, especially with eType2 for which the legacy test cases are with 16TX. We assume this was mistakenly added in RAN plenary. We propose to ignore the 4TX-4RX follow PMI case.

Observation #7: During Rel-19 SCM SI, no PMI cases in 4TX-4RX setup were considered. This case is listed in the WID as a new scenario.

Proposal #7: RAN4 not to consider 4TX-4RX follow PMI test.

Rank for eType II PMI reporting (if introduced)

- Candidate options:

  - Option 1: Only consider rank 2

  - Option 2: Consider rank 4

During Rel-19 SCM SI, PMI reporting cases were simulated for alignment for ranks 2 and 4 with 8TX-4RX setup. We believe those simulation cases can be used as basis for new tests and performance requirements.

According to the WID, new eType2 PMI tests should be specified. The simulation results [6] showed that some gain from eType2 over Type1 can be achieved with rank 2. However, with rank4, Type1 seemed to perform equal or better than eType2. We believe a reasonable test case for advanced codebooks should be such that gain over simpler codebooks can be achieved. Therefore, we propose that the new eType2 test cases for 8TX-4RX are specified with rank 2 only. It was observed during the SI that to get eType2 test scenario feasible for rank 4, 32TX should be applied.

Observation #8: According to Rel-19 SCM simulations in CDL channels, eType2 PMI feedback provides no gain against Type1 PMI feedback with 8TX-4RX rank 4.

Proposal #8: Specify new eType2 test cases for 8TX-4RX with rank 2 only (option 1).

Test metric for eType II PMI reporting (if introduced)

- Candidate options:

  - Proposal 1: Reuse the same test metric for the existing eType II requirement based on TDL, i.e., Thput of following PMI (eType II)/ Thput of Random PMI (Type I codebook) with SNR @ 90 % of the maximum throughput for following PMI.

  - Proposal 2: Check PMI simulation results on both normalized throughput and gamma values and do down selection.

We support Proposal 2. It may be easier to align SNR @ 90% tput than gamma values. Thus, we propose not to exclude the possibility of specifying normalized tput based requirements for PMI tests at this point.

Proposal #9: Keep test metric options open for eType2 PMI tests and select one when simulation alignment is achieved.

# 2.8 Duplex mode and SCS

Duplex mode and SCS

- Agreement:

  - Cover TDD 30kHz SCS with 40MHz CHBW and FFS whether to also cover FDD 15kHz SCS with 10MHz CHBW

We believe it is adequate to set requirements for TDD 30kHz SCS with 40MHz CHBW only, and not to consider FDD. These new tests are merely complementing the existing legacy test cases with no new features introduced. Therefore, the scope of the new requirements can be kept more limited.

Proposal #10: Only cover TDD 30kHz SCS with 40MHz CHBW, and do not consider FDD.

# 2.9 Test applicability for SCM SU-MIMO requirements

Test applicability for SCM SU-MIMO requirements

- Candidate options:

  - Proposal 1: Introduce new declaration for the SCM testing. RAN4 needs to discuss whether to define one single declaration on SCM requirements or define multiple declarations for each individual SCM test cases.

  - Proposal 2: Introduce new optional UE capability “SCM compatible MIMO receiver” and corresponding applicability rules

  - Proposal 3: If requirements with rCDL are introduced, their applicability should be optional and based on UE declaration to support these requirements.

We consider it necessary to introduce a new UE declaration if requirements with rCDL are introduced. The details of the UE declaration can be discussed after the requirements to be introduced have been confirmed.

Proposal #11: Introduce a new UE declaration if requirements with rCDL are introduced.

# Conclusion

In this paper we provided our views on performance requirements for FR1 SU-MIMO with spatial channel model. The following observations and proposals are made:

Observation #1: We applied the updated AS scaling process, and the resulting channel achieves the desired ASD, ASA, ZSA, and ZSD values.

Proposal #1: RAN4 companies to do angular scaling using the updated process in TR 38.901 V19.1.0 and cross-check the results.

Observation #2: Current post-truncation delay scaling strategy increases the antenna element radiation pattern weighted delay spread (AEP-RMS-DS) compared to the original 24-cluster model.

Observation #3: Without delay scaling, the 12-cluster and 16-cluster variants of rCDL-C2 are almost identical in terms of frequency coherence.

Observation #4: Post-truncation delay scaling alters the frequency selectivity of the truncated rCDL-C2 model compared to the original untruncated model.

Proposal #2: Omit the post-truncation delay scaling step in rCDL-C2 model derivation.

Proposal #3: Keep the UE antenna array assumptions as in TR38.753 due to time limitations (option 2).

Observation #5: As observed during the Rel-19 SCM study item, aligning the link-level CDL models between companies is particularly difficult.

Proposal #4: To enable CDL model alignment, we propose to measure and align channel statistics such as Spatial Domain Power Density (SDPD), Time Coherence (TC), and Frequency Coherence (FC) as described in TR 38.753 Section 6.4.

Proposal #5: To reduce PMI bias, choose option 1a: apply the existing simple beam steering approach specified in TS 38.101-4 Annex B (B.2.3.2.3).

Observation #6: 8TX-8RX-8L simulation results in rCDL-C2 with random precoding show clear performance difference between CW0 and CW1.

Proposal #6: For 8TX-8RX-8L demod, use random precoding, assign different MCS levels (13, 10) for the two codewords, and set single SNR requirement for per-CW 70% relative tput.

Observation #7: During Rel-19 SCM SI, no PMI cases in 4TX-4RX setup were considered. This case is listed in the WID as a new scenario.

Proposal #7: RAN4 not to consider 4TX-4RX follow PMI test.

Observation #8: According to Rel-19 SCM simulations in CDL channels, eType2 PMI feedback provides no gain against Type1 PMI feedback with 8TX-4RX rank 4.

Proposal #8: Specify new eType2 test cases for 8TX-4RX with rank 2 only (option 1).

Proposal #9: Keep test metric options open for eType2 PMI tests and select one when simulation alignment is achieved.

Proposal #10: Only cover TDD 30kHz SCS with 40MHz CHBW, and do not consider FDD.

Proposal #11: Introduce a new UE declaration if requirements with rCDL are introduced.

# Reference

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- TR 38.753, Study on spatial channel model for demodulation performance requirements for NR

- RP-252957, “3GPP Work Item Description: NR demodulation performance: Phase 6”, Apple

- R4-2514823, WF on NR_demod_Ph6_Ph5, China Telecom

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 19), V19.1.0 (2025-09)

- R4-2505594, ”Simulation results of Spatial Channel Modelling”, MediaTek