3GPP TSG-RAN4 Meeting #117	R4-2520264

Dallas, TX, US, Nov. 2025

Title:	Reply LS on Beam Correspondence Initial Access

Source:	Qualcomm, Sony, Lenovo

Agenda item:	11.3

Document for:	Approval

# 1.	Introduction

The LS from RAN5 [1] seeks clarifications on the Rel-18 initial access beam correspondence core requirement in the form of three questions. Our view is provided below.

# 2.	Discussion on beam correspondence

It is useful to first converge on a common understanding of what the initial access beam correspondence (IABC) requirement is. A way-forward [5] listed out some of the questions that were felt to be important, even though the WF itself was not approved. These questions are listed below along with our views on them:

In the field, a UE must register its MSG1 at the gNB with sufficient SINR during a RACH occasion as part of a successful RACH procedure. The wording of the core requirement addresses this exact field behavior, with the one deviation being a need to demonstrate ‘1 ms’ worth of RACHs, rather than just one successful MSG1 event. The core requirement is therefore relevant to field operation, but the relevance of this requirement is compromised if UEs use special non-field behaviour to meet it (test mode).

Observation 2-1a: The wording in the RAN4 core requirement is largely consistent with field operation, but the relevance of the test is compromised if UEs use special behavior (test functions) during compliance.

The question of identifying bad UEs is always good to revisit, but it requires an agreed notion of ‘bad’. For this, we can only turn to 38.101-2, where the requirement defines a failing UE is one that cannot muster sufficient EIRP while transmitting MSG1 in the direction of the downlink. Any other definition of ‘bad’ UE in the IABC context needs justification via standards references.

Observation 2-1b: A bad UE in the IABC context is a UE that cannot demonstrate sufficient MSG1 EIRP in the direction of the downlink. The standards reference is the requirement on statistics of EIRP in DL direction, across all direction as outlined in section 6.2.1 of 38.101-2.

Observation 2-1c: Since the notion of ‘bad’ UE is derived from the core requirement, the core requirement is automatically good at identifying good and bad UEs.

The DL RS must be measured by the UE primarily for timing and frequency alignment. UEs are also allowed to use the DLRS to search for an optimal beam. Hypothetically, a UE could have a single beam from an FR1-style discrete antenna with low gain but high TRP and meet the beam correspondence requirements. This example illustrates that it is up to UE implementation what the UE does with the DLRS for beam management, and not visible outside the UE. A UE’s dependence on DLRS is checked only in terms of the UL EIRP that manifests at the prescribed time intervals.

Observation 2-2: The DL RS is necessary for the beam correspondence test. A UE cannot reliably meet the beam correspondence core requirement without timing and frequency alignment.

As is well understood, the network expects the UE to send a MSG1 in certain predefined time intervals known as RACH occasions (RO) that depend on SSB index and other information in the SSS. The UE uses the next valid occasion in time after detecting the SSB, but there is no stipulation on which SSB is the ‘start of the procedure’. In the field, it is in the UE’s interest to respond at the earliest opportunity but usually there is a variable amount of delay for executing its beam selection or refinement logic. This behavior is legal because the network cannot stipulate which RO must be used by a UE, instead it must monitor all of them.  The beam correspondence requirement accordingly is not a dynamic test that measures RACH response time, and the TE must monitor all RO just like the gNB would.

Observation 2-3: In the field, the choice of RACH occasion to send MSG1 to the network is up to UE implementation. This is also the expectation during for the core requirement.

Autonomous in the beam correspondence context means ‘without network assistance’ and by using only the UE’s logic. RAN1 and RAN2 design do not have a notion of ‘best’ beam. The RAN4 core requirement checks it the UE’s beam selection behavior allows the UE to demonstrate sufficient MSG1 EIRP in all directions. This is the only implicit requirement on the ‘quality’ of the autonomously chosen beam.

Observation 2-4: There is no notion of ‘best’ beam in RAN4 UE RF. The IABC requirement implicitly requires the UE’s chosen beams to be good enough so it can demonstrate sufficiently good statistics of MSG1 EIRP in the direction of the downlink.

The beam correspondence requirement is based on EIRP in the direction of DL. It does not check or constrain the EIRP in directions other than of the DL. (The only Rel-18 requirements that measure EIRP in directions other than DL are those that have a TRP metric, rather than EIRP.)

Observation 2-5: Per 38.101-2, the IABC requirement only concerns itself with EIRP in the direction of DL. The IABC requirement applies no constraint on EIRP in directions other than that of the DL.

Proposal 2-1: RAN4 to achieve common understanding on the Rel-18 initial access beam correspondence requirement in terms of the questions recorded in R4-2515162.

# 3.	Reply

## 3.1	RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

In general, BL (beam lock) and other test-related functions (‘test modes’) exist to facilitate testing in some way (RAN5 domain). UE core requirements (RAN4 domain) on the other hand are meant to establish design goals for field performance for UEs. For relevance to field performance, it is preferred for UEs to not rely on behavior-modifying test modes (like BL) for compliance.

In the context of the Rel-18 IABC (initial access and RRC_inactive beam correspondence), RAN4 have discussed BL extensively as part of testability of the IABC requirement. While there was no technical consensus among participating companies on the need for BL, a compromise was reached where RAN4 agreed to a set of requirements based on existence of a BL function for verification [2]. In our understanding of both intent and wording in the LS or the core requirement, there is no mandate from RAN4 for the UE to implement such a function for compliance.

Observation 3-1: There is no mandate from RAN4 for the UE to implement a test function like beam lock for compliance.

Based on observations in section 2, it is evident that the wording in the RAN4 core requirement is consistent with ensuring a certain expectation in field operation, and use of beam lock in fact reduces the relevance of the requirement.

Beyond the technical argument above, one must also consider where RAN4 is headed, as we start discussing 6G. If RAN4 ties compliance with core requirements to implementation of a test function, it implies that a UE that can meet the core requirements without the test function is non-compliant. This is not RAN4 intent (even if some companies may want this) and is a dangerous precedent.

Observation 3-2: If RAN4 ties compliance with core requirements to implementation of a test function, a worrisome implication for the future of RAN4 is that ‘Even if a UE that can meet core requirements without an available test function, it is non-compliant.’

Proposal 3-1: Discuss if OK for RAN4 to enforce ‘Even if a UE that can meet core requirements without an available test function, it is non-compliant.’

Proposal 3-2: RAN4 to clarify in the reply LS, the intent of the Rel-18 initial access beam correspondence core requirement and confirm it does not include implementation of a test function.

Consequently:

Answer 1: Yes, either procedure can be used. The initial access beam correspondence core requirements require the UE to transmit sufficient MSG1 EIRP in the direction of the gNB during 1ms of RACH occasions. The core requirements do not mandate implementation of a beamlock test function. It is up to RAN5 how best to verify UE compliance.

# 4. 	Conclusions

Towards establishing common understanding on the initial access beam correspondence requirement, questions posed in R4-2515162 are answered:

Observation 2-1a: The wording in the RAN4 core requirement is largely consistent with field operation, but the relevance of the test is compromised if UEs use special behavior (test functions) during compliance.

Observation 2-1b: A bad UE in the IABC context is a UE that cannot demonstrate sufficient MSG1 EIRP in the direction of the downlink. The standards reference is the requirement on statistics of EIRP in DL direction, across all direction as outlined in section 6.2.1 of 38.101-2.

Observation 2-1c: Since the notion of ‘bad’ UE is derived from the core requirement, the core requirement is automatically good at identifying good and bad UEs.

Observation 2-2: The DL RS is necessary for the beam correspondence test. A UE cannot reliably meet the beam correspondence core requirement without timing and frequency alignment.

Observation 2-3: In the field, the choice of RACH occasion to send MSG1 to the network is up to UE implementation. This is also the expectation during for the core requirement.

Observation 2-4: There is no notion of ‘best’ beam in RAN4 UE RF. The IABC requirement implicitly requires the UE’s chosen beams to be good enough so it can demonstrate sufficiently good statistics of MSG1 EIRP in the direction of the downlink

Observation 2-5: Per 38.101-2, the IABC requirement only concerns itself with EIRP in the direction of DL. The IABC requirement applies no constraint on EIRP in directions other than that of the DL.

Proposal 2-1: RAN4 to achieve common understanding on the Rel-18 initial access beam correspondence requirement in terms of the questions recorded in R4-2515162.

Observation 3-1: There is no mandate from RAN4 for the UE to implement a test function like beam lock for compliance.

Observation 3-2: If RAN4 ties compliance with core requirements to implementation of a test function, a worrisome implication for the future of RAN4 is that ‘Even if a UE that can meet core requirements without an available test function, it is non-compliant.’

Proposal 3-1: Discuss if OK for RAN4 to enforce ‘Even if a UE that can meet core requirements without an available test function, it is non-compliant.’

Proposal 3-2: RAN4 to clarify in the reply LS, the intent of the Rel-18 initial access beam correspondence core requirement and confirm it does not include implementation of a test function.

# 5. 	References

- R5-253653, ‘LS on Beam Correspondence Initial Access’, RAN5, RAN5#107, May. 2025

- R4-2317764, ‘LS on testability issues of beam correspondence requirement for initial access’, RAN4#108Bis, Oct. ‘23

- R4-2511345, ‘Reply LS on Beam Correspondence Initial Access‘, Qualcomm et al., RAN4#116, Aug. ‘25

- R4-2514421, ‘Reply LS on Beam Correspondence Initial Access‘, Qualcomm et al., RAN4#116-Bis, Oct. ‘25

- R4-2515162, ‘WF for LS reply to RAN5 on Rel-18 initial access beam correspondence’, Ericsson, RAN4#116-Bis, Oct. ‘25

3GPP TSG-RAN4 Meeting #117	R4-2520264

Dallas, TX, US, Nov. 2025

Title:	[Draft] Reply LS on Beam Correspondence Initial Access

Response to:	(R5-253653) LS on Beam Correspondence Initial Access

Source:	RAN4

To:	RAN5

Contact Person:

Name	: Sumant Iyer

E-mail Address	: sumanti _at_ qti_dot_qualcomm_dot_com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN4 would like to thank RAN5 for their questions, which are clarified thusly:

- RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

Answer1: Yes, either procedure can be used. The initial access beam correspondence core requirements require the UE to transmit sufficient MSG1 EIRP in the direction of the gNB during 1ms of RACH occasions. The core requirements do not mandate implementation of a beamlock test function. It is up to RAN5 how best to verify UE compliance.

- Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams? Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

Answer2: RAN4 is of the following view:

- Using beamlock function in testing does not prevent UE from choosing UL beams in the real field.

- In the beamlock function based test, UE should still be able to autonomously choose the UL beams based on DL measurement before the beamlock function is activated.

  - RAN4’s understanding is that beamlock function should be activated after the UL beam is autonomously chosen.

- Once the beam is locked, UE cannot autonomously change the UL beams.

- The exact test setup should be decided by RAN5.

- Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

Answer3: Please refer to the answer to Q1 above.

2. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #116-Bis	Oct. 2025         		Prague, CZ

TSG-RAN WG4 Meeting #117	Nov. 2023         		Dallas, TX, US