# 3GPP TSG-RAN WG4 Meeting #117	R4-2520294

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	7.6.4.1

Source: 	Sharp

Title: 	OTA test method for NTN Ka and Ku bands

Document for:	Discussion and decision

Introduction

The following way forward in R4-2514820 was approved at RAN4#116bis meeting.

### Sub-topic 2-6: Polarizaion

RAN4 start to work on whether and how to support of CP testing in IFF system.

Linear polarization is supported as basis based on the WID scope.

The core requirements of NTN will be investigated to define the applicability of polarization on the requirements in the next RAN4 meeting.

### Sub-topic 2-3: Quiet zone size

The [63] cm QZ for a [45 x 45] antenna appears to be the smallest viable size

For commercial reasons, a larger QZ would be considered

### Sub-topic 2-4: DUT size and weight

VSAT vendors/NTN operators to provide the maximum VSAT device size, antenna size and relative position, and weight limits for each NTN VSAT type.

### Sub-topic 2-10: Beamlock

RAN4 further check whether beam steering is needed for VSAT conformance testing.

### Sub-topic 2-12: Antenna gain measurement

The antenna gain is not testable from a conformance perspective and should be considered as a system level requirement.

Clarification of core spec for this antenna gain topic should be done from Rel-18/19 NTN WIs.

The vendors’ input is summarized in the moderator’s contribution [R4-2520340]. In this document, additional views on top of the open issues above are provided.

Discussions

Antenna position of full duplex VSAT

For the typical implementation of terrestrial UEs, Tx and Rx antenna are physically shared, and hence the antenna position for Tx and Rx are identical. On the other hand, a full-duplex VSAT with Ka/Ku-band may employ physically separate aperture for Tx and Rx, respectively. This aspect should be taken into account when OTA test enhancements for VSAT are studied.

Proposal 1: RAN4 to take into account that a full duplex VSAT UE may employ separate Tx and Rx arrays, resulting in a different phase center for each Tx and Rx antenna aperture.

Necessity of beam steering tests for NGSO

In NGSO network, a VSAT UE needs to track satellite directions considering UE and satellite movement. For the UE RF tests, it is possible to obtain the necessary OTA Tx/Rx performance results by using explicit beam steering commands other than UE autonomous control. However, UE signaling tests may be challenging under an OTA test environment because the physical position of satellite Tx/Rx point cannot be physically moved in IFF chamber. Instead, satellite movement can be imitated by rotating a VSAT UE, which requires to turn off the sensors and GNSS used for UE autonomous beam steering. This approach may work if the signaling test is the major purpose. However for this case, it is not clear to us why satellite movement needs to be considered. Furthermore, it wouldn’t be appropriate as well if the purpose of the tests includes UE beam steering capability test because the UE behavior is different from that under a commercial network.

Based on the analysis above, we want to understand the purpose, technical challenges and solutions better before making the decision as this is an important and complicated issue for VSAT vendors. More specifically, RAN4 should firstly discuss and clarify if antenna steering test for GNSO (i.e. moving satellite) is essential or not. After that, RAN4 should summarize the technical challenges and potential solutions for that for the better understanding of the group.

Proposal 2:  RAN4 to firstly discuss and clarify the necessity and feasibility of OTA tests for antenna steering to the moving satellite. Then, the technical challenges and potential solutions are summarized for the group’s better understanding.

Conclusion

The proposals in this contribution are summarized as follows:

Proposal 1: RAN4 to take into account the different antenna position for Tx and Rx in a full duplex VSAT UE

Proposal 2:  RAN4 to firstly discuss and clarify the necessity and feasibility of OTA tests for antenna steering to the moving satellite. Then, the technical challenges and potential solutions are summarized for the group’s better understanding.