3GPP TSG-RAN WG4 Meeting #117	R4-2520303

Dallas, US, November 17th – 21st, 2025

Source:	Tejas Networks

Title:	Discussion on RRM measurement prediction

Agenda item:	7.12.3

Document for:	Discussion

Introduction

The study on AI/ML mobility in NR were concluded in RAN4#116 in the scope of Rel-19 SI. The study outcome is captured in TR 38.744 V1.2.0 [3].

Discussion

The RRM specification impacts are captured in TR 38.744 section 6, and few of the potential impacts are discussed in detail below.

Prediction accuracy requirements

Agreement: Further discuss based on the open issues in [1] and the following way forward: - Discuss whether the prediction accuracy has the same or different accuracy requirements as non-AI/ML RRM measurement. - Discuss whether the following parameters need to be the side condition of the accuracy requirements: - OW/PW length - MRRT - Skipping pattern - UE speed - L1 filtering approach and sample number

AI/ML based mobility was extensively studied in Rel-19, and the conclusions are captured in TR 38.744. The study concluded that the UE can be configured with periodic or event-triggered reporting of predicted and/or actual measurement results. For intra-frequency temporal domain case A, one or more predicted measurement results across a prediction window can be reported, while case B allows reporting of the latest predicted or actual result. For inter-frequency prediction, the latest predicted result is reported.

Observation 1: In temporal domain case A, more than one predicted measurement result across a window can be reported.

For temporal domain case A, more than one predicted measurement result can be part of the measurement report. In this case the accuracy of the predicted result may not be the same for all prediction.

Proposal 1: RAN4 to decide whether we need separate accuracy requirement for multiple predicted results in a prediction window.

Cluster-based prediction

Rel-19 Study conclusions: - It is up to UE’s implementation to decide on model related choices including cluster-based vs single-cell-based approach, RRM sub-use cases and OW length - Both cluster approach (where measurement results from more than one cells are used as input to the model) and single cell approach (where measurement results from single cell are used as input to the model) were used by different companies. Both approaches are valid implementations.

As per the Rel-19 study, using a group of measurement values as input for prediction can improve RRM measurement accuracy compared to using a single measurement. And the Rel-19 study concluded that, both cluster approach (where measurement results from more than one cells are used as input to the model) and single cell approach (where measurement results from single cell are used as input to the model) are valid implementation. Furthermore, it is up to UE’s implementation to decide on model related choices including cluster-based vs single-cell-based approach.

Cluster approach example as per TR 38.744,

  - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

  - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

In line with the above conclusions, for a UE configured to predict the measurement values for a specific cell or frequency, the AI/ML model may use actual measurement values from another cell or frequency along with predicted measurement values from another cell or frequency as inputs to the model. Cluster based approach can benefit from incorporation of both actual and predicted measurement results, as a result enhance the model’s ability to generalize and improve performance in predicting the target measurements.

Proposal 2: Measurement prediction can be based on actual measurement values or predicted measurement values or combination of predicted and measured values.

In scenarios where cluster-based approach is applied, measurement prediction may depend on inputs derived from other cells or frequencies, which can include either actual measurement values or predicted measurement values. Therefore, how to handle LCM in such cases needs to be clearly defined to ensure consistency of prediction and to avoid unnecessary fallback or model switching.

Proposal 3: RAN4 to specify the handling of LCM for cluster-based approaches, where predicted measurements are used as inputs for inference.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Observation 1: In temporal domain case A, more than one predicted measurement result across a window can be reported.

Proposal 1: RAN4 to decide whether we need separate accuracy requirement for multiple predicted results in a prediction window.

Proposal 2: Measurement prediction can be based on actual measurement values or predicted measurement values or combination of predicted and measured values.

Proposal 3: RAN4 to specify the handling of LCM for cluster-based approaches, where predicted measurements are used as inputs for inference.

- References

- RP-251968, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, Beijing, China, Sept. 15-18, 2025.

- RP-251965, Status Report to TSG, Beijing, China, Sep. 15-18, 2025.

- 3GPP TR 38.744 V1.2.0 (2025-9)