3GPP TSG-RAN WG4 Meeting #117	R4-2520307

# Dallas, USA, Nov. 17-21, 2025

Source: 	KDDI Corporation

Title: 	Discussion on 6G spectrum

Agenda Item:	8.5

Document for:	Discussion

- 1. Introduction

In RAN4#116-bis, RAN4 began the discussion on 6G spectrum and agreed the WF on the numerology as follows[1]. However, the issue on new spectrum/bands for consideration during the 6G SI isn’t captured in the WF. In this contribution, we focus on new spectrum/bands for consideration during the 6G SI and propose some issues.

- 2. Discussion

  - 6G spectrum and bands

New spectrum and bands

ITU-R identified Upper 6 GHz (6.425-7.125) for IMT in WRC-23 and is discussing 4.4-4.8, 7.125-8.4 and 14.8-15.35 GHz toward WRC-27. At the same time, some regions/countries are proactively discussing to allocate Upper 6 GHz to IMT as follows.

- China has identified the band 6425-7125 MHz for IMT, this came into effect 1st July 2023, see Chinese table of allocations and has given certainty that the band will be for 5G/6G, here. - UAE has added the band 6425-7125 MHz to their IMT holdings expects actual operation to begin between the years 2025 and 2026 by 2025/2026, here. - RCC Recommendation 1/21 to harmonize the technical conditions for 6425-7125 MHz, including channel used by 5G/IMT-2020 systems can consist of one or more adjacent frequency blocks but should not exceed 400 MHz. - The Indian Government decided to allocate the entire Upper 6 GHz band for IMT (3GPP n104), with different parts of the band being made available through to 2030, Cabinet Approves Spectrum Refarming for 5G and Future 6G Services: Report - Brazil's National Telecommunications Agency (Anatel) identified the upper 6 GHz for IMT in Jan, 2025 and has now announced a draft schedule for frequency auctions over the next decade (2026–2036) , including 6 GHz in 2026, SEI/ANATEL - 13277590 - Draft Resolution - European Commission issued a mandated “to study feasibility of and develop least restrictive harmonized technical conditions” for the 6425-7125 MHz frequency band” based on the outcome of WRC-23 regarding upper 6GHz. The harmonization of this band is supposed to be finalized by July 2027.

Observation 1: Some regions/countries are proactively discussing to allocate Upper 6 GHz to IMT.

Proposal 1: Study several candidates as 6G bands for 6.425-7.125GHz identified in WRC-23 and 4.4-4.8, 7.125-8.4 and 14.8-15.35 GHz being discussed toward WRC-27 considering the regulatory discussions are still ongoing.

Existing spectrum and bands

We expect that 6G will support existing bands already have defined as 5G NR so that 5G bands which MNOs are operating can be re-farmed to 6GR. On the other hand, there are unfortunately some bands not used in commercial networks actually for example n90, and also in RAN plenary there is the discussion to optimize and re-define the existing 5G band for 1.5GHz [3]. We propose d to study whether some existing 5G bands can be optimized and re-defined in 6G.

Proposal 2: Study whether some existing 5G bands can be optimized and re-defined in 6G.

  - 6G frequency range

RAN Plenary discussed 6G frequency ranged and according to the moderator’s summary report, the following options were proposed [4].

- Issue 1: How to categorize the frequency range? Should the spectrum resource from 7.125GHz to 8.4GHz be categorized into the lower group or the higher group? - Option 1: extend FR1 to 8.4 GHz and define a separate mid-high band (8.4–24.25 GHz) with its own single numerology. - Option 2: extend FR1 to 8.4 GHz and merge 8.4–24.25 GHz into FR2, creating a unified FR2 family. - Option 3: define separate FR for 7.125 – 24.25 GHz.

On the other hand, as mentioned in section 2.1, the regulatory discussions for some of these bands are still ongoing. Then, we propose to study the frequency ranges considering frequencies used first time in 6G and the progress/outcomes of the ongoing regulatory discussions.

Proposal 3: Study the frequency ranges considering frequencies used first time in 6G and the progress/outcomes of the ongoing regulatory discussions.

- 3. Conclusion

Observation 1: Some regions/countries are proactively discussing to allocate Upper 6 GHz to IMT.

Proposal 1: Study several candidates as 6G bands for 6.425-7.125GHz identified in WRC-23 and 4.4-4.8, 7.125-8.4 and 14.8-15.35 GHz being discussed toward WRC-27 considering the regulatory discussions are still ongoing.

Proposal 2: Study whether some existing 5G bands can be optimized and re-defined in 6G.

Proposal 3: Study the frequency ranges considering frequencies used first time in 6G and the progress/outcomes of the ongoing regulatory discussions.

- 4. Reference

[1] R4-2514626, WF on 6G Spectrum, Nokia, RAN4#116bis