# 3GPP TSG-RAN WG4 Meeting #117	R4-2520313

# Dallas, US, Nov. 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6084 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_enh-Core) CR on satellite switching with resync |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 6.1C.3, some occurrences of “cell” were incorrectly used instead of “satellite”. The correction ensures consistent terminology with the NR SAN satellite switching context. The definition of T∆ and Tmargin cannot directly refer to section 6.1C.1.2.2.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replaced “cell” with “satellite” in clause 6.1C.3 to correctly describe satellite switching. Corrected the definition of T∆ and Tmargin in 6.1C.3.2.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement text will remain imsleading, implying cell switching instead of satellite switching, which may cause implementation confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of change>>

6.1C.3	NR SAN Satellite switching with re-synchronization

6.1C.3.1	Introduction

The purpose of NR SAN Satellite switching with re-synchronization is to switch satellite in the same SSB frequency and same gNB without PCI changing. The requirements in this clause are applicable to SA NR SAN.

6.1C.3.2	NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization

The requirements in this clause are applicable to both hard and soft switch over quasi-earth fixed scenario from NR SAN FR1 satellite to NR SAN FR1 satellite. The requirements in this clause apply provided that UE has the valid and applicable parameters of ephemeris information, common TA, DL and UL Polarization information, Koffset, and Kmac for target NR SAN satellite during Dswitch_unchangedPCI, otherwise interruption time may be longer than the requirements in clause 6.1C.3.2.2 for hard satellite switch and satellite switch delay may be longer than the requirements in clause 6.1C.3.2.3 for soft satellite switch.

Requirements for soft satellite switching are applicable for UEs that support softSatelliteSwitchResyncNTN-r18 [14] when network configures soft satellite switching with resynchronization [2].

Requirements for hard satellite switching are applicable for UEs that support hardSatelliteSwitchResyncNTN-r18 or softSatelliteSwitchResyncNTN-r18 [14] when the network configures hard satellite switching with resynchronization [2]; or for UEs that support hardSatelliteSwitchResyncNTN-r18 but do not support softSatelliteSwitchResyncNTN-r18 when the network configures soft satellite switching with resynchronization.

6.1C.3.2.1	Satellite switching delay

When the UE receives a broadcast message implying satellite switching within re-synchronization, the UE shall be ready to start the transmission of the new uplink PRACH channel or transmission of the new uplink PUSCH channel within Dswitch_unchangedPCI msec.

Where:

-	Dswitch_unchangedPCI equals to the interruption time stated in clause 6.1C.3.2.2 for hard satellite switch and satellite switch delay stated in clause 6.1C.3.2.3 for soft satellite switch.

6.1C.3.2.2	Interruption time for hard satellite switch with re-sync

The interruption time is the time between t-service and the time when the UE is ready to receive any DL channel/signal and transmit any UL channel/signal from/to the target satellite.

When intra-frequency hard switch to NR SAN satellite is commanded,

the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + Tprocessing + T∆ + Tmargin ms

Otherwise, no interruption time requirement is applied.

Where:

-	Tsearch is the time required to search the target NR SAN satellite assuming the target satellite is not already known when UE starts synchronizing with target satellite. If the target satellite Es/Iot ≥ -2 dB, then Tsearch = Tfirst_SSB ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target satellite search times.

-	T∆ is time for fine time tracking and acquiring full timing information of the target cell.

T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW.

T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

-	Tprocessing is time for UE processing. Tprocessing can be up to 10 ms.

-	Tmargin is time for SSB post-processing. Tmargin can be up to 2 ms.

-	Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, and if ssb-TimeOffset is configured, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the SSB-TimeOffset reference point as defined in TS 38.331 [2] to UE.

UE is allowed to skip measurements for other cells and satellites than the target satellite and source satellite from T-service until the satellite switch completion.

##### 6.1C.3.2.3	Satellite switch delay for soft satellite switch with re-sync

The Satellite switch delay is from t-serviceStart to the time instance when UE is ready to receive any DL channel/signal and transmit any UL channel/signal from/to the target satellite.

When intra-frequency soft switch to NR SAN satellite is commanded,

the satellite switch time shall be less than Tsoft_switch

Tsoft_switch = max(t-service-t-seviceStart, Tsearch + T∆ + Tmargin) + Tprocessing  ms

Where:

-	Tsearch is the time required to search the target NR SAN satellite assuming the target satellite is not already known when the handover command is received by the UE. If the target satellite Es/Iot ≥ -2 dB, then Tsearch = Tfirst_SSB ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target satellite search times.

-	T∆ is time for fine time tracking and acquiring full timing information of the target cell.

T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW.

T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

-	Tprocessing is time for UE processing. Tprocessing can be up to 10 ms.

-	Tmargin is same as the one defined in section 6.1C.1.2.2.1.

-	Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from SSB-TimeOffset reference point as defined in 38.331 [2] to UE.

During the time period from t-seviceStart to t-service, scheduling restriction as defined in clause 9.2C.5.3 is allowed, with the exception that the locations of SSB symbols of target satellite where scheduling restriction applies are determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the SSB-TimeOffset reference point as defined in TS 38.331 [2] to UE.

UE is allowed to skip measurements for other cells and satellites than the target satellite and source satellite from t-seviceStart to the satellite switch completion.

The requirement in this clause applies and UE is required to keep the connection (DL and UL) with the source NGSO satellite, under the following conditions:

-	SSBs from the two satellites are spaced apart from each other at least by 1 OFDM symbol in the time domain at UE Rx side.

<<End of change>>