3GPP TSG-RAN WG4 Meeting #117                                          R4-2520322

Dallas, USA, Nov 17 – 21, 2025

Source: 	Huawei, HiSilicon

Title: 	Onn system parameters for 6G —— Channel bandwidth

Agenda Item:	8.2.3

Document for:	Approval

Introduction

6G system parameters have been discussed extensively in last RAN4 meeting [1], and WF was agreed in [2].

This contribution provides our further consideration on system parameters for 6G on channel bandwidth, including max CBW, min CBW, numerology, spectrum utilization and flexible channel bandwidth, etc.

Discussion

Channel bandwidth

Max channel bandwidth

RAN4 agreement on max channel bandwidth is excerpted as below.

RAN4 Agreement In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths. - RF performance evaluation with proposed max CBW - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum. - Assess the implementation feasibility and complexity and power consumption - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1. - Study the feasibility to support different NW and UE max CBW. - Collaborate with RAN1 on feasibility findings - Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations.

Notably, RAN1 reached the following agreement regarding max channel bandwidth.

RAN1 Agreement - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

To support such channel bandwidth, current RAN1 agreement provides no indication regarding the maximum FFT size. During RAN4 discussions, many companies proposed to adopt an 8192 FFT size as the baseline. While it is understood that a single 8192 FFT cannot support a 400 MHz channel bandwidth with 30 kHz subcarrier spacing, FFT size alone may not be the determining factor for the maximum channel bandwidth. Multi-FFT remains a viable solution and could be implemented in NR, which is up to vendors’ choice. Therefore, while the 8192 FFT size can be regarded as a baseline or typical implementation for 6G, it should not be considered a limiting factor in determining the maximum channel bandwidth, consistent with the network-side agreement. The feasibility and methods for enabling UE support of 400 MHz bandwidth will be further studied in RAN1. From RAN4 perspective, potential spectrum availability, implementation complexity, performance and power consumption should also be thoroughly evaluated.

In addition, we note that some companies have suggested considering a 4096 FFT size for FDD or lower frequency bands. In our view, if an 8192 FFT size can be supported, the use of a reduced FFT size becomes merely an implementation choice Therefore, for different bands, different FFT size could also be considered in UE implementation and it is not necessary to consider a separate assumption of 4096 FFT for specific scenarios.

Proposal 1-1: If a single FFT size is considered as a baseline, other FFT size(s) are not precluded just as implementation choice considering the wide frequency range.

While the increased implementation complexity for UE to support 400 MHz channel bandwidth is acknowledged, qualitative analysis alone offers limited value to RAN4 and RAN1 discussions. In line with the RAN4 agreement to compare implementation options, e.g., for cases like 200MHz/400 MHz, and evaluate RF performance and architectural trade-offs, RAN4 should conduct quantitative implementation comparisons. Providing such implementation relevant evaluation to RAN1 is essential for reaching a pertinent joint conclusion with a meaningful technical basis.

Proposal 1-2: RAN4 should conduct quantitative comparisons of implementation options, assessing their RF performance and architectural trade-offs, to support a joint conclusion on the maximum channel bandwidth.

Min channel bandwidth

RAN4 agreement on min channel bandwidth is excerpted as below.

Agreement - Compare options of defining min CBW, considering pros and cons - Study the following aspects from RAN4 perspective, meanwhile tracking RAN1/RAN progress - Whether 5MHz could be considered as a general baseline while 3MHz is allowed for particular bands - SCS-dependent framework - Other aspects are not precluded - Provide early feedback to RAN1 with RAN4's initial findings on min CBW from implementation and spectrum perspective.

The SSB (tentatively used term for 6GR here) design has not yet been finalized in RAN1, which is critical for determining the minimum supported channel bandwidth during initial access. The minimum channel bandwidth required for initial access should be evaluated separately from minimum channel bandwidth for specific operating bands. If the baseline RB number for SSB size remains similar to that in NR, the minimum channel bandwidth for initial access could adopt the same assumptions as 5G, which are tied to different numerologies—for example, 5 MHz for 15 kHz SCS and 10 MHz for 30 kHz SCS.

Given global spectrum allocation patterns, we recognize that some operators may hold 3 MHz of spectrum in certain cases. For such scenarios, the approach used in NR could still be considered: treating 3 MHz as an exceptional case while optimizing the physical layer design for most scenarios and typical bandwidths. However, this should be a RAN1 decision as part of the initial access design.

In the current RAN4 specifications, the minimum channel bandwidth is determined on a case-by-case basis for specific operating bands within different frequency ranges. As usual, this process incorporates operator inputs and reflects actual spectrum allocation conditions.

Proposal 2-1: Decouple the minimum channel bandwidth defined for specific spectrum/operating bands from the minimum channel bandwidth required for initial access. The latter should be determined by RAN1 as part of the initial access design.

Numerology

RAN4 agreement on numerology is excerpted as below.

Agreement - Evaluate the following proposals regarding numerology from RAN4 perspective - "Single numerology" proposal - Frequency sub-range/Band specific SCS values proposal - Compare perf gain and implementation complexity for different SCS with same frequency range or specific band - Study numerology for SSB of initial cell search from RAN4 perspective - Other proposals not presented in this meeting are not precluded - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

Numerology is a topic extensively discussed in both RAN1 and RAN meetings. So far, "Single numerology" proposal is popular in RAN4, such as:

For sub-6GHz

- 15kHz SCS for FDD, 30kHz SCS for TDD

For around 7GHz

- 30kHz SCS

For between 24.25GHz - 52.6GHz

- 120kHz SCS

Building on the 5G experience, a single numerology for sub-6 GHz and FR2 can sufficiently address the majority of deployment scenarios. Adopting a unified numerology helps simplify implementation for both UE and network sides, making it a suitable starting point for 6G studies. The trade-offs involving the combination of FFT size, channel bandwidth, and subcarrier spacing can be discussed under the topic of max channel bandwidth. In the RAN4 study, the focus should be placed on the numerology for the SSB in the initial cell search procedure.

Proposal 3-1: Single numerology is considered as starting point for 6G.

- Single SCS for each band

  - 15kHz for FDD bands

  - 30kHz for TDD bands

- Single SCS for both data and SSB

Spectrum utilization

RAN4 agreement on spectrum utilization is excerpted as below.

Agreement - Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM). - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G - Evaluate the RF performance impact (complying with the affected requirements) of advanced spectral confinement techniques (e.g., better filtering, windowing) to understand how many RBs can be enabled - Considering trade-offs between SU, RF performance, and UE/BS complexity - Channel bandwidth and SCS with smaller SU should be prioritized - SU for larger channel bandwidth shall be evaluated based on progress on CBW - Closely coordinate with RAN1 on different waveform candidates and SCS configurations.

Table below listed the SU for all supported NR channel bandwidths.

Table 1: Maximum transmission bandwidth configuration NRB and SU ratio

| SCS (kHz) | 3 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 35 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) |
| 15 | 15 | 90.0% | 25 | 90.0% | 52 | 93.6% | 79 | 94.8% | 106 | 95.4% | 133 | 95.8% | 160 | 96.0% | 188 | 96.7% |
| 30 |  |  | 11 | 79.2% | 24 | 86.4% | 38 | 91.2% | 51 | 91.8% | 65 | 93.6% | 78 | 93.6% | 92 | 94.6% |
| 60 |  |  |  |  | 11 | 79.2% | 18 | 86.4% | 24 | 86.4% | 31 | 89.3% | 38 | 91.2% | 44 | 90.5% |
| SCS (kHz) | 40 |  | 45 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
|  | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) |
| 15 | 216 | 97.2% | 242 | 96.8% | 270 | 96.5% |  |  |  |  |  |  |  |  |  |  |
| 30 | 106 | 95.4% | 119 | 95.2% | 133 | 96.5% | 162 | 97.2% | 189 | 97.2% | 217 | 97.7% | 245 | 98.0% | 273 | 98.3% |
| 60 | 51 | 91.8% | 58 | 92.8% | 65 | 94.0% | 79 | 94.8% | 93 | 95.7% | 107 | 96.3% | 121 | 96.8% | 135 | 97.2% |

As shown in the SU table above, spectrum utilization for smaller channel bandwidths remains relatively low compared to larger bandwidths. This is primarily due to implementation constraints, particularly in channel filter design, where appropriate guard bands should be considered to meet spectrum related requirements. Improving SU in 6GR, especially for smaller channel bandwidths, is critically important, given that low-frequency bands with narrower bandwidths can offer enhanced coverage, which is essential for 6GR deployment.

In accordance with the WF from the last meeting, which states that " channel bandwidth and SCS with smaller SU should be prioritized," we believe it is more appropriate to begin evaluations in FR1. Given the intertwined nature of RF impairment studies with UE RF, a practical approach for early SU investigation is to leverage existing 5G requirements and assumptions, while incorporating advanced spectrum confinement techniques. The evaluation assumptions discussed in Section 2.1.2 for waveform analysis may also serve as a basis for initial SU evaluations.

Regarding spectrum confinement techniques, as in 5G, no specific implementation is restricted. Both frequency-domain and time-domain methods are permissible, provided that companies clearly declare the techniques used in their evaluations.

Proposal 4-1: The evaluation assumptions for waveform analysis could also serve as a basis for initial SU evaluations, leveraging existing 5G requirements and assumptions, while incorporating advanced spectrum confinement techniques.

Proposal 4-2: No limitation on the adopted spectrum confinement techniques, provided that companies clearly declare the techniques used in their evaluations.

Flexible channel bandwidth

RAN4 agreement on irregular channel bandwidth is excerpted as below.

Agreement - 5G concepts from TR 38.844 can be considered as starting point to study a more generic solution for 6G - The listed main proposals and identified issues should be taken into account for the following study of irregular/flexible/scalable channel bandwidths - Other options not presented in this meeting are not precluded

In existing LTE/NR specification, a fixed set of regular channel bandwidths are defined, e.g. to fulfil the operator’s request.

FR1: {3 MHz, 5 MHz, 6 MHz, 7 MHz, 10 MHz, 15 MHz, 20 MHz, 25 MHz, 30 MHz, 35 MHz, 40 MHz, 45 MHz, 50 MHz, 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz}

The number of channel bandwidths is already 18 for NR FR1. And it is expected that 6G would introduce wider channel bandwidth larger than 100 MHz. The granularity for regular channel bandwidth for larger than 100 MHz is to be discussed. The number would be increased significant if small granularity is defined as below 100 MHz. Too many bandwidth types increase the design and test/verification workload and implementation complexity for UE implementation. Hence from the UE implementation and test coverage perspective, only a finite set of channel bandwidth would be possible. If a generic approach can be adopted to handle the irregular channel bandwidth in 6G, it could help to reduce the number of channel bandwidths which would be benefit for the UE design and test effort. It is proposed to study the possibility to reduce the number of regular channel bandwidths.

Observation 5-1: A generic approach to handle irregular channel bandwidth may help to reduce the number of regular channel bandwidths be supported for 6G UE.

Proposal 5-1: It is proposed to study a generic solution with the goal of reducing the number of regular channel bandwidths.

And among these schemes discussed in TR 38.844, overlapping UE CBW and larger CBW has less impact to UE complexity, however, the UE throughput is limited to legacy CBW in UL or both DL and UL, which need more study for 6G. For larger CBW scheme in TR 38.844, it is only applicable to DL and new asymmetric bandwidth combinations for UE are needed. In our view for 6G we should study whether/how larger CBW scheme can apply to UL and avoid to introduction of asymmetric bandwidth combinations.

And for the overall generic solution, we can consider to define a finite set of channel bandwidths in the specification, and for other channel bandwidth in between two standard channel bandwidths is supported by scalable bandwidth. BS RF requirements are based on corresponding channel bandwidth filter is implemented, and UE additional RF requirements if needed are based on next wider channel bandwidth.

On BS side, the channel bandwidth set is declared by the vendors, hence it would be acceptable to support the additional irregular channel bandwidth for the specific band and operator’s spectrum holding. In general, for the RF requirement definition, we should aim to define the RAN4 requirements as many as possible in a BW independent/scalable manner. And on UE side, only regular channel bandwidth set is defined.

Proposal 5-2: it is proposed to study whether/how larger CBW scheme can apply to UL and avoid to introduction of asymmetric bandwidth combinations.

Conclusion

This contribution provides our consideration on system parameters for 6G.

Maximum channel bandwidth

Proposal 1-1: If a single FFT size is considered as a baseline, other FFT size(s) are not precluded just as implementation choice considering the wide frequency range.

Proposal 1-2: RAN4 should conduct quantitative comparisons of implementation options, assessing their RF performance and architectural trade-offs, to support a joint conclusion on the maximum channel bandwidth.

Minimum channel bandwidth

Proposal 2-1: Decouple the minimum channel bandwidth defined for specific spectrum/operating bands from the minimum channel bandwidth required for initial access. The latter should be determined by RAN1 as part of the initial access design.

Numerology

Proposal 3-1: Single numerology is considered as starting point for 6G.

- Single SCS for each band

  - 15kHz for FDD bands

  - 30kHz for TDD bands

- Single SCS for both data and SSB

Spectrum utilization

Proposal 4-1: The evaluation assumptions for waveform analysis could also serve as a basis for initial SU evaluations, leveraging existing 5G requirements and assumptions, while incorporating advanced spectrum confinement techniques.

Proposal 4-2: No limitation on the adopted spectrum confinement techniques, provided that companies clearly declare the techniques used in their evaluations.

Flexible channel bandwidth

Observation 5-1: A generic approach to handle irregular channel bandwidth may help to reduce the number of regular channel bandwidths be supported for 6G UE.

Proposal 5-1: It is proposed to study a generic solution with the goal of reducing the number of regular channel bandwidths.

Proposal 5-2: it is proposed to study whether/how larger CBW scheme can apply to UL and avoid to introduction of asymmetric bandwidth combinations.

- References

[1] R4-2514508, Feature lead summary for [116bis][101] 6G system parameter

[2] R4-2514641, WF on 6GR system parameter

[3] R4-2514642, WF for 6G general RF and UE RF