3GPP TSG-RAN WG4 #117	R4-2520339

Dallas, US, November 17-21, 2025

Agenda item:	6.22.2		UE demodulation for time/frequency interleaving

Source:	Qualcomm Incorporated

Title:						Impairment results for time-frequency interleaving for 5G Broadcast

Document for:	Discussion

# Introduction

In RAN4 #116, RAN4 work on Rel-19 LTE-based 5G Broadcast Phase 2 was started according to the work item description in [1]. In [3] we provided our alignment results for the way forward agreed in [2]. In this paper we provide impairment results.

# Discussion

In [3] we provided our alignment results for the way forward agreed in [2]. All our proposals for the simulation parameters have been agreed in [4].

Agreement:

- RAN4 to confirm that demod requirements are defined for SCS = 1.25 kHz.

- RAN4 to confirm that demod requirements are defined for (M, N) = (8, 4) and (M, N) = (32, 16).

- RAN4 to confirm the TB sizes of 31704 bits for (M, N) = (8, 4) and of 45352 for (M, N) = (32, 16).

- RAN4 to confirm that demod requirements are defined for 8 MHz channel bandwidth.

- RAN4 to confirm that demod requirements are defined for the channel model channel model in section B.2.6.2-1 in [3] for 5 Hz and 50 Hz.

- RAN4 to confirm that both time and frequency domain interleaving is enabled in the demod test cases.

## Simulation Results

The way forward outlines all parameters that are needed to provide simulation results. In the following we provide our alignment results according to these agreements.

| Bandwidth [MHz] | RMC | Interleaving parameter M, N | TBS [bits] | Code rate over N RVs per TB | Doppler [Hz] | Alignment SNR [dB] | Impairment SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8 MHz | FRC-1 | M = 8, N = 4 | 31704 | 0.41 | 5 Hz | 10.2 dB | 12.2 dB |
|  |  |  |  |  | 50 Hz | 8.6 dB | 10.6 dB |
| 8 MHz | FRC-2 | M = 32, N = 16 | 45352 | 0.3 | 5 Hz | 1 dB | 3 dB |
|  |  |  |  |  | 50 Hz | 0.4 dB | 2.4 dB |

We propose to accept our impairment results as SNR requirements for time-frequency interleaving demod test cases for 5G Broadcast.

Proposal 1: RAN4 to accept our impairment results as SNR requirements for time-frequency interleaving demod test cases for 5G Broadcast.

## Details of the Reference Measurement Channel

In this section, we provide the details of the fixed reference channels used in the simulations above.

Table 2-1: Fix Reference Channel for Time/Frequency Interleaving

| Parameter | PMCH |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | FRC-1 | FRC-2 |
| Channel bandwidth | MHZ | 8 | 8 |
| Subcarrier spacing | kHz | 1.25 | 1.25 |
| Allocated resource blocks |  | 39 | 39 |
| Allocated slots per 40 ms (Note 1) |  | 32 | 39 |
| Time domain spreading and scaling (M, N) |  | (8, 4) | (32, 16) |
| Frequency domain interleaving |  | Enabled Codebook cycling disabled | Enabled Codebook cycling disabled |
| Modulation |  | 16QAM | QPSK |
| Target coding rate |  | 0.41 | 0.3 |
| Information bit payload (Note 2) |  |  |  |
| For each slot | Bits | 31704 | 45352 |
| Number of code blocks per subframe |  | 6 | 6 |
| Binary channel bits per subframe |  |  |  |
| For each slot | Bits | 19200 | 9600 |
| Transport block size before scaling with N | Bits | 7992 | 2825 |
| Transport block size after scaling with N | Bits | 31704 | 45353 |
| MBMS UE category |  |  |  |
| Note 1: First subframe every 40 ms is allocated for non-MBMS transmission Note 2: Zero OFDM symbols are reserved for PDCCH and no CRS allocated as per TS 36.211 [4] Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 bits is allocated to each Code Block (otherwise L = 0 bits) |  |  |  |

# Conclusions

In this paper we provided our views on the demodulation performance requirements for time-frequency interleaving in LTE-based 5G broadcast. Our observations and proposals are summarized as follows:

Proposal 1: RAN4 to accept our impairment results as SNR requirements for time-frequency interleaving demod test cases for 5G Broadcast.

# References

- RP-250794, Revised WID on LTE-based 5G Broadcast Phase 2, RAN #107, March 2025

- R4-2512620, WF on 5G_Broadcast_Ph2, RAN4 #116, August 2025

- R4-2513073, Demodulation for time-frequency interleaving for 5G Broadcast, Qualcomm, RAN4 #116bis, October 2025

- RAN4 #116bis RRM_Demod_OTA session meeting report, RAN4 Vice Chair (China Telecom), RAN4 #116bis, October 2025