3GPP TSG-RAN WG4 Meeting # 117                                                             R4-2520349

Dallas, USA, 17th – 21st Nov., 2025,

Agenda item:	7.12.4

Source:	MediaTek Inc.

Title:	Discussion on measurement event prediction

Document for:	Discussion

# Introduction

RAN#109 plenary has agreed the updated version of the WID with the following RAN4 objectives [1]:

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

In this contribution, we would like to discuss the related RRM requirements for measurement event prediction.

# Discussion

As discussed in our another contribution R4-2513081, for temporal prediction, measurement event prediction should be the focus and RAN4 only need to define test cases for measurement event prediction. And it is up to UE implementation when to do the prediction as long as UE can meet the requirements.

Proposal 1: For measurement event prediction, it is up to UE when to do the prediction.

But when defining the requirements, we may need a reference UE behavior. We simulated the RSRP prediction accuracy for different PW windows. In the simulation, the settings are as below:

- Scenario: temporal prediction

- OW: 4 x 40ms, PW: 40ms, 80ms, 120ms, 160ms, 200ms, 240ms, 280ms, 320ms

- Sliding window

- L3 filtering: k=4

- Input: RSRP of two cells (Serving cell and the target cell) at 4 occasions in OW (2x4), Output: RSRP of two cells at 8 predicted occasions (2x8)

- Measurement error is not added

- Inference performed when enter condition of A3 event is met as in Fig.1.

Fig. 1

Table. 1 shows the performance of absolute RSRP difference at 5% and 95% tile points with different PW. It can be observed that the longer PW is, the worse the absolute RSRP accuracy is.

Table 1

| Absolute RSRP accuracy (dB) |  | future time instance |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 40ms | 80ms | 120ms | 160ms | 200ms | 240ms | 280ms | 320ms |
| Serving cell | 5% | -0.18 | -0.46 | -0.76 | -1.09 | -1.54 | -1.92 | -2.35 | -2.9 |
|  | 95% | 0.21 | 0.44 | 0.76 | 1.07 | 1.56 | 2.03 | 2.37 | 2.81 |
| Target cell | 5% | -0.21 | -0.48 | -0.83 | -1.18 | -1.69 | -2.17 | -2.64 | -3.08 |
|  | 95% | 0.18 | 0.44 | 0.76 | 1.17 | 1.58 | 1.89 | 2.42 | 2.74 |

RAN4 need to discuss how to define the requirements, e.g., define different requirements for several TTT values or define a unified requirement. From our point, we prefer not to define a set of requirements. As far as we know, 320ms is a typical value for TTT. If to define a unified requirement, we prefer to choose TTT as 320ms. Regard the reference UE behaviour, we prefer UE do the prediction when enter condition of the event is met as in Fig.1. If some UE can predict more accurate, those UE can do the prediction earlier and PW may be longer.

Observation 1: The longer PW is, the worse the absolute RSRP accuracy is.

Proposal 2: Prefer not to define a set of requirements for different TTT values. Further discuss which TTT value to define requirements for.

Propose 3: For measurement event prediction, for the purpose of defining requirements, the assumed UE behaviour is that UE starts to predict when enter condition of the event is met.

In SI phase, it was agreed that “If the report includes the predicted RSRPs corresponding to the predicted event occurrence time, the absolute and/or relative accuracy requirement for the predicted RSRP will be defined.” Last meeting, there were many proposals discussing other metrics except predicted RSRP accuracy.

In legacy event triggered L3 report, UE will also report the measurement results. For event prediction, we think UE will report L3 results, either measurement results or predicted results. Whether UE will report more information, it is still pending on RAN2 discussion. We think it is too early to conclude anything but RAN4 can discuss the motivation of other proposed metrics to align the understanding at first.

Propose 4: For the metrics used to evaluate the performance of measurement event prediction, it is too early to conclude anything but RAN4 can align the understanding of other proposed metrics at first.

In our understanding, there are mainly three other metrics to discuss:

- Event prediction latency: how far ahead of the ground truth event occurrence time the event is predicted

- Successful event prediction rate: The rate that UE can predict the event successfully if an event will occur.

- False alarm rate: The rate that an event will not occur, but UE predicts that there will be an event.

For event prediction latency, whether to have this metric highly depends on whether UE will report when the predict event occurs.

For the other two, we take A3 event for temporal predication Case A as an example for further discussion. According to RAN2 discussion, there can be two kinds of NW implementations upon receiving predicted event.

One implementation is that NW will trigger HO based on the prediction report without waiting for the measurement report. Under this implementation, even if there is an event to occur but UE fails to predict it accurately. There is no harm comparing to not using AI for future prediction as anyway a measurement report will be sent to NW finally. That is, successful event prediction rate is not very important. However, if there is no event to occur but UE predicts there will be. In this case, if NW triggers HO to the target cell based on the predicted result, then HO may fail, or the overall performance may be degraded due to poor quality of target cell. That is, false alarm rate is more important.

The other NW implementation is that NW still triggers HO based on measurement report. NW only starts HO preparation after receiving prediction report. Under this implementation, even if there is an event to occur but UE fails to predict it accurately, then there will be no gain comparing to not using AI for future prediction. But there will be no harm or no impact. On the other way, if there is no event to occur but UE predicts there will be. This will lead to some waste of NW resources (HO preparation at NW is all in vain). In our view, either successful event prediction rate or false alarm rate is not important.

Considering that there is no limitation on NW implementation, all in all, false alarm rate is more important than successful event prediction rate. Even though, we are not sure it is possible to test false alarm rate. In our understanding, false alarm may happen when neighbour cell gets better and better than serving cell in the observation window, but suddenly there is a blocker between UE and neighbour cell. If we want to test false alarm rate, the test data set must contain enough related scenario. We don’t think in the normal mobility, we can get enough data to test false alarm rate. And we also don’t think it is a good way to create lots of this scenario artificially in the test.

Propose 5: Compared to successful event prediction rate, false alarm rate is more important.

Propose 6: When considering other metrics, whether the metric is testable should be also considered.

# Conclusion

In this contribution, we provide our views for measurement event prediction. Observations and proposals are summarized as follows.

Observation 1: The longer PW is, the worse the absolute RSRP accuracy is.

Proposal 2: Prefer not to define a set of requirements for different TTT values. Further discuss which TTT value to define requirements for.

Propose 3: For measurement event prediction, for the purpose of defining requirements, the assumed UE behaviour is that UE starts to predict when enter condition of the event is met.

Propose 4: For the metrics used to evaluate the performance of measurement event prediction, it is too early to conclude anything but RAN4 can align the understanding of other proposed metrics at first.

Propose 5: Compared to successful event prediction rate, false alarm rate is more important.

Propose 6: When considering other metrics, whether the metric is testable should be also considered.

# Reference

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.