3GPP TSG-RAN WG4 Meeting # 117	R4-2520359

# Dallas, USA, 16th - 22th, Nov 2026

Source: 	Samsung

Title: 	Discussion on spectrum for 6GR

Agenda Item:	8.5

Document for:	Discussion

- Introduction

Spectrum serves as a fundamental enabler for each cellular generation, and 6G is expected to utilize a broader range of frequencies than ever before. Consequently, it is essential to discuss the definition of bands, band combinations and frequency ranges. Furthermore, with the introduction of an increasing number of new bands, it becomes more necessary to explore the potential for further simplifying band combination, and to conduct spectrum related regulatory survey.

This paper discussed the issues captured in WF [1] and FL summary [2].

- Discussion

## 2.1 Band and band combination definition

### 2.1.1 Approaches to secure 6G spectrum

Our general views on 6G spectrum are outlined as follow,

- New band exploration for new spectrum to be licensed:

  - Identified new band widely accepted by regulators and industry: The exploration of new bands for 6G spectrum licensing is underway, with identified bands gaining widespread acceptance from regulators and industry stakeholders. This approach mirrors the successful adoption of previous generations, such as the European Pioneer/Primary bands for 5G (700MHz n28, C band, and 26GHz n258).

  - Capacity and Coverage: The new bands should be designed to provide sufficient bandwidth to meet the anticipated capacity and coverage requirements of 6G, enabling advanced technologies and high data throughput.

  - Economy perspective: Harmonized 6G pioneer bands, supported by regulatory certainty, will establish a unified 6G ecosystem. This fosters economies of scale, driving affordability and widespread deployment of 6G technologies.

  - Spectrum management: Effective spectrum management is critical to prevent spectrum fragmentation, particularly during spectrum refarming. This ensures efficient resource utilization and creates opportunities for new market entrants, enhancing competition and innovation.

- Spectrum clearing: important for candidate bands congested with incumbent services, e.g., 7-24GHz.

  - Example: C-band auction in US, satellite operators agreed to clear spectrum to exchange for relocation cost.

- Spectrum refarming: potentially for all spectrum used for 2G, 3G and 4G, and smooth migration from 5G to 6G.

Observation 1: Our general views on approaches to secure 6G spectrum are outlined as above, includes new band exploration, spectrum clearing and spectrum refarming.

### 2.1.2 Band prefix

For 6G, it is proposed to adopt “s” as band prefix, resulting in designations such as s77 or t77, s256 or t256 for single band, and CA_s1-s77 or CA_t1-t77 for 6G-6G carrier aggregation.

Proposal 1: It is proposed to adopt “s” as 6G band prefix.

  - Band combination simplification

Since Rel-15, tens of thousands of band combination have been introduced for NR, yet only less than 10% have been implemented. Significant efforts have been dedicated to new BC requests, requirements analysis(including MSD, UE-UE co-existence, ΔTIB and ΔRIB), TP/CR preparation and review, errors correcting, and new rules adaption. Particularly in Rel-19, RAN4 spectrum work explode with tremendous of new band combinations, leading to a common consensus that a change is necessary for 6G. We elaborate our views in the following sections.

### 2.2.1 Band group concept

PA/LNA/Transceiver ports are already “band-group” basis, whereas duplexer/filter are implemented on a per-band basis to ensure emissions conformance. The addition of multi-plexer is necessary for carrier aggregation support with bands in same “band group” such as Mid-Mid, Low-Low. As illustrated in Figure 1, band combination implementation increases the complexity of UE RFFE design. Additionally, it leads to more complex conformance test and increased signaling overhead.

Fig1. Illustration for UE RFFE for CA_n1-n3 and CA_n25-n30-n66

The so called “Band group concept” has been discussed offline among UE vendors for some time. It seems intrigue the reconsideration of whether carrier aggregation is implementation favourably in 6G day-1 considering that the UE primarily operates in single CC mode for most of the time.

The fundamental idea is to divide FRs into several band groups, such as 5 band groups for FR1 and only 1 band group For FR2-1. Within one band group, only one band is allowed to operate at a time. The anticipated advantages include improved UE performance and reduced cost due to less insertion loss, a lighter RFFE design, more room for FR3, which are benefit from the removal of the multi-plexer.

Further, within one band group, there are two alternatives to be further considered.

- Alt 1) CA via switching

- Alt 2) CA is allowed by decreased Rx Chains per band

Observation 2: The fundamental idea for band group concept is to divide FRs into several band groups. Within one band group, only one band is allowed to operate at a time. Further, within one band group, CA via switching and/or decreased Rx chains per band can be considered.

Observation 3: The anticipated advantages include improved UE performance and reduced cost due to less insertion loss, a lighter RFFE design, more room for FR3, which are benefit from the removal of multi-plexer.

We are open to discuss the band group concept, with the understanding that it should be thoroughly checked with all stakeholders, particularly the operators. In the meanwhile, traditional CA, like what we have for 5G-5G carrier aggregation, should not be precluded for 6G.

Proposal 2: We are open to discuss the band group concept, with the understanding that it should be thoroughly checked with all stakeholders, particularly the operators. In the meanwhile, traditional CA (like what we have for 5G-5G carrier aggregation) should not be precluded for 6G.

### 2.2.2 ΔTIB, ΔRIB

Though the agreement [1] mention that it is not precluded to discuss band combination related structure/use of the ∆TIB requirements under this agenda, i.e. simplification of its use. It must be firstly determined whether the ∆TIB and ∆RIB requirements will be removed from CA requirements, in the UE RF thread. More details can be found in our paper [3].

Proposal 3: It must be firstly determined, in the UE RF thread, whether the ∆TIB and ∆RIB requirements will be removed from CA requirements or kept, before discussing any kind of simplification in this thread.

### 2.2.3 Removal of BCS concept

In 5G era, BCS has been introduced for NR-CA, but not for EN-DC. For EN-DC, one band within a band combination are supposed to support all CBWs that a single band supports, unless indicated otherwise. In contrast, for NR-CA, BCS(s) shall be indicated to NW. Multiple BCSs have been introduced for many combos due to the addition of new channel bandwidths in later releases, which significantly exploding the band combination specification work and increasing the signalling overhead. Though BCS 4/5 was introduced in Rel-17 motivated by the desire for band combination simplification which alleviates the specification workload to some extent, UE indication still remains necessary.

In 6G, we are open to discuss the removal of BCS concept, which means that one band within a band combination is supposed to support all CBWs that a single band supports, unless otherwise indicated. If certain channel bandwidths, such as the maximum channel bandwidth, cannot be supported due to interference concerns to other bands, this can be indicated additionally (e.g., via supportedBandwidthDL and supportedBandwidthUL).

Proposal 4: It is proposed to discuss the removal of BCS concept for band combination.

- This means that one band within a band combination is supposed to support all CBWs that a single band supports, unless otherwise indicated

- If certain channel bandwidths, such as the maximum channel bandwidth, cannot be supported due to interference concerns to other bands, this can be indicated additionally

## 2.3 Definition of frequency ranges

In 5G NR, spectrum usage has been structured into two ranges: Frequency Range 1 (FR1, below 7.125 GHz) and Frequency Range 2 (FR2, 24.25–71 GHz). This categorization was mainly driven by propagation characteristics, device implementation constraints, and regulatory allocation at the time of standardization.

As 6G evolves, there is growing consensus that 7–24 GHz mid-band spectrum, will play a critical role in bridging coverage and capacity between FR1 and FR2. This range offers more favorable propagation compared to FR2 while enabling significant wider bandwidth than FR1. In addition, discussions have emerged on whether further refinements (e.g., sub-ranges between FR1 and FR2 or extensions toward sub-THz spectrum) are necessary to address diverse deployment needs.

During last meeting, eight options were identified by Feature lead according to the proposals submitted. They have been documented in an in-official slides and circulated among companies by FL to facilitate companies’ work.

In our view, a key consideration in defining frequency ranges for 6G is the appropriate choice of subcarrier spacing (SCS) and numerology. The propagation delay spread, phase noise, and hardware limitations vary significantly across frequency ranges, making it impractical to adopt same SCS configuration across all bands. For example, smaller SCS values are desirable in lower bands to improve coverage and robustness, while larger SCS values are necessary in higher bands to manage phase noise and support wideband channels. To simply implementation and improve efficiency, “single -SCS principle” is desirable, and following recommendation is made.

| Frequency range | SCS (KHz) |
| --- | --- |
| FDD bands≤ 7.125GHz | 15 |
| TDD bands≤ 7.125GHz | 30 |
| 7.125GHz – 8.4GHz | 30 |
| 8.4 GHz – 24.25 GHz | 60 |
| 24.25 GHz – 52.6 GHz | 120 |

Moreover, from the perspective of UE RF front-end architecture (whether antenna connector available or not), having antenna connectors (with requirements still based on conductivity) is generally more preferred at least for frequency range below 15GHz, considering test complexity and cost for OTA-based requirements. For 15 to 24 GHz, we remain open for further discussion.

With above initial consideration, we propose the following options (i.e., Option 9a/option9b). We are open to name 8400MHz – 24250 MHz as either FR1-X or FR? which can be further considered. We are also open to explore the possibility to further sub-divide this range. It is important to note that naming it as FR1-X does not necessarily ensure conductive-based characteristics.

Proposal 5: For frequency range definition/separation, following new options (i.e., Option 9a/b) are proposed, by considering single-SCS principle and UE RF front-end architecture.

# 3. Conclusion

Observation 1: Our general views on approaches to secure 6G spectrum are outlined as above, includes new band exploration, spectrum clearing and spectrum refarming.

Proposal 1: It is proposed to adopt “s” as 6G band prefix.

Observation 2: The fundamental idea for band group concept is to divide FRs into several band groups. Within one band group, only one band is allowed to operate at a time. Further, within one band group, CA via switching and/or decreased Rx chains per band can be considered.

Observation 3: The anticipated advantages include improved UE performance and reduced cost due to less insertion loss, a lighter RFFE design, more room for FR3, which are benefit from the removal of multi-plexer.

Proposal 2: We are open to discuss the band group concept, with the understanding that it should be thoroughly checked with all stakeholders, particularly the operators. In the meanwhile, traditional CA(like what we have for 5G-5G carrier aggregation) should not be precluded for 6G.

Proposal 3: It must be firstly determined, in the UE RF thread, whether the ∆TIB and ∆RIB requirements will be removed from CA requirements or kept, before discussing any kind of simplification in this thread.

Proposal 4: It is proposed to discuss the removal of BCS concept for band combination.

- This means that one band within a band combination is supposed to support all CBWs that a single band supports, unless otherwise indicated

- If certain channel bandwidths, such as the maximum channel bandwidth, cannot be supported due to interference concerns to other bands, this can be indicated additionally

Proposal 5: For frequency range definition/separation, following new options (i.e., Option 9a/b) are proposed, by considering single-SCS principle and UE RF front-end architecture.

# 4. Reference

[1] R4-2514626, WF on 6G Spectrum, Feature lead (Nokia), RAN4#116bis

[2] R4-2514511, [116bis][104] Feature Lead Summary for 6G Spectrum, Feature lead (Nokia), RAN4#116bis

[3] R4-2520358, Discussion on general RF and UE RF for 6GR, Samsung, RAN4#117