# 3GPP TSG-RAN WG4 Meeting #117	R4-2520371

# Dallas, TX, USA, November 17 – 21, 2025

Agenda item:			8.1

Source:	Feature lead (Samsung)

Title:	Running Summary of 6G AI

Document for:	Information

# Introduction

Based on the latest approved RAN working group level SID on 6G Radio [RP-252912], RAN4 study on AI/ML includes AI/ML framework and new RAN4-driven AI/ML use cases, with the detailed objective provided as:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this running summary, it is intended to include the following aspects relevant to the RAN4 study on 6G AI:

- Status update for 6G RAN4 AI/ML issues, which is used to track the status of different objectives

- RAN4 key Tdocs and key agreements achieved before this meeting.

- RAN1 summary for relevant and essential agreements related to RAN4 6G AI/ML

- RAN2 summary for relevant and essential agreements related to RAN4 6G AI/ML

The information for feature lead is provided:

| Feature Lead | Company | Email address |
| --- | --- | --- |
| He Wang (Jackson) | Samsung | h0809.wang@samsung.com |

# Status update

The following status is provided based on RAN4 discussion till the end of RAN#116bis:

RAN4 AI/ML General scope and timeline:

| Issue | Description (summarized by FL based on agreed WF) | Latest Agreements | Expected Timeline (if any) | Issue Status |
| --- | --- | --- | --- | --- |
| RAN4 6G AI/ML study scope | RAN4 scope description | R4-2514637 (RAN4#116bis) | N/A | Closed |
| RAN4 6G AI/ML study timeline | RAN4 timeline | R4-2514637 (RAN4#116bis) | N/A | Closed |

RAN4 AI/ML framework:

| Issue | Description (summarized by FL based on agreed WF) | Latest Agreements | Expected Timeline (if any) | Issue Status |
| --- | --- | --- | --- | --- |
| Post-deployment enhancement | Potential post-deployment testing enhancements scope in 6G | R4-2514637 (RAN4#116bis) | N/A | Open |
| Model generalization | Generalization of UE and BS based AI/ML functionality for 6G and identify the potential standard impact | R4-2514637 (RAN4#116bis) | Start no earlier than Q2 2026 | Open |
| AI/ML UE category | Potential scope of metrics related to AI/MI complexity supported by UE/NW | R4-2514637 (RAN4#116bis) | Start no earlier than Q2 2026 | Open |

RAN4-driven use cases:

| Issues | Description (summarized by FL based on agreed WF) | Latest Agreements | Expected Timeline (if any) | Issue Status |
| --- | --- | --- | --- | --- |
| Use Case#1: AI-nonlinearity compensation | The scope of potential sub-use cases will be further discussed: (1) AI-DPoD and (2) AI-DPD. | R4-2514637 (RAN4#116bis) | RAN4-led AI/ML use cases selected in Stage 1 (Q4’25 to Q2’26) | Open |
| Use Case#2: AI-based channel estimation | Sub-use case definition and study, including AI-based CE with DMRS reduction and with CSI-RS reduction | R4-2514637 (RAN4#116bis) |  | Closed (unless triggered by RAN1) |
| Use Case#3: AI-SRS-assisted channel reconstruction | The scope of potential sub-use cases will be further discussed. | R4-2514637 (RAN4#116bis) |  | Open |
| Use Case#4: AI-PRACH receiver | The scope of potential sub-use cases will be further discussed. | R4-2514637 (RAN4#116bis) |  | Open |
| Use Case#5: AI-RRM | The scope of potential sub-use cases will be further discussed. | R4-2514637 (RAN4#116bis) |  | Open |

# RAN4 - 6G AI/ML agreements

## RAN4 #116bis meeting (Oct-2025)

### Key Tdocs

| R4-2514636 | Topic Summary for [116bis][107] 6G AI |
| --- | --- |
| R4-2514637 | WF on 6G AI |

### Key agreements

#### General scope and timeline

Issue 1-1: RAN4 6G AI/ML study scope

Agreement:

- In 6G AI/ML study, the detailed RAN4 scope shall include

  - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

    - RAN4-driven use cases: RAN4 shall identify, select and study only those AI/ML use cases that fall within RAN4 core responsibility. The detailed principles are captured under 3-1.

    - Other WG-driven use cases: RAN4 shall assist other working groups in these use cases by studying RAN4 impacts with RAN4 analysis and evaluation relevant to implementation and performance.

  - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

    - LCM procedures: RAN4 can study relevant RAN4 requirements for LCM procedures. If the use cases are identified by other WG, RAN4 related study should be based on the agreements and/or requests by other WG. The exact LCM context will be separately discussed and decided in the responding WGs.

    - RAN4 may study the feasibility and testability of the AI/ML functionalities. The study may include different aspects. It will be further decided how to manage the discussion under different agenda items, e.g. AI or testability.

Issue 1-2: RAN4 6G AI/ML study timeline

Agreement:

- RAN4 timeline for Rel-20 6G AI/ML study

  - Stage 1: Q4'25 and Q1'26

    - To clarify study scope for RAN4 AI/ML framework

    - To select the top-prioritized RAN4-led AI/ML use cases, with the detailed evaluation plan concluded for Stage-2

  - Stage 2: Q2’26 to Q2’27

    - To analyse and conclude RAN4 AI/ML framework (including RAN4 aspects for LCM procedures and other RAN4 specific issues)

    - To study comprehensively the RAN4-led use case(s) selected in Stage 1.

      - Other new use case(s) are not precluded depending on the consensus and the limited RAN4 bandwidth

    - Based on the agreement and/or the request from other WG, RAN4 will study the selected other WG-driven use cases with RAN4 analysis and evaluation. RAN4 related bandwidth limitation should be taken into consideration.

#### RAN4 AI/ML framework

Issue 2-2: Post-deployment enhancement

Agreement:

- RAN4 will discuss the potential post-deployment testing enhancements scope in 6G

Issue 2-4: Model generalization

Agreement:

- For AI/ML functionality generalization, RAN4 shall study generalization of UE and BS based AI/ML functionality for 6G and identify the potential standard impact.

  - The study will begin no early than Q2 2026.

  - Study and identify the typical parameters/aspects that are expected to be generalized by AI/ML functionality for each use case firstly, and then discuss how to test the generalization for the corresponding use case.

  - RAN4-led use cases will be prioritized.

  - The related study on other WG-led use cases will be based on the requests from the corresponding WG.

Issue 2-6: AI/ML UE category

Agreement:

- Regarding metrics related to AI/MI complexity supported by UE/NW, it can be discussed if a study is needed in 6G no earlier than Q2 2026.

  - The definition and the context of AI/MI complexity needs to be further clarified if the study is agreed.

#### RAN4-driven AI/ML use case (General Issues)

Issue 3-1: Principle for AI/ML use cases selection

Agreement:

- Principles for AI/ML RAN4-driven use cases selection

  - RAN4 shall closely coordinate with other WG and strive not to duplicate the efforts

    - RAN4 should work with other WG to clarify the split between RAN4 driven AI/ML work and RAN1/2/3 driven work.

    - Coordinated discussions with RAN1/2/3 are required to avoid overlapping scope.

  - Typical RAN4-driven use case shall

    - ensure performance gain or gain from other aspects, by focusing on some RF/Demod/RRM topics

  - Prioritize the use case by taking into account:

    - use cases with gain over non-AI counterpart

      - gain may include performance gain or gain from other aspects, like overhead reduction.

      - trade-off between performance and complexity/overhead should also considered.

    - use cases with potential standard impact.

    - Note: the prioritization principle is the general principle and no intention to have implication on RAN4 6G AI work plan.

Issue 3-2: Aspects of RAN4-driven use case to be studied

Agreement:

- Aspects of RAN4-driven use case to be studied

  - By following RAN1 agreement, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following:

    - Definition of each (sub-)use case, including at least AI model input/output

    - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results

    - Assumption on training types, e.g.,

      - offline training, online training, finetuning

      - Label construction (if applicable), including whether/how to obtain label data for model training

    - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model

    - Collaboration/interaction between UE and NW, e.g.,

      - no collaboration/interaction

      - UE/Network collaboration targeting at separate or joint ML operation

    - High level potential specification impact

  - Additionally, the following RAN4 aspect(s) are encouraged to study:

    - Feasibility issues, including complexity, and other aspects related to implementation

    - Other aspects identified by companies are not precluded.

- Here is the exemplary template for company to study/report the details of one certain use case:

| Sub-use case X |  | Company-A | Example |
| --- | --- | --- | --- |
| AI model input | Input in training |  | e.g., estimated DMRS |
|  | Input in inference |  | e.g., estimated data signal |
| AI model output | Label in training (if applicable) |  | e.g., original DMRS |
|  | Output in inference |  | e.g., corrected signal from non-linearity |
| Assumption on training | Training type |  | e.g., offline training, online training, finetuning, etc. |
|  | Label construction (if applicable) |  | whether/how to obtain label data for model training |
| model location for inference |  |  | e.g., UE-sided model, NW-sided model, and two-sided model |
| Collaboration/interaction between UE and NW |  |  | e.g., no collaboration/interaction, UE/Network collaboration targeting at separate or joint ML operation |
| Evaluation methodology |  |  | e.g., system level simulation, link level simulation, with detailed description |
| Evaluation assumption |  |  | Detailed table can be provided for evaluation assumption |
| Evaluation KPI |  |  | e.g., Tput |
| Evaluation benchmark |  |  | Description of non-AI solution to be compared with |
| Preliminary evaluation results |  |  | Detailed results can be provided. |
| High level potential specification impact |  |  | e.g., impacted existing clause, potential new requirement, impacts to other WG’s specifications |
| Feasibility issues, including complexity, and other aspects related to implementation |  |  | e.g., description of model complexity, other implementation complexity etc. |

#### AI/ML use case for RF issues (Per-use case discussion)

Issue 4-1b: Sub use case definition and selection for AI-DPD/DPoD

Agreement:

- The scope of potential RAN4-led AI use case, AI-DPD/DPoD, will be further discussed in the next meeting. The candidate sub-use cases include

- UL:

  - UE: AI-DPD

  - NW: AI-DPoD

- DL:

  - NW: AI-DPD

  - UE: AI-DPoD

- To facilitate the 6G AI scope discussion, the interested companies are encouraged to timely provide more analysis, including the performance benefit, spec impact, the assumption behind their analysis, on the specific sub-use case(s).

Issue 4-1c: Evaluation for AI-DPoD

Agreement:

- Non-linearity model(s) of transmission signals:

  - Discussed in other 6G agenda (e.g., system parameter agenda and UE RF agenda), which is intended to discuss the common model as much as possible.

    - Using separate models for AI and non-AI DPoD is not precluded.

- Evaluation Procedures and some detailed discussion points

  - Step-1: Non-linearity model(s) of transmission signals is determined to be used in the evaluation.

  - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

    - FFS a joint session or separate session of AI and non-AI non-linearity compensation.

    - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to

      - FFS the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA impairment including PA and other RF components non-linearity.

      - FFS suitable AI/ML model, type of training.

      - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs).

      - FFS how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations

      - Focusing on higher modulation orders focusing on high-SNR conditions

      - Other details are not precluded

  - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation.

    - The potential MPR reduction enabled by AI-DPoD is evaluated.

      - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

    - The impact on UL coverage and UE transmit energy efficiency is evaluated.

- Other design principle to be considered:

  - UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply.

Issue 4-1d: Evaluation for AI-DPD

Agreement:

- Non-linearity model(s) of transmission signals:

  - Discussed in other 6G agenda, which is intended to discuss the common model as much as possible.

#### AI/ML use case for Demod (Per-use case discussion)

Use Case#2: AI-based channel estimation

Agreement:

- Postpone RAN4 discussion on AI-based channel estimation including both sub-use cases (AI-based channel estimation with DMRS reduction and AI-based channel estimation with CSI-RS reduction)

  - If triggered by RAN1, RAN4 will start the study to provide RAN4 analysis and evaluation relevant to implementation and performance.

Use Case#3: AI-SRS-assisted channel reconstruction

Agreement:

- The scope of potential RAN4-led AI use case of AI-SRS assisted channel reconstruction, will be further discussed in the next meeting.

Use Case#4: AI-enabled PRACH receiver

Agreement:

- The scope of potential RAN4-led AI use case of AI-enabled PRACH will be further discussed in the next meeting.

#### AI/ML use case for RRM (Per-use case discussion)

Use Case#5: AI-based measurement prediction

Issue 6-1c: Additional principle/method for AI-RRM use case selection

Agreement:

- Additional principle/method for AI-RRM use case selection

  - Whether the proposed AI-RRM use case should be driven by RAN4:

    - Existing 5G-A use cases included in RAN2 and RAN1 WIs should be precluded in RAN4 6G study.

    - Coordination with RAN1 and RAN2 when potential AI-RRM use cases are agreed in RAN4 to avoid overlapping scope.

Issue 6-1a: Use case definition and supporting level from companies

Agreement:

- The scope of potential RAN4-led AI-RRM use case(s) will be further discussed in the next meeting.

# RAN1 – AI/ML related agreements summary

## AI/ML in 6GR interface

### RAN1 #122bis (Oct-2025)

No agreement was achieved in RAN1 #112bis. Different 6GR AI/ML use cases are identified/categorized with detailed information summarized.

### RAN1 #122 (Aug-2025)

Agreement

For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following:

- Definition of each (sub-)use case, including at least AI/ML model input/output

- The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results

- Assumption on training types, e.g.,

  - offline training, online training/finetuning

  - Label construction (if applicable), including whether/how to obtain label data for model training

- Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model

- Collaboration/interaction between UE and NW, e.g.,

  - no collaboration/interaction

  - UE/Network collaboration targeting at separate or joint ML operation

- High level potential specification impact

# RAN2 – AI/ML related agreements summary

## Data collection / management / transfer

### RAN2 #131bis (Oct-2025)

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

## LCM framework and requirements

### RAN2 #131bis (Oct-2025)

Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

## RAN2-driven AI/ML use cases

RAN2-driven AI/ML use cases are not starting in November. Here is just a placeholder.