# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2520427

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.2.1

Source:	CMCC

Title:	Discussion on 6GR waveform

Document for:	Discussion

Introduction

In RAN#108, the technology study item for 6G radio (6GR) has been approved [1]. The anticipated services of 6GR range from immersive communication and massive communication to integrated sensing and communication. Furthermore, 6G services can be provided by both terrestrial network (TN) and non-terrestrial network (NTN). in last meeting, WF[2] and LS[3] are approved which show the high level guideline for further discussion.

- In this contribution, we show our initial analysis on waveform and PA modelling.

Discussion

PA modeling

- Agreement - Consider PA modelling at least for the following aspects for RAN4 discussion - Memory effects for UE supporting larger CBW - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3) - Calibration conditions - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc. - PA models for different device types, handheld UE is prioritized - Applicable requirements - 5G-A requirements as starting point - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

PA modeling will be used for RF front end non-linearity simulation, e.g. MPR requirement analysis and as the baseline for RF AI analysis. In 5G phase, polynomial PA modeling is widely used among different work items but lack of detailed analysis of the difference from commercial PA device. Some companies also use actual testing data for MPR requirements analysis but without unified PA modeling.

For 6G, higher PAE with lower power consumption is required, following add some solutions for 6G higher PAE with corresponding drawbacks

| target | technology | drawbacks |
| --- | --- | --- |
| High PAE | Higher Vcc supply | Need DC-DC boost The most effective solution based on a comprehensive comparison |
|  | ET PA | ET PA needs high efficiency ET modulator ET PA can’t support larger than 100MHz |
|  | High accuracy APT PA | Further study how to model it with PA modeling. |
|  | Increase PA load-line | Needs increase Vcc for higher output power but the enhancement is limited |
|  | Push-Pull or Doherty PA | Induce linearity issues, no use for current <6GHz UE |
| High linearity | DPD | Adapt the separate Rx chain and DPD algorithm |

Observation 1: above list the candidate solutions to enhanced PA PAE and linearity performance with corresponding drawbacks, among which, higher Vcc supply seems like the most effective solutions.

For max 100MHz bandwidth, legacy NR ignore the memory effect. Now RAN1 approved to support max 400MHz at BS side and FFS for UE to support such CBW. 400MHz CBW will induce more memory effect which can’t be ignored for PA modelling, further more, 400MHz CBW will induce more power consumption in DPD processing.

Observation 2: when UE support max 400MHz CBW, the memory effect should be taken into consideration for PA modeling.

The PA model will impact ACLR, MPR, A-MPR, etc. Last meeting WF suggest to list the RF impairment for MPR evaluation. Following list the assumption for 5G/LTE MPR evaluation which can be used as the baseline for further enhancements.

|  | NR | LTE |
| --- | --- | --- |
| IQ suppression | 28dBc | 25dBc |
| Carrier leakage | 28dBc | 25dBc |
| Single carrier CIM3 | 60dBc | 60dBc |

Based on above enhancement, further check whether IQ suppression, carrier leakage, CIM3 can be further enhanced, e.g. enhanced by 3dBc.

Proposal 1: further check whether the IQ suppression and carrier leakage can be enhanced from 28dBc.

  - Waveform

- Agreement - The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1 - To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects - Tx assumption including PA model - Related RF requirements which should be taken into consideration - Both UL and DL are considered - Take the candidate waveforms identified/discussed in RAN1 as the baseline - Identify the evaluation metric, e.g. net gain - Implementation constraints - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs - Evaluation cases could be selected step-by-step, based on RAN1 progress - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement

Agreement Draft LS R1-2508068 is endorsed with following revision: - removing “Additionally, if time permits, any feedback for CP-OFDM PAPR reduction/MPR values achievable by implementation is also appreciated.” Agreement Final LS R1-2508069 is endorsed. Agreement - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding Agreement - Study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

RAN1 has approved to use NR waveform design as the baseline for the 6GR study. Besides, the PAPR reduction design is proposed in RAN1 with benefit of RF front-end simplification of both BS and UE in many services. Better PAPR performance will contribute to better ACLR, leading to less MPR and better UL coverage performance. On the other side, the PAPR reduction design will have potential impact on signal quality (e.g. degraded EVM performance) and implementation complexity of both transmitter and receiver.

- Proposal 2： Related RF requirements for lower PAPR evaluation include ACLR, MPR and EVM requirements

- In previous coverage enhancement evaluation, RAN4 use net gain as performance metric. Besides, RAN1 also approved to use net gain for low-PAPR evaluation in last meeting. It’s suggested to use this net gain as performance metric for low-PAPR evaluation.

- Proposal 3：It’s suggested to use this net gain as performance metric for low-PAPR evaluation.

- Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

Conclusion

This contribution makes the following observations and proposals.

| target | technology | drawbacks |
| --- | --- | --- |
| High PAE | Higher Vcc supply | Need DC-DC boost The most effective solution based on a comprehensive comparison |
|  | ET PA | ET PA needs high efficiency ET modulator ET PA can’t support larger than 100MHz |
|  | High accuracy APT PA | Further study how to model it with PA modeling. |
|  | Increase PA load-line | Needs increase Vcc for higher output power but the enhancement is limited |
|  | Push-Pull or Doherty PA | Induce linearity issues, no use for current <6GHz UE |
| High linearity | DPD | Adapt the separate Rx chain and DPD algorithm |

Observation 1: above list the candidate solutions to enhanced PA PAE and linearity performance with corresponding drawbacks, among which, higher Vcc supply seems like the most effective solutions.

Observation 2: when UE support max 400MHz CBW, the memory effect should be taken into consideration for PA modeling.

Proposal 1: further check whether the IQ suppression and carrier leakage can be enhanced from 28dBc

- Proposal 2： Related RF requirements for lower PAPR evaluation include ACLR, MPR and EVM requirements

- Proposal 3：It’s suggested to use this net gain as performance metric for low-PAPR evaluation.

- Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO

- R4-2514641, WF on system parameter, HW

- R4-2514643, LS on 6G parameter, HW

- R1-2508069, LS on PA models in 6GR waveform evaluations, HW