# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2520430

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.2.4

Source:	CMCC

Title:	Discussion on 6GR channel arrangement

Document for:	Discussion

Introduction

In RAN#108, the technology study item for 6G radio (6GR) has been approved [1]. The anticipated services of 6GR range from immersive communication and massive communication to integrated sensing and communication. Furthermore, 6G services can be provided by both terrestrial network (TN) and non-terrestrial network (NTN). in last meeting, WF[2] and LS[3] are approved which show the high level guideline for further discussion. According to chairman guideline, this sub-agenda includes but not limited to channel raster, sync raster, channel spacing etc.

- In this contribution, we show our initial analysis on channel raster, sync raster and channel spacing etc.

Discussion

Channel raster

LTE use channel raster to identify carrier center. Different from LTE, the channel raster in 5G is not that important and is mainly used in RAN4 for RF requirements conformance testing which is assumed that carrier center is aligned with channel raster. But there is no such limitation of the carrierbandwidth signaling in RAN2 spec and RAN4 conclude that the carrierbandwidth in SIB is not mandatory to be aligned with channel raster.

During initial access, NR UE use SSB position, kSSB, offset-to-pointA to identify the point A position, the value of which in NR is an NR-ARFCN. Then UE dedicated CBW downlinkChannelBW-PerSCS-List and uplinkChannelBW-PerSCS-List are calculated by specific offset to point A. Here the CBW fields are only used for the purpose of configuring the UE with a size and location to ensure compliance with regulatory requirements.

Observation 1: channel raster is mainly used for RAN4 conformance testing which assume that carrier center is aligned with channel raster. Nevertheless, there is no such limitation of the carrierbandwidth field in RAN2 spec.

In 6G, it’s suggested to conclude the relationship between channel raster and carrier center at first with following options:

- Option 1: UE dedicated CBW has to be aligned with channel raster, which should be reflected into final RAN2 spec for UE design

- Option 2: channel raster is only used for RF requirements testing and no need to be reflected into RAN2 spec

Proposal 1: RAN4 is suggested to study whether the carrier bandwidth in SIB and in UE dedicated CBW have to be aligned with channel raster, i.e. whether the channel raster is only defined for conformance testing.

In NR, the carrier center is the LO location which is also the center of IFFT/FFT, i.e. 0 sub-carrier. For 6G, the same CP-OFDM and DFT-s-OFDM are still reused. So the carrier center is still the center of IFFT/FFT, i.e. 0 sub-carrier. The question is whether such cattier center must be the LO location. 5G allow UE to report its DC location which can even be placed into the guard band. As a result, it seems no need to limit the LO location and the carrier center could not be aligned with channel raster.

NR use NR-ARFCN as the candidate value for frequency indication which method could be reused with less spec impact. Therefore, if there is no alignment requirements of carrier bandwidth and the channel raster, RAN4 can only retain the global raster concept which is for NR-ARFCN definition, i.e. 5kHz for FR1

Proposal 2: if Alignment of the carrier bandwidth with the channel raster is not mandatory, RAN4 can only retain global raster definition, i.e. 5kHz to align with NR-ARFCN definition for FR1.

If alignment of carrier bandwidth with the channel raster is mandatory just like NR, following list the method used for NR for further enhancement

- Option 1: 100kHz channel raster + 10kHz enhanced channel raster

- Option 2: SCS based

5G experience show both above options are robust from co-existence, spectrum aggregation etc point of view. The key difference is the impact for sync raster design. For above option 1, current sync raster is designed based on 100kHz channel raster and can be used for 10kHz raster but with following drawback that “The synchronization raster and the corresponding SS block do not cover all possible RF channel bandwidths and locations on Enhanced channel raster”. if we want to address above drawback and at the time maintain the principle that channel raster is aligned with carrier bandwidth, then there will be denser syn raster as denser enhanced channel raster.

Observation 2: current sync raster is based on 100kHz channel raster rather than 10kHz which lead to the SSB doesn’t cover all RF CBW and locations. If RAN4 want to address such drawback and retain the alignment between channel raster and CBW, sync raster must be designed more denser.

The drawback of SCS basis channel raster is the inflexibility when co-current with 100kHz channel raster network especially when migration from 5G to 6G. careful design is required for two carriers co-existence when one is 10kHz raster basis and the other is SCS basis.

Observation 3: the main drawbacks of SCS basis channel raster is inflexibility when co-current with 10kHz channel raster network especially when migration from 5G to 6G.

Following table show the summary based on above analysis.

| Alignment between CBW and channel raster | Only define 5kHz global raster | 100kHz + 10kHz enhanced channel raster | SCS basis channel raster |
| --- | --- | --- | --- |
| mandatory |  | Drawback: denser sync raster to cover all RF CBW and locations | Drawback: inflexibility when co-current with 10KHz channel raster carrier |
| No limitation | reuse the definition of ARFCN (preferred) |  |  |

From sparse sync raster point of view, the SCS basis channel raster is preferred if CBW has to be aligned with channel raster.

Proposa1 3: SCS basis channel raster is preferred if CBW has to be aligned with channel raster.

Synchronization raster

- Agreement - Evaluation on sync raster from RAN4 perspective: - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4 - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving - Collaboration and planning: - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset. - Develop a flexible evaluation framework in RAN4 that can quickly assess different sync raster proposals once key parameters (like final SSB bandwidth and min CBW) are stabilized by RAN1.

In 5G NR, the sync raster is defined based on minimum transmission bandwidth configuration, SSB bandwidth and channel raster. The minimum CBW for <3GHz, >3GHz FR1 and FR2 frequency range is 5MHz, 10MHz and 50MHz respectively. Such minimum CBW based sync raster will introduce much denser raster and enlarge search time with higher power consumption. Usually operators have wider spectrum than above minimum channel bandwidth. For example, CMCC’s spectrum for n41 and n79 is 160MHz and 100MHz respectively. Following show the candidate sync raster numbers for different CBW for <3GHz. UE has to search more sync raster for larger CBW based on current spec, which is not necessary for actual deployment.

RAN4 is suggested to study the possibility of defining sparser sync raster based on larger CBW which align with operators wider spectrum profile with the benefits of shorter search time and lower power consumption.

- Proposal 4: RAN4 further study the possibility of defining sparser sync raster based on larger CBW which align with operators wider spectrum profile with the benefits of shorter search time and lower power consumption.

- NR define sync raster based on unified minimum CBW, e.g. 5MHz for <3GHz FR1, 10MHz for other FR1 and 50MHz for FR2. Different operators have different spectrum number, if we still reuse the unified minimum CBW which will lead to the denser sync raster or doesn’t meet all operators’ demand. A solution is to implement a multi-tiered synchronization raster. For instance, one tier would correspond to a unified minimum channel bandwidth, while another would support larger bandwidths.

- Proposal 5: A proposed solution is to implement a multi-tiered synchronization raster. For instance, one tier would correspond to a unified minimum channel bandwidth, while another would support larger bandwidths.

Conclusion

This contribution makes the following observations and proposals.

Observation 1: channel raster is mainly used for RAN4 conformance testing which assume that carrier center is aligned with channel raster. Nevertheless, there is no such limitation of the carrierbandwidth field in RAN2 spec.

Observation 2: current sync raster is based on 100kHz channel raster rather than 10kHz which lead to the SSB doesn’t cover all RF CBW and locations. If RAN4 want to address such drawback and retain the alignment between channel raster and CBW, sync raster must be designed more denser.

Observation 3: the main drawbacks of SCS basis channel raster is inflexibility when co-current with 10kHz channel raster network especially when migration from 5G to 6G.

| Alignment between CBW and channel raster | Only define 5kHz global raster | 100kHz + 10kHz enhanced channel raster | SCS basis channel raster |
| --- | --- | --- | --- |
| mandatory |  | Drawback: denser sync raster to cover all RF CBW and locations | Drawback: inflexibility when co-current with 10KHz channel raster carrier |
| No limitation | reuse the definition of ARFCN (preferred) |  |  |

Proposal 1: RAN4 is suggested to study whether the carrier bandwidth in SIB and in UE dedicated CBW have to be aligned with channel raster, i.e. whether the channel raster is only defined for conformance testing.

Proposal 2: if Alignment of the carrier bandwidth with the channel raster is not mandatory, RAN4 can only retain global raster definition, i.e. 5kHz to align with NR-ARFCN definition for FR1.

Proposa1 3: SCS basis channel raster is preferred if CBW has to be aligned with channel raster.

- Proposal 4: RAN4 further study the possibility of defining sparser sync raster based on larger CBW which align with operators wider spectrum profile with the benefits of shorter search time and lower power consumption.

- Proposal 5: A proposed solution is to implement a multi-tiered synchronization raster. For instance, one tier would correspond to a unified minimum channel bandwidth, while another would support larger bandwidths.

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO

- R4-2514641, WF on system parameter, HW

- R4-2514643, LS on 6G parameter, HW

- R1-2508069, LS on PA models in 6GR waveform evaluations, HW