3GPP TSG-RAN WG4 Meeting #117			                           R4-2520439

Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.11

Source: 	CMCC

Title: 	Discussion on 6GR Testability and OTA

Document for:	Discussion

# 1. Introduction

After the discussion at the last meeting, some preliminary conclusions regarding 6G Testability and OTA were reached. The main focus of the discussion was on being able to complete the design of the test system in the early stage of 6G and avoiding the problems left over from the NR period.

This document will build upon the conclusions reached at the last meeting and continue the discussion.

- Discussion

### 2.1 Improved testability of Conducted Requirements

After last meeting discussion, we have reached the following agreement.

- Sub-topic 1-1 Antenna assumption of FR1 transmitter and receiver Issue 1-1-1: Whether RAN4 should consider a More realistic antenna efficiency assumption for FR1 Agreements: - move this discussion to general RF/UE RF from next meeting. Issue 1-2-1: Whether it is valuable to improve some conducted requirements to be verified via OTA approach Agreements: - RAN4 can further study the testability limitations of some conducted test cases, consider the following aspects as starting point, e.g., - FR1 conducted test cases already consider antenna-performance impacts but not verified via radiated approach in a case-by-case manner, e.g. MSD - FR1 test case simplification with both conducted and radiated considered, e.g., spurious emission - Discuss which conducted conformance testing could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern.

From our perspective, the motivation of improving some conducted requirements to be verified via OTA approach is good because it is undeniable that the OTA test is closer to the real scenario. However, from the testing perspective, cost and time also need to be considered. Compared to the conducted test, the OTA test is more expensive and time-consuming. Considering all aspects, it is recommended to retain the conducted test at this stage and analyze it using the case-by-case method.

Observation 1: Although the OTA test is closer to the real scenario, its testing cost is high and it takes a long time.

Proposal 1: It is suggested to retain the conducted test at this stage and analyze it using the case-by-case method.

### 2.2 New test methodologies for new 6GR frequencies

Issue 2-1-1: Testability for New Frequencies between FR1 and FR2 Agreements: RAN4 study the testability including conducted testing and radiated testing for the new frequencies. The following aspects can be considered: - The frequency range definition will be discussed under the spectrum agenda. - For conducted test, use existing test method as a starting point - For OTA test - Further study potential test methodologies including the full-package, i.e., supported frequency range, test setup, configuration, positioning, procedure, validation/calibration, quiet-zone/test zone, MU, testing time reduction

The discussion on 6G frequency categorization is still ongoing in 6G spectrum, especially the issue whether to define a new separate FR has not yet reached a conclusion.

Therefore, in terms of testability, it is more necessary to consider from the perspective of frequency characteristics. If option 2 is adopted for frequency categorization, the testability will most likely be consistent with the corresponding FR. For the frequency around 7GHz, considering that its frequency range is closer to FR1, it is recommended that its testability use FR1 conduction test as the baseline.

Proposal 2: For the testability around 7GHz, FR1 conduction test should be used as the baseline.

### 2.3 Testability for different Device types

Issue 4-1-1: OTA testability applicability for different UE types in 6GR day-1 Agreements: - RAN4 consider the study of OTA test system to better accommodate different UE types, new feature/functionalities, new performance metrics and test cases. - The UE types and form factor considered in testability can be further discussed in this SI. - RAN4 study the feasibility of developing a single system to cover above aspects.

In addition to the new frequencies, new test cases also need to be discussed. Many testing requirements for different device types were raised during 5G, such as AIOT devices and NTN UEs. Due to the differences in device architectures and characteristics, the testing requirements for these devices in some cases cannot fully match the existing testability. However, due to the high cost of setting up the testing system and the difficulty in modifying, it is difficult to adjust the testing system for these new device types.

During last meeting, the discussion on device type has not reached a conclusion. Due to the significant differences among devices, it is quite difficult to completely use a single system to cover all the above aspects. However, for the convenience of future device testing, we can first refer to the more common device types in the NR network to design a unified testing system, such as handheld devices, NTN terminals, FWA, etc.

Proposal 3: Discuss the testing system based on the more common devices in the NR network.

Altering a proven test architecture to support new device categories would incur extra expense and technical debt—something that becomes prohibitively difficult once the system is already deployed. Facing the diverse new device of 6GR, if it is impossible to use a completely identical testing system, then we can retain as much flexibility as possible for the testing system to adapt to the new devices that may appear in the future of 6G.

Observation 2: It is very difficult to make modifications to the tested system that has already undergone verification.

Proposal 4: Retain as much flexibility as possible for the unified testing system to develop and expand for other 6G devices.

### 2.4 Efficiency improvement (Test and requirements)

Issue 9-1-1: Improve OTA test efficiency in 6GR Agreements: - 6G OTA testability study should consider system complexity, cost and test time reduction.

To make 6G testing both affordable and fast without sacrificing trust-worthy results, when conducting 6G tests, it is necessary to comprehensively consider various factors in order to achieve a balance in terms of system complexity, cost, and test time reduction., equipment cost and operational complexity. In response to the rapid growth of 6G devices and application scenarios, prioritize testing based on the business impact for each 6G scenario, and give priority to testing high-value cases. This will effectively improve the testing efficiency. Additionally, it is also advisable to consider further simplifying the testing steps and methods in order to achieve the goal of reducing costs and testing time.

Observation 3: By setting test priorities and simplifying test procedures and methods, the time and cost can be further reduced.

### 2.5 AI/ML testability

Issue 5-1-1: RAN4 study on enhancement of AI/ML testability for 6GR Agreements: - The corresponding test method including both OTA and conducted, to verify the AI/ML features (FFS details) should be studied in RAN4 in 6G SI. - The methodologies identified in 5G will be considered as the starting point for both conducted and OTA.

For AI/ML, one important topic is generalization. The goal of generalization is to verify whether the performance gain/minimum level of performance of AI/ML functionality/model can be achieved/maintained under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. In 5G-A discussion on testing environment/framework for AI/ML, both static and non-static scenarios/configurations are dicsussed. Finally, due to limited timeline and workload, static conditions are used as baseline. In detail, RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability (environment differs in each test but not changing dynamically during the test). There are some drawbacks for generalization based on static test. Firstly, the test with fixed configurations/channel conditions cannot reflect the real situation. Secondly, to guatantee the generalization performance of AI/ML model in real-world, multiple tests are needed will increase the test burden. Above issues can be solved by adopting dynamic test. Dynamic test, i.e. environment changes dynamically during the test, is more realistic. And the dynamic test could help in reducing the number of tests. It is proposed to study dynamic test for AI/ML in 6G.

Proposal 5: for AI/ML, it is proposed to consider dynamic test (i.e. environment changes dynamically during the test) in 6GR.

# 3. Conclusions

Observation 1: Although the OTA test is closer to the real scenario, its testing cost is high and it takes a long time.

Proposal 1: It is suggested to retain the conducted test at this stage and analyze it using the case-by-case method.

Proposal 2: For the testability around 7GHz, FR1 conduction test should be used as the baseline.

Proposal 3: Discuss the testing system based on the more common devices in the NR network.

Observation 2: It is very difficult to make modifications to the tested system that has already undergone verification.

Proposal 4: Retain as much flexibility as possible for the unified testing system to develop and expand for other 6G devices.

Observation 3: By setting test priorities and simplifying test procedures and methods, the time and cost can be further reduced.

Proposal 5: for AI/ML, it is proposed to consider dynamic test (i.e. environment changes dynamically during the test) in 6GR.

# 4. Reference

- R4-2514647, WF for [116bis][110] 6G testability and OTA, Feature lead (vivo)