# 3GPP TSG-RAN WG4 Meeting #117                             R4-2520442

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	8.8.1

Source: 	CMCC

Title: 			 Discussion on general aspects for 6G AI/ML

Document for:	Discussion

- Introduction

In RAN #108 meeting, the SID on 6G radio was approved [1]. The objectives on AI/ML are duplicated as following.

- (8)AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In last RAN4 meeting, there was discussion 6G AI/ML, and a WF was approved [2]. This contribution provides discussion on general parts for RAN4 study on 6G AI/ML.

- Discussion

Topic#1: General scope and timeline Issue 1-1: RAN4 6G AI/ML study scope Agreement: - In 6G AI/ML study, the detailed RAN4 scope shall include - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… - RAN4-driven use cases: RAN4 shall identify, select and study only those AI/ML use cases that fall within RAN4 core responsibility. The detailed principles are captured under 3-1. - Other WG-driven use cases: RAN4 shall assist other working groups in these use cases by studying RAN4 impacts with RAN4 analysis and evaluation relevant to implementation and performance. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures: RAN4 can study relevant RAN4 requirements for LCM procedures. If the use cases are identified by other WG, RAN4 related study should be based on the agreements and/or requests by other WG. The exact LCM context will be separately discussed and decided in the responding WGs. - RAN4 may study the feasibility and testability of the AI/ML functionalities. The study may include different aspects. It will be further decided how to manage the discussion under different agenda items, e.g. AI or testability. Topic#2: RAN4 AI/ML framework Issue 2-2: Post-deployment enhancement Agreement: - RAN4 will discuss the potential post-deployment testing enhancements scope in 6G Issue 2-4: Model generalization Agreement: - For AI/ML functionality generalization, RAN4 shall study generalization of UE and BS based AI/ML functionality for 6G and identify the potential standard impact. - The study will begin no early than Q2 2026. - Study and identify the typical parameters/aspects that are expected to be generalized by AI/ML functionality for each use case firstly, and then discuss how to test the generalization for the corresponding use case. - RAN4-led use cases will be prioritized. - The related study on other WG-led use cases will be based on the requests from the corresponding WG. Issue 2-6: AI/ML UE category Agreement: - Regarding metrics related to AI/MI complexity supported by UE/NW, it can be discussed if a study is needed in 6G no earlier than Q2 2026. - The definition and the context of AI/MI complexity needs to be further clarified if the study is agreed.

Three issues are identified for RAN4 AI/ML framework: post-deployment enhancement, model generalization, AI/ML UE category. According to the agreed WF in last meeting, RAN4 study on model generalization and AI/ML UE category will begin no early than Q2 2026, this contribution focus on the discussion on post-deployment enhancement.

post-deployment is important to guarantee the performance in real field. The change/update/modification of AI/ML model/functionality may be very frequent. It is questionable whether a device that has passed the conformance test with one version of AI/ML model/functionality can also pass the test after the change/update/modification. What’s more, it is also possible that a totally new model is loaded to the device after deployment, which means its performance are not tested and cannot be guaranteed. Taking above into account, it is necessary to study post-deployment to guarantee the performance.

In REL-19 study, there are two options. Option 1 is post-deployment pre-activation functionality/configuration update testing. In detail, the the validation of a change in AI functionality is conducted before its deployment/activation in already deployed UEs. Option 2 is post-deployment post-activation functionality testing based on performance monitoring. The performance monitoring and LCM procedures will be used. RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. The detailed description of options 1 and 2 and sub-options can be found in table below [3]. In RAN4#115 meeting, it was agreed that Option 2 (Post-deployment post-activation functionality testing based on performance monitoring) is prioritized [4].

- Option 1: Post-deployment pre-activation functionality/configuration update testing

- Option 2: Post-deployment post-activation functionality testing based on performance monitoring

|  | Option 1 | Option 2 |
| --- | --- | --- |
| New name | Post-deployment pre-activation functionality/configuration update testing | Post-deployment post-activation functionality testing based on performance monitoring |
| Description | Conduct the validation of a change in AI functionality before its deployment/activation in already deployed UEs • Validation takes into account the UE hardware in which the model is to be deployed/activated. | Using performance monitoring and LCM procedures • Performance monitoring will be designed in other groups • RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. |
| Sub-options | Possible collection of input data to a model during conformance testing for use later on. |  |
| Further potential clarifications (not resolved) | The following proposals have been made for option 1 but not discussed. o Whether the testing of a new model is on a device or in a lab. o Whether models that have been updated post conformancetesting should be kept in a device. o Whether parallel operating of one model and (in-device) testing of another can be assumed. o Whether signaling from a UE of changes/updates are needed. - Whether there is any relation to GCF procedures. - Whether it is possible to differentiate “major” and “minor” model changes and only apply option 1 to major changes. - Definition of Fine-tuning and Model update - Model update can have a performance impact. - Fine-tuning might have little performance impact. - Hybrid Validation approach - A hybrid approach integrating both pre-activation functionality/model update testing (Option 1) and post-deployment management based on LCM (Option 2) aims to provide a balanced validation strategy. | The following proposals have been made for option 1 but not discussed. - Whether the metric used for performance testing and the metric used for monitoring can be aligned. - Whether monitoring used for post-deployment testing should be NW sided only. |
| Significant issues | Whether the option is reasonable or would cause too much complexity and test time burden when introducing updates. | Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.) Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs. |

For 6G study, the study in 5G can be used as baseline. Considering that it was agreed that option 2 (Post-deployment post-activation functionality testing based on performance monitoring) is prioritized, option 2 can be used as starting point for 6G study.

Proposal 1: for post-deployment, 5G study can be used as baseline. And option 2 (Post-deployment post-activation functionality testing based on performance monitoring) can be used as starting point for 6G study.

During 6G study phase, there are multiple candidate AI/ML use cases, including RAN4-driven use cases and RAN1/2-driven use cases. And there may be more use case introduced in the feature. For post-deployment, one issue is whether the reliable and testable monitoring procedures can be established for all use cases or need to be discussed case by case. From the perspective of workload and forward compatibility, a unified monitoring procedures which can be applied to all the use cases to guarantee the performance of post-deployment is preferred. It is necessary to study whether the unified solution for post-deployment applied to all use cases is feasible.

Proposal 2: for post-deployment based on performance monitoring, it is proposed to study whether monitoring procedures can be established in a general way which can be used for all use cases or need to be discussed separately in a case by case manner.

- Conclusion

This contribution provides discussion on general parts for RAN4 study on 6G AI/ML. The observations and proposals are:

Proposal 1: for post-deployment, 5G study can be used as baseline. And option 2 (Post-deployment post-activation functionality testing based on performance monitoring) can be used as starting point for 6G study.

Proposal 2: for post-deployment based on performance monitoring, it is proposed to study whether monitoring procedures can be established in a general way which can be used for all use cases or need to be discussed separately in a case by case manner.

- Reference

- RP-251881, New SID: Study on 6G Radio

- R4-2514637, WF on 6G AI, Samsung

- R4-2503953, WF on AI/ML air interface study, Ericsson

- R4-2508051, WF on AI/ML part 2, Ericsson

- R4-2511652, RAN4 6G Plan and Meeting Arrangement