# 3GPP TSG-RAN WG4 Meeting #117                             R4-2520447

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	8.6

Source: 	CMCC

Title: 			 Discussion on RRM for 6GR study

Document for:	Discussion

- Introduction

In RAN #108 meeting, the SID on 6G radio was approved [1]. The objectives on RRM aspects for 6GR is highlighted in blue as following.

- (5)6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4]

In last RAN4 meeting, there was initial discussion on RRM aspects for 6GR, and a WF was approved [2]. This contribution provides discussion on RRM aspects for 6G.

- Discussion

## 2.1 Measurement gap(MG) and interruption

Issue 4: Measurement gap(MG) and interruption Issue 4-1: MG related scope Agreement in main session (Tue): Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion. [Way forward] - FFS followings in RAN4#117 (other sub-topics are not precluded) - based on the majority views from companies, RAN4 RRM to discuss which MG related sub-topics can be prioritized from the following candidate list, and then discuss the solutions for the selected topics in 6G SI: - MG pattern reduction from 5G (10 companies: Apple, MTK, QC, OPPO, Sony, Xiaomi, vivo, Ericsson, Samsung, Nokia) - Gap-less measurement and its side conditions (10 companies: MTK, Samsung, CMCC, ZTE, NTT DCM, HW, Ericsson, Nokia, QC, Xiaomi) - In scenarios with and without an available RF chain. - Unified MG (7 companies: Apple, OPPO, LGE, Xiaomi, ZTE, Ericsson, QC), e.g., - Unified MG configuration - Unified MG and scheduling restriction - Unified MG for different feature related measurements - Adapative MG operation, e.g., activation/deactivation/cancellation/skipping (7 companies: Apple, OPPO, CMCC, Xiaomi, ZTE, Ericsson, LGE) - Using which 5G MG enhancement features to 6G day 1 (OPPO, CMCC, Xiaomi, vivo) - E.g., needforGap, NCSG, concurrent MG, preconfigured MG and etc. - MG sharing(Apple, OPPO, vivo), e.g., - among intra-frequency, inter-frequency, and inter-RAT measurement (including L3 and L1 measurement) - UE assisted MG configuration (Apple, QC) - e.g., MG requesting by UE (Apple, QC) - Multi-CC measurements in MG (Apple, CMCC, ZTE(searcher related)) - Optimization on MGL and RF tuning/retuning (Apple, MTK, QC) - MG applicability for per-UE, per-FR, per-CC, or per-CC group (OPPO, Xiaomi, ZTE(granularity)) Issue 4-2: interruption related scope [Way forward] - FFS followings in RAN4#117 (other sub-topics are not precluded) - Proposal 1(QC): - RAN4 should identify and evaluate mechanisms that enable interruption-free measurements, with a focus on deployability from the beginning of 6GR. - RAN4 should study all existing interruption scenarios in 5G NR whether they are still applicable to 6GR, whether the existing requirements should be refined and whether interruptions are needed in certain scenarios not yet covered in 5G NR. - RAN4 should study for the case of 15 kHz subcarrier spacing whether the unused half slot (0.5 ms) in the duration of a measurement gap can be used efficiently, e.g., for data transmission. - Proposal 2(ZTE): - Finer granularity of interruption such as symbol level is foreseen in 6G to benefit the throughput via avoid the vital symbols within a slot used for DCI or UCI. Awareness of the location of interruption by NW could lesson the impact on throughput. - Proposal 3(Ericsson): - RAN4 shall strive for reducing the amount of interruptions in UEs. - The interruptions can be specified at a symbol level. - The existing interruption requirements may need to be revisited for 6G, if the 6G UEs can achieve a better performance. - Proposal 4(Nokia): - RAN4 to aim at removing all UE autonomous interruptions in 6G. - Proposal 5(MTK): - RAN4 (RRM/RF) to further study the possibility of reducing the interruption time including the RF retuning time for different UE procedures.

Measurement plays an important role in RAN4 RRM. Measurement enhancement, including multiple measurement gap types, introduced in different release in 5G. Per-UE gap and per-FR gap are the baseline. Inter-frequcy measurement without gap for the case that SSB is completely contained in the active BWP of the UE is introduced in Rel-16. NeedForGap is introduced by RAN2 in Rel-16, but the requirements are not specified until Rel-18. Concurrent MGs, Pre-MG, NCSG are introduced in Rel-17. Rel-18 focus on enhancements of pre-configured MGs, multiple concurrent MGs and NCSG, NR SSB-based inter-frequency and intra-frequency measurements without gaps, and inter-RAT measurement without gaps. XR transmission/reception in measurement gaps (MG skipping/canceling) is supported in Rel-19.

Some useful features.e.g. measurement without gaps, are fully supported in Rel-18 when the related RAN4 requirements are specified. Since the standardization support for this feature is late, the market support is not well, it is challenging for companies to implement and deploy these feature. To avoid the same issue in 6GR, it is necessary that the feature with market demand are supported from 6G day-one.

Proposal 1: it is proposed that the feature with market demand are supported from 6G day-one.

From the perspective of operator, measurement without gaps is one of our interest. Measurement gap will result in throughput loss, e.g. 15% throughput loss is observed for MG pattern with 40ms MGRP and 6ms MGL. Gap-less measurement enable UE to measure the MOs outside its operating BWPs without measurement gaps,  which is beneficial for network since the impact on throughput is reduced. As mentioned above, for NR, NeedForGap is introduced by RAN2 in Rel-16, but the requirements are not specified until Rel-18. And NCSG is introduced in Rel-17. As a result, these features are not supported well in the real field. To avoid the same issue, for 6GR, it is necessary that measurement without gaps are supported from 6G day-one.

Proposal 2: it is proposed that measurement without gaps are supported as mandatory from 6G day-one.

For measurement without gap, in NR, there are two candidate solutions: NeedForGapNCSG, NeedForGap-InfoNR. The common part of these two solutions is that measurement without MG are performed by using the spare RF chain. The difference is whether the interruption location is known or not to network. The location of interruption for NCSG is clear between network and UE, which is the VIL of the NCSG pattern. While for NeedForGap, the interruption location are not specified, only maximum interruption ratio are defined.

For 6GR, to support measurement without gaps, the above 5G design can be used as starting point. However, one issue observed in 5G is that NCSG and NeedForGap are similar solutions, considering both solutions will lead to increased WG workload and fragmented market. It is better to focus on one of the solutions, not both.

Proposal 3: To support measurement without gaps for 6GR, 5G MG features, e.g. NCSG or NeedForGap can be used as starting point.

A related consideration is about interruption. For measurement without gap, no matter NeedForGapNCSG or NeedForGap-InfoNR, there is UE capability about whether there is interruption. Even though UE capability is introduced, the common case is assuming interruption is needed. On one hand, no measurement gap but with interruption will impact throughput. On the other hand, with or without interruption complex the specification, as duplicated as following as one example.

TS 38.133 The inter-frequency measurement requirements in clause 9.3.4 and clause 9.3.5 applies for the following scenarios: - SSB-based inter-frequency measurement object with measurement gap. - SSB-based inter-frequency measurement object without measurement gap for UE capable of interFrequencyMeas-NoGap, when - all of the SMTC occasions of this inter-frequency measurement object are overlapped with the measurement gap or associated measurement gap in concurrent measurement gaps, or - part of the SMTC occasions of this inter-frequency measurement object are overlapped with the associated measurement gap and all the SMTC occasions of this inter-frequency measurement object are overlapped with the union of concurrent measurement gaps, or - part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or associated measurement gap in concurrent measurement gaps and the flag interFrequencyConfig-NoGap-r16 is not configured by the Network. - part of the SMTC occasions of this inter-frequency measurement object are overlapped with the associated measurement gap and all the SMTC occasions of this inter-frequency measurement object are overlapped with the union of concurrent measurement gaps or the union of concurrent measurement gaps and MUSIM gaps if MUSIM gaps are configured. - no requirement applies if all of the SMTC occasions of this inter-frequency measurement object are overlapped with MUSIM gaps if MUSIM gaps are configured. - SSB-based inter-frequency measurement object without measurement gap for UE capable of NeedForInterruptionInfoNR-r18, when - all of the SMTC occasions of this inter-frequency measurement object are overlapped with the measurement gap or associated measurement gap in concurrent measurement gaps for the UE indicating ‘no-gap’ via NeedForGapsInfoNR and ‘no-gap-with-interruption’ or ‘no-gap-no-interruption’ via NeedForInterruptionInfoNR-r18 for the inter-frequency measurement, or - part of the SMTC occasions of this inter-frequency measurement object are overlapped with the measurement gap or associated measurement gap in concurrent measurement gaps for the UE indicates ‘no-gap’ via NeedForGapsInfoNR and ‘no-gap-with-interruption’ via NeedForInterruptionInfoNR-r18 for the inter-frequency measurement. The inter-frequency measurement requirements in clause 9.3.9 applies for the following scenarios: - SSB-based inter-frequency measurement with no measurement gap, when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network. - SSB-based inter-frequency measurement with no measurement gap, when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network. - SSB-based inter-frequency measurement with no measurement gap, when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps or MUSIM gaps if MUSIM gaps are configured, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network. - SSB-based inter-frequency measurement with no measurement gap, when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps or MUSIM gaps if MUSIM gaps are configured, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network. - for UE indicating NeedForInterruptionInfoNR-r18, when - none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps for the UE indicates ‘no-gap’ via NeedForGapsInfoNR and no-gap-with-interruption or no-gap-no-interruption via NeedForInterruptionInfoNR for the inter-frequency measurement. - part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps, for the UE indicates ‘no-gap’ via NeedForGapsInfoNR and no-gap-no-interruption via NeedForInterruptionInfoNR for the inter-frequency measurement. - none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps or MUSIM gaps if MUSIM gaps are configured, for the UE indicates ‘no-gap’ via NeedForGap-InfoNR and ‘no-gap-with-interruption’ or ‘no-gap-no-interruption’ via NeedForInterruptionInfoNR-r18 for the inter-frequency measurement. - part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps or MUSIM gaps if MUSIM gaps are configured, for the UE indicates ‘no-gap’ via NeedForGap-InfoNR for the inter-frequency measurement.

In our view, this is about UE implementation. We are not sure whether it is good to have the same assumption as 5G, since it is expected 6GR is enhanced compared with NR. For 6GR, spectrum efficiency improvement is one of the key considerations of 6GR study, it is preferred not to consider interruption for measurement without gap, which is beneficial for throughput improvement, and also simplify RAN4 discussion and requirements definition.

Proposal 4: for 6GR, it is proposed to discuss whether interruption are still needed for measurement without gap. Could we assume no interruption for gap-less measurement as a basic feature.

Another important feature is parallel measurement. For NTN, parallel measurement is supported. In detail, parallelSMTC-r17 (Parallel measurements on multiple SMTC-s for a single frequency carrier), parallelMeasurementGap-r17 (Parallel measurements with multiple measurement gaps), parallelMeasurementWithoutRestriction-r17 (Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell), and maxNumber-NGSO-SatellitesWithinOneSMTC-r17 (Parallel measurements on multiple NGSO satellites within a SMTC) are considered. Taking parallelMeasurementGap-r17 (Parallel measurements with multiple measurement gaps) as an example, 2 is mandatory and 4 is optional. These parallel measurement for NTN need to be inherited from NR to 6GR.

For TN, concurrent measurement gap is introduced in Rel-17. Concurrent gaps enable different gap configurations for different MOs, which could avoid the resource waste due to configure one MG with long MGRP and/or short MGRP try to cover as much MOs as possible. Concurrent gaps is also helpful in reducing measurement delay. It is proposed to consider concurrent measurement gap from 6G day-1.

However, parallel measurement for the colliding cases is not supported. Two gaps are considered as colliding when there is fully or partially overlapping between two gaps or no overlapping but the distance between two gaps is less than 4ms. For the colliding case, only one gap is kept based on priority, the other gap is dropped. It is proposed to support Parallel measurement for colliding gaps. On one hand, it is beneficial for both throughput and measurement delay reduction. On the other hand, this issue is also about searcher assumptions. For 6GR, we believe the assumption about searchers will be enhanced compared with NR, which enable the parallel measurement.

Proposal 5: it is proposed to support concurrent measurement gaps from 6G day-1.

Proposal 6: it is proposed to support parallel measurement for the colliding measurement gaps.

Measurement gap has impact on throughput. During the measurement gaps, UE is not required to conduct reception/transmission from/to the serving cells except the reception of signals used for RRM measurement(s) and the signals used for random access procedure. How UE perform measurement is up to UE implementation, network has no information about whether measurement is performed in a certain measurement occasion. As a result, network will not schedule UE in any measurement gap occasion. For a typical measurement gap configuration with 40ms MGRP and 6ms MGL, the data loss is 15%.

Transmission/reception in measurement gaps (MG skipping/canceling) is supported in Rel-19, but it is only limited to XR service. Dynamic indication is used to indicate whether to skip the first gap occasion after a minimum time offset required between the last symbol of the PDCCH carrying the indication and the start of corresponding skipped gap/restriction occasion indicated by the DCI. In addition, UAI about a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s is introduced. MG skipping/canceling is a good mechanism which is beneficial for throughput improvement. It is proposed to consider MG skipping/canceling in 6GR, and MG skipping/canceling is considered as a generic approach, i.e. it is applied to all the service, not limit to XR.

Proposal 7: it is proposed to consider MG skipping/canceling from 6G day-1, and MG skipping/canceling is a generic approach, not limit to XR.

Multiple measurement gap patterns, in total more than 20 gap patterns, are introduced in 5G, as duplicated in following table. Too many gap patterns increase the implementation complexity for both network and UE.  Among those gap patterns, only limited a few patterns are mandatory. While many MG patterns are optional based on UE capability. According to the discussion in last meeting, some companies propose reduce the number of gap patterns. In our view, the MG pattern reduction can be considered together with increased mandatory gap patterns. It is expected that all the selected MG patterns for 6GR can be mandatory supported.

Table 9.1.2-1: Gap Pattern Configuration

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |
| 24 | 10 | 80 |
| 25 | 20 | 160 |

Proposal 8: for measurement gap design for 6GR, reducing MG patterns and increasing number of mandatory MG patterns can be discussed together.

Measurement gap pattern are defined in terms of measurement gap length (MGL) and measurement gap repetition period (MGRP). MGL consists of measurement duration and RF re-tuning time (RRT) before and after measurement duration. In 5G, the working assumption on RRT is 0.5 ms for FR1 and 0.25ms for FR2. As discussed in last meeting, UE vendors propose to consider shorter RF re-tuning time since UE has better implementation. Is is beneficial to consider shorter RRT, which not only reduce the measurement gap length, and also reduce the interruption due to RF re-tuning.

Proposal 9: it is proposed to consider reduced RF re-tuning time when discuss the measurement gap pattern for 6GR.

## 2.2 RRM framework: Measurement capability/delay/overhead/accuracy/quantity/unified-measurement

Issue 5: RRM framework: Issue 5: RRM framework: Measurement capability/delay/overhead/accuracy Agreement: RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. Note: the title of the main topic can be revised after selection of the sub-topics. [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - Measurement capability - measurement capability for number of cells, beams and frequency layers (MTK, OPPO, Ericsson) - Virtual RRM UE group (Apple, ZTE) - Measurement requirements depending on purpose of the configured measurement: mobility or data (CA) (Nokia) - RF retuning time, baseband processing time for typical use cases - UE reference architecture for 6G spectrums - Baseline assumptions of RRM requirements for different UE device types - Measurement delay/overhead - Searcher number for enhanced simultaneous measurements (OPPO, HW, Samsung, ZTE, vivo, Ericsson) - Rx beam sweeping factor reduction (QC, Ericsson) - Identification/measurement/tracking/reporting delay reduction (QC) - RRM with NW aided measurement priority (Ericsson) - Virtual RRM UE group (Apple, ZTE) - Measurement requirements depending on purpose of the configured measurement: mobility or data (CA) (Nokia) - Unified UE measurement requirements, including cell detection status and measurements, across state transitions and cell changes. (Nokia) - SSB evaluation (Samsung) - Intra and inter-frequency definition - Unified measurements - United/integrated cross-layers measurement and/or report between L1 and L3 (Samsung, OPPO, CMCC, LGE, Xiaomi) - United/integrated cross-functions measurement and/or report for L1 (e.g., integration of MIMO and LTM, or integration of RLM/BFD/CBD)(OPPO, CMCC, Xiaomi) - RAN4 to study the flexible and adaptive measurement behaviour for L1 measurement. Unification in terms of reusing L3 measurements for L1 before certain point (e.g., before TCI state activation) (Ericsson)

Measurement delay is related with number of searchers. In 5G, 2 searchers are assumed until Rel-19 with the introduction of 3 searchers. However, the applied scenarios for 3 searchers are limited. For 6GR, it is expected that the assumption about searchers will be enhanced compared with NR. With increased number of searchers, the parallel measurement is enabled, which could help in reducing measurement delay.

Proposal 10: to reduce measurement delay, it is proposed to study the increased number of searchers in order to enable simultaneous measurement.

One lesson learned from NR is that RAN4 spend a lot effort in the definition of intra-frequency/inter-frequency measurement, but the actual impact is only about how to categorize requirements from RAN4 perspective. And the tough discussion is repeated again and again for different type of measurement. Following is the summary of definition on intra-frequency/ inter-frequency measurement.

SSB based L3 measurement A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell and the centre frequency of the SSB of the neighbour cell indicated for measurement are the same, and the subcarrier spacing of the two SSBs are also the same CSI-RS based L3 measurement A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement SSB based L1 measurement A measurement is defined as an SSB based intra-frequency L1-RSRP measurement provided the centre frequency of the SSB of the serving cell and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. CSI-RS based L1 measurement A measurement is defined as a CSI-RS based intra-frequency L1-RSRP measurement provided that: - the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and - for SCS of 60KHz, the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and - at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP.

In our understanding, from RAN4 perspective, the original motivation to have definition on intra-frequency/ inter-frequency measurement is that it can be combined whether measurement gap is needed or not based on the measurement type. For LTE, for most cases, intra-frequency measurement means no measurement gap, inter-frequency measurement means measurement gap is needed. However, the situation is different for NR. Whether measurement gap is needed or not is not decided by the the intra-frequency measurement definition. The key point is whether target reference signal is within UE active BWP or not. Taking SSB based L3 measurement as an example, requirements are categorized into 4 cases: intra-frequency measurement without gap, intra-frequency measurement with gap, inter-frequency measurement without gap, inter-frequency measurement with gap. This categorization results in a lot workload when discussing enhancement in later release.

Observation 1: in NR, with the definition on intra-frequency/inter-frequency measurement, measurement requirements are categorized into 4 cases: intra-frequency measurement without gap, intra-frequency measurement with gap, inter-frequency measurement without gap, inter-frequency measurement with gap, which results in complex specification and increased RAN4 workload.

For 6GR, one general issue nee to be careful considered is how to handle the definition on intra-frequency/ inter-frequency measurement. Do we need the definition. If the definition is only about requirements categorization, maybe no definition is also OK. One option is that no definition on intra-frequency/ inter-frequency measurement. RRM requirements are categorized as measurement with gap and measurement without gap.

Proposal 11: it is proposed to discuss whether to have the definition on intra-frequency/ inter-frequency measurement in 6GR. Suggest to discuss whether following consideration is feasible

- Option 1: no definition on intra-frequency/ inter-frequency measurement. RRM requirements are categorized as measurement with gap and measurement without gap

In NR, both L3 based mobility and L1/L2-Triggered Mobility are supported. L3 based mobility is robust, while LTM could reduce the latency, overhead. It is proposed to consider both L3 based mobility and L1/L2-Triggered Mobility from 6G day-1.

In NR, L3 based mobility and L1/L2-Triggered Mobility are independent. Taking intra-frequency measurements as an example, the requirements are duplicated as following. Table 9.2.5.2-1 is the requirements for L3 based measurement, Table 9.14.5.1-1 and Table 9.14.5.1-2 are the requirements for LTM.

Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.14.5.1-1: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is the configured periodicity for reporting. NOTE 2: K = 1.5. NOTE 3: The requirements apply when the max RTD between the serving cell and any candidate cell on which UE is required to perform L1-RSRP measurement on the same frequency layer is not larger than CP length of the cell on the frequency layer. |  |

Table 9.14.5.1-2: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC*NLayer) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC) *NLayer) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX*NLayer |
| NOTE 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is the configured periodicity for reporting. NLayer = the number of intra-frequency layers configured for L1-RSRP measurement with LTM-CSI-ResourceConfig-r18 + the number of inter-frequency layers without measurement gaps which are configured for L1-RSRP measurement with LTM-CSI-ResourceConfig-r18. NOTE 2: K = 1.5. NOTE 3: UE shall at least measure neighbour cells which have at least one TCI state on the LTM candidate cell active TCI state list within UE capability and additional cells to measure is up to UE implementation. |  |

It can be seen that the requirements are defined differently with different assumption, e.g. the whether to differentiate the relationship between RTD and CP, whether to scale with the number of intra-frequency layers. The L1 measurement delay may be even longer than L3 measurement delay. And it is also possible that UE supporting L3 measurement may not support L1 measurement for mobility.

Treating L3 based mobility and LTM separately results in a lot issues. The requirements are complex. UE behaviour is inconsistent. And additional UE power consumption is introduced due to redundant measurement. To avoid these issue, it is proposed to consider unified measurement framework for L3 measurement and L1 measurement for mobility.

According to the discussion in last meeting, some companies think this issue is pending on RAN1/2 design. We have different views. Our motivation is to unify UE measurement behaviour, not about configuration or RS design. It is expected that UE measurement behaviour are unified no matter the measurement purpose, e.g. UE perform RS measurement with fine beam, and the measured results can be further used to derive the target results requested by the measurement purpose.

Proposal 12:  for L3 measurement and L1 measurement for mobility, from the perspective of UE measurement, it is proposed to consider unified measurement framework and define unified measurement requirements.

RLM, BFD, CBD are necessary functionalities to guarantee the radio link quality. However, the RLM, BFD, CBD are treated independently in NR.

In detail, RLM is performed on RLM-RS resources. On each RLM-RS resource, the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout and Qin. The threshold Qout is defined as the level at which the downlink radio link cannot be reliably received and the out-of-sync block error rate (BLERout) is 10%. The threshold Qin is defined as the level at which the downlink radio link quality can be received with significantly higher reliability than at Qout and the in-sync block error rate (BLERin) is 2%. Both thresholds are derived based on the hypothetical PDCCH transmission parameters. The evaluation period for in-sync is based on 5 samples, and the evaluation period for out-of-sync is based on 10 samples.

BFD resources are within set q0. On each RS resource configuration in the set , the UE shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of accessing downlink radio link quality of the serving cell beams. The threshold Qout_LR is defined as the level at which the downlink radio level link of a given resource configuration on the set cannot be reliably received and shall correspond to the BLERout = 10% block error rate of a hypothetical PDCCH transmission. The evaluation period for BFD is based on 5 samples

CBD resource are in the set q1. Upon request the UE shall deliver configuration indexes from the set to higher layers, and the corresponding L1-RSRP measurement provided that the measured L1-RSRP is equal to or better than the threshold Qin_LR, which is indicated by higher layer parameter rsrp-ThresholdSSB. The evaluation period for CBD is based on 3 samples.

It can be seen that RLM, BFD, CBD are handled independently, even though all of them are related with radio link quality. As a result, the requirements are complex, UE behaviour is inconsistent, and introduce additional UE power consumption due to redundant measurement. To avoid these issue, it is proposed to consider unified measurement framework for RLM\BFD\CBD.

Proposal 13: from the perspective of UE measurement, it is proposed to consider unified measurement framework and define unified requirements for RLM, BFD, CBD.

## 2.3 Mobility related RRM

Issue 6: Mobility related RRM Issue 6: Mobility related RRM [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to start from mobility related RRM solutions with less RAN1/2-dependency. - RAN4 to identify which of the following topics can be starts directly in RAN4: - Latency and/or interruption reduction for mobility through RAN4-defined components (Apple, MTK(interruption only), QC, LGE, ZTE, HW, vivo, Ericsson, Nokia, CMCC, Samsung) - discuss RRM part reduction during mobility, e.g., L1/L3 measurement, beam sweeping and etc. - discuss the scenarios/conditions for such reduction (known, unknown, or other status) - discuss NW controlled and UE initiated L1/L3 measurement report - Solutions for longer SSB periodicity in mobility (MTK, Samsung, Ericsson) - Early RRC decoding, and/or, DL/UL sync, and/or, early T/F tracking for mobility (MTK, Nokia) - Virtual RRM UE group (Apple, ZTE) - Unified measurement and mobility framework (QC) - E.g., based on 5G LTM - End-to-end handover latency target (QC) - RAN4 to study the practically achievable end-to-end handover latency target, taking into account user-plane data forwarding latency, to better align handover requirements with practical effectiveness. - Unified UE measurement requirements across cell changes (Nokia)

In 5G, there are multiple evolution to UE mobility in different release. Rel-15 is based on the baseline handover. Conditional handover and Dual Active Protocol Stack (DAPS) are introduced in Rel-16. Inter-Cell Beam Management (ICBM) is specified in Rel-17. Requirements for L1/L2 triggered mobility (LTM) are defined in Rel-18 and the requirements for enhanced LTM are defined in Rel-19.

Taking HO as an example, handover delay are defined. The main component of handover delay is interruption time, during which UE cannot perform data reception and/or reception. No matter in HO, CHO, or LTM, interruption cannot be avoided. For 6G, we may aim for a solution to minimize the handover delay including interruption time. Normally, handover interruption comprises the time required to search the target cell, time for fine time tracking, time for UE UE processing. One way to reduce interruption delay is to discuss whether the components of interruption can be reduced and whether the time for each components can be reduced.

Proposal 14: it is proposed to study how to reduce the latency and/or interruption for mobility.

## 2.4 RRM related energy efficiency

Issue 7: RRM related energy efficiency [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to identify candidate topics can be studied in RAN4 directly: - Network energy saving: - RRM for new SSB design(e.g., SSB periodicity extension, OD-SSB/OD-SIB1) (vivo, Ericsson, Nokia, ZTE) - SSB-less based RRM (Ericsson, Nokia, ZTE) - UE power saving: - UE type/state based RRM relaxation (Apple, CMCC, Ericsson(scalable set of measurement requirement), Nokia) - LR based solutions for UE power saving (Sony, vivo, Ericsson) - DRX/eDRX based measurement (Ericsson) - Energy sensing based RRM strategy (CATT) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other energy efficiency related RRM topics if there are sufficient conclusions from other WGs - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other energy efficiency related RRM topics once there are sufficient conclusions from other WGs.

RAN1 achieved some agreements for energy efficiency topic, some of them which have impact on RAN4 RRM are identified, we copy them below for information

Agreement Study and evaluate NW energy savings and the impact on UE performance and user experience with respect to 20ms and longer periodicities of sync signal(s) at least for initial access with the following consideration, but not limited to: BS assumptions: - Cell-common signaling (e.g., sync signal(s), broadcast PDCCH, SIB-1, SIB, paging, PRACH), e.g., - Clustered provisioning of different cell-common signaling, - On-demand provisioning of different cell-common signaling, - UE-specific signaling (for low, light, medium loads), e.g., - Clustered provisioning with cell-common signaling, - Unclustered provisioning with cell-common signaling, UE impact: - Cell search complexity and latency, including frequency search latency, - UE Power consumption, - Sync signal detection, coverage and tracking performance, - RRM, mobility, - Beam management, - Other properties are not precluded, - Improvements to address identified impact, e.g., - Additional sync signal needs, - Adaptation of sync signal transmission periodicity, - Sparser synch raster. Agreement Study and evaluate on-demand and/or periodic SIB-1 transmission with respect to - NW energy savings potential and UE power consumption impact, - SIB-1 acquisition delay, - NW and UE complexity, - Coverage, - Applicable deployment scenarios, e.g.,: - Standalone cell/carrier, - Multiple TRPs/cells/carriers. Agreement Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects: - Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other) - Measurements and/or synchronization. - System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS. - RRC states - Other functionalities Agreement Study and evaluate on-demand sync signal(s) mechanisms for 6GR energy efficiency, considering, e.g.,: - On-demand sync signal(s) for single cell/carrier, multi-carrier/cell, multi-TRP, - Network-triggered and UE-triggered on-demand sync signal(s), - Idle and/or connected modes, - Other mechanisms/aspects/signals/channels are not precluded. Agreement Study and evaluate multi-carrier/cells/TRPs mechanisms for 6GR NES, considering, e.g.,: - Sync signal-less carriers/cells/TRPs for at least intra-band and collocated inter-band multi-carrier/cell/TRPs, including potential extensions to additional deployments and scenarios, - RRC states, - UE energy consumption and complexity, Other mechanisms/aspects/signals/channels are not precluded.

Sync signals plays an critical role on measurement, synchronization, timing and system information acquisition. Therefore, the design of sync signal is highly related to RRM requirement.

As listed above, several sync signal mechanisms are currently under study in RAN1, including the 20ms and longer periodicities of sync signal(s), on-demand sync signal(s), signal-less carriers/cells/TRPs and on-demand SIB-1, these can also be early involved in RAN4 study scope. These mechanisms were initially introduced in 5G stage with scenario limitation, as optional features on top of 5G baseline design. The work accomplished in 5G can be the starting point for 6G-Day1 requirement discussion, with necessary revisions and updates based on RAN1’s further design on scenarios, procedures and signals.

- Proposal 15: For network energy efficiency, the 20ms and longer periodicities of sync signal(s), on-demand sync signal(s), signal-less carriers/cells/TRPs, on-demand SIB-1 mechanisms can be involved in RAN4 study scope, and the corresponding work in 5G can be the starting point.

From the perspective of UE energy efficiency, as mentioned above, DL-WUS of OFDM based sequence and corresponding mechanisms are being studied in RAN1 Group. Similarly, in 5G, the feature of LP-WUS/LP-SS(both OOK based and OFDM based sequence) has been introduced, and RAN4 defined the corresponding measurement requirement for both MR and LR for this feature. This existing framework can be used as the starting point with necessary revision and update based on RAN1’s further design on scenarios, procedures and signals.

Additionally, from RAN4 RRM standpoint, the L1/L3 measurement occupy the main energy consumption. From initial 5G design, the measurement requirements are bounded with DRX cycle configuration, which can help UE unified behavior in DRX ON/OFF period, target to energy efficiency purpose. Some further power saving features has been introduced in subsequent releases, including the measurement relaxation in RRC IDLE/Inactive, RLM/BFD measurement relaxation, LP-WUR assistance and so on. All aim at measurement relax under good link quality and low mobility state. For 6G Day1, we think all lessons from NR can be considered for measurement design of energy efficiency purpose, we list our initial consideration below. However, most of them need the PHY and higher layer support, RAN4 need to wait the RAN1/RAN2 progress and then refine our design strategy.

- Whether to continue the DRX cycle bounded measurement requirement in 6G

- Whether to integrate the link quality and mobility state with measurement requirement in 6G Day1

- Omitting repeated measurement behaviors. Unified L1/L3 measurement as we discussed in Clause 2.2 can be the starting point.

- Proposal 16: For UE energy efficiency, RAN4 need to wait the RAN1/RAN2 progress and then refine measurement design strategy, our initial thinking is as below:

- DL-WUS mechanism can be involved in RAN4 study scope, the LP-WUR related work in 5G can be the starting point

- Whether to continue the DRX cycle bounded measurement requirement in 6G

- Whether to integrate the link quality and mobility state with measurement requirement in 6G Day1

- Merge repeated measurement behaviors. Unified L1/L3 measurement as we discussed in Clause 2.2 can be the starting point.

## 2.5 Spectrum aggregation and CA related RRM

Issue 8: Spectrum aggregation and CA related RRM Issue 8: Spectrum aggregation and CA related RRM [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to identify which of the following topics can be starts directly in RAN4 RRM with less RAN1/2 and RAN4 RF session dependency: - SCell activation/deactivation, deactivated SCell measurement, fast carrier setup based on 6G UE implementations (MTK(SCell activation), QC, vivo(activation), Ericsson, Nokia) - RRM conditions and requirements for Single Cell Multi-Carriers (MTK, vivo, Samsung, OPPO) - RRM impacts of DL and UL decoupling (Samsung, Ericsson) - Carrier switch enhancements for UL and DL (MTK) - RRM impacts of realistic SCS for spectrum (Samsung) - relaxation of the requirement on timing alignment between carriers (Ericsson) - RRM for MRSS (Samsung)

For 6G, to realize efficient and flexible IDLE/INACTIVE mode multiple carriers operation, and to reduce cell management overhead of existing CA framework, there could be a single cell with multiple carriers as a hyper cell, which can. In detail, for hyper cell (i.e. single cell with multiple carrier), System information can be broadcast on one of the multiple carriers, i.e., so called anchor carrier. Both anchor and non-anchor carrier can be selected by UE for initial access and data transmission. Flexible intra-cell multiple carriers switching, e.g. L1/L2 based multiple carriers activation and deactivation, can be considered for connected mode. The RRM impact due to hyper cell (i.e. single cell with multiple carrier) can be studied if the RAN1/2 agreed to introduce hyper cell (i.e. single cell with multiple carrier) solution.

Proposal 17: it is proposed to study RRM impact due to hyper cell (i.e. single cell with multiple carrier).

## 2.6 NTN related RRM

From 6GR SID, the harmonized 6G Radio design for TN and NTN is the one of the baseline design principles of 6G technology framework.

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

RAN1 achieved following related agreement, the specific discussion on NTN agenda is still pending.

Agreement

- For harmonized 6GR design for TN and NTN, RAN1 studies to identify the technical aspects affected by NTN characteristics, as well as lessons learned from NR/IoT NTN

In 5G, the RRM requirements for TN network and SAN were defined separately. Considering the large propagation delay difference and doppler difference caused by NTN network deployment, the main UE measurement difference between TN and NTN at least including the following:

- For NTN, One SMTC offset can not cover inter-frequency cells due to large propagation delay

- For NTN, on a single frequency carrier, limited number of SMTC that UE can process in parallel

- For NTN, limited number of MG that UE can process in parallel

- For NTN, within one SMTC, limited number of measurement on NGSO satellites that UE can process in parallel

- For NTN, on a single carrier, limited number of target NGSO satellites that UE can monitor

- For NTN, UE normal operation on serving cell in parallel with measurement on difference NGSO satellite depends on UE capability.

Towards 6G, the baseline UE measurement capability shall be reconsidered, which shall be further enhanced from NR assumption. From our perspective, at least the UE capability of parallelSMTC-r17, parallelMeasurementGap-r17, parallelMeasurementWithoutRestriction-r17 need to be inherited as mandatory to 6G.

Regarding the harmonized 6G Radio design for TN and NTN, from RAN4 RRM perspective, if both TN measurement and NTN measurement are configured to UE, the measurement priority shall be under network control. Considering the continuous and extensive deployment of TN network, the measurement on TN carrier shall have higher priority as the default assumption.

- Proposal 18: Towards 6G, the baseline UE measurement capability for NTN shall be reconsidered, at least the UE capability of parallelSMTC-r17, parallelMeasurementGap-r17, parallelMeasurementWithoutRestriction-r17 need to be inherited as mandatory to 6G NTN.

Proposal 19: Within the harmonized 6G Radio design for TN and NTN, if both TN measurement and NTN measurement are configured to UE, the measurement priority shall be under network control, the measurement on TN carrier shall have higher priority as the default assumption.

## 2.7 Other PHY signal/channel/procedure related RRM

CGI (Cell Global Identity) is a basic function for ANR (Automatic Neighbor Relation), which is helpful in network planning and optimization. When requested by the network for the purpose of reportCGI, the UE shall identify and report the CGI of a NR target cell. The UE may make autonomous gaps in both downlink reception and uplink transmission for receiving MIB and SIB1 message. In 5G, CGI reading related requirements, including CGI identification delay and CGI reporting delay, are not defined in Rel-15. From the perspective of operator, CGI reading is a very important feature. It is proposed to support CGI reading from 6G day-1.

Proposal 20: it is proposed to support CGI reading and define RAN4 requirements from 6G day-1.

- Conclusion

This contribution provides consideration on RRM for 6GR. The proposals are:

Proposal 1: it is proposed that the feature with market demand are supported from 6G day-one.

Proposal 2: it is proposed that measurement without gaps are supported as mandatory from 6G day-one.

Proposal 3: To support measurement without gaps for 6GR, 5G MG features, e.g. NCSG or NeedForGap can be used as starting point.

Proposal 4: for 6GR, it is proposed to discuss whether interruption are still needed for measurement without gap. Could we assume no interruption for gap-less measurement as a basic feature.

Proposal 5: it is proposed to support concurrent measurement gaps from 6G day-1.

Proposal 6: it is proposed to support parallel measurement for the colliding measurement gaps.

Proposal 7: it is proposed to consider MG skipping/canceling from 6G day-1, and MG skipping/canceling is a generic approach, not limit to XR.

Proposal 8: for measurement gap design for 6GR, reducing MG patterns and increasing number of mandatory MG patterns can be discussed together.

Proposal 9: it is proposed to consider reduced RF re-tuning time when discuss the measurement gap pattern for 6GR.

Proposal 10: to reduce measurement delay, it is proposed to study the increased number of searchers in order to enable simultaneous measurement.

Observation 1: in NR, with the definition on intra-frequency/inter-frequency measurement, measurement requirements are categorized into 4 cases: intra-frequency measurement without gap, intra-frequency measurement with gap, inter-frequency measurement without gap, inter-frequency measurement with gap, which results in complex specification and increased RAN4 workload.

Proposal 11: it is proposed to discuss whether to have the definition on intra-frequency/ inter-frequency measurement in 6GR. Suggest to discuss whether following consideration is feasible

- Option 1: no definition on intra-frequency/ inter-frequency measurement. RRM requirements are categorized as measurement with gap and measurement without gap

Proposal 12:  for L3 measurement and L1 measurement for mobility, from the perspective of UE measurement, it is proposed to consider unified measurement framework and define unified measurement requirements.

Proposal 13: from the perspective of UE measurement, it is proposed to consider unified measurement framework and define unified requirements for RLM, BFD, CBD.

Proposal 14: it is proposed to study how to reduce the latency and/or interruption for mobility.

- Proposal 15: For network energy efficiency, the 20ms and longer periodicities of sync signal(s), on-demand sync signal(s), signal-less carriers/cells/TRPs, on-demand SIB-1 mechanisms can be involved in RAN4 study scope, and the corresponding work in 5G can be the starting point.

- Proposal 16: For UE energy efficiency, RAN4 need to wait the RAN1/RAN2 progress and then refine measurement design strategy, our initial thinking is as below:

- DL-WUS mechanism can be involved in RAN4 study scope, the LP-WUR related work in 5G can be the starting point

- Whether to continue the DRX cycle bounded measurement requirement in 6G

- Whether to integrate the link quality and mobility state with measurement requirement in 6G Day1

- Merge repeated measurement behaviors. Unified L1/L3 measurement as we discussed in Clause 2.2 can be the starting point.

Proposal 17: it is proposed to study RRM impact due to hyper cell (i.e. single cell with multiple carrier).

Proposal 18: Towards 6G, the baseline UE measurement capability for NTN shall be reconsidered, at least the UE capability of parallelSMTC-r17, parallelMeasurementGap-r17, parallelMeasurementWithoutRestriction-r17 need to be inherited as mandatory to 6G NTN.

Proposal 19: Within the harmonized 6G Radio design for TN and NTN, if both TN measurement and NTN measurement are configured to UE, the measurement priority shall be under network control, the measurement on TN carrier shall have higher priority as the default assumption.

Proposal 20: it is proposed to support CGI reading and define RAN4 requirements from 6G day-1.

- Reference

- RP-251881, New SID: Study on 6G Radio

- R4-2514644, WF on 6G RRM, Apple

- R4-2511652, RAN4 6G Plan and Meeting Arrangement