# 3GPP TSG-RAN WG4 Meeting #117                              R4-2520449

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	7.9.2

Source: 	CMCC

Title: 			Discussion on general aspects for enhancement on AI/ML for NR air interface

Document for:	Discussion

- Introduction

In RAN #108 meeting, the new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements was approved [1]. The objectives for RAN4 are duplicated as following.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In last meeting, there was discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, and a WF was approved [2]. This contribution provides further discussion on this topic.

- Discussion

The steps to develop the test decoder and reference encoder are agreed as following:

- Step 1:  Create dataset

  - Step 1-1: Align on configurations for mixed dataset construction, taking factors such as scalability over Tx port number and subband size and payload size into account

  - Step 1-2 Decide how align and on which metrics can be used for evaluating dataset alignment, if any

  - Step 1-3 Companies create datasets

  - Step 1-4 Identify which company datasets to include in the mixed dataset(s)

  - Step 1-5 Create new dataset(s)

- Step 2: Align on the model backbone, model hyperparameters and the scalability solution.

  - Step 2-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution.

    - This can be in parallel with step 1.

    - Transformer is agreed as baseline for backbone

    - Same backbone for test decoder and reference encoder

  - Step 2-2: Preliminary selection of structures, taking into account complexity

  - [Step 2-2a Align on training parameters]

  - Step 2-3: Agree on metrics and steps for evaluation

  - Step 2-4: Check on performance alignment -> see simulation results from contributing companies.

  - Step 2-5: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed.

  - Step 2-6: Select one or more model structures considering complexity and robustness and decide how many models.

  - Step 2-7: Select one or more scalability solutions. This step can be done in parallel with Steps 2-2 to 2-4.

- Step 3: Specify the model for the following work in WID. Where and how to specify may need further discussion.

Note: At this stage, models using the agreed structures, dataset, scalability have already been created by most companies

  - Step 3-1 Decide how to select from the trained model(s) from each company

  - Step 3-2: Freeze model(s)

  - Step 3-3: Agree on how and where to capture the models, the dataset and relevant information

According to the discussion in last meeting, one of the open issues is that RAN4 will discuss how to introduce the new mixed dataset for model structure evaluation purpose, by taking into consideration of the scalability and/or various configurations.

It was agreed that unless the significant issues are identified, RAN4 is going to only discuss the scalability issues and options, which have been agreed in RAN1. For the model structure feasibility study of Case 0 with precoding matrix in spatial-frequency domain as input, following alternatives on scaling over token dimension (e.g., number of subbands, number of Tx ports, or combination thereof), feature dimension (e.g., number of Tx ports, number of subbands, or combination thereof), and payload configurations are studied in RAN1.

Agreements in RAN1#120 meeting Agreement For model structure scalability study for temporal domain Case 0, - For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters - Notes - Other Alternatives are not precluded. - Different Alternatives may be used in combination. - Same/similar approach is applied at the decoder side. - Evaluations to consider: - Case 1 (scalable structure): Scalable model structure described above - Using model structure as indicated in above diagram with fixed hyperparameters, companies may train a single parameter set or different parameter sets across different {number of Tx ports, CSI feedback payload size, bandwidth} assuming a common model structure. - To report whether a single parameter set or different parameter sets were used across different {number of Tx ports, CSI feedback payload size, bandwidth}. (e.g., single parameter set across different payload sizes and bandwidths, different parameter set across different number of Tx ports) - Case 2 (dedicated structure): Using model structure as indicated in above diagram with different hyperparameters, where the input and the output related hyperparameters are chosen optimally corresponding to each specific {number of Tx ports, CSI feedback payload size, bandwidth} without scalability considerations. - Different parameter sets are trained across different number of Tx ports, CSI feedback payload sizes, and bandwidths. - For each scalable model structure choice, to evaluate the SGCS performance of the non-AI/ML benchmark (e.g., Rel-16 eType2), Case 1, and Case 2, for each of {number of Tx ports, CSI feedback payload size, bandwidth}, and report the average gain (%) in SGCS of Case 1 and Case 2 over the non-AI/ML benchmark, as well as the loss (%) in the average gain of Case 1 w.r.t. Case 2. The average is performed by first calculating the SGCS gain (%) for each {number of Tx ports, CSI feedback payload size, bandwidth} and then averaging the SGCS gain (%) values over {number of Tx ports, CSI feedback payload size, bandwidth}. Agreement For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values. - Number of Tx ports - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports) - [N1, N2, P] = [4,4,2] (32 CSI-RS ports) - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports) - CSI feedback payload size - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z. - Bandwidth & subband size - All subbands - A subset of all the subbands.

Proposal 1: for scalability, it is proposed to follow RAN1 study to consider number of subbands, number of Tx ports, and CSI feedback payload size.

In Rel-19 evaluation about the feasibility of test options, dataset are created based on agreed system level simulation assumption [3], which are duplicated as following.

System-level simulation parameters [3]

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

System-level simulation parameters used in Rel-19 to create dataset can be used baseline, and further update on number of subbands, number of Tx ports, and CSI feedback payload size to consider the scalability.

Proposal 2: for dataset creation, it is proposed that the system-level simulation parameters used in Rel-19 to create dataset can be used baseline, and further update on number of subbands, number of Tx ports to consider the scalability impact.

- Number of Tx ports

  - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports)

  - [N1, N2, P] = [4,4,2] (32 CSI-RS ports)

  - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports)

- Bandwidth & subband size

  - All subbands

  - A subset of all the subbands.

- Conclusion

This contribution provides discussion on inter-vendor training collaboration and interoperability for two-sided AI/ML models. The proposals are:

Proposal 1: for scalability, it is proposed to follow RAN1 study to consider number of subbands, number of Tx ports, and CSI feedback payload size.

Proposal 2: for dataset creation, it is proposed that the system-level simulation parameters used in Rel-19 to create dataset can be used baseline, and further update on number of subbands, number of Tx ports to consider the scalability impact.

- Number of Tx ports

  - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports)

  - [N1, N2, P] = [4,4,2] (32 CSI-RS ports)

  - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports)

- Bandwidth & subband size

  - All subbands

  - A subset of all the subbands.

- Reference

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

- R4-2514614, WF on R20 NR_AIML_air_Ph2, Ericsson

- R4-2401570, WF on AI/ML, Qualcomm Incorporated

- R4-2503953, WF on AI/ML air interface study

- R4-2511849, WF on [116][132] NR_AIML_air_part2, Ericsson