3GPP TSG-RAN WG4 Meeting # 117		R4-2520586

Dallas, TX, USA, November 17 – 21, 2025

Agenda item:	4.8.1

Source:	Apple

Title:	DL RMC requirements for LB-LB CA via switching with 40mSec periodicity

WI/SI:	NR_LBCA_Sw-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

In the previous RAN4 meeting (Meeting #116-bis, Prague, Czech Republic, October 13-17, 2025), three proposals on RMC definition for Rel-19 low NR band carrier aggregation via switching (NR_LBCA_Sw) were summitted. Discussions were held on the switching pattern and the switching pattern periodicity that should be used to define the RMC. A common understanding from most companies is that a periodicity of 40mSec could be used.

In this contribution paper, we modify our previously proposed RMC definition based on periodicity of 10mS to create a new one with 40mSec periodicity. 8 switching events within the 40mSec are assumed.

# Discussion

  - RMCs for REFSENS performance measurements

In Rel-19, low NR band aggregation via switching was introduced to address challenges that exist for LB-LB inter-band carrier aggregation when traditional aggregation techniques (single antenna with multiplexers) are used. Only CA combinations involving an SDL band and a FDD band were considered. RAN4 has recently completed the definition of the core UE RF requirements. For the UE RF performance evaluation phase, the introduction of new DL RMCs was considered in the previous meeting based on contributions from three companies. No agreement was reached on the DL RMC to be used, and the debate was postponed to the upcoming meetings.

In the previous meeting, our proposal on DL RMC was based on 10mSec of switching priority as shown below.

DL RMC for evaluating the UE DL performance for low NR band aggregation via switching:

This was based on the fact that all the existing RMCs in the 3GPP specifications are based on 1 NR frame.

For this meeting, we will define a new RMC based on 40mSec periodicity. To do this, we will assume that the switching  pattern for 40mSec can be derived by repeating 4 times the switching pattern used for 10mSec.

Observation: The switching pattern with 40mSec periodicity can be obtained by repeating 4 times the 10mSec-based pattern.

To calculate the maximum throughput for the reference channel, we need to determine the information bit payload per slot by first calculating the transport block size (TBS) based on the configurations for the reference channel shown in Tables A.3.2.2-1 and in Table A.3.1-1 of TS38.101-1, as well as the procedure highlighted in clause 5.1.3.2 of the TS38.214 specifications.

The algorithm on calculating the TBS is highlighted below, taken from clause 5.1.3.2 of the TS38.214 specifications:

1)	The UE shall first determine the number of REs (NRE) NRE)within the slot.

-	A UE first determines the number of REs allocated for PDSCH within a PRB () by, where is the number of subcarriers in a physical resource block, Nsymbslot is the number of symbols of the PDSCH allocation within the slot, NDMRSPRB is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 1_1, 1_2 or 1_3 or as described for format 1_0 in Clause 5.1.6.2, and

-	A UE determines the total number of REs allocated for PDSCH ()NRE) by NRE= NRE'* nPRB, where nPRB is the total number of allocated PRBs for the UE.

2)	Unquantized intermediate variable (Ninfo) TBStemp) is obtained by TBStemp= NRE*R*Qm*ʋ.

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-1 find the closest TBS that is not less than .

Using the above algorithm and assuming a CBW of 10MHz as an example and a switching period of 140uSec ( 2 OFDM symbols with 15kHz SCS), we can calculate the maximum throughput for 10nSec as follows:

- For slots 2, 3, 9, 12, 13, 14, and 19, the information bit payload remains the same because those slots are not affected by the switching period: 3368 Bits

- For slot 4 which has a switching period of 140uSec (2 OFDM symbols) the payload is calculated to be 2600 Bits when only 12 OFDM symbols are assumed instead of 14 symbols.

- The total payload for the whole frame will be: 7 x 3368 + 2600 = 26176 Bits

- This will result into a maximum throughput of 26176/10mSec = 2.6176 Mbps for one frame

It can be observed that there is a throughput loss of (2.6944-2.6176) = 0.0768 Mbps or 2.8% when LB CA via switching is implemented compared to the case without switching.

Proposal: We recommend using the RMC defined in the table below for evaluating the receiver performance for Rel-19 LB-LB CA vias switching.

Fixed reference channel for receiver requirements configured for low NR band inter-band carrier aggregation via switching [supportedLowBandSwitching-r19] (SCS 15 kHz, FDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0,1,5,6,7,8 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,9 | Bits | 1672 | 3368 | 5120 | 6912 | 8712 |
| For Slot 4 | Bits | 1064 | 2600 | 4064 | 5576 | 7088 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0,1,5,6,7,8 | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,9 | CBs | 1 | 1 | 1 | 1 | 2 |
| For Slot 4 | CBs | 1 | 1 | 1 | 1 | 2 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |
| For Slots 0,1,5,6,7,8 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,9 | Bits | 5400 | 11232 | 17064 | 22896 | 28728 |
| For Slot 4 | Bits | 4200 | 8736 | 13272 | 17808 | 22344 |
| Max. Throughput averaged over 1 frame | Mbps | 0.6080 | 1.2704 | 1.9424 | 2.6312 | 3.3224 |

# Conclusions

Observation: The switching pattern with 40mSec periodicity can be obtained by repeating 4 times the 10mSec-based pattern.

Proposal: We recommend using the RMC defined in the table below for evaluating the receiver performance for Rel-19 LB-LB CA vias switching.

Fixed reference channel for receiver requirements configured for low NR band inter-band carrier aggregation via switching [supportedLowBandSwitching-r19] (SCS 15 kHz, FDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0,1,5,6,7,8 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,9 | Bits | 1672 | 3368 | 5120 | 6912 | 8712 |
| For Slot 4 | Bits | 1064 | 2600 | 4064 | 5576 | 7088 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0,1,5,6,7,8 | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,9 | CBs | 1 | 1 | 1 | 1 | 2 |
| For Slot 4 | CBs | 1 | 1 | 1 | 1 | 2 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |
| For Slots 0,1,5,6,7,8 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,9 | Bits | 5400 | 11232 | 17064 | 22896 | 28728 |
| For Slot 4 | Bits | 4200 | 8736 | 13272 | 17808 | 22344 |
| Max. Throughput averaged over 1 frame | Mbps | 0.6080 | 1.2704 | 1.9424 | 2.6312 | 3.3224 |

# 4	References

R4-2513854, “On RMCs definition for Rel-19 low NR band CA via switching”, by Apple,  3GPP TSG RAN Prague, Czech Republic, October 13 – 17, 2025