3GPP TSG RAN WG4 #117 	                         			   		   R4-2520610

Dallas, Tx, USA, Nov 17th – 21st, 2025

Agenda Item:	8.2.5

Source:	InterDigital France R&D, SAS

Title:	6G Device Type analysis

Document for:	Discussion and Decision

- Introduction

RAN approved a new SID on 6G Radio at RAN#108 [1]. As indicated in the SID, the development of 6G Radio is needed to meet a range of future use cases requiring extreme performance, flexibility and scalability. RAN also initiated study on RAN requirements [2][3] to define specific quantitative targets with respect to KPI’s such as peak data rate, spectral efficiency, user experienced data rate, area capacity, user and control plane latency, reliability, mobility, and energy efficiency. Such targets are expected to surpass those of 5G in most or all cases.

The SID also emphasizes some key requirements and principles that should guide the design of the new system from the start. At a high-level, the requirements can be summarized as follows:

- Support new and expanded usage scenarios

- Enhance performance from user and system perspective

- CAPEX / OPEX reduction, including smooth migration from 5G

To achieve these requirements, 3GPP is to design 6GR with the following guiding principles:

- Focus on energy efficiency for NW and device

- Support enhanced spectral efficiency

- Enable wider bandwidth and improve support for diverse spectrum allocations

- Enable scalable and forward compatible design for diverse device types

- Common radio design to meet vertical needs (mobility broadband high priority)

- Enhanced overall coverage (UL), enabling 7 GHz operation with same site grid as 3.5 GHz

- Harmonized TN/NTN

- Avoid excessive configurations/options/capabilities

This contribution presents our views on device type requirements of the future 6GR along with proposals on assumptions and aspects that may involve RAN4.

- General aspects of 6G Device Types discussion

In the last meeting, Device Types topic discussions lead to a WF within the 6G system parameters agenda in [4] that we listed here for convenience:

- Topic #7: Device types - Agreement - The following aspects could be further studied - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range. - Study on the NW impact should also be considered - Study how to better support variety devices from RAN4 perspective - E.g. differentiate devices, use cases, features - Other aspects are not precluded - Collaborate on the framework with other WGs: - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

  - Device types and their baseline features

The traditional devices listed in the WF are typical for the 5G and 5G-A as well:

      - Smartphone

      - MTC

      - IoT

      - Wearable

      - FWA

The predominant impact is expected from the Smartphone. However, since we must support all these types of devices in the future and probably other form factors to come as well, a kind of baseline for each device type may be needed to simplify several RF and RRM requirements with the aim of reducing signalling overhead and general operation.

We believe that it is essential for the RAN4 SI progress to have a better understanding of future UE capabilities so a proper analysis can be conducted in RAN1 and RAN plenary.

  - 6G Device types and their characteristics in NR and LTE

The device types may have different requirements based on their typical usage scenario. For example, an FWA device has different mobility requirements than a Smartphone, as these devices are of a very different usage. Certainly, their RF requirements are also different in terms of TX/Rx antennas. Thus, we believe that defining the right RF and RRM characteristics for each device is required as an initial step to find commonalities and differences.

Moreover, from the network point of view, certain requirements are not necessarily coming from the commonalities, but rather from the extreme cases and the diversity of the devices it must support.

As the traditional 5G use case will expand in 6G, we believe 6G use cases have a strong foundation in the 5G. This may be re-enforced due to MRSS compatibility requirements.

It will be a good practice to start the 6G device type analysis from the current and past 5G use cases and supported form factors that were derived from the main usage scenarios:

      - eMBB

      - mMTC

      - URLLC

These three usage scenarios will be marked by an evolution into 6G as follows:

      - eMBB will become optimized and improved for Immersive Communications

      - mMTC will get probably an order of magnitude in capacity

      - URLLC will become HRLLC

For the Immersive Communication usage scenario, the UE and network will have to support a variety of traffic loads with specific latency and jitter requirements. These requirements are affecting RAN4 RF requirements just in terms of possible channel bandwidth increase and symbol length.

The mMTC part for RAN4 is just about the minimum channel bandwidth and their support by the network. HRLLC will have a similar impact as the 5G case.

Observation 1: 5G usage scenarios and their evolution into 6G are a good starting point for device type capabilities analysis.

In Table 1 below we present a few characteristics of the above listed devices from 4G and 5G:

Table 1. 5G devices capabilities and use cases

Proposal 1: Consider the 5G and 5GA requirements as a starting point for 6G RAN4 related device characterization.

  - RF characteristics

From the RF requirements standpoint, the UE capabilities may be impacted by the maximum and minimum supported channel bandwidth in 6G. The current proposals for maximum channel bandwidth are pointing to a 200MHz channel for FR1 and 400MHz for FR2. Obviously, this applies only to Smartphone and FWA device types.

Proposal 2: The maximum supported channel bandwidth apply to Smartphone and FWA devices.

The minimum channel bandwidth of 3MHz is also limited to a certain type of device in 5G. In R18, 3GPP specified a new 3 MHz channel bandwidth to support operation in bands used for rail communications.

Since 6GR study is to target a scalable and forward compatible design for diverse device types, the decision on minimum bandwidth necessary for initial access needs to consider the needs of the most basic devices (e.g. LPWA) from the start. Setting a lower minimum bandwidth allows supporting lower complexity devices. However, the minimum bandwidth significantly impacts the design of basic signals (e.g. sync signals, PBCH) and associated functionality (e.g. cell search, idle mode operation) which also needs to be considered. A larger bandwidth results in higher frequency diversity and possibly faster acquisition. For 6GR, a minimum bandwidth of 3 MHz should satisfy future market needs for low-power devices while still enabling high performance for advanced devices.

Proposal 3: Support 3MHz minimum channel bandwidth for all device types.

Similarly, CA capability is limited to Smartphone, FWA and optionally to RedCap/Wearable devices.

Proposal 4: The CA capability apply to Smartphone and FWA devices and optionally to RedCap.

The power classes are device specific, and they may stay the same unless new device types are developed. However, one issue that must be addressed in the 6G is the TxD transparency versus UL MIMO related power class ambiguity.

Proposal 5: RAN4 to address the TxD versus UL MIMO power class ambiguity during 6G development.

  - RRM characteristics

On the RRM side, one problem during 5G was the proliferation of the gap types for the measurements and the multiple types of interruptions. One target for the 6G would be to improve the gap management and simplify its operation. However, this is mostly a problem for the devices like Smartphones, RedCap and Wearables that support handover and support all mobility classes. There are essentially two ways improve measurement gap management:

      - On intra-frequency: Improve beam sweeping/measurements

      - On inter-frequency: Have an enough minimum of RF chains

Obviously, on the beam sweeping improvement, the design decisions are taken in RAN1. But for the minimum RF chains for devices like Smartphone and RedCap as a baseline, may be influenced by RAN4. Since these devices may support carrier aggregation and most likely they will, two RF chains can be considered for minimum requirement.

Proposal 6: Two RF chains are minimum requirement for 6G Smartphones and RedCap devices.

On the interruptions side, there are many factors that are involved, and some are directly related to the physical layer design and thus at least partially out of RAN4 control. Still RAN4 may make remarks and send LS-es to RAN1 concerning design consequences on interruptions.

- Conclusion

This contribution discussed the 6GR devices minimum requirements from the perspective of RAN4. Based on the discussions, we propose the following:

Observation 1: 5G usage scenarios and their evolution into 6G are a good starting point for device type capabilities analysis.

Proposal 1: Consider the 5G and 5GA requirements as a starting point for 6G RAN4 related device characterization.

Proposal 2: The maximum supported channel bandwidth apply to Smartphone and FWA devices.

Proposal 3: Support 3MHz minimum channel bandwidth for all device types.

Proposal 4: The CA capability apply to Smartphone and FWA devices and optionally to RedCap.

Proposal 5: RAN4 to address the TxD versus UL MIMO power class ambiguity during 6G development.

Proposal 6: Two RF chains are minimum requirement for 6G Smartphones and RedCap devices.

- References

[1] RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator).

[2] RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom.

[3] 3GPP TR 38.914: Study on 6G Scenarios and Requirements, v0.1.0.

[4] R4-2514650, “WF on 6GR system parameters”, Huawei, HiSilicon, Prague, RAN4 #116bis, Oct 2025