# 3GPP TSG-RAN WG4 Meeting #117                                                     R4-2520619

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	7.9.3

Source:	Apple

Title:	Inter-vendor training collaboration and interoperability for two-sided AI/ML models

Document for:	Discussion

Introduction

RAN#108 has approved a new WI on AI/ML for NR air interface enhancements in Rel-20 [1], where the objective includes CSI feedback enhancement with two-sided model and inter-vendor training collaboration for two-sided model.

Discussion on Scalability

In AI/ML-based CSI compression, the encoder at the UE processes a precoding matrix whose dimension depends on the transmit antenna configuration, typically expressed as [N_TX × N_SB], where N_TX denotes the number of transmit ports and N_SB the number of subbands. Current transformer-based encoder designs learn a mapping from this precoding matrix to a latent representation (payload) that is subsequently quantized and reported to the network. However, the number of transmit ports can vary significantly across deployments and device classes, from 8 or 16 ports in handheld and compact UEs up to 32, 64, or more ports in high-end or multi-panel configurations.

Because the first embedding or fully-connected (FCN) layer in the encoder expects a fixed input dimension, any change in N_TX alters the input size and invalidates the trained model weights. As a result, a model trained for one array geometry (e.g., 32 ports) cannot directly handle a different geometry (e.g., 24 or 64 ports) without structural modification or retraining. This lack of scalability over transmit ports would require maintaining multiple distinct models for each antenna configuration, leading to excessive memory usage, signaling overhead, and interoperability complexity. Furthermore, it would limit forward compatibility with future, larger-port configurations that may appear in upcoming releases.

Therefore, it is essential to identify a scalable model structure that maintains stable performance (e.g., negligible SGCS degradation relative to dedicated models) while accommodating variable N_TX through mechanisms such as input padding, configuration-specific embeddings, or tokenization strategies. Such scalability would allow a single standardized backbone model to operate efficiently across different antenna configurations, minimizing implementation fragmentation and ensuring long-term applicability of AI/ML-based CSI compression within the 3GPP framework.

To enable scalable operation, where a single standardized encoder structure can support multiple Tx port configurations (e.g., 8, 16, 32 ports) and variable bandwidth partitioning, the model must handle changes in input dimensionality without retraining or re-architecting.
Scalability over Tx ports therefore becomes a key study area for ensuring forward compatibility and interoperability across gNB vendors and deployments.

During the Case 0 scalability study, RAN1 agreed that the most effective baseline structure for transformer-based CSI compression is the Subband-as-Token configuration (Alternative 1).
In this formulation, the precoding matrix is reorganized such that each token corresponds to one frequency subband, and the features of each token represent the transmit (Tx) ports. The transformer then operates along the frequency dimension, learning correlations across subbands while treating the Tx-port domain as feature inputs encoded through the embedding layer.

This design provides a physically meaningful and computationally efficient representation of CSI data and was selected as the baseline for scalability evaluation because of the following advantages:

- Physical interpretability:
Each token directly corresponds to a subband CSI realization, consistent with the frequency-domain structure used in conventional 3GPP CSI reporting. This ensures backward alignment with existing measurement and reporting granularity.

- Scalability over Tx ports:
The Tx-port dimension can vary across configurations and can be normalized through a simple linear embedding operation (for example, by zero-padding to a predefined base dimension). This enables a single transformer backbone to operate across different antenna configurations.

- Transformer efficiency:
The sequence length equals the number of subbands, which is typically small (e.g., 12). This keeps the computational load of self-attention moderate while maintaining sufficient spectral resolution.

- Cross-subband correlation modeling:
The self-attention mechanism learns frequency-domain correlation patterns naturally, providing a data-driven complement to traditional subband interpolation or frequency smoothing methods.

Consequently, “Subband = Token” was adopted as the baseline model structure for scalability evaluation in RAN1, providing a balanced trade-off between interpretability, flexibility, and model efficiency.

Alt 1: Scalable Transformer-Based CSI Compression

The goal is to design one scalable encoder–decoder model that supports multiple 3GPP CSI configurations, differing in:

- number of Tx ports (N_TX)

- number of subbands (SB)

- payload size (K bits per report)

without retraining or redefining the model structure for each case.

In Alt 1, scalability over the Tx-port dimension is achieved through dedicated linear embedding layers,  each responsible for mapping the input dimension N_TX to the Transformer’s latent feature dimension E.

In the Alt 1 scalable model design, the Transformer backbone operates in a fixed latent dimension E (e.g., 64), which is independent of the number of transmit ports. Each antenna geometry employs a dedicated linear embedding layer that maps the physical CSI vector of size N_TX for each subband into the shared latent space of dimension E. As a result, the encoder always processes inputs of uniform size [SB × E], regardless of the actual antenna configuration. The same principle applies symmetrically at the decoder, where a matching de-embedding layer maps the reconstructed latent features back from E to N_TX. This approach decouples the Transformer’s architectural width from the physical array size, allowing the same backbone to be reused for multiple geometries without modification. Only lightweight geometry-specific adapters are added, ensuring that smaller or larger arrays can be supported by training or activating the corresponding embedding/de-embedding pair. Unlike zero-padding schemes, Alt 1 avoids performance degradation from unused dimensions and learns an explicit spatial projection for each geometry, achieving clean normalization of the input space. The Transformer’s parameter count and computation cost remain fixed, determined solely by E, while the adapters scale linearly with the number of supported array configurations. Empirical results in RAN1 #120bis show that this design achieves minimal (< 1 %) SGCS loss compared to configuration-specific models, providing an efficient and forward-compatible baseline for scalable CSI compression across diverse Tx port configurations.

The Alt 1 scalable architecture incorporates several modular components to ensure adaptability across different antenna, subband, and payload configurations. First, the Tx-specific embedding and de-embedding layers handle variations in antenna geometry by learning configuration-dependent linear mappings between the physical transmit port domain (N_TX) and the Transformer’s latent feature space (E). These adapters allow each geometry (e.g., 2×4, 4×4, 2×8) to project its CSI observations into the same latent dimension without altering the shared backbone. Second, the K-specific linear heads  in the encoder and  in the decoder provide flexibility for different feedback payload sizes by compressing or expanding the latent vector to the desired payload dimension K. This enables the same model to support multiple quantization or reporting bitrates with minimal retraining. The Transformer cores themselves are shared between all configurations on both encoder and decoder sides, forming a common latent processing backbone that learns frequency-domain and spatial correlations invariantly across scenarios. Finally, padding and masking mechanisms are applied to manage variability in the number of subbands (SB = 6–19), ensuring consistent sequence lengths for attention operations while preventing gradient updates on padded positions. Together, these elements provide full scalability over transmit ports, subband granularity, and payload configurations using a single unified Transformer framework.

Alt 1: Training Methodology

The scalable Transformer-based CSI compression model is trained using a mixed-configuration dataset that aggregates samples from multiple antenna, subband, and payload configurations. For example, the dataset may include Case A with (8 Tx ports, 6 subbands, 16-bit payload), Case B with (16 Tx ports, 12 subbands, 32-bit payload), and Case C with (32 Tx ports, 19 subbands, 64-bit payload). Each training sample is labeled with its configuration tuple (N_TX, SB, K), ensuring that the model can identify the active structure during training. This composite dataset exposes the network to diverse operating conditions, promoting generalization across a wide range of practical scenarios.

During training, a dynamic branch activation mechanism is used to enable configuration-specific processing while maintaining a shared backbone. For each mini-batch drawn from the mixed dataset, the model activates only the submodules corresponding to that batch’s configuration. On the encoder side, the active pair consists of the geometry-specific embedding layer and the K-specific projection head (Embedding_i + FC_K_enc); on the decoder side, the corresponding K-specific expansion and geometry-specific de-embedding layers are activated (FC_K_dec + De-Embedding_i). The Transformer encoder and decoder cores remain shared and continuously updated across all configurations.

During back-propagation, the model updates the shared Transformer parameters (common to all) and the active adapter weights (specific to the current configuration), while keeping inactive branches frozen for that iteration. This setup implements a multi-task learning framework, where every configuration contributes gradients to the shared backbone, while its associated adapters specialize to the unique characteristics of that configuration.

This training strategy provides several practical advantages. First, the continuous exposure of the shared Transformer to all configurations mitigates catastrophic forgetting, a common issue in sequential fine-tuning. Second, the approach ensures latent-space alignment across configurations, which simplifies decoder reuse and cross-configuration inference. Third, it enables faster and more stable convergence, as the backbone learns universal spectral-spatial structures such as frequency correlation and antenna-array manifolds that are shared across environments. Finally, by unifying all configurations into a single training process, overall training cost is significantly reduced compared to running separate training sessions for each Tx, subband, or payload configuration.

Alt 1: Standardization Implication for RAN4

If RAN4 agrees to standardize the Alt 1 scalable model, then to ensure interoperability between UEs and gNBs, three model components must be standardized together, because each part contributes to end-to-end reconstruction consistency:

- Inputs & configuration

  - Payload vector of size K (post-quantization de-mapped to reals).

  - Configuration identifiers carried by RRC (or equivalent): target Tx geometry ID (determines NTX), active subband set / SB count, and the payload size K.

  - Padding/masking map for subband tokens when SB<SBmax.

- K-specific expansion head (decoder-side)

  - A lightweight linear layer FCKdec:RK→Rlatent that inverts the encoder’s K-specific projection.

  - Output is reshaped to SB×E (or produced directly as [SB×E] depending on implementation).

  - One such head per standardized payload option (e.g., K∈{32,64,128}); only the matching head is active.

- Shared Transformer decoder backbone

  - Operates within the latent token space: SB×E → SB×E.

  - Uses positional encoding over subband indices and applies attention masks to ignore padded tokens.

  - Learns cross-subband (frequency) correlations and refines latent representations independent of Tx geometry and K.

- Tx-specific de-embedding head

  - Geometry-specific linear adapter DeEmbi:RE→RNTX applied per token.

  - Restores the spatial (port) dimension for the selected antenna layout (e.g., 2×4, 4×4, 2×8).

  - Produces the final tensor NTX×SB in the spatial-frequency domain (or mapped back to the exact precoder domain used for training).

- Output domain & post-processing

  - Decoder outputs the reconstructed precoding matrix in the agreed domain (e.g., spatial-frequency eigenvector or W2 angular-delay form), matching Case-0 conventions.

  - Optional denormalization / unit-norm projection to meet PMI constraints.

- Scalability behavior

  - K-scalability via per-K expansion heads (no retraining of backbone).

  - Tx-scalability via per-geometry de-embedding adapters (backbone unchanged).

  - SB-scalability via padding/masking; backbone attention cost scales with sequence length only.

- Standardization implication (decoder side)

  - To guarantee interoperability, RAN4 should standardize:

    - the shared Transformer decoder [SB×E]→[SB×E],

    - the set of K-specific expansion heads (K→latent/[SB×E]), and

    - the set of Tx-specific de-embedding layers (E→NTX).

  - These three elements ensure a received payload of length K is reconstructed identically for any standardized Tx geometry and subband configuration.

If Alt-1 is adopted for standardization, then following will need to be specified:

- Shared Backbone

  - A common Transformer-based decoder core that operates on latent tensors of size [SB × E].

  - This block remains fixed across all configurations and is reused for every geometry and payload.

- Geometry-Specific Layers (4 total)

  - Four Tx-specific de-embedding layers Linear(E → N_TX) — one per antenna geometry configuration (e.g. 2×4, 4×4, 4×8, 8×8).

  - Each de-embedding layer converts latent [SB × E] features back to [SB × N_TX] CSI realizations.

  - Only one is active during inference based on the known Tx geometry.

- Payload-Specific Layers (3 total)

  - Three K-specific expansion heads FC_K_dec: Linear(K → latent) , one per standardized payload size (e.g. K = 16, 32, 64 bits).

  - Each head reconstructs the latent feature space from the compressed payload before entering the shared Transformer decoder.

Alt 2 — Shared Embedding with Padding and Masking

Alt 2 achieves full scalability in Tx geometry, subband count, and payload size (K) through a single shared encoder–decoder model trained at the maximum expected configuration. Instead of defining separate branches per geometry or per payload, Alt 2 relies on padding and masking to normalize input and output dimensions. The Transformer core remains unchanged, operating only along the frequency (subband) dimension, with all feature-domain variability handled by preprocessing.

All Tx-port layouts (e.g. 8, 16, 32 ports) are projected into a common feature space using a shared embedding layerLinear(N_TX_base → E), where N_TX_base is the maximum array size used during training (e.g. 32).

- For smaller geometries, the CSI input [N_TX × SB] is zero-padded along the Tx dimension up to N_TX_base.

- The Transformer itself does not perform attention across features , only across subbands ,therefore no feature-domain attention mask is required.

- The zeros are absorbed by the linear embedding, allowing the network to generalize across geometries while maintaining consistent feature dimensionality E.

This design eliminates the need for geometry-specific embedding or de-embedding layers; a single embedding layer can handle all Tx sizes.

The Transformer treats each subband as one token. To support variable numbers of subbands (e.g. 6, 12, 19), sequence-level masking is applied within the self-attention mechanism:

- Inputs with fewer subbands are padded to the maximum sequence length used during training.

- A token mask disables attention on padded subbands, ensuring that cross-subband correlation is learned only from valid tokens.

Thus, padding operates on features (Tx ports), while masking applies to tokens (subbands).

For payload compression, Alt 2 uses a single linear projection and expansion pair sized for the maximum payload length Kₘₐₓ (e.g. 64 bits):

- On the encoder side, smaller payload targets (e.g. K = 16 or 32) are derived by truncating the first K components of the Kₘₐₓ output.

- On the decoder side, the same expansion layer is reused, smaller payloads are zero-padded back to Kₘₐₓ before decoding.

This eliminates the need for K-specific heads, making payload scalability intrinsic to the model.

Alt 2 is trained once on a mixed-configuration dataset that includes all expected Tx-port and subband configurations, each padded or masked to the base dimensions (N_TX_base, SB_base, Kₘₐₓ).

- Each batch includes the appropriate masks identifying valid subbands.

- The Transformer core learns a universal latent representation that ignores inactive tokens and zero-padded features.

- Gradients are automatically restricted to active portions of the input, enabling a single shared model to generalize across all configurations.

Compared with Alt 1, Alt 2 requires significantly fewer configuration-specific adapters:

| Component | Alt 1 | Alt 2 |
| --- | --- | --- |
| Tx-specific embeddings | 4 (separate per geometry) | 1 (shared with zero-padding) |
| K-specific heads | 3 (separate per payload size) | 1 (shared) |
| Transformer backbone | 1 (shared) | 1 (shared) |
| Total small trainable adapters | 7 | 2 |

Figure 1: CSI Compression Scalability Options

Figure 2: CSI encoder and CSI decoder that proposed in RAN1 R19

In figure 2 the CSI encoder and CSI decoder that proposed in RAN1 R19 is shown. Below is an explanation of the symbols used:

| Symbol | Meaning | Scope / Layer | Explanation |
| --- | --- | --- | --- |
| N_token | Number of tokens | Transformer sequence length | Corresponds to the number of subbands (SB). Each subband is represented as one token in the sequence. |
| d_model | Transformer feature dimension | Inside Transformer encoder/decoder | Internal embedding dimension (E) for each token. Defines the width of the feature vectors that self-attention operates on. |
| N_head, d_head | Number and size of attention heads | Inside Transformer | Used to split d_model into multiple attention subspaces (d_model = N_head × d_head). |
| d_FF | Feed-forward hidden dimension | Inside Transformer | Intermediate expansion size in the MLP block, typically 2–4× d_model. |
| z_dim | Latent dimension per token after encoder projection | Output of encoder linear head | Each token’s feature vector of size d_model is linearly projected to a compressed latent code of length z_dim. |
| N_bit | Quantization depth per latent element | Quantizer | Defines how many bits each latent element is represented with (e.g., 2 bits). |
| K | Total feedback payload dimension | CSI feedback interface | Represents the total compressed feedback after flattening and quantization. Equivalent to the total number of transmitted elements or bits. |

Regarding the dimension z_dim and payload size we provide the following explanation:

| Quantity | Interpretation | Formula / Relation |
| --- | --- | --- |
| z_dim | Latent vector length per subband token | Internal model hyperparameter (Linear(d_model → z_dim)) |
| K | Total payload dimension transmitted | K = N_token × z_dim × N_bit |
| RAN4 simplification | Normalized total payload (after quantization) | z_dim ≈ K / (N_token × N_bit) |

The key trade-off between Alt 1 and Alt 2 lies in specialization versus universality. Alt 1 maintains distinct embedding and de-embedding layers per antenna geometry and separate projection/expansion heads per payload size, enabling the model to learn geometry-specific spatial manifolds and payload-specific quantization behaviors. This yields slightly higher accuracy: RAN1’s #120 bis results showed median SGCS gains of about 1–2 % for Alt 1 relative to Alt 2, especially for asymmetric arrays (e.g., 2×8 vs 8×2). However, Alt 1 increases implementation complexity, each geometry and payload combination requires dedicated adapters, version management, and training data balancing.Maintaining multiple heads and embeddings could increase total model size and cross-vendor alignment burden.

By contrast, Alt 2 consolidates all geometries and payload sizes into a single shared model trained at the maximum configuration. It uses zero-padding along the Tx dimension and token masking along the subband dimension to handle variable shapes dynamically, with a shared Transformer backbone and one linear projection/expansion pair sized to Kₘₐₓ. This design minimizes parameter count and simplifies standardization, RAN4 can specify a single encoder/decoder pair without managing multiple branches. The trade-off is a minor performance loss (≈ 1–3 % SGCS) when array aspect ratios diverge, since one embedding must generalize to all geometries. In summary, Alt 1 offers higher per-scenario fidelity and better geometry sensitivity, whereas Alt 2 provides cleaner scalability, simpler training, and a more practical path for standardization and interoperability.

Scalability Method for Transformer-Based CSI Compression: RAN4 should standardize a single baseline backbone to avoid model fragmentation. Alt 2 (shared embedding with padding/masking) is proposed as the default scalability framework, while Alt 1 (geometry-aware adapters) remains an optional enhancement for vendors seeking higher accuracy in geometry-sensitive cases.

- Baseline (Alt 2): shared embedding, zero-padding for Tx scaling, token masking for subbands, 
• Optional (Alt 1): geometry-specific adapters and payload-specific heads.
• Result: one interoperable framework, with optional performance boosts where needed.

Discussion on datasets: scalability, and channel models.

Channel format

RAN4 must specify a single, standardized dataset format that all companies can use to train the standardized test decoder and candidate reference models (Direction C / 3a-1) and run comparable performance evaluations. Today, encoder inputs and latent interfaces vary significantly across proposals, creating a fundamental interoperability barrier. Some proposals operate directly on raw channel matrices, which introduces several critical challenges: the raw channel representation is inherently high-dimensional and computationally expensive to process, different vendors apply proprietary pre-processing techniques (normalization schemes, filtering, spatial/temporal windowing) that make training datasets non-comparable, raw channel data contains substantial redundant information that could be irrelevant for CSI reconstruction.

Eigenvector-based representation solves these fundamental issues by providing a mathematically principled, canonical channel representation that eliminates preprocessing ambiguity while dramatically reducing data dimensionality. The eigenvector domain captures the essential channel subspace structure the dominant spatial directions of signal propagation which is exactly what CSI compression fundamentally needs to preserve. By standardizing on eigenvectors derived through a deterministic procedure (per-slot/per-subband covariance construction → rank selection → eigendecomposition → basis ordering/phase unwrapping → normalization), RAN4 ensures that all vendors work with identical input representations, making training datasets truly portable and performance comparisons genuinely meaningful.

Operational and technical advantages of eigenvector standardization include: dimensionality reduction from full channel matrices to dominant eigenvectors which reduces memory requirements; elimination of vendor-specific preprocessing variations that currently make dataset sharing impossible.

For CSI compression, RAN4 shall standardize the dataset format as eigenvector-based channel representation derived through a deterministic procedure: per-slot/per-subband covariance matrix construction → rank selection based on signal subspace energy threshold → eigenvalue decomposition → eigenvector basis ordering (descending eigenvalue magnitude) → phase unwrapping/alignment → amplitude normalization

Selection of Dataset for Training and Testing

A central challenge for RAN4 in specifying a reference AI/ML model for CSI compression is ensuring that the model trained under laboratory-controlled conditions continues to perform reliably under real network deployments.
Past study-item experience (Rel-19 → Rel-20) showed a consistent gap between link-level performance (excellent SGCS in CDL/TDL tests) and field performance, where models trained only on link-level data degraded sharply when evaluated on live urban macro environments. Conversely, system-level simulations (SLS) approximate deployment behavior but lack the precise calibration RAN4 requires for reproducible, normative testing. Therefore, neither dataset alone is sufficient, RAN4 needs a training methodology that inherits the calibration stability of LLS and the generalization capacity of SLS.

To build a reference model that performs robustly both in controlled link-level testing and in real field deployments, RAN4 should standardize a mixed-domain training dataset combining System Level Simulation (SLS) and Link Level Simulation (LLS) data.

- LLS (TDL/rCDL-C1) channels are calibrated within RAN4 and ensure that all companies can reproduce identical conditions when verifying performance requirements (e.g., SGCS, throughput).

- SLS (UMa, UMi, RuralMacro) channels represent spatial geometry, and user channel characteristics observed in real networks. They generalize far better to field data, which was consistently shown in Rel-19 and Rel-20 studies.

Empirical results demonstrate that [R4-2510342]:

- Models trained only on CDL or TDL achieve high SGCS in their native test domain but degrade severely when evaluated under SLS/field conditions.

- Models trained on UMa (SLS) generalize moderately well across CDL/field domains.

- Mixed training (SLS + LLS) achieves nearly optimal SGCS in both domains with negligible penalty (≤ 1–2 %), indicating that the latent representation learns domain-invariant spatial–spectral features.

Here are the key characteristics of the mixed dataset:

Balanced Composition: Include both SLS (e.g., UMa, UMi) and LLS (e.g., TDL, rCDL) data to capture realistic deployment diversity and maintain calibration consistency.

Field Robustness: SLS data improves generalization to real network environments, addressing the poor field performance observed in TDL/CDL-only training.

Reproducibility: LLS (TDL) data provides a stable, calibrated baseline for performance verification and inter-vendor comparability.

Unified Dataset Schema: Use a single dataset structure with metadata tags (channel model, geometry, Tx ports, subbands, payload K) to support mixed-configuration scalability training.

The combined dataset ensures that models passing TDL-based performance tests will also exhibit stable, field-aligned behavior, reducing domain mismatch and improving long-term interoperability.

Mixed SLS + LLS Dataset for Reference Model Training: To ensure that the standardized AI/ML-based CSI compression model performs consistently in both laboratory tests and real-world deployments, RAN4 should adopt a mixed-domain training dataset combining System Level Simulation (SLS) and Link Level Simulation (LLS)

TDL is the only fully calibrated and validated channel model within RAN4 for demodulation and CSI prediction. CDL/rCDL models remain under evaluation for PMI alignment and may introduce vendor-specific deviations. Using TDL ensures consistent, reproducible, and comparable SGCS/throughput results across companies without calibration drift.

RAN4 shall adopt the TDL channel model under LLS as the reference for defining and verifying AI/ML-based CSI compression performance requirements.

Incorporating Scalability Configurations into the Mixed Dataset

While RAN4’s Rel-19 dataset focused on fixed input/output sizes, typically (Subbands = 13, Tx ports = 32, Payload = 64 bits), the introduction of scalability in Rel-20 and beyond changes the problem fundamentally.
The AI/ML-based CSI compression model is now expected to handle variable input dimensions (different Tx geometries and subband allocations) and variable output payload sizes (K) using a single trained model.
Without explicit inclusion of these variations in the training dataset, the model cannot learn the invariances or normalization behavior needed to generalize across configurations. Hence, RAN4 must extend its reference dataset to become scalability-aware.

This requires constructing a mixed-configuration dataset that systematically covers the range of configurations used in scalability studies, such as those in the RAN1 Case 0 and Case 1 evaluations.
The goal is not only to validate a model’s scalability at inference, but also to teach the model during training how to interpret differences in the number of Tx ports, subbands, and payload granularity within a unified latent space.

The mixed scalability dataset should include multiple configuration tuples of the form (N_TX, SB, K) where:

- N_TX – number of transmit ports or geometry (e.g., 8, 16, 32), including asymmetric layouts (2×8, 4×4, 8×2).

- SB – number of subbands (e.g., 6, 12, 19) representing varying frequency-domain resolution.

- K – payload size (e.g., 16 bit, 32 bit, 64 bit) defining different compression ratios.

Each sample should be tagged with its configuration metadata to allow selective batching.
To efficiently train a single scalable model, a mixed-configuration training loop should be employed:

- Configuration-Tagged Sampling:
Each mini-batch is drawn from the mixed dataset and contains samples with configuration identifiers (N_TX, SB, K).

- Dynamic Branch Activation (Alt 1) or Padding/Masking (Alt 2):

  - For Alt 1, the model activates the corresponding geometry-specific embedding and K-specific head for that configuration.

  - For Alt 2, all inputs are padded to the maximum configuration and masked accordingly; the same Transformer backbone is shared.

- Shared Backbone Optimization:
Back-propagation updates the common Transformer encoder/decoder weights for all configurations, ensuring a unified latent representation that captures spectral–spatial correlation independent of geometry size.

- Balanced Gradient Aggregation:
Loss terms (e.g., SGCS-based) are averaged per configuration to maintain equal training pressure across small and large geometries.

RAN4 can define one mixed-dataset to test scalability performance under aligned assumptions, avoiding the need to generate separate datasets per configuration.

Unified Mixed Dataset for Channel Domain and Scalability Coverage: RAN4 should establish a single unified mixed dataset that merges both channel-domain diversity (SLS + LLS) and scalability configuration diversity (Tx ports, subbands, and payload sizes)

Unified Dataset: Combine LLS (TDL/rCDL) for calibration and SLS (UMa/Umi) for field realism in one reference

Scalability Coverage: Include multiple (N_TX, SB, K) configurations to expose the model to variations in geometry, frequency granularity, and compression ratios.

Efficient Training: Use configuration-tagged batches to train a single scalable model that learns invariances across domains and configurations.

Single Benchmark: Avoids separate datasets for each use case, ensuring interoperability, reproducibility, and simplified testing within one aligned mixed-dataset framework.

# 4	Number of Test Decoders for Option 3 and Generalization

The WF from RAN4#114bis was as follows:

Agreements

- The encoder that corresponds to the standardized test decoder is stored within 3GPP

  - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS)

  - FFS whether several encoders would be captured (e.g. due to different complexity)

For further discussion:

The following topics should be further discussed:

- For the encoder, the TBD and FFS above; i.e., where the encoder is stored/referred to and whether several encoders would be captured.

- Whether it is agreeable to assume that the training dataset for the standardized test decoder is stored within 3GPP, if available

  - For example in case UE or network vendors could find access to the dataset used for the test encoder useful for their developments

- Whether a test dataset is needed in the specification, or some other alternative (such as a channel model etc.)

For option 3 track 1, the endorsed observations are as follows:

- It is possible for UE vendors to train an encoder based on another company’s frozen test decoder that is functional

- The performance of the encoder is very dependent on the dataset used for training the encoder and the dataset used for training the frozen decoder, and the testset

  - If the encoder is trained using a dataset of the UE vendor, the UE vendor dataset should contain the [features] captured in the dataset used to develop the test decoder.

- The variation in the performance depending on the training datasets and test datasets can be as much as 30-50% in SGCS. Further details are available in the results spreadsheet.

- For option 3, track 1 to be useful, more careful attention would be needed to the alignment of datasets

- Only one encoder structure was considered for option 3 track 1. Other lower complexity encoder structures were not examined.

Reference encoder and number of test decoders

Option 3 fixes a single neural-network test decoder in the TE and retains the encoder that was jointly trained with it as a reference model. What remains unresolved is the cardinality and scope of these models: do we need only one decoder (and thus one reference encoder) to cover all requirement scenarios and UE capability classes, or do fundamentally different propagation conditions and capability constraints force multiple frozen decoders? The current simulations trained one encoder–decoder pair and then derived new encoders, but they have not yet proven that a single decoder generalizes across all envisaged scenarios. If several decoders were ultimately required, each would demand its own reference encoder, increasing storage and maintenance overhead inside 3GPP.

A second, related issue is whether multiple reference encoders should be kept for a single decoder to reflect different encoder complexity levels. If low-complexity encoders can meet the same performance target as higher-complexity ones, it is unclear why vendors would not simply adopt the simplest solution and why 3GPP should store more than one reference encoder. Conversely, if different complexity levels deliver materially different, standardized performance tiers, several encoders might be justified but that would imply defining multiple performance levels for the same scenario, something RAN4 rarely does. Clear criteria are therefore needed to decide when additional decoders or reference encoders are warranted, how they are trained, and how their complexity is bounded, so that interoperability is guaranteed without proliferating models beyond what is strictly necessary.

We believe that RAN4 should keep one frozen test decoder and one associated reference encoder the encoder used to train that decoder stored in a 3GPP repository with versioning and checksums. Additional decoders are only added if objective evidence (e.g., an agreed NMSE/SGCS spread threshold across distinct requirement scenarios) shows the single decoder cannot meet all needs. The decoder is trained once with the reference encoder; retraining with others is unnecessary unless new, approved scenarios or tiers appear.

To determine the required cardinality of different test decoders to cover all applicable scenarios, we propose conducting a Test-Decoder Deployment Variability Assessment to draw conclusions on the appropriate granularity of the specified decoders.

In the worst-case, testing requirements for CSI compression could demand multiple distinct test decoders to cover fundamentally different scenarios (CDL vs Uma vs Umi vs field data, etc) , for example, varying frequency ranges, antenna configurations, mobility profiles, or channel models. If each scenario requires its own fully trained and frozen test decoder, RAN4 would need to manage, store, and maintain several large models in parallel. Any change, such as a dataset refresh, preprocessing adjustment, or bug fix would have to be replicated across all decoders, multiplying the verification, version control, and interoperability checks by the number of scenarios. This increases TE storage requirements, and complexity.

The scalability challenge intensifies over time. As new channel models or deployment scenarios are introduced in later releases, more frozen decoders would need to be added, each trained and validated independently. Older decoders can quickly become outdated while still being part of the certification scope, creating long-term maintenance burdens. This rigid “one full model per scenario” approach also limits flexibility, as each decoder is tied to its original training assumptions, making adaptation to future conditions cumbersome and inefficient.

We would like to introduce a data-driven and statistically grounded method for defining and managing standardized AI/ML test decoders in RAN4. We can leverage the previously proposed PSE metric to find similarities across datasets and train test decoders accordingly. Rather than relying on a single dataset or subjective scenario boundaries (e.g., UMa, UMi, rural), the approach characterizes channel datasets using the statistical distribution of Power Spectral Entropy (PSE). These distributions capture the structural characteristics of the radio environment through spatial and delay entropy patterns that represent the underlying channel behavior.

Backbone + Adapter Concept for multiple test-decoders 
Instead of maintaining multiple full-size decoders, RAN4 could standardize a single backbone model, a shared, frozen core architecture and weight set that defines the common latent space for encoder–decoder interoperability. For scenario-specific requirements, lightweight adapters (e.g., small fine-tuned modules or layers) can be trained on a targeted dataset to specialize the backbone for that environment. The backbone remains unchanged across scenarios; only the adapter varies.

In this model, a new scenario (e.g., high-speed railway at mmWave) would not require creating and freezing an entirely new decoder. Instead, a scenario-specific adapter would be trained using a small, well-defined dataset. The adapter, combined with the fixed backbone, functions as the test decoder for that scenario. This approach decouples the common interoperability layer from the scenario-specific tuning, enabling RAN4 to maintain a single core model while allowing flexible, lightweight extensions.

This proposed approach reduces the number of large decoder models RAN4 must store and version, lowering maintenance costs and administrative effort. It also mitigates the risk of divergence between decoders, since all scenarios share the same backbone. Scenario-specific testing is more efficient because only the adapter needs validation, not the full decoder. Future-proofing is improved new scenarios can be supported by releasing a small dataset and fine the adapter without retraining or replacing the backbone.

If RAN4 standardizes a single frozen backbone and a common adapter architecture, there is no need to store multiple pre-trained adapters for different scenarios. Instead, each new scenario, whether defined by a specific channel model, environment, or frequency band, would only require an agreed scenario-specific dataset and simulation assumptions. At certification time, the adapter is trained on this dataset to produce the specialized decoder on the fly, while the backbone remains unchanged. This approach minimizes stored artifacts to just the backbone and adapter architecture, eliminates per-scenario versioning overhead and ensures that any future updates (e.g., changes to preprocessing rules) require only generating a new scenario-specific dataset and training the adapter, while the backbone remains frozen. This keeps the process scalable, reduces re-certification effort, and maintains consistency across all scenarios.

Single Backbone Decoder with Scenario-Specific Adapters: To avoid storing multiple full models, RAN4 maintains one frozen backbone decoder and attaches lightweight adapters trained per MPSE-clustered scenario. The backbone defines the interoperable core; adapters specialize it for new environments without retraining or replacing the core model.

- Freeze one backbone + reference encoder as the 3GPP baseline for all scenarios.

- Define a common adapter architecture trained on each cluster’s dataset.

- Store only backbone and adapter spec; new scenarios release new datasets, not new frozen decoders.

If multiple encoder complexity levels are explored, the reference encoder is simply the lowest-complexity one that still meets the target. Only if RAN4 explicitly defines different performance tiers tied to UE capabilities would multiple reference encoders be justified. Remaining FFS items include setting the exact convergence metric and thresholds, defining any capability tiers and their performance floors, and establishing lifecycle rules for refreshing datasets/models without complicating certification. This approach preserves interoperability and limits model proliferation while leaving a controlled path for expansion if truly needed.

RAN4 shall retain a single reference encoder per frozen test decoder, selecting the lowest-complexity encoder that still meets the agreed performance target; additional reference encoders would be justified only if RAN4 explicitly defines distinct performance tiers tied to UE capability classes.

If RAN4 defines distinct performance tiers, RAN4 needs to set the exact convergence metric (SGCS, NMSE) define the capability tiers and their minimum performance floors .

# 5	Construction of the synthetic dataset and data alignment using PSE

The central problem is ensuring that the dataset used to train the test decoder realistically represents the range of channel conditions the model will encounter in actual deployments. In previous simulations, even though companies used the same agreed-upon assumptions, differences emerged in datasets due to inconsistencies like amplitude or phase normalization. Such mismatches mean the latent space learned by the decoder might not reflect real-world conditions, causing degraded interoperability and inconsistent encoder performance across vendors.

For a Work Item (WI), accumulating extensive real-world measurement data within 3GPP RAN4 is impractical. Therefore, the dataset must come from carefully designed simulations covering sufficient diversity frequency bands, environments/channel models, mobility levels, antenna configurations, and SNR ranges to be representative of real deployment scenarios. Equally critical is ensuring strict alignment between datasets produced by different companies, through standardized preprocessing steps and verification procedures, so all encoders trained against the resulting latent space remain interoperable and robust in real-world networks.

To address dataset realism and alignment, it is proposed that RAN4 define a standardized simulation framework covering a diverse set of scenarios relevant to real-world deployments. This framework would explicitly specify frequency bands, environments (urban macro, urban micro, indoor, rural), mobility levels (static, pedestrian, vehicular, high-speed), antenna configurations, and SNR distributions. A common preprocessing procedure including precise amplitude/phase normalization, quantization methods, shall be documented and strictly followed by all contributing companies.

Datasets generated under this unified procedure must pass statistical alignment tests based on predefined metrics such as power spectral entropy (PSE). Companies wishing to introduce proprietary data must demonstrate statistical equivalence with the reference dataset; if alignment fails, companies must augment or re-weight their data accordingly or default to the official dataset provided by RAN4. This disciplined, reproducible approach ensures the resulting dataset captures a realistic, interoperable latent space for the test decoder, maintaining consistency and reliability in certification.

Standardized Dataset Collection for Test Decoder Training

RAN4 should ensure every test decoder is trained on realistic, interoperable data:

- Simulation framework: Define common frequency bands, environments, antenna setups, and SNR distributions.

- Dataset alignment: Verify consistency across vendors using metrics such as power spectral entropy (PSE).

- Outcome – Guarantee that the trained test decoder reflects true wireless channel scenarios and provides a common latent space for interoperability.

Power Spectral Entropy (PSE) is a way to summarize how the power of a wireless channel is distributed across the physical dimensions of space and delay. We start with the channel state information (CSI), which describes how each transmit port communicates with each receive port on each frequency subcarrier. This raw CSI contains the fine details of multipath propagation but is tied to a specific antenna layout and is hard to compare across different systems.

The first step is to represent the CSI on universal physical axes that everyone can share. The channel is transformed from the antenna-port view into a beamspace view so that received and transmitted angles of arrival and departure are represented on a common grid. At the same time, a delay transform converts the frequency information into multipath delays. The result is a three-dimensional cube showing how much signal power lies in each combination of transmit angle, receive angle, and delay.

Next, only the magnitude of the energy matters. The complex channel coefficients are converted into power values, so each small cell of the cube shows the amount of power coming from that direction and delay. All these power values are then normalized so that they sum to one. This creates a probability map, meaning each cell can be thought of as the probability that a unit of signal energy comes from that particular angle and delay.

Finally, PSE measures how evenly this probability is spread. If most of the power is concentrated in a few cells, as in a clear line-of-sight channel, the entropy is low. If power is spread across many cells, as in a rich scattering environment, the entropy is high. In this way a single number, or a compact set of numbers, captures the structural richness of the wireless channel. Because PSE is based on a shared physical grid and on power rather than phase, it is well suited for aligning and comparing datasets from different companies or measurement setups.

We can consider the following options for dataset alignment:

Option A – PSE-Based Statistical Alignment
Option A relies on a structural comparison of datasets through Power Spectral Entropy (PSE) computed from beamspace–delay power maps derived from raw CSI.
Because PSE captures the spatial and delay richness of propagation independently of antenna configuration or phase, it provides a vendor-neutral quantitative fingerprint of each dataset.
By comparing PSE distributions across vendors using metrics such as Wasserstein or KS distance, RAN4 can decide whether datasets are statistically equivalent, ensuring that all training data populate a similar latent space.
This approach is objective, lightweight, and simulation-agnostic; it does not depend on any specific encoder–decoder implementation.
However, it requires access to standardized raw CSI formats and strict preprocessing, which adds an initial setup cost but yields a reproducible, model-independent reference for long-term maintenance.

Option B – Performance-Based Alignment
Option B assesses alignment indirectly by testing each vendor’s dataset on a pre-trained reference encoder–decoder pair (e.g., the Option 3 test decoder).
If the resulting SGCS or NMSE performance is close to that obtained on the reference mixed dataset, the new dataset is considered aligned.
This method provides a functional measure of compatibility that reflects the model’s sensitivity to distribution shift. Its limitation is that performance similarity can arise even when underlying data statistics differ significantly. Performance-based alignment evaluates datasets indirectly, by observing whether a pre-trained encoder–decoder model performs similarly on a new dataset and on a reference dataset. However, two datasets can yield nearly identical model metrics (e.g., SGCS or NMSE) even when their underlying channel statistics differ in ways that matter for generalization. For example, a dataset with strong line-of-sight bias and one with rich scattering could both achieve similar reconstruction error if the test model happens to be less sensitive to angular spread; yet, they would lead to different latent-space representations once new encoders are trained.

We are proposing the following steps:

## 1. Transform CSI to a 3-D power cube

Each channel snapshot provides a complex-valued frequency response H(f) between all transmit and receive antenna ports.
This raw matrix is not directly comparable across different antenna layouts, so we first convert it to a common physical grid.

Step 1.1 – Beamspace projection
Apply fixed, agreed-upon DFT beamforming matrices:

- B_rx of size N_r × N_rx for the receive array

- B_tx of size N_t × N_tx for the transmit array

These matrices express the channel in terms of directional beams instead of antenna elements, which removes dependence on each vendor’s exact antenna geometry.

Step 1.2 – Delay transform
Apply an inverse FFT across subcarriers (size K) to resolve multipath delays.
This converts the frequency response into the time (delay) domain.

Step 1.3 – Build the power cube
Combine the two spatial transforms and the delay transform to form a 3-D non-negative power cube:

S[u,v,τ] = | ∑_f ( B_rx[u,:] · H(f) · B_tx[:,v] ) e^{j2π f τ Δτ} |^2

where

- u = 0 … N_r − 1 is the RX beam index,

- v = 0 … N_t − 1 is the TX beam index,

- τ = 0 … K − 1 is the delay-bin index.

This cube S tells us how much signal energy arrives from each RX direction, leaves via each TX direction, and at each excess delay.

## 2. Convert to a probability map and compute PSE

Different channel snapshots can have different absolute powers, so we normalize each cube to make them comparable.

Step 2.1 – Probability normalization
p[u,v,τ] = S[u,v,τ] / Σ_{u',v',τ'} S[u',v',τ'}

This converts the power cube into a probability distribution over all N_r * N_t * K bins.

Step 2.2 – Power Spectral Entropy
Calculate Shannon entropy and normalize it by the maximum possible entropy:

PSE = - Σ_{u,v,τ} p[u,v,τ] * log2(p[u,v,τ]) / log2(N_r * N_t * K)

- PSE close to 0 means power is concentrated in a few bins (sparse, line-of-sight channels).

- PSE close to 1 means power is spread across many bins (rich, highly scattering channels).

This single number is a compact, geometry-independent fingerprint of each channel snapshot.

## 3. Compare PSE distributions across datasets

Now each dataset is simply a collection of PSE numbers.

Step 3.1 – Empirical CDFs
Compute empirical cumulative distribution functions (CDFs) F_A(x) and F_B(x) for datasets A and B.

Step 3.2 – Wasserstein-1 distance 
W1 = ∫ |F_A(x) - F_B(x)| dx

This is the minimum “work” needed to reshape one PSE distribution into the other and captures overall distributional shift.

Step 3.3 – Kolmogorov–Smirnov statistic
D_KS = sup_x |F_A(x) - F_B(x)|

This is the largest vertical gap between the two CDFs and reveals localized mismatches even when means are similar.

Together, W1 and D_KS provide a global and local picture of how different the two datasets are in terms of spatial–temporal channel richness.

## 4. Make an alignment decision

Define acceptable tolerances (for example, W1 ≤ 0.05 and D_KS ≤ 0.1).
Optionally compute a single alignment score:

Score = 100 * [1 - γ1 * (W1 / τ_W1) - γ2 * (D_KS / τ_KS)]

where γ1 and γ2 weight the two distances and τ_W1, τ_KS are the tolerances.

- Score ≥ 80 → Aligned: datasets are aligned and can be used to evaluate or deploy the trained CSI compression model with confidence.

- Score 60–80 → Partially aligned: Consider reweighting the test dataset, filtering outlier snapshots, or collecting additional samples before relying on performance metrics

- Score < 60 → Misaligned: datasets differ too much for direct model sharing.

PSE can also be used for performance monitoring as an input-distribution drift metric.

Embedding PSE into network performance monitoring lets operators track input-distribution drift and act before compression accuracy degrades. Each trained CSI compression model can be shipped with a reference PSE “envelope” (mean, variance, or histogram). During operation, the UE or gNB computes rolling PSE statistics from live CSI feedback and compares them with the reference envelope. Persistent deviations signal that the deployed model is encountering channel conditions different from those it was trained on, prompting retraining or an on-the-fly model switch. Standardizing PSE computation and reporting would give operators and vendors a common, low-overhead guardrail for maintaining CSI compression performance throughout a network’s lifetime.

PSE-Based Dataset Alignment Methodology

RAN4 should adopt a power spectral entropy (PSE) method to verify cross-vendor dataset consistency:

- Raw channel requirement – Provide raw CSI/channel data so that each CSI snapshot can be converted into a common beamspace-delay power map for PSE calculation.

- PSE computation – Compute PSE to quantify how widely signal power is spread across beams and delays.

- Statistical comparison – Treat PSE values from different datasets as distributions and compare them with distance metrics such as Wasserstein distance.

- Alignment criterion – Datasets are considered aligned if the measured distances stay below agreed thresholds, ensuring consistent training data for standardized test decoders.

– PSE for Online Performance Monitoring and OOD Detection

RAN4 should use power spectral entropy (PSE) as a live KPI to detect input-distribution drift and monitor model performance:

- Continuous PSE computation – Calculate PSE in real time from live CSI feedback during network operation.

- Reference PSE envelope – Each deployed CSI compression model carries a PSE envelope derived from its training dataset.

- OOD detection – Compare live PSE statistics to the reference envelope; persistent deviations signal out-of-distribution (OOD) conditions.

- LCM action triggers – Detected drift can trigger model retraining, adapter updates, or switching to a more suitable model.

- Unified methodology – The same PSE framework used for dataset alignment in certification ensures consistency between offline validation and online performance monitoring.

# 6 Inter-vendor training collaboration and interoperability for two-sided AI/ML

In RAN Meeting #109 a revised WID was captured in RP-252445 with the following aggrements:

Inter-vendor training collaboration for two-sided AI/ML models

- Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback

  - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback

- Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

Interoperability and RRM requirement [RAN4]:

- To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability

  - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration

- Necessary RRM requirement for specified LCM procedures.

- NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

Below we give an overview of Direction C, Direction A, sub-option 3a-1 and Option 3 for fully specified test decoder:

Direction C:

Direction C is the fully specified reference-model approach for two-sided AI/ML CSI feedback.
Its purpose is to guarantee inter-vendor interoperability and minimum field performance by standardizing at least one side of the model: the encoder, the decoder, or both. The standardized side would be fully defined in 3GPP: its architecture, layer configuration, input/output format, and, if required, trained weights are all fixed in the specification. With this fixed anchor, any vendor can develop or fine-tune the opposite side of the model so that the end-to-end CSI compression and reconstruction meet the agreed KPIs.

This flexible structure allows multiple deployment choices. If the decoder is standardized, every UE vendor can train its own encoder to generate compressed CSI that is guaranteed to work with the reference decoder. If the encoder is standardized, network vendors can train their own decoders to match the standardized encoder output. And if needed, both encoder and decoder may be specified together as a complete reference model for direct use.
In every case, vendors retain freedom to re-engineer or fine-tune their non-standardized side, while the common interface ensures compatibility.

Direction C also embeds the scalability and generalization principles studied in RAN1.
The reference model must handle a wide range of antenna-port counts, sub-band sizes, and payload budgets, and it must be robust when real field data differ from synthetic training data.
Vendors may fine-tune the non-standardized side to local conditions, but the standardized reference side guarantees that any compliant encoder–decoder pair will interoperate and deliver at least the minimum required performance.

Finally, Direction C serves as a foundation for other inter-vendor collaboration methods, such as Direction A sub-option 3a-1 (parameter exchange).
The RAN4 test decoder and reference encoder developed for Option 3 can be built on the Direction C backbone, ensuring that laboratory conformance tests and field deployments share the same standardized anchor. In this way Direction C provides the gold-standard reference model for commercial deployment, while remaining flexible enough to accommodate vendor innovation on the complementary side of the AI/ML CSI feedback chain.

Option3 test decoder:

Option 3 is RAN4’s fully specified test-decoder approach for AI/ML-based CSI feedback.
Its purpose is to give every vendor a common, reproducible benchmark for conformance testing.
RAN4 defines the complete structure of the decoder: including backbone type, layers, dimensions, activation functions, and any normalization or attention mechanisms, and fixes the trained weights.
This “test decoder” becomes the gold-standard receiver for compressed CSI: any UE encoder that passes conformance testing against it is proven to interoperate at or above the agreed performance floor.
By freezing both the architecture and the parameters, RAN4 ensures that test results are repeatable across labs and independent of any individual vendor’s implementation.

To make the test decoder meaningful, RAN4 also specifies a co-trained reference encoder.
This reference encoder is the exact partner that was used when training the test decoder.
It is stored in 3GPP documentation or linked repositories so that test equipment vendors and implementers can reproduce the decoder’s original training and verify its performance.
While commercial UEs are not required to deploy this reference encoder, it serves several critical roles:

- providing the dataset and latent outputs used to validate the test decoder,

- acting as a baseline against which any new UE encoder can be compared, and

- supporting inter-vendor training collaboration by giving all parties a known, trusted encoder side when developing or adapting their own models.

The goal of deriving the test decoder with its reference encoder is therefore twofold.
First, it provides a single, authoritative test environment to certify UE encoders for Release-20 CSI feedback.
Second, it supplies a stable reference model that RAN1 and other working groups can leverage for inter-vendor training collaboration: whether in Direction C (fully specified reference model) or Direction A sub-option 3a-1 (parameter exchange without CSI sharing).
Through this work, RAN4 guarantees that AI/ML-based CSI compression achieves minimum interoperability and performance across all vendors and network deployments.

Direction A 3a-1:

Direction A 3a-1 is a parameter-exchange method for inter-vendor training collaboration in two-sided AI/ML CSI feedback.
Its goal is to let a network vendor train an encoder–decoder model and then share only the trained encoder parameters and structural metadata with UE vendors, while keeping proprietary CSI data private.
No target-CSI samples are exchanged.

The procedure begins with network-side joint training of an encoder and decoder using network data.
Once training is complete, the network exports the encoder’s weights and any necessary model-structure information.
A UE vendor then imports these parameters and trains its own decoder locally, using only UE-collected channel data, to reconstruct the CSI from the shared encoder outputs.
After freezing this decoder, the UE fine-tunes its own encoder so that the final UE encoder–decoder chain performs optimally under real operating conditions.
Inference in the field then runs with the UE’s trained encoder and a compatible network decoder, all without exchanging raw CSI.

This approach provides a secure and efficient path to interoperability.
Because every vendor uses the same standardized encoder structure specified by RAN4, the shared parameters can be interpreted unambiguously.
The UE’s locally trained decoder guarantees compatibility with the network’s decoder family, while allowing each vendor to adapt to its own antenna layouts, RF chains, and processing preferences.
Direction A 3a-1 thus enables joint or semi-joint model development across vendors, meets performance and RAN4 requirements, and preserves data privacy, fulfilling a key part of the Release-20 AI/ML work on inter-vendor training collaboration.

Fully specified test decoder with standardized reference encoder for Option 3 testing and Direction C field use, supporting Direction A encoder-parameter exchange

Investigate feasibility of one RAN4-specified test decoder together with a fully specified reference encoder structure and parameters that serves multiple purposes:
(1) provides the Option 3 lab conformance benchmark,
(2) acts as the Direction C standardized reference side for field deployments, and
(3) supplies the standardized encoder structure and parameter format needed for Direction A 3a-1 inter-vendor collaboration (which does not require a standardized decoder).

- Complete model specification – Specify the test-decoder design and trained weights plus the full reference-encoder structure
Applicability: (1) ensures reproducible Option 3 testing; (2) provides the reference side for Direction C; (3) provides a standardized encoder architecture and a common parameter format so that encoder weights can be exchanged and interpreted unambiguously for Direction A parameter exchange.

- Unified test and field use – Allow the same decoder/encoder pair to serve as the Option 3 test object and as the Direction C field reference, while the standardized encoder structure alone enables Direction A training collaboration without mandating a decoder.

Study LCM requirements on KPIs and triggers for model lifecycle actions for CSI compression in inter-vendor collaboration

RAN4 should investigate LCM requirements for AI/ML CSI-compression models, covering when to activate, switch, or fall back

- Clarify key performance indicators (KPIs) to be observed, ensuring they reflect both end-to-end link quality

- Define the principles for decision criteria, such as how KPIs indicate stable improvement or persistent degradation before a model change.

- RAN4 to discuss requirements for minimum observation and hold times to ensure KPI issues persist and new models run long enough, preventing rapid toggling that could disrupt scheduling or RRM performance.

- Outline monitoring cadence expectations, balancing early detection of issues with acceptable signaling and processing load.

- Specify how these high-level requirements will be tested and validated in RAN4 conformance procedures.

# Conclusion

In the previous sections we made the following observations:

Discussion on Scalability:

- Scalability Method for Transformer-Based CSI Compression: RAN4 should standardize a single baseline backbone to avoid model fragmentation. Alt 2 (shared embedding with padding/masking) is proposed as the default scalability framework, while Alt 1 (geometry-aware adapters) remains an optional enhancement for vendors seeking higher accuracy in geometry-sensitive cases.

- Baseline (Alt 2): shared embedding, zero-padding for Tx scaling, token masking for subbands, 
• Optional (Alt 1): geometry-specific adapters and payload-specific heads.
• Result: one interoperable framework, with optional performance boosts where needed.

Discussion on Dataset:

For CSI compression, RAN4 shall standardize the dataset format as eigenvector-based channel representation derived through a deterministic procedure: per-slot/per-subband covariance matrix construction → rank selection based on signal subspace energy threshold → eigenvalue decomposition → eigenvector basis ordering (descending eigenvalue magnitude) → phase unwrapping/alignment → amplitude normalization

Mixed SLS + LLS Dataset for Reference Model Training: To ensure that the standardized AI/ML-based CSI compression model performs consistently in both laboratory tests and real-world deployments, RAN4 should adopt a mixed-domain training dataset combining System Level Simulation (SLS) and Link Level Simulation (LLS)

RAN4 shall adopt the TDL channel model under LLS as the reference for defining and verifying AI/ML-based CSI compression performance requirements.

Unified Mixed Dataset for Channel Domain and Scalability Coverage: RAN4 should establish a single unified mixed dataset that merges both channel-domain diversity (SLS + LLS) and scalability configuration diversity (Tx ports, subbands, and payload sizes)

Unified Dataset: Combine LLS (TDL/rCDL) for calibration and SLS (UMa/Umi) for field realism in one reference

Scalability Coverage: Include multiple (N_TX, SB, K) configurations to expose the model to variations in geometry, frequency granularity, and compression ratios.

Efficient Training: Use configuration-tagged batches to train a single scalable model that learns invariances across domains and configurations.

Single Benchmark: Avoids separate datasets for each use case, ensuring interoperability, reproducibility, and simplified testing within one aligned mixed-dataset framework.

Discussion on Number of Test Decoders:

Single Backbone Decoder with Scenario-Specific Adapters: To avoid storing multiple full models, RAN4 maintains one frozen backbone decoder and attaches lightweight adapters trained per PSE-clustered scenario. The backbone defines the interoperable core; adapters specialize it for new environments without retraining or replacing the core model.

- Freeze one backbone + reference encoder as the 3GPP baseline for all scenarios.

- Define a common adapter architecture trained on each cluster’s dataset.

- Store only backbone and adapter spec; new scenarios release new datasets, not new frozen decoders.

RAN4 shall retain a single reference encoder per frozen test decoder, selecting the lowest-complexity encoder that still meets the agreed performance target; additional reference encoders would be justified only if RAN4 explicitly defines distinct performance tiers tied to UE capability classes.

If RAN4 defines distinct performance tiers, RAN4 needs to set the exact convergence metric (SGCS, NMSE) define the capability tiers and their minimum performance floors .

Discussion on Standardized Dataset:

Standardized Dataset Collection for Test Decoder Training

RAN4 should ensure every test decoder is trained on realistic, interoperable data:

- Simulation framework: Define common frequency bands, environments, antenna setups, and SNR distributions.

- Dataset alignment: Verify consistency across vendors using metrics such as power spectral entropy (PSE).

- Outcome – Guarantee that the trained test decoder reflects true wireless channel scenarios and provides a common latent space for interoperability.

Discussion on Dataset Alignment:

PSE-Based Dataset Alignment Methodology

RAN4 should adopt a power spectral entropy (PSE) method to verify cross-vendor dataset consistency:

- Raw channel requirement – Provide raw CSI/channel data so that each CSI snapshot can be converted into a common beamspace-delay power map for PSE calculation.

- PSE computation – Compute PSE to quantify how widely signal power is spread across beams and delays.

- Statistical comparison – Treat PSE values from different datasets as distributions and compare them with distance metrics such as Wasserstein distance.

- Alignment criterion – Datasets are considered aligned if the measured distances stay below agreed thresholds, ensuring consistent training data for standardized test decoders.

– PSE for Online Performance Monitoring and OOD Detection

RAN4 should use power spectral entropy (PSE) as a live KPI to detect input-distribution drift and monitor model performance:

- Continuous PSE computation – Calculate PSE in real time from live CSI feedback during network operation.

- Reference PSE envelope – Each deployed CSI compression model carries a PSE envelope derived from its training dataset.

- OOD detection – Compare live PSE statistics to the reference envelope; persistent deviations signal out-of-distribution (OOD) conditions.

- LCM action triggers – Detected drift can trigger model retraining, adapter updates, or switching to a more suitable model.

- Unified methodology – The same PSE framework used for dataset alignment in certification ensures consistency between offline validation and online performance monitoring.

Discussion on Feasibility:

Fully specified test decoder with standardized reference encoder for Option 3 testing and Direction C field use, supporting Direction A encoder-parameter exchange

Investigate feasibility of one RAN4-specified test decoder together with a fully specified reference encoder structure and parameters that serves multiple purposes:
(1) provides the Option 3 lab conformance benchmark,
(2) acts as the Direction C standardized reference side for field deployments, and
(3) supplies the standardized encoder structure and parameter format needed for Direction A 3a-1 inter-vendor collaboration (which does not require a standardized decoder).

- Complete model specification – Specify the test-decoder design and trained weights plus the full reference-encoder structure
Applicability: (1) ensures reproducible Option 3 testing; (2) provides the reference side for Direction C; (3) provides a standardized encoder architecture and a common parameter format so that encoder weights can be exchanged and interpreted unambiguously for Direction A parameter exchange.

- Unified test and field use – Allow the same decoder/encoder pair to serve as the Option 3 test object and as the Direction C field reference, while the standardized encoder structure alone enables Direction A training collaboration without mandating a decoder.

Discussion on LCM:

Study LCM requirements on KPIs and triggers for model lifecycle actions for CSI compression in inter-vendor collaboration

RAN4 should investigate LCM requirements for AI/ML CSI-compression models, covering when to activate, switch, or fall back

- Clarify key performance indicators (KPIs) to be observed, ensuring they reflect both end-to-end link quality

- Define the principles for decision criteria, such as how KPIs indicate stable improvement or persistent degradation before a model change.

- RAN4 to discuss requirements for minimum observation and hold times to ensure KPI issues persist and new models run long enough, preventing rapid toggling that could disrupt scheduling or RRM performance.

- Outline monitoring cadence expectations, balancing early detection of issues with acceptable signaling and processing load.

- Specify how these high-level requirements will be tested and validated in RAN4 conformance procedures.

# References

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, Qualcomm.