# 3GPP TSG-RAN WG4 Meeting #117	                                                           R4-2520624

# Dallas, USA, Nov. 17-21, 2025

Source:	Apple

Title:	Discussion on AI/ML-Based 6G Use Cases

Agenda item:	8.8.2

Document for:	Discussion

Introduction

This paper presents candidate AI/ML-based use cases for 6G RRM aimed at reducing measurement gaps, lowering UE power consumption, boosting throughput, and strengthening mobility robustness. Building on 3GPP studies in TR 38.744 [1] and TR 38.843 [2], it extends prediction from cell-level to beam- and multi-domain L3 RSRP across time, frequency, and spatial dimensions, and further advances spatial-domain prediction in beam management across Rx spatial and temporal domains.

Discussion On AI/ML use case for RRM

During RAN4#116 bis the following agreements were reached:

Agreement:

- Additional principle/method for AI-RRM use case selection

  - Whether the proposed AI-RRM use case should be driven by RAN4:

    - Existing 5G-A use cases included in RAN2 and RAN1 WIs should be precluded in RAN4 6G study.

    - Coordination with RAN1 and RAN2 when potential AI-RRM use cases are agreed in RAN4 to avoid overlapping scope.

##### Issue 6-1a: Use case definition and supporting level from companies

Agreement:

- The scope of potential RAN4-led AI-RRM use case(s) will be further discussed in the next meeting.

Figure 1: Generic view of AI/ML RRM and measurement reductions.

Figure 2 presents a structured view of L3 mobility prediction for 6G radio-resource management, organizing the study areas by domain (time, spatial, frequency) and Step level. It distinguishes between L3 beam-level RSRP prediction (left branch) and L3 cell-level RSRP prediction (right branch), showing how each evolves through increasingly complex multi-domain correlations. The framework is designed to guide RAN4’s study sequencing, ensuring that simpler, high-impact investigations are completed first before expanding into broader, cross-domain prediction cases.

Figure 2a. Structured view of L3 mobility prediction for 6G

### Time-Domain Beam-Level Prediction (Intra-Cell)

In this use case, the AI/ML model learns how a beam’s L3-RSRP evolves over time, without changing the beam set or the serving cell. The input to the model is a sequence of historical L3 beam-level measurements for a subset of beams (Set B) at earlier time instances (e.g., t-3, t-2, t-1). Using this temporal history, the model predicts the future L3-RSRP of the same beams at a future time (e.g., t+Δt) without requiring the UE to perform new measurements.

- Input:
L3-filtered RSRP values of beams in Set B at previous time steps
(e.g., [RSRP_b1(t-3), RSRP_b1(t-2), RSRP_b1(t-1)] … for all beams in Set B)

- Output:
Predicted L3-RSRP for the same Set B beams at a future time instance
(model does not predict new beams, only future values of existing beams)

### Spatial Inter-Cell Beam Prediction (Beam-Level)

In this use case, the AI/ML model learns to predict the best downlink transmit beam(s) across multiple neighboring cells, based only on a partial set of measured beams. Unlike Rel-18/19 intra-cell prediction (where input and output beams belong to the same serving cell), this case extends prediction across M cells, enabling the UE or network to identify the optimal beam,  and possibly the optimal target cell,  without sweeping all beams in every cell.

### Model Input (Set B – Measured Beams)

- L3 filtered RSRP measurements of a subset of beams from M cells

- Example: 4 beams per cell × M cells → 4 × M input beams

- Beam-level features only (no time history for this spatial-only sub-case)

- Each input sample corresponds to:

- (cell ID, beam ID, measured RSRP)

- Purpose: reduce measurement overhead (do not sweep all beams in each cell)

### Model Output (Set A – Predicted Beams)

Two possible output modes

| Mode | Output Meaning | Example Output |
| --- | --- | --- |
| Global Top-1 / Top-K | Best beam across all cells | (best_cell, best_beam, predicted_RSRP) |
| Per-Cell Top-1 | Best beam per cell | [ (cell-1, beam-3), (cell-2, beam-6), ... ] |

- Output set contains full beam set per cell: Set A = 8 × M beams (in example)

- Model outputs beam index + predicted L1-RSRP

- Can support Top-1, Top-K, or ranked beam list depending on use case

### Inter-Frequency + Inter-Cell (Non-Collocated) Beam Prediction

In this scenario, the UE measures a subset of beams on Frequency-1 (f₁) from Cell-A, and the AI/ML model predicts the best beam(s) on Frequency-2 (f₂), which may belong to a different physical cell/TRP (e.g., Cell-B or Cell-C). Unlike the classical CA case where beams on f₁ and f₂ are co-sited or share similar codebooks, the non-collocated case has no guaranteed alignment in beam index, direction, array geometry, or SSB/CSI-RS mapping. Therefore, the model must learn both spatial and frequency-domain correlation simultaneously.

This case is harder than standard inter-cell or inter-frequency prediction, because both dimensions change:

- Not the same frequency

- Not the same cell/TRP

- Not the same beam codebook / array pattern

- No natural one-to-one mapping between beams

The model must answer the question:

“Given the spatial pattern I observe on f₁ from Cell-A, what is the most likely strongest beam on f₂, possibly from Cell-B or Cell-C?”

### Model Input (Set B – Measured Beams on f₁)

| Element | Description |
| --- | --- |
| L3-RSRP of subset beams on f₁ | e.g., 8 beams from Cell-A (same as Rel-19 setup) |
| Beam-level features | (cell ID, beam ID, f₁ frequency, RSRP) |
| Optional time history | if temporal prediction is included |
| Optional metadata | UE location class, mobility state, Doppler, etc. |
| Assistance information | describes how beams on f₁ relate to beams on f₂ |

Total input size example: 8 beams × 1 cell (or more)

### Model Output (Set A – Predicted Beams on f₂)

| Mode | Output |
| --- | --- |
| Top-1 / Top-K | Best beam(s) on f₂, may belong to Cell-B or Cell-C |
| Output fields | (predicted_cell, predicted_beam_index, predicted_RSRP) |
| Output space size | e.g., 32 beams across 2–3 cells on f₂ |

In the non-collocated inter-frequency case, there is no inherent one-to-one relationship between the beams on f₁ and the beams on f₂, because the cells may use different antenna arrays, different beam codebooks, and different physical orientations. As a result, the model cannot “learn” the mapping unless it is given some form of assistance information that provides structural clues about how the two beam spaces relate. This assistance may include deployment information (e.g., relative TRP positions or sector orientation), beam/codebook characteristics (e.g., wide vs. narrow beams, analog vs. hybrid panels), array metadata (e.g., panel count, element spacing, downtilt settings), frequency-dependent scaling factors (e.g., pathloss offset, wavelength ratio), or even UE-side characteristics (e.g., single-panel vs. multi-panel UE). Without such side information, the problem becomes mathematically non-identifiable, because the model would be attempting to learn a mapping between two unrelated beam spaces with no shared reference, making reliable prediction impossible.

### Time + Spatial Inter-Cell Beam Prediction

This use case combines temporal prediction (how beam quality evolves over time) with spatial prediction (which beam/cell will be best among multiple neighboring cells). Unlike the pure spatial case, which predicts the best beam “now,” this version predicts the best future beam(s) across cells, based on historical multi-cell measurements. It is therefore a joint learning problem across time, space, and cells.

### Model Input

| Component | Description |
| --- | --- |
| Beam measurements | L3 RSRP history of a subset of beams (Set B) from M cells |
| Time horizon | Sliding window of past measurements (e.g. last 3–5 samples) |
| Total input size | (# beams in Set B × # cells × # time steps) |
| Optional fields | beam index, cell ID, UE speed/angle, Doppler, timing info |
| Purpose | Capture both temporal trend (future prediction) and spatial correlation (cross-cell behavior) |

Example input tensor:

RSRP[cell, beam, time]  →  [M cells × B beams × T history samples]

### Model Output

Two output modes are possible:

| Output mode | Meaning |
| --- | --- |
| Global Top-1 / Top-K | Predict the best beam across all cells at future time t+Δ |
| Per-cell Top-1 | Predict the best beam per cell at future time t+Δ |

Output fields include:

(predicted_cell_ID, predicted_beam_ID, predicted_future_RSRP)

So the Ai/ml mapping is described as follows:

Input  = Past RSRP of Set B beams across M cells

Output = Future best beam(s) from full Set A beams across M cells

### The Model learns the following correlations:

- Temporal evolution of beam strength (mobility, fading, blockage, Doppler)

- Spatial correlation between cells (how beams of neighboring cells relate)

- Early mobility cues → future serving/target cell decision

The benefits of this prediction are described below:

- Reduces measurement overhead: only a subset of beams is measured instead of sweeping all beams across all cells

- Reduces time overhead: prediction replaces repeated measurement

- Reduces mobility latency: the network/UE can make handover or beam-switch decisions before the next measurement reporting period

- Enables proactive mobility: allows early beam/cell selection before degradation or RLF occurs, instead of reacting to it

- Supports seamless mobility at high speed: useful when channel conditions evolve faster than the measurement/reporting periodicity

### Frequency + Spatial Inter-cell Beam Prediction

This use case refers to AI/ML-based prediction where both the frequency layer AND the cell/TRP change between input and output. The UE measures only a subset of beams on Frequency-1 (f₁) from Cell-A, and the model predicts the best beam(s) on Frequency-2 (f₂), which may belong to Cell-B or Cell-C. Because the source and target beams come from different cells and different frequency layers, there is no guaranteed alignment of beam index, codebook structure, antenna array geometry or SSB/CSI-RS mapping.

This case is exactly equivalent to previous case called “inter-frequency, non-collocated beam prediction.”
In other words:

Frequency + Spatial Inter-cell  ≡  Inter-frequency, non-collocated prediction

It is the most general and most difficult version of beam prediction, because the model must jointly learn:

- Frequency-domain correlation → IF f₁ beam is strong, what does that imply on f₂?

- Spatial / inter-cell correlation → IF Cell-A beam looks like this, which beam of Cell-B or Cell-C is likely to be best?

- Cross-domain mapping → no beam-to-beam alignment unless assistance information is provided.

### Model Input

RSRP of Set-B beams on f₁ from Cell-A

+ Assistance information (deployment geometry, beam class, array metadata, frequency offset, etc.)

### Model Output

Top-K predicted beams on f₂ from Cell-B / Cell-C

(predicted_cell_ID, predicted_beam_ID, predicted_RSRP)

Figure 2b. Frequency domain prediction categories

### Time + Frequency + Spatial Beam Prediction

This use case extends all previous prediction modes by combining temporal evolution, cross-frequency generalization, and inter-cell spatial correlation into a single AI/ML model. The UE measures only a subset of beams on Frequency-1 (f₁) from one or more cells over a history window (past time steps), and the model predicts the future best beam(s) on Frequency-2 (f₂), possibly belonging to a different cell/TRP.

It is effectively the union of three dimensions:

| Dimension | Meaning |
| --- | --- |
| Time | Predict future beam strength instead of current |
| Frequency | Predict beam(s) on another carrier (f₂) |
| Spatial / Inter-cell | Predicted beam may come from another cell/TRP |

This is the most complete form of AI-based mobility/beam management useful for proactive handover, FR1→FR2 offloading, CA activation, and multi-layer mobility.

### Model Input

| Component | Description |
| --- | --- |
| RSRP history | Time sequence of L3 RSRP for subset beams on f₁ |
| Cells | Measurements may come from Cell-A only or multiple cells |
| Time horizon | Sliding window (e.g., last 3–5 samples per beam) |
| Assistance information | deployment geometry, frequency scaling, codebook class, TRP metadata, |
| Optional features | UE speed, Doppler, beam index, PCI |

Tensor-like input example:

RSRP[cell, beam, time] on f₁  →  [M cells × B beams × T samples]

+ Assistance Information vector

### Model Output

| Output type | Description |
| --- | --- |
| Top-1 / Top-K beams | Predicted future best beam(s) on f₂ |
| Target cell | Cell-B or Cell-C may differ from input cell |
| Predicted metric | Future L3-RSRP (or probability score) |

So the mapping is:

Input:   Past measurements on f₁ (subset beams, Cell-A, time history)

Output:  Future best beams on f₂ (full beam set, Cell-B/C)

A general description of Cell-A, Cell-B, Cell-C is shown below. The tables clarify the roles of Cell-A, Cell-B, and Cell-C in AI/ML-based mobility prediction and show how different deployment scenarios map across cells and frequency layers. Cell-A represents the serving cell where the UE performs measurements and where model inputs originate. Cell-B and Cell-C represent candidate neighbor cells that may become the next serving or secondary cell, with the model predicting their future signal quality even before actual measurements occur.

The second table shows how prediction complexity increases as we move from single-cell, single-frequency (classic intra-cell beam management), to multi-cell same-frequency (inter-cell beam prediction), to single-cell multi-frequency(collocated CA), and finally to multi-cell, multi-frequency non-collocated, the most general and challenging case, where prediction spans time, frequency, and spatial domains simultaneously.

### These are the correlations that the AI/ML Model Learns Jointly:

| Learns | Example of behavior it captures |
| --- | --- |
| Temporal evolution | Beam fading & mobility trend → future prediction |
| Cross-frequency mapping | If beam X is strong on f₁, which beam is likely strong on f₂? |
| Inter-cell spatial correlation | If Cell-A beam weakens, which neighbor cell (B/C) is likely stronger? |
| Cross-domain mobility triggers | Predict not just strongest beam, but strongest future cell on another frequency |

The benefits of this prediction are described below. The table highlights the key benefits of AI/ML-based prediction for mobility and beam management, showing how it reduces both measurement delay and signaling overhead while enabling faster and more proactive mobility decisions. By predicting cell and beam quality across time, frequency, and cells, the approach eliminates the need for repeated reference signal measurements, supports seamless FR1→FR2 mobility, and allows a single prediction step to replace multiple classical measurement procedures. This makes it especially valuable for heterogeneous deployments such as macro-to-hotspot, relay, NTN, or multi-band CA scenarios.

The table below categorizes the different AI/ML beam and cell prediction scenarios based on whether the input and output belong to the same cell/TRP, the same frequency layer, and the same beam/codebook structure. This classification highlights how complexity increases as prediction moves from simple intra-cell cases to full frequency + spatial inter-cell scenarios. The final row represents the most general and challenging case, where neither the cell, frequency, nor codebook is shared between input and output.

AI/ML-Assisted L1 Rx-Beam Prediction for L3 Beam-Level Reporting

In many mobility scenarios (e.g., multi-TRP deployments, CFRA access, network-controlled beam reselection, FR2 mobility, CA beam correlation, etc.), the network requires beam-level information, not just a single cell-level RSRP value. This means the UE may need to report X best beams per cell, which in turn requires it to first measure all Rx beams per Tx beam at L1.

Today, this requires sequential L1 beam sweeps → high RF retuning overhead, especially in inter-frequency mobility where measurements happen inside measurement gaps.
As the number of Rx beams increases the number of required gaps grows proportionally.

This case introduces AI/ML to reduce the number of measured Rx beams while still generating a full beam vector for L3 reporting.

The AI/ML Model I/O is described below:

| Element | Description |
| --- | --- |
| Input | Partial Rx-beam measurements (e.g., 2 measured out of 8) for each Tx beam |
| Prediction Target | Full Rx-beam vector (per Tx beam) |
| Output to L3 (A¹) | K Tx beams, each collapsed to its best Rx beam (measured + predicted) |
| Consumes Output | Beam consolidation (cell-level), L3 beam filtering (beam-level) , Beam selection for reporting (X beams) |
| Visible to network? | No. Only A¹, C, and F are visible as in baseline. |

Figure 2c. AI/ML-Assisted L1 Rx-Beam Prediction for L3

Time + Frequency + Spatial domain Tx-Rx prediction for L3 beam level mobility

This corresponds to the most advanced form of AI/ML beam prediction, where the model jointly learns:

- temporal evolution of beams

- cross-frequency correlation between layers (f₁ ↔ f₂)

- spatial structure across cells and beams (Tx beams × Rx beams)

and produces future beam predictions rather than just filling missing values.

This use case represents the most general and most capable form of AI/-assisted prediction within the L3 beam-based mobility framework. Unlike simpler variants that operate in only one dimension (e.g., spatial prediction for Rx-beam completion, or frequency-domain prediction for inter-frequency mobility), this case allows the algorithm to jointly learn how beam quality evolves simultaneously over time, across carrier frequencies, and across cells and beams (Tx-Rx pairs). In practical terms, the UE provides the AI/ML model with a short history of partial beam measurements:e.g., a sliding window over the last N reporting intervals, possibly across multiple frequencies and cells,and the model produces a multi-dimensional prediction of the future beam state, not only for the serving cell but also for candidate handover or carrier aggregation cells.

As a result, the UE can predict which beams (and at which frequency layer) will be the strongest at a future time t + Δ, enabling the network to trigger handover, carrier activation, or TRP reselection before the channel degradation is physically measured.

Model Inputs:

| Dimension | Example Input Data |
| --- | --- |
| Time | Past K measurement windows |
| Frequency | L1 beam RSRP from f₁ + f₂ (intra + inter-frequency) |
| Spatial (Tx) | Per-beam RSRP per gNB Tx beam |
| Spatial (Rx) | Partial Rx-beam samples per Tx beam |

| Output Type | Meaning |
| --- | --- |
| Future beam RSRP per (Tx,Rx) pair | Full predicted beam cube at time t+Δ |
| Top-X beams per cell (for reporting at F) | Input to beam selection block |

## What the AI/ML Model Learns

The model is not just learning a single mapping (e.g., “beam 4 on f₁ → beam 6 on f₂”).
Instead, it learns joint structure:

- Temporal beam evolution

  - How beams drift, rotate, or fade due to motion and channel changes

  - Predicts the future best beam before the measurement happens

- Cross-frequency mapping

  - Learns how the spatial field at f₁ correlates with f₂, even when cells are not co-located

  - E.g., mmWave beams change faster and are narrower, but stay aligned with sub-6 beams in direction space

- Multi-cell spatial correlation

  - Beams from Cell-A, Cell-B, Cell-C are not independent; reflections and geometry create shared structure

  - The “beam cluster” across neighbors is a more stable predictor than any single beam

- Tx-Rx structure

  - Learns which Rx beams respond strongest to each Tx beam in a given propagation environment

  - Allows partial measurement (subset of Rx beams) + inference of missing beams

- Mobility-driven prediction

  - Predicts not only “what is best now” but “what will be best soon,” enabling mobility anticipation

Benefits of Full-Dimension Prediction (Compared to single-domain)

| Dimension | Traditional Measurement | AI/ML Full-Dimension Prediction |
| --- | --- | --- |
| Time | Reacts after degradation | Predicts before degradation |
| Frequency | Needs extra gaps & CSI search | Predicts beams on f₂ from f₁ |
| Spatial (Tx-only) | Requires full sweep of beams | Predicts best beam per cell |
| Spatial (Tx-Rx pairs) | Needs Rx sweep per gap | Predicts missing Rx beams |
| Multi-cell | Measures neighbors one-by-one | Predicts best beam across group |
| Signaling impact | Multiple reports | One report replaces many |

### Frequency-domain Inter-cell L3 cell-layer Prediction (non-collocated)

In this use case, the AI/ML model predicts the L3-filtered RSRP of a target cell on another frequency layer (f₂) without the UE performing actual measurements on that frequency. Unlike single-cell input approaches, the prediction model takes input from a cluster of cells on the source frequency (f₁) not just the serving cell. This cluster may include Cell-A (serving), plus Cell-B, Cell-C, etc., that are spatially correlated but all operating on the same frequency layer.

In a non-collocated scenario, the serving cell on frequency f₁ and the target cell on frequency f₂ belong to different physical TRPs, sectors, or deployment layers. Since there is no guaranteed alignment in beam index, antenna geometry, transmit power, or codebook structure between the two frequency layers, the UE cannot rely on a simple 1:1 mapping between cells or beams. Instead, the prediction model must infer the expected L3-filtered RSRP on f₂ purely from the spatial “fingerprint” observed on f₁.
To make this inference robust, the model does not rely on a single serving-cell input but on a cluster of measurements from multiple cells on the same frequency layer (f₁). This cluster captures the spatial propagation context more accurately than any single cell and enables the model to learn cross-frequency correlations statistically rather than geometrically.

This approach is needed because in non-collocated deployments, the target cell on f₂ is not guaranteed to have any geometric, beam, or codebook alignment with the cells on f₁. The correlation must instead be learned statistically using spatial patterns observed across multiple cells at f₁.

This reduces measurement gaps, SSB overhead, and mobility latency.

In non-collocated deployments, the serving cell on frequency f₁ and the target cell on frequency f₂ do not share any guaranteed structural relationship. Unlike collocated CA scenarios where the same gNB panel may serve both carriers, and beams at f₁ and f₂ may follow similar boresight directions or array layouts in non-collocated deployments the cells may belong to different sites, use different antenna geometries, have independent downtilts, and operate with completely different beam codebooks. This means there is no deterministic mapping such as “beam #3 on f₁ → beam #3 on f₂,” nor even a reliable assumption that the strongest cell on f₁ corresponds to the strongest cell on f₂.

As a result, the relationship between f₁ and f₂ cannot be defined analytically or through a fixed geometric transformation. Instead, the correlation must be learned statistically from data. The model must infer, for example, that “when this combination of L3-RSRP values is observed from Cell-A, Cell-B, and Cell-C on f₁, then Cell-X on f₂ is likely to be the strongest,” even though the UE has never directly measured f₂ at that moment. This becomes possible only when the AI/ML model is given multi-cell input, because the joint spatial fingerprint across several cells contains far more predictive information than a single-cell snapshot.

This is precisely why the cluster-based approach is required. A single serving-cell measurement on f₁ represents only one spatial view of the radio environment and can easily mislead the model if that cell is temporarily weak due to fading, blockage, beam misalignment, or simply because the UE is closer to a different sector. A cluster of cells, however, provides a richer set of spatial cues: even if one cell experiences a transient dip, the others may still reflect the underlying large-scale pathloss, angular structure, or UE trajectory. The AI model can therefore learn a more stable and generalizable mapping between f₁ and f₂.

In essence, the cluster is a proxy for spatial diversity: it allows the model to reconstruct the radio environment more accurately than any single cell can. This is especially important in heterogeneous deployments (macro + micro + mmWave hotspot + relay, etc.), where no single cell fully describes the UE’s surroundings. The learned correlation between frequencies emerges not from geometry or beam index alignment, but from statistical consistency across many cells and many UEs exactly the type of information neural models can exploit when trained on large datasets.

Even when clustered measurements are used as input, the AI/ML model still does not have direct visibility into key deployment characteristics that influence how signal strength on f₁ correlates with signal strength on f₂. In non-collocated scenarios, the target cell on f₂ may differ in frequency band, antenna height, array geometry, beamwidth, downtilt, transmit power, or propagation domain (macro ↔ hotspot ↔ indoor ↔ relay ↔ mmWave). Because these factors are not observable from RSRP values alone, the model needs side information—a compact set of descriptors that tell it whycertain frequency mappings behave the way they do. Assistance Information provides this missing semantic context, enabling the model to learn conditional correlations instead of blindly fitting statistical patterns.

Cluster-Based Input (f₁ Layer)

| Input Element | Description |
| --- | --- |
| L3-RSRP values from a cluster of cells on f₁ | {Cell-A, Cell-B, Cell-C …} (serving + neighbors) |
| Optional beam-level features | strongest beam index, Top-K beam distribution per cell |
| Optional UE context | Doppler class, speed, mobility state, UE antenna type |
| Assistance information (optional but useful) | Δf, deployment type, co-siting flag, TRP class, power class |

So the model input is:

Input = { L3_RSRP(Cell-A @ f₁),

L3_RSRP(Cell-B @ f₁),

L3_RSRP(Cell-C @ f₁), ... }  (+ assistance info)

Model Output (f₂ Layer)

| Output Element | Description |
| --- | --- |
| Predicted L3-filtered cell RSRP on f₂ | e.g., Predicted L3_RSRP(Cell-X @ f₂) |
| Optional output | Top-K ranked cells on f₂, confidence score, fallback indicator |

Mapping becomes: Cluster at f₁  →  Predicted target cell(s) at f₂

Frequency-domain prediction eliminates the need for the UE to perform real-time measurements on the target frequency layer, allowing the network to anticipate which inter-frequency cell will provide the strongest coverage before any measurement gaps, beam sweeps, or SCell configuration take place. By replacing reactive measurement-based mobility with predictive mobility, the approach removes delay, reduces signaling and RS overhead, and enables faster, smoother inter-frequency handovers and CA activation: especially in FR1→FR2 transitions where measurement gaps are costly and beam searches are expensive. When combined with a cluster-based input design, the model becomes more accurate, more robust to fading or anomalies, and more scalable across heterogeneous deployments

The key benefits are listed below:

### Key Benefits

- Eliminates inter-frequency measurement gaps → no need to pause data transmission to scan f₂

- Reduces RRM reporting latency → prediction is instant, measurement has more delay

- Enables proactive mobility → target cell on f₂ can be prepared before serving cell degrades

- Cuts RS overhead and UE energy consumption → no need to activate RF chains on f₂ unless needed

- Accelerates CA and SCell addition → prediction replaces multi-step measurement + filtering + reporting

- Scales to multi-band systems → inference cost stays constant even as number of carriers grows

- Improves robustness in heterogeneous networks (macro + hotspot + relay + mmWave) via cluster input

- Allows early FR1→FR2 offloading without full mmWave sweep, reducing beam training and access delay

- Supports simplified mobility logic where the network selects the best band/cell based on predicted RSRP

- Creates a foundation for AI-based multi-layer mobility in 6G (prediction instead of measurement-driven mobility)

Joint Time + Frequency L3 cell-level prediction

Joint time + frequency AI/ML prediction enables the UE to anticipate the future RSRP of a secondary-frequency cell before performing any inter-frequency measurement. This removes the need for repeated measurement gaps, eliminates the L3 filtering latency, and allows the network to trigger mobility or CA activation proactively rather than reactively.

The UE predicts the future RSRP of a target cell on a different frequency layer (f₂) based on the historical RSRP evolution on the serving frequency (f₁), without actually measuring f₂ yet.

| Dimension | Meaning |
| --- | --- |
| Time | Predict RSRP at t + Δ (future, not yet filtered) |
| Frequency | Predict cell RSRP on f₂ (not yet measured) |
| Spatial / cluster | Use multiple cells on f₁ instead of only serving cell |

Inputs to the AI/ML Model (joint time + frequency)

| Input Type | Example Signals | Source |
| --- | --- | --- |
| Time history (f₁) | L3-RSRP(t-4…t), L3-RSRQ, SINR trend | Serving cell & neighbors on f₁ |
| Spatial / cluster inputs | RSRP of Cell-A/B/C on f₁ | Intra-frequency |
| Mobility context | TA evolution, Doppler, speed class | PHY / MAC |

Outputs of the AI/ML Model

| Output | Format | Consumed by |
| --- | --- | --- |
| Predicted RSRP of Cell-X@f₂ at t + Δ | 1 scalar in dBm | Feeds point C (L3 filtered output) |
| Optional RSRP slope | ΔdB/100 ms | Allows TTT skipping |

The Model Learns the following correlations:

| Correlation Type | Meaning |
| --- | --- |
| Temporal | RSRP decay/growth trend (UE moving, blockage, fading) |
| Inter-cell spatial | How Cell-A / Cell-B behavior predicts Cell-C |
| Cross-frequency mapping | How conditions on f₁ imply future f₂ quality |
| Lead-time compensation | Predicting before UE reaches coverage edge |

Below we describe the associated benefits:

#### Measurement Gap Reduction

- UE measures only on f₁, predicts RSRP on f₂ → fewer gaps

- Gaps triggered only when prediction confidence is low, not periodically

- One f₁ multi-cell cluster input replaces multiple f₂ scans (no Rx-beam sweeps on f₂)

#### Mobility Reliability / Precaution

- Predicted RSRP + slope enables early handover before link degrades

- Slope (dB/100 ms) allows adaptive / skipped TTT instead of fixed timers

Time + Frequency + Spatial Tx-Rx Domain (multi-dimensional) L3 cell level

This is the most advanced form of AI-assisted mobility prediction, where the model does not only predict future cell-level RSRP, but also the future full beam cube across time, frequency, and spatial (Tx/Rx) dimensions.

The model learns the following correlations:

| Domain | What is learned | Why it matters |
| --- | --- | --- |
| Time | Temporal evolution of RSRP / beam ranking | Enables proactive mobility (HO before degradation) |
| Frequency | Mapping from f₁ beams/cells → f₂ beams/cells | Eliminates or reduces inter-frequency gaps |
| Spatial (Tx-Rx) | Joint response of Tx beams + UE Rx beam patterns | Enables beam-level reporting without full sweep |
| Cross-domain coupling | How time trend at f₁ predicts spatial structure at f₂ | Key for non-collocated deployments |

Below we describe the model inputs:

| Input Type | Example Data | Notes |
| --- | --- | --- |
| Temporal history | Last N RSRP/L1,3-cell snapshots | Time correlation |
| Multi-cell cluster features | Serving + neighbor cells on f₁ | Improves spatial robustness |
| Beam grid (Tx × Rx) | K Tx beams × M Rx beams (partial or full) | May include measured + predicted beams |
| Assistance Info | TRP geometry class, codebook type, Δf, scaling | Needed for non-collocated cells |
| Mobility context | Speed class | Optional but increases stability |

Below we describe the model outputs:

| Output Type | Used By | Purpose |
| --- | --- | --- |
| Full predicted Tx×Rx beam cube | Point E/F path | Beam-level mobility / BF reporting |
| Top-X beams per cell | Beam Selection block | No L1 sweep needed on f₂ |
| Predicted cell-level RSRP at f₂ | L3 event logic (A3/A5/A6) | Replaces gap-based measurements |
| RSRP slope & confidence | TTT skip / fallback / safety logic | Reliability guardrail |

### Key Benefits of Multi-Domain (Time + Frequency + Spatial Tx-Rx) Prediction

- Measurement gap saving:  Reduction in inter-frequency gaps for f₂/f₃, since RSRP on target layers is predicted instead of measured.

- Beam sweep reduction: Eliminates full Rx-beam scans (e.g., 8–16 beams), replacing them with AI-predicted beam cubes.

- Mob speed-up – Handover or CA activation can be triggered before serving-cell degradation, providing a positive lead-time (Δ > 0 ms).

- Unified decision logic – One prediction simultaneously provides beam + cell + frequency ranking, replacing multiple measurement and filtering procedures.

- Supports non-collocated cells – No dependency on shared TRP geometry, beam codebooks, or fixed mapping assumptions; correlations are learned statistically.

We propose to distinguish and define priorities of study items.

Step 1 – Foundational Intra- and Inter-Cell Domains

At Step 1, the focus is on fundamental single-domain predictions.

- On the beam side, this includes intra-cell time-domain prediction, forecasting future L3 RSRP of a beam based on its historical evolution, and spatial-domain and frequency + spatial-domain prediction across neighboring cells. Time domain is been investigated in previous releases therefore it is greyed out.

Spatial domain for inter-cell beam prediction takes as input two-dimensional vectors formed from L3-RSRP measurements of a subset of DL Tx beams (Set B) from multiple cells . Specifically, the UE measures a reduced set of beams (e.g., 4 beams per cell) across M cells, with unmeasured beam positions zero-padded in the input vector, and these measurements from all cells are concatenated to form the complete input tensor to the model. The model output provides either the Top-1 or Top-K best predicted beams along with their corresponding predicted L1-RSRP values from the full beam set (Set A, e.g., 8 beams per cell) across all M cells. The prediction can be performed in two modes: (1) predicting the absolute best beam(s) across all cells simultaneously, or (2) predicting the best beam per cell (cell-specific prediction), where each cell gets its own optimal beam recommendation.

Figure 2d. Inter-cell beam level prediction (spatial prediction)

- On the cell side, studies include time-domain (studied in previous releases), frequency-domain, and spatial-Rx-domain prediction.
These serve as the building blocks for understanding how AI/ML models can predict L3 measurements in one domain at a time, forming the foundation for multi-domain extensions.

Step 2 – Early Cross-Domain Extensions

Step 2 integrates time with frequency or spatial correlation, moving from independent domains to joint prediction tasks.

- Beam-level studies examine time + spatial relationships, how beam strength evolves across time and direction.

- Cell-level studies investigate time + frequency relationships, how L3 RSRP changes jointly over time and frequency layers.

### Step 3 – Full Multi-Domain Prediction

At Step 3, the study extends to time + frequency + spatial prediction, the most comprehensive correlation space.
Here, the UE measures only a reduced subset of Tx/Rx beams and frequencies ( generalized Set B) and predicts the full multi-dimensional Set A.
This enables reconstruction of complete L3 RSRP information across domains, significantly reducing measurement gaps and enabling predictive handover and frequency layer selection.

Step 4 – Application of Prediction for RRM Efficiency

Step 4 shifts from model development to RRM utilization of the predicted information.
The predicted L3 metrics are now applied as inputs to RRM functions, such as:

- Dynamic Measurement-Gap (MG) reduction ,  optimizing the MGRP, MGL, and MO parameters to minimize interruption and resource loss.

- CFRA mobility optimization , leveraging prediction to pre-arm access resources and improve random-access success.

- Scell-related procedures, using predicted inter-frequency or spatial metrics to assist secondary-cell setup and maintenance.

At this stage, AI/ML prediction becomes an operational enabler for efficiency improvements across multiple procedures.

Step 5 – End-to-End Mobility Performance Evaluation

Finally, Step 5 investigates system-level benefits derived from these prediction tools.
This includes measuring gains in throughput, latency, mobility robustness, and UE power efficiency, as well as studying implications for Carrier Aggregation (CA) and multi-cell coordination.
These evaluations quantify the overall impact of AI-assisted measurement prediction once the underlying mechanisms from earlier priorities are validated.

Therefore, we propose the following:

Stepwise Prioritization Rationale

The hierarchical structure ensures that RAN4’s study proceeds in a methodical and cumulative manner.
Each layer builds on validated results from the previous one:

- Begin with domain-specific learning (Step 1).

- Extend to correlated domains (Step 2 – 3).

- Apply the resulting predictions to concrete RRM processes (Step 4).

- Quantify network-level benefits (Step 5).

This progressive prioritization guarantees that every step is grounded in measurable accuracy and feasibility, while keeping RAN4’s scope aligned with its expertise in measurement definition, performance, and efficiency evaluation.

We present more details below on the functionality of the predictions described in Figure 2:

Time-Domain Beam-Level Prediction (Intra-Cell)

At the beam level, the time-domain model predicts the temporal evolution of the L3 RSRP for a given set B of beams. The UE (or network) uses historical RSRP sequences measured on the same beams to forecast future L3 RSRP at upcoming time instances. Inputs are time-stamped L3 measurements; the output is a predicted L3 RSRP trace on the same set B. Training can be done offline, and inference can run either on the UE or gNB side. This is the simplest case, no spatial or frequency extrapolation is involved, and it provides the temporal continuity required for short-term beam tracking or proactive mobility decisions within a single cell.

Time + Spatial Beam-Level Prediction (Intra-Cell)

The time + spatial case extends prediction into joint Tx–Rx beam space. Here the UE measures a small subset (set B) of Tx–Rx beam pairs across time, and the AI/ML model reconstructs or predicts the full set A of Tx–Rx beam combinations. Since set B ≠ set A, the model captures spatial correlation between beams, effectively learning the mapping between limited measured directions and the unmeasured ones. This allows the UE to skip exhaustive Rx-beam sweeps and rely on model inference to identify likely optimal directions. The reduction in sweeping directly cuts measurement-gap length (MGL) and frequency of measurement occasions (MOs), improving throughput and power efficiency while maintaining L3 accuracy.

Frequency + Spatial Beam-Level Prediction (Inter-Cell)

At the next level, frequency + spatial prediction operates across frequencies in non-collocated deployments. The UE observes Tx–Rx beam pairs at one or more serving frequencies (B) and the model predicts corresponding L3 RSRPs at target frequencies (A). The AI/ML model thus learns how spatial propagation characteristics map between frequency layers. This cross-frequency inference allows the UE or gNB to anticipate signal conditions on unmeasured layers, reducing the need for direct inter-frequency gaps. The approach supports advance CFRA preparation by predicting the most promising frequency/beam pairs for future handover.

Frequency + Spatial + Time Beam-Level Prediction (Inter-Cell)

The most comprehensive beam-level model adds temporal continuity to the frequency–spatial framework. The inputs include time-series measurements of Tx–Rx beam pairs across serving frequencies (B), and the output is a forecast of the L3 RSRP distribution for target frequencies (A) over future time intervals. This prediction offers multi-domain prediction, supporting dynamic MG adaptation and inter-frequency handover decisions with minimum delay. It also provides the basis for parallel PRACH resource preparation, multiple frequency/beam hypotheses can be armed in advance, ensuring one remains valid when the handover is executed.

Time-Domain Cell-Level Prediction (Intra-Cell)

At the cell level, the time-domain model predicts future L3 RSRP values of the serving cell based on past measurement samples at the same frequency. It models the slow fading and temporal trends caused by mobility, blockage, or rotation, providing early insight into when the current cell may degrade. This allows network-side mobility controllers to trigger event-based measurements more intelligently or initiate pre-handover evaluation before degradation becomes critical.

Frequency-Domain Cell-Level Prediction (Inter-Cell)

In the frequency-domain cell-level case, the UE measures L3 RSRP at one or more serving frequencies (B) and the model predicts RSRP at neighboring frequencies (A). This enables inter-frequency link quality estimation without immediate measurement-gap scheduling. The prediction can be combined with layer prioritization rules, allowing the network to pre-select the most suitable frequency candidate for handover or CA operation while saving UE measurement energy.

Time + Frequency Domain Cell-Level Prediction (Inter-Cell)

The time + frequency model combines historical time evolution with cross-frequency correlation. The UE’s past RSRP traces at frequency B are used to infer how the cell’s quality at frequency A is expected to evolve. This joint domain learning supports predictive inter-frequency event triggering, e.g., predicting when a neighbor frequency will surpass the serving one, reducing latency between degradation detection and handover execution.

Time + Frequency + Spatial Domain Cell-Level Prediction (Inter-Cell)

The multi-domain cell-level model integrates all three correlations: time, frequency, and Rx-Tx beam spatial dimension. It uses historical measurements across multiple frequencies (B) and multiple Rx-Tx beams (B) to predict full-resolution RSRPs for target frequencies (A) and beam sets. This unified prediction produces a dense “L3 RSRP cube” representing expected signal quality across space, spectrum, and time. It is the most powerful configuration for minimizing redundant measurements while maintaining full mobility awareness, especially valuable for CA and multi-TRP operation in 6G.

Prediction as Enabling Tools and Future Investigation

Across both beam-level and cell-level hierarchies, these AI-based predictors should be viewed as tools rather than endpoints. The immediate goal of RAN4 is to verifying feasibility and performance benefits. Once validated, subsequent priorities will focus on evaluating the benefits these tools bring to concrete RRM procedures, such as dynamic MG reduction (MGRP, MGL, MO), CFRA mobility optimization, and Scell-related procedures. Performance metrics including throughput gain, reduced latency/delay, and UE power savings will be quantified in later stages. This stepwise progression ensures that prediction capabilities are first standardized as a reliable mechanism within RAN4’s domain, before being integrated into end-to-end mobility and CA optimization frameworks.

- RAN4 is invited to first study L3 beam-level prediction as the primary AI/ML problem, since it represents the more general case where per-beam RSRP across cells, frequencies, and time must be predicted. L3 cell-level prediction can then be treated as a post-processing function (beam consolidation / averaging) applied to the predicted beam set, rather than a separate prediction problem.

- AI/ML-Based L3 Mobility Prediction Framework for 6G RRM:  Study AI/ML-based L3 mobility prediction as a structured framework covering beam-level and cell-level L3 RSRP prediction across time, frequency, and spatial domains, prioritized for progressive investigation. The objective is to reduce measurement gaps, improve mobility, and enable predictive RRM operations through multi-domains. The study will proceed as follows:

- Step 1: Establish single-domain prediction (time, frequency, spatial) for intra/inter-cell RSRP at beam and cell levels.

  - Time: L3 beam level, UE sided model , intra-cell

  - Frequency: non-collocated for both beam and cell level, cluster based

  - Spatial: Joint TX-RX

- Step 2: Extend to joint-domain prediction (time + frequency / time + spatial)

- Step 3: Investigate full multi-domain prediction (time + frequency + spatial) to reconstruct complete L3 RSRP sets from measurements.

- Step 4: Apply prediction outputs to RRM procedures such as dynamic measurement-gap reduction, CFRA mobility optimization, and Scell operations.

- Step 5: Quantify end-to-end performance gains, throughput, latency, power efficiency, and mobility robustness, as derived benefits.
The prediction models will be treated as enabling tools, with RAN4 focusing on defining principles, input/output metrics, and accuracy evaluation before benefit assessment in later phases.

The proposed L1 beam-level prediction under the Long-Term Mobility (LTM) framework exhibits the same underlying characteristics and modeling principles as the L3 beam-level prediction already defined in the AI/ML-based L3 mobility study. Both rely on learning the temporal, spatial, and frequency evolution of beam-level RSRP (or equivalent signal strength indicators) to enable prediction-based RRM functions such as beam tracking, handover preparation, and measurement-gap reduction.

Therefore, RAN4 considers that the L1 beam-level prediction should not be studied as an independent use case, but rather as an extension or refinement of the existing L3 beam-level framework. The same AI/ML modeling insights: feature construction, input/output relationships, and domain prioritization (time, spatial, frequency): can be directly borrowed and reused for L1-level evaluations for LTM mobility.

Figure 3: LTM related AI/ML predictions (L1-beam level)

- Extend the AI/ML-based L3 beam-level mobility prediction framework to L1 beam-level (LTM) mobility, as both share similar temporal, spatial, and frequency characteristics. The study will reuse L3 prediction principles to predict L1 beam evolution for mobility enhancement.

Figure 4: Beam Management AI/ML prediction

Figure 4 illustrates the proposed AI/ML-based beam management prediction framework, organized into progressive priorities that expand from spatial to spatio-temporal learning. The objective is to predict beam-level behavior across transmit (Tx) , receive (Rx) and time domains, enabling efficient beam tracking, reduced sweeping overhead, and optimized RRM procedures. The framework starts with foundational single-domain learning and evolves toward joint-domain prediction, ultimately targeting the optimization of RLM, BFD, and CBD procedures while reducing scheduling and measurement restrictions.

In Rel-19 AI/ML beam management, the model predicts only DL Tx beams (Set A) based on measurements from Set B, but the corresponding Rx beam(s) remain unknown when the predicted Tx beam is not in Set B, causing high TCI state activation delays due to unknown TCI state handling procedures. Additionally, Rel-19 assumes Rx beam determination depends on SSB transmission, which may not be feasible in 6G scenarios focused on energy efficiency where SSB transmissions are reduced. To address these limitations, Tx-Rx beam pair prediction jointly predicts both the optimal Tx and Rx beams simultaneously, eliminating the Rx beam uncertainty and reducing activation delays. The AI/ML model takes as input L1-RSRP measurements from a subset of DL Tx-Rx beam pair combinations (Set B) and outputs the predicted best Tx-Rx beam pair(s) from the full set of all possible beam pair combinations (Set A), avoiding the need for separate Rx beam determination procedures and enabling faster TCI state activation since both beams are known at prediction time.

At Step 1, the focus is on single-domain spatial prediction, covering Spatial Tx, Spatial Rx, and Spatial Tx & Time domains. These represent the first step toward understanding how AI/ML models can learn and reproduce beam-directional characteristics based on spatial geometry and temporal domains. Time and Tx spatial domain will not be investigated since RAN1 has studied them thoroughly.

At Step 2, the study advances to joint Rx–Tx domain prediction, where the model learns the mutual coupling between transmit and receive beam pairs. This joint prediction enables the UE to reconstruct or anticipate the full Tx–Rx beam space from a limited subset of measured pairs, drastically reducing the number of Rx sweeps required per Tx beam. Such compression directly translates to lower measurement overhead, shorter measurement gaps, and improved scheduling flexibility for user traffic, particularly in dense beamforming environments.

Finally, Step 3 introduces spatio-temporal joint prediction, integrating the temporal evolution of the beam-pair relationship. Here, the AI/ML model predicts how both Tx and Rx beams evolve over time, maintaining coherence even as mobility, blockage, or rotation alter the channel. The resulting predictions feed into Step 4 outcomes, including optimized RLM/BFD/CBD procedures and reduced measurement and scheduling restrictions, ultimately minimizing delay (sweep factor) and overhead.

- AI/ML-Based Beam Management Prediction Framework. The study aims to reduce Rx sweeping, measurement gaps, and scheduling restrictions by predicting optimal Tx–Rx beam pairs and their temporal evolution. The work will need to cover the topics:

- Single-domain prediction: Spatial Rx, to establish baseline prediction accuracy.

- Joint Rx–Tx spatial prediction to reconstruct full beam-pair relationships from minimal measurements.

- Spatio-temporal joint prediction to forecast dynamic beam evolution under mobility and blockage.

- Apply prediction to optimize RLM/BFD/CBD procedures and reduce measurement/scheduling overhead and delay, enabling efficient AI-driven beam management for 6G.

As we describe in a later section we would also like to propose another related use case AI/ML for Multi-TRP Beam Prediction as shown in figure below.

Figure 5:  AI/ML for Multi-TRP Beam Prediction

The prediction problems in L3 mobility (beam-level), LTM (L1 beam-level reporting), BM (beam/TRP selection), and multi-TRP inter-cell clustering are not four separate AI problems: they are the same tensor prediction problem viewed from different consumption points in the measurement chain. All of them operate on the same underlying structure: a time-, frequency-, and spatial-indexed RSRP/RSRQ/SINR matrix across cells, beams, and (optionally) Rx branches or TRPs. The only difference is where the predicted values are used:

- L3 mobility uses the predicted beam cube → consolidation → cell RSRP

- LTM uses the predicted top-X beams → MAC CE reporting

- BM uses the predicted Tx–Rx beam pairs → TRP/beam ranking

- Multi-TRP clustering uses the same beam cube → multi-cell beam selection instead of single-cell handover

Because the domain structure is shared, the same AI/ML "prediction block" can be reused as a plug-in module across these use cases, with only lightweight post-processing differences (cell averaging, ranking, confidence filtering, etc.). A separate prediction model should only be studied if a use case truly requires a different input/output domain (e.g., CSI compression, PHY-layer weights). Otherwise, L3 beam-level prediction acts as the common superset, enabling reuse, common datasets, shared KPIs, and aligned evaluation across mobility, LTM, BM, and multi-TRP cluster-based inter-cell prediction.

- RAN4 is invited to study L3 beam-level prediction as a superset prediction domain, enabling reuse of the same time/spatial/frequency prediction blocks across mobility, LTM, BM, and multi-TRP scenarios. Since different use cases may involve different site conditions or input/output formats, the study should evaluate when the prediction domains are transferable or generalizable across use cases.

Discussion on Other AI/ML Use-Cases

Discussion On Measurement Gaps and MGL

In current NR systems, measurement gaps (MGs) are long and frequent because the UE must suspend traffic to measure SSB bursts across multiple frequency layers. The issue worsens when the SSBs to be measured are spread far apart within a burst (e.g., indices 2 and 28), forcing the UE to stay muted for the entire duration. This leads to high resource loss (up to 25–30% throughput impact), increased UE power consumption, and reduced responsiveness during handover and beam management procedures, especially in FR2 and inter-frequency scenarios.

We propose to leverage AI/ML-driven solutions that shortens MGs and MGLs by predicting which frequency layers and SSB subsets actually need measurement. The UE measures a small, contiguous Set B of SSBs and uses AI/ML to predict the unmeasured Set A, reconstructing full beam-level RSRPs without exhaustive full sweep measurements. If some required SSBs are temporally distant, they can be substituted by aperiodic CSI-RS transmitted within the same short window, allowing compact and efficient measurement scheduling. The approach reduces both MG frequency and MGL duration while maintaining accuracy for mobility and beam management.

- RAN4 is invited to study AI/ML-based SSB subset prediction and packing to reduce MGL by reconstructing the full SSB set (Set A) from a smaller measured subset (Set B)

Current 5G NR mobility relies on static measurement-gap (MG) configurations, with fixed periodicity and duration (e.g., 5 ms every 20 ms in FR2). As the number of frequency layers and beam combinations grows, this static configuration results in excessive measurement overhead, high power drain, and degraded throughput. Even with AI/ML-based prediction, a static Set B (fixed subset of beams) remains inefficient, as it cannot adapt to varying mobility, Doppler, or channel conditions. The result is either wasted resources when conditions are stable or missed measurements when dynamics increase.

We propose a framework that introduces dynamic Set B (generalized set B) adaptation and dynamic MG control, where the AI/ML model continuously adjusts the measurement subset size and corresponding MG resources based on its prediction confidence.

- RAN4 is invited to study AI/ML-driven dynamic measurement-gap control with adaptive Set-B resizing.

Discussion On Non-collocated Scenario Predictions

6G is now examining how the Rel-20 AI/ML inter-frequency prediction concept can extend from collocated to non-collocated deployments, where the core limitation of current methods becomes clear. In collocated cases, the AI/ML model can assume a deterministic one-to-one relationship between frequencies.

In non-collocated heterogeneous layouts, this assumption collapses. Macro, micro, and small cells may sit on different sites, bands, and clock domains, creating multiple candidate targets for any predicted RSRP value. Further ambiguity arises because the RF and signaling characteristics themselves differ across sites: TX architectures may use different array sizes, beam-widths, sweep orders, SSB densities, etc. As a result, even if a model predicts “–88 dBm on f₂,” the network cannot infer which PCI, beam set, or timing reference that value belongs to, and the UE cannot act on it without extra context. Conversely, forcing the UE to perform full detection on every carrier negates the intended gap-reduction benefit of prediction.

This produces a structural tension: AI/ML aims to suppress inter-frequency measurements, while mobility control still requires cell-identified, architecture-aware predictions. Studying how to reconcile these needs—potentially via lightweight assistance information that narrows the search space for inference—becomes essential if AI/ML-based inter-frequency prediction is to remain operationally useful in non-collocated 6G deployments.

- Study mechanisms to enable cell-identified AI/ML inter-frequency prediction in non-collocated 6G deployments.

Discussion On AI SRS Channel Reconstruction

AI/ML-based SRS-assisted channel reconstruction is intended to infer downlink CSI from uplink SRS, removing the need for frequent CSI-RS transmission and feedback. The core assumption is UL–DL reciprocity, but in practice this is violated by UE-side transmit-chain imbalances—amplitude and phase mismatches that differ across antenna ports, bands, devices, and even thermal states. These hardware-driven distortions contaminate the SRS input, causing the AI/ML model to learn from biased data and leading to degraded DL reconstruction accuracy, especially in FR2 and large-array systems where small phase errors compound across bandwidth and ports.

Current workarounds, such as coarse “imbalance level” reporting, provide only a scalar indicator and fail to convey the true spatial or frequency structure of the impairment. As a result, two UEs with the same reported level may still present very different imbalance patterns, limiting model generalization and forcing either per-UE model variants or accepting residual reconstruction error. The fundamental challenge is how to maintain a single scalable AI/ML model while compensating for diverse, slowly varying, UE-specific reciprocity errors that directly undermine the intended CSI overhead reduction.

- Study mechanisms to compensate UE-specific transmit-chain imbalances that break UL–DL reciprocity in AI/ML-based SRS-assisted channel reconstruction, to capture the true spatial and frequency structure of distortion

Discussion On Top-K Beam/TRP Prediction for CFRA Handover

Current AI/ML work in 3GPP enables the UE to predict when a target cell will become stronger than the serving cell, but the prediction stops at the cell level. This leaves a critical execution gap: handover in 5G-Advanced and 6G relies on beam-resolved CFRA, where the UE must select the correct SSB beam of the target cell to reserve the right PRACH resource and align with the gNB’s receive beam. A cell-only prediction cannot guarantee that the correct beam will be used when Msg1 is sent, meaning that even a technically correct cell prediction can still lead to CFRA failure, forced CBRA fallback, and increased interruption or RLF risk. In other words, time-domain cell prediction solves “where to hand over,” but does not solve “how to enter the cell successfully.”

The problem is further amplified by the fragility of single-beam assumptions. Small channel changes—Doppler, blockage, panel switch, or user orientation, can shift the strongest beam within milliseconds, even though the target cell itself remains correct. AI/ML can reduce average error but cannot eliminate residual mismatch, so relying on one predicted “best beam” creates a binary outcome: either the prediction is perfect, or the handover fails at Msg1. This failure mode becomes increasingly risky in dense beamformed deployments and multi-TRP topologies, where multiple beams and reception points may be valid at handover time. As long as prediction remains restricted to a single beam, proactive mobility will continue to inherit a single point of failure at exactly the moment when the network is trying to eliminate handover latency.

- Study the extension of AI/ML-based mobility prediction from cell-level to beam/TRP-level, addressing the critical gap where correct cell prediction still fails if the wrong SSB beam is used at CFRA execution.

AI/ML-Assisted Uplink TRP, Beam, and PRACH Resource Selection for CFRA in Asymmetric Multi-TRP Deployments

In legacy CFRA, the UE assumes that the downlink TRP providing the SSB is also the uplink TRP that will decode Msg1. This assumption no longer holds in multi-TRP deployments, where the strongest downlink and uplink paths may originate from different TRPs. As a result, the UE is forced to transmit Msg1 toward the DL-SSB TRP even when a different, UL-optimized TRP would provide better coverage, beam alignment, or timing. This causes avoidable Msg1 failures, repeated PRACH attempts, and unnecessary access delay—despite the network having multiple available TRPs capable of receiving the preamble. The root issue is that the UE has no mechanism to infer the correct UL-TRP, beam, timing, or PRACH resource when the UL-optimized TRP does not transmit SSB or CSI-RS.

This breaks three fundamental assumptions of CFRA: the DL beam used for SSB does not guarantee the best UL beam, the timing advance learned from the DL TRP may be wrong for the UL TRP, and the configured PRACH RO may not align with the TRP capable of decoding Msg1. Since the UL-only TRP does not transmit DL reference signals, the UE cannot measure it directly, and simply adding more DL reference signals would increase overhead and defeat the UL-only design. The resulting gap is architectural rather than procedural: successful initial access now depends on predicting which TRP, beam, timing, and PRACH resource will succeed, even when only downlink-side fingerprints are observable.

- RAN4 is invited to study an AI/ML-based CFRA enhancement where the UE, using only DL measurements, predicts the optimal UL TRP in asymmetric multi-TRP deployments.

UL-Aware and TRP-Aware Inter-Frequency Prediction and Mobility Optimization

Inter-frequency mobility in current systems is primarily driven by downlink measurements, implicitly assuming that the cell (or TRP) with the strongest DL RSRP will also be reachable on the uplink. In practice, this assumption frequently breaks down: UE power limits, timing-advance constraints, uplink interference, and asymmetric multi-TRP layouts make UL viability, not DL RSRP, the true limiting factor. As a result, handovers that appear valid from a DL perspective may still fail at Msg1, trigger PRACH timeouts, or require repeated measurement gaps and fallback procedures. The gap between “DL-best” and “UL-reachable” becomes wider in FR2, high mobility, power-limited devices, and disaggregated RAN architectures, revealing a structural flaw in DL-only prediction and measurement frameworks.

This limitation becomes even more pronounced in multi-TRP topologies, where a single logical cell can be served by multiple spatially separated TRPs with different uplink paths, power budgets, and interference conditions. Traditional mobility procedures treat the cell as a single DL entity, but the real execution success depends on which TRP actually receives the uplink. Thus, the fundamental problem is no longer selecting the best cell, but selecting the best <cell, TRP>pair across frequency layers, something current DL-centric measurements and AI/ML models are not designed to infer. As 6G moves toward wider deployments of mTRP, UL-only TRPs, and power-asymmetric architectures, mobility decisions must be driven by a combined DL+UL perspective rather than DL alone, motivating the need to rethink how prediction inputs and decision metrics are defined.

- RAN4 is invited to study UL-aware, TRP-aware inter-frequency mobility prediction that combines DL metrics with UL-reachability indicators to ensure the selected target is not just strongest but uplink-viable.

AI/ML for Multi-TRP Beam Prediction

As networks evolve toward multi-TRP operation in 5G-Advanced and 6G, the fundamental beamforming problem changes from “which beam should a single TRP use?” to “which TRPs should transmit, using which beams, and with what coordination?”. Exhaustive measurement-based approaches become impractical, because the number of TRP–beam combinations grows combinatorially and decisions will be needed at timescale required by mobility, interference, and load dynamics. This creates a clear opportunity for AI/ML-based prediction: learning optimal mTRP strategies from a small set of observed measurements instead of requiring full measurement sweeps.

The AI/ML model learns two different but complementary behaviors. In coherent mode, it predicts beam pairs and phase offsets that cause signals from multiple TRPs to add constructively at the UE, effectively synthesizing a virtual “super-antenna” across distributed sites. In non-coherent mode, the same model predicts which TRPs should transmit independently, which beams they should use, and how to avoid harmful interference, maximizing overall spectral efficiency and fairness. In both cases, AI replaces brute-force search with learned spatial reasoning: the model infers optimal transmission decisions from sparse RSRP/CSI-RS observations, mobility indicators, and historical outcomes.

The key advantage is generalization. Instead of trying every possible beam combination (thousands per TRP pair), the model learns patterns such as: “when UE measurements look like this, TRP1 beam X and TRP2 beam Y will align well” or “when TRP1 is dominant and TRP2 is marginal, only activate TRP1”. This enables low-overhead measurement, and dynamic switching between coherent and non-coherent modes depending on mobility, synchronization quality, or interference risk. The model continuously improves as more field data is collected.

Figure 12:  AI/ML for Multi-TRP Beam Prediction

- Investigate an AI/ML-based framework for multi-TRP beam prediction that supports both coherent and non coherent mode.

Conclusions

In this paper, we discussed candidate AI/ML-based use cases for 6G RRM, highlighting approaches to reduce measurement gaps, lower UE power consumption, increase throughput, and enhance mobility robustness. We made the following proposals:

AI/ML RRM Use Case:

- RAN4 is invited to first study L3 beam-level prediction as the primary AI/ML problem, since it represents the more general case where per-beam RSRP across cells, frequencies, and time must be predicted. L3 cell-level prediction can then be treated as a post-processing function (beam consolidation / averaging) applied to the predicted beam set, rather than a separate prediction problem.

- AI/ML-Based L3 Mobility Prediction Framework for 6G RRM:  Study AI/ML-based L3 mobility prediction as a structured framework covering beam-level and cell-level L3 RSRP prediction across time, frequency, and spatial domains, prioritized for progressive investigation. The objective is to reduce measurement gaps, improve mobility, and enable predictive RRM operations through multi-domains. The study will proceed as follows:

- Step 1: Establish single-domain prediction (time, frequency, spatial) for intra/inter-cell RSRP at beam and cell levels.

  - Time: L3 beam level, UE sided model , intra-cell

  - Frequency: non-collocated for both beam and cell level, cluster based

  - Spatial: Joint TX-RX

- Step 2: Extend to joint-domain prediction (time + frequency / time + spatial)

- Step 3: Investigate full multi-domain prediction (time + frequency + spatial) to reconstruct complete L3 RSRP sets from measurements.

- Step 4: Apply prediction outputs to RRM procedures such as dynamic measurement-gap reduction, CFRA mobility optimization, and Scell operations.

- Step 5: Quantify end-to-end performance gains, throughput, latency, power efficiency, and mobility robustness, as derived benefits.
The prediction models will be treated as enabling tools, with RAN4 focusing on defining principles, input/output metrics, and accuracy evaluation before benefit assessment in later phases.

- Extend the AI/ML-based L3 beam-level mobility prediction framework to L1 beam-level (LTM) mobility, as both share similar temporal, spatial, and frequency characteristics. The study will reuse L3 prediction principles to predict L1 beam evolution for mobility enhancement.

- AI/ML-Based Beam Management Prediction Framework. The study aims to reduce Rx sweeping, measurement gaps, and scheduling restrictions by predicting optimal Tx–Rx beam pairs and their temporal evolution. The work will need to cover the topics:

- Single-domain prediction: Spatial Rx, to establish baseline prediction accuracy.

- Joint Rx–Tx spatial prediction to reconstruct full beam-pair relationships from minimal measurements.

- Spatio-temporal joint prediction to forecast dynamic beam evolution under mobility and blockage.

- Apply prediction to optimize RLM/BFD/CBD procedures and reduce measurement/scheduling overhead and delay, enabling efficient AI-driven beam management for 6G.

- RAN4 is invited to study L3 beam-level prediction as a superset prediction domain, enabling reuse of the same time/spatial/frequency prediction blocks across mobility, LTM, BM, and multi-TRP scenarios. Since different use cases may involve different site conditions or input/output formats, the study should evaluate when the prediction domains are transferable or generalizable across use cases.

- Other Use Cases:

- SSB packing and MGL reduction : RAN4 is invited to study AI/ML-based SSB subset prediction and packing to reduce MGL by reconstructing the full SSB set (Set A) from a smaller measured subset (Set B)

- Dynamic MG control: RAN4 is invited to study AI/ML-driven dynamic measurement-gap control with adaptive Set-B resizing for MGL reduction

- Cell-identification for noncollocated: Study mechanisms to enable cell-identification for AI/ML inter-frequency prediction in non-collocated 6G deployments.

- UE specific compensation for IL in SRS: Study mechanisms to compensate UE-specific transmit-chain imbalances that break UL–DL reciprocity in AI/ML-based SRS-assisted channel reconstruction, to capture the true spatial and frequency structure of distortion

- CFRA optimization with beam level prediction: Study the extension of AI/ML-based mobility prediction from cell-level to beam/TRP-level, addressing the critical gap where correct cell prediction still fails if the wrong SSB beam is used at CFRA execution.

- CFRA enhancement in asymmetric DL-UL: RAN4 is invited to study an AI/ML-based CFRA enhancement where the UE, using only DL measurements, predicts the optimal UL TRP in asymmetric multi-TRP deployments.

- UL-aware mTRP mobility prediction: RAN4 is invited to study UL-aware, TRP-aware inter-frequency mobility prediction that combines DL metrics with UL-reachability indicators to ensure the selected target is not just strongest but uplink-viable.

- multi-TRP beam prediction: Investigate an AI/ML-based framework for multi-TRP beam prediction that supports both coherent and non coherent mode.

References

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR, Draft Technical Report, RAN #109.

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Technical Report