# 3GPP TSG-RAN WG4 Meeting #117	R4-2520659

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	10.6.4

Source:	Apple

Title:	On R18 NTN RRM core and performance requirement maintenance

Document for:	Discussion

# Introduction

In last RAN4#116 meeting, moderator has captured all open issues in the topic summary [1]. There are still some opens issues for further discussion, and in this contribution, we continue the discussion on remaining RRM requirements for eNTN.

Issue 2-1: How to reflect “when to start DL synchronization between t-serviceStart and t-Service is up to UE implementation” Candidate options: - Proposal 1: (Apple) - to reflect “when to start DL synchronization between t-serviceStart and t-Service is up to UE implementation”, extra clarification is needed for existing soft satellite switching latency requirement. Either of the following alternatives is fine to us: - Alt 1: clarify the current requirement equation for soft satellite switching is applied for the case if UE starts DL synchronization from t-serviceStart. - Alt 2: clarify the Tfirst_SSB is the time to the end of the target satellite’s first complete SSB burst where UE starts performing DL synchronization. - Alt 3: clarify the current requirement equation for soft satellite switching is applied for the case if UE starts DL synchronization from t-service. Recommended WF: - Discuss proposal 1.

# Discussion on Core part maintenance

### In RAN4 #110 meeting WF R4-2403492, RAN4 agreed that,

### Issue 5-2-S2: (‘Soft’ Satellite switch) Interruption time Agreement: (Needs online discussion) - The starting point of the downlink synchronization time/interruption time. - Between t-serviceStart and t-Service, the exact starting time is up to UE implementation - The starting point of the interruption time. - t-Service

### And RAN2 also has similar definition in their spec TS38.331,

### However, in the current RAN4 requirement for soft satellite switching without PCI change, it was specified that,

### The above equation means RAN4 assumed the DL synchronization starts from t-seviceStart, and it’s conflicting with the previous RAN4 agreements and RAN2 spec.

Figure 1. the existing equation is unclear on when UE to start DL sync

In the equation, Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when the handover command is received by the UE. If the target cell Es/Iot ≥ -2 dB, then Tsearch = Tfirst_SSB ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from SSB-TimeOffset reference point as defined in 38.331 [2] to UE.

In our view, the existing requirement applicability shall be clarified to avoid the ambiguity for UE implementation, i.e., when to perform DL synchronization of new SAT is up to implementation. We think the alt 2 itself exactly means when to start synchronization is up to UE implementation, and we didn’t see necessity to change the current test case setup since no one claim on testing failure so far. However, requirement needs to be technically or theoretically precise and needs to be consistent among different WGs and specs, we prefer to clarify such UE implementation based behavior in the spec without test case change.

We prefer the Alt2 but can also be fine with Alt3 as compromise.

- Alt 1: clarify the current requirement equation for soft satellite switching is applied for the case if UE starts DL synchronization from t-serviceStart. (Apple, CMCC, Nokia)

- Alt 2: clarify the Tfirst_SSB is the time to the end of the target satellite’s first complete SSB burst where UE starts performing DL synchronization. (Apple, CMCC, E///, HW)

- Alt 3: clarify the current requirement equation for soft satellite switching is applied for the case if UE starts DL synchronization from t-service. (MTK)

- Alt 4: no change (QC, HW, vivo, Nokia, ZTE, CMCC)

Proposal 1: to reflect “when to start DL synchronization between t-serviceStart and t-Service is up to UE implementation”, extra clarification is needed for existing soft satellite switching latency requirement. Either of the following alternatives is fine to us:

- Alt 1: clarify the current requirement equation for soft satellite switching is applied for the case if UE starts DL synchronization from t-serviceStart.

- Alt 2: clarify the Tfirst_SSB is the time to the end of the target satellite’s first complete SSB burst where UE starts performing DL synchronization.

- Alt 3: clarify the current requirement equation for soft satellite switching is applied for the case if UE starts DL synchronization from t-service.

# Conclusion

In this contribution, we discuss the NR-NTN core requirement maintenance.

Proposal 1: to reflect “when to start DL synchronization between t-serviceStart and t-Service is up to UE implementation”, extra clarification is needed for existing soft satellite switching latency requirement. Either of the following alternatives is fine to us:

- Alt 1: clarify the current requirement equation for soft satellite switching is applied for the case if UE starts DL synchronization from t-serviceStart.

- Alt 2: clarify the Tfirst_SSB is the time to the end of the target satellite’s first complete SSB burst where UE starts performing DL synchronization.

- Alt 3: clarify the current requirement equation for soft satellite switching is applied for the case if UE starts DL synchronization from t-service.

# References

[1] R4-2509046, Topic summary for [116][203] Maintenance_R18_R19, Moderator (Apple), RAN4 #116