3GPP TSG-RAN WG4 Meeting #117														R4-2520680

Dallas, USA, Nov. 17-21, 2025

Agenda item:	11.3

Source:	Apple

Title:	FR2 TX requirements verification in beam locked mode

Document for:	Discussion

# Introduction

The issue of whether to use beamlock function in RAN5 testing has been discussed for several meetings including how to reply to the RAN5 LS [1]. The disagreement lies in the fact that in R18, the beam correspondence requirements in initial access and RRC_INACTIVE are derived under the condition that beamlock function is used when the requirements are verified. However, this agreement has not been captured in 38.101-2, although it is capture in TR38.891 [2]. Since it is not in the TS, some companies argued that RAN5 can specify the test without the use of beamlock function.

In this contribution, we share our views on how to move forward.

# Discussion

The beam correspondence requirements in initial access and RRC_INACTIVE are derived in R18 under the condition that beamlock function is used when the requirements are verified. Although this agreement has not been captured in 38.101-2, it has been captured in Section 6.4 of TR38.891 [2], copied below:

- Beam lock function Currently, the beam lock function is only available in the RRC_CONNECTED state, and if the beam lock function is absent during the verification in RRC_IDLE, many complex issues will appear, e.g., when the RAR is held to trigger PRACH retransmission, UE may attempt other beams since the RAR is not received which may lead to unstable test. It also will be difficult to measure the θ (or φ) component EIRP under φ (or θ) link polarization without beam lock function. In [3], the LS from RAN5 concludes it is feasible to introduce beam lock function for RRC_IDLE, so the concern above can be solved. RAN4 concludes that dedicated beam lock function for RRC_IDLE state is needed, and the details of new beam lock function design can be left to RAN5.

Furthermore, the RAN4 agreement was sent to RAN5 by LS [3].

To our knowledge, in recent RAN5 discussion the RAN4 agreement of using beam lock function for testing seems to be in question. To move forward, RAN5 sent an LS to RAN4 as copied below [1], seeking RAN4 clarification.

1. Overall Description: RAN5 has discussed the test procedure and feasibility of 6.6.4 Beam Correspondence in RRC_INACTIVE requirements. RAN5 seeks the clarification from TS38.101-2 sub clause 6.6.4 Beam correspondence for power class 3 to support test procedure with UE Beamlock test function in the IDLE mode and without UE Beamlock test function in the IDLE mode. As stated in 6.6.4.1, UE shall meet the spherical coverage requirement for initial access and RRC_INACTIVE according to Table 6.2.1.3-3, with its autonomously chosen UL beams and without uplink beam sweeping, using the side conditions for initial access and in RRC_INACTIVE as defined in Clause 6.6.4.3.4. RAN5 seeks the following clarifications: - RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements? - Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams? - Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such? - Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined? 2. Actions: To RAN4 group. ACTION: RAN5 respectfully requests RAN4 to respond to the above questions.

RAN4 made the following agreements on the answers as captured by the text highlighted in green in the text box. We provide our views on the agreements below the text box.

Q1: RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either  procedure be used to verify RAN4 core requirements?

Agreement: - RAN4 will reply with some clarification on the requirements and assumption defined in RAN4. - If no better wording can be identified, the wording in the related WF/LS and agreements will be used as much as possible. - RAN4 will ask RAN5 to make the decision based on their judgement.

We consider the above agreements on Q1’s answer as a reasonable compromise as it makes no conclusion from RAN4 perspective on whether the test procedure with or without UE beamlock test function can be used to verify RAN4 core requirement, which falls into the scope of RAN5. Instead, RAN4 just shares what agreement/assumption was made when RAN4 specified the core requirements. Furthermore, we would like to note that since the core requirements were specified under the assumption that beamlock test function is used, it is uncertain if the test procedure without beamlock test function can properly verify the RAN4 requirements. There is a likelihood such a test procedure may require RAN4 to revisit the core requirements.

Q2: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

Agreement: - Using beamlock function in testing does not prevent UE from choosing UL beams in the real field. - In the beamlock function based test, UE should still be able to autonomously choose the UL beams based on DL measurement before the beamlock function is activated. - RAN4’s understanding is that beamlock function should be activated after the UL beam is autonomously chosen. - Once the beam is locked, UE cannot autonomously change the UL beams. - The exact test setup should be decided by RAN5.

The above agreements on Q2’s answer is clear and complete. As such, it can be directly captured in the reply LS without any wording change.

Q3: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

Agreement: Refer to the answer in Q1. In the replied LS, the wording in the agreed WF/LS can be referred to.

We also think the agreements on Q3’s answer can refer to Q1’s answer if Q1’a answer provides sufficient info on the agreements and assumptions RAN4 made when specifying the core requirements.

After the agreements were made, there was discussion on the detailed wording of the draft LS. We propose to use the following as the basis for further discussion and agreement.

Q1: RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

Answer: RAN4 core requirements were derived under the assumption that UE beamlock function is used. During the RAN4 discussion of deriving the core requirements, the working assumptions were shared with RAN5 in LS R4-2317764.

RAN4 has agreed on the PRACH EIRP spherical coverage requirement based on working assumptions of test condition as below. RAN4 respectfully asks RAN5 to evaluate following working assumptions and provide feedback if RAN5 identifies any issue related to the working assumptions. - RAN4 has agreed to inform RAN5 to develop the beamlock function in initial access. - UE locks the beam direction after requested by test equipment - UE is kept in RRC_IDLE state to ensure at least 1ms EIRP measurement period for PRACH. - Enable multiple PRACH transmissions in testing mode, including holding RAR. - UE transmits with the optimal Tx beam that is autonomously selected by UE. - UE achieves Pcmax before PRACH EIRP measurement. - UE transmits PRACH with gap <=20ms. To accelerate the testing, it’s recommended to use the longest applicable PRACH preamble format and minimum gap between PRACH transmission.

RAN4 request RAN5 to consider the above-mentioned assumption in developing their test procedures.

Q2: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

Answer: RAN4 is of the following view:

- Using beamlock function in testing does not prevent UE from choosing UL beams in the real field.

- In the beamlock function based test, UE should still be able to autonomously choose the UL beams based on DL measurement before the beamlock function is activated.

  - RAN4’s understanding is that beamlock function should be activated after the UL beam is autonomously chosen.

- Once the beam is locked, UE cannot autonomously change the UL beams.

- The exact test setup should be decided by RAN5.

Q3: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

Answer: Please refer to the answer to Q1 above.

If at this meeting there is still no agreement on the reply LS, RAN4 can consider one of the two options:

Option 1: Agree to disagree, i.e., acknowledge there is no agreement and stop further discussion.

Option 2: Reopen the core requirement discussion, i.e., to check if companies are OK to keep the core requirement without the assumption of using beamlock function in requirement verification. Once that discussion is concluded, RAN4 can provide a reply LS to RAN5 based on the discussion outcome.

Proposal 2: If at this meeting there is still no agreement on the reply LS, RAN4 can consider one of the two options:

- Option 1: Agree to disagree, i.e., acknowledge there is no agreement and stop further discussion.

- Option 2: Reopen the core requirement discussion, i.e., to check if companies are OK to keep the core requirement without the assumption of using beamlock function in requirement verification. Once that discussion is concluded, RAN4 can provide a reply LS to RAN5 based on the discussion outcome.

# Conclusion

In this contribution, we propose to use the following answers to the questions in the RAN5 LS as the basis for further discussion and agreement

Proposal 1: To provide the following clarification to RAN5:

Q1: RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

Answer: RAN4 core requirements were derived under the assumption that UE beamlock function is used. During the RAN4 discussion of deriving the core requirements, the working assumptions were shared with RAN5 in LS R4-2317764.

RAN4 has agreed on the PRACH EIRP spherical coverage requirement based on working assumptions of test condition as below. RAN4 respectfully asks RAN5 to evaluate following working assumptions and provide feedback if RAN5 identifies any issue related to the working assumptions. - RAN4 has agreed to inform RAN5 to develop the beamlock function in initial access. - UE locks the beam direction after requested by test equipment - UE is kept in RRC_IDLE state to ensure at least 1ms EIRP measurement period for PRACH. - Enable multiple PRACH transmissions in testing mode, including holding RAR. - UE transmits with the optimal Tx beam that is autonomously selected by UE. - UE achieves Pcmax before PRACH EIRP measurement. - UE transmits PRACH with gap <=20ms. To accelerate the testing, it’s recommended to use the longest applicable PRACH preamble format and minimum gap between PRACH transmission.

RAN4 request RAN5 to consider the above-mentioned assumption in developing their test procedures.

Q2: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

Answer: RAN4 is of the following view:

- Using beamlock function in testing does not prevent UE from choosing UL beams in the real field.

- In the beamlock function based test, UE should still be able to autonomously choose the UL beams based on DL measurement before the beamlock function is activated.

  - RAN4’s understanding is that beamlock function should be activated after the UL beam is autonomously chosen.

- Once the beam is locked, UE cannot autonomously change the UL beams.

- The exact test setup should be decided by RAN5.

Q3: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

Answer: Please refer to the answer to Q1 above.

Proposal 2: If at this meeting there is still no agreement on the reply LS, RAN4 can consider one of the two options:

- Option 1: Agree to disagree, i.e., acknowledge there is no agreement and stop further discussion.

- Option 2: Reopen the core requirement discussion, i.e., to check if companies are OK to keep the core requirement without the assumption of using beamlock function in requirement verification. Once that discussion is concluded, RAN4 can provide a reply LS to RAN5 based on the discussion outcome.

# References

- R5-253653, LS on Beam Correspondence Initial Access, RAN5

- TR 38.891

- R4-2317764, LS on testability issues of beam correspondence requirement for initial access, RAN4