3GPP TSG-RAN4 Meeting  #117	R4-2520684

Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.2.4

Source:	Apple

Title:	On 6G system parameters – channel arrangement

Document for:	Discussion

# 1	Introduction

At the last meeting, RAN4 started 6G discussion. A WF on 6G system parameters was agreed [1], including the following aspects on channel arrangement:

- Channel raster

- Sync raster

- Channel spacing

In this contribution, we continue the discussion on 6G channel arrangement based on the WF above.

# 2	Discussion

## 2.1 Channel raster

The WF captures the following agreements:

### Sub-topic 4-1: Channel raster - Main proposals - The SCS-based raster as baseline: A dominant proposal is to adopt an SCS-based channel raster as the primary or baseline method for 6G, especially for new bands and frequencies above ~3 GHz. - Unified and finer-granularity rasters: For lower frequencies (typically below 3 GHz), there is a push for rasters finer than 100 kHz to increase channel placement flexibility: - 10 kHz raster: Proposed as a baseline for refarmed bands below 2.4/3 GHz, continuing the 5G "enhanced raster" approach. - 5 kHz raster: Proposed as a unified "common channel raster" for all refarmed FR1 bands, considering it provides maximum flexibility and is a GCD of common SCS values. - Migration and co-existence approaches: - Band-specific hybrid approach: Many proposals recommend a dual or hybrid strategy: using a finer raster (5/10 kHz) for refarmed low-band spectrum and an SCS-based raster for new and higher-frequency bands. - Raster shifting: Suggesting the use of a fixed frequency shift to align a new 6G SCS-based raster with existing 5G raster points, easing coexistence. - Study migration paths: Explicitly proposing to study the migration of existing NR bands from a 100 kHz raster to an SCS-based model. - Agreement - Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster - Investigate the interaction between the channel raster and the synchronization raster - Investigate the necessity of channel raster or alternative ways for the channel configuration - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported - Study the listed main proposals especially for the migration and co-existence approaches - Other options not presented in this meeting are not precluded - Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based).

When the concept of channel raster was being discussed at the beginning of 5G, the need was questioned because a UE no longer needs it for initial access. Instead, the UE uses synchronization raster to perform system acquisition and identify the RF channel location in frequency domain based on the acquired system information. Therefore, from initial access point of view, channel raster is not needed. In the end, the main argument for the need is to identify a channel location in UE testing. However, this argument may still be questionable since during the testing, the UE is either able to complete the system acquisition to obtain RF channel location or be informed with dedicated signaling.

Observation 1: From initial access point of view, channel raster is not needed in 5G.

At the same time, there is a need to indicate the absolute frequency in RRC signaling, i.e., through ARFCN. For example, the 5G ARFCN is defined as follows:

Clearly, the range of ARFCN, i.e., the number of bits needed to signal the absolute frequency in RRC, would depend on the granularity of the global frequency raster FGlobal. From this perspective, the larger the granularity is, the smaller number of bits is required.

Observation 2: In 5G, the number of bits needed to signal the absolute frequency in RRC depends on the granularity of the global frequency raster.

The granularity of the channel raster does affect the sync raster design. In 5G, 100kHz channel raster was adopted to ensure symmetrical guard band at the channel edge for the purpose of coexistence with LTE. As a result, three sync raster points are specified to ensure that each channel has a corresponding sync raster that lies on the same SCS grid for bands of 100kHz channel raster. However, if the channel raster is specified with a granularity that is the common factor of both 100kHz and SCS (15/30/60/120kHz), such complication can be avoided.

Observation 3: In 5G, because of the adoption of 100kHz, three sync raster points are specified to ensure that each channel has a corresponding sync raster that lies on the same SCS grid for bands of 100kHz channel raster. However, if the channel raster is specified with a granularity that is the common factor of both 100kHz and SCS (15/30/60/120kHz), such complication can be avoided.

MRSS (multi-RAT spectrum sharing) allows 6G to be deployed on a 5G carrier, dynamically sharing the spectrum with 5G. Since many 5G channels have been deployed with 100kHz channel raster, there is a need to allow 6G to use the exact same channel location to ensure SCS grid alignment. This means 6G channel raster design at least needs to include the channel raster points of 5G channels, especially those located on the 100kHz raster grid.

Observation 4: From MRSS perspective, 6G channel raster design at least needs to include the channel raster points of 5G channels, especially those located on the 100kHz raster grid.

To sum up, we propose to study the need of specifying channel raster in 6G. If a need is identified, the largest common factor of 100kHz and SCS (15/30/60/120kHz) can be considered as a starting point. In other words, we can consider 5kHz raster points instead of 100kHz or SCS (15/30kHz) for FR1 to increase channel placement flexibility and to avoid too many sync raster points. Meanwhile, we are also open to considering any other options that take into account the above reasoning.

Proposal 1: It is proposed to study the need of specifying channel raster in 6G. If a need is identified, we can consider specifying 5kHz raster points instead of 100kHz or SCS (15/30kHz) for FR1 to increase channel placement flexibility and to avoid too many sync raster points.

## 2.2 Sync raster

The WF captures the following agreements:

### Sub-topic 4-2: Sync raster - Main proposals - The "Sparser is Better" principle: - Coarser step sizes: The predominant proposal is to design a sparser, coarser-grained sync raster for 6G. This involves using larger step sizes and basing the design on larger assumed channel bandwidths to reduce the total number of raster points. - Minimize raster entries: A key goal is to explicitly minimize the number of sync raster entries defined per operating band. - Enhanced flexibility and efficiency: - Adaptive SSB periodicity: Propose supporting longer and adaptive SSB periodicities (e.g., 40ms, 80ms, 160ms) to enable significant network energy saving, with the possibility of a dual raster (sparse and dense) for different deployment scenarios. - Decouple data and sync carriers: A forward-looking proposal suggests studying the feasibility of "pure data channels" that do not contain an SSB, allowing the sync raster to be optimized solely for initial access. - Simplified and unified design: - Unified Framework: Propose a unified sync raster design that works across different bands, min CBW values, and SCS configurations to avoid the fragmentation seen in 5G. - Leverage coarser reference grids: Specifically propose using a "1 RB reference channel raster" to achieve RB-level alignment and eliminate K_ssb, leading to a significant reduction in sync raster points. - Consistent SSB design: Advocate for a consistent SSB design across bands to facilitate harmonized deployment and avoid risk of deployment delays. - Prudent timeline and collaboration: - Postpone for RAN1 input: Several proposals recommend postponing detailed RAN4 sync raster discussion until after RAN1 has made sufficient progress on the fundamental SSB and initial access design. - Early RAN4 involvement: It is also proposed by companies that RAN4 should be involved early in any study related to initial access performance to provide RF and implementation perspectives. - Agreement - Evaluation on sync raster from RAN4 perspective: - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4 - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving - Collaboration and planning: - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset. - Develop a flexible evaluation framework in RAN4 that can quickly assess different sync raster proposals once key parameters (like final SSB bandwidth and min CBW) are stabilized by RAN1.

In 6G, there are proposals to increase SSB periodicity for energy saving by the network. Therefore, the need for reduced sync raster points to reduce UE cell search complexity and enable fast cell search becomes even stronger. While any detailed sync. raster design would depend on SSB bandwidth, minimum CBW, and channel spectrum utilization, RAN4 can agree on the goal of reduced sync. raster points. This view was shared in several contributions submitted to the last meeting, as can be seen from the WF above.

One way to achieve fast cell search is that sync raster granularity is completely restricted by the minimum CBW, as is the case in 5G design. Instead, multiple sync rasters are designed for different CBWs, which is called “CBW-dependent sync raster design”. When the UE starts cell search, it starts SSB frequency search from more typical channel BW and larger sync raster step size for a band or frequency range (FR), as shown below.

Proposal 2: It is proposed to study CBW-dependent sync raster design as an option to enable fast cell search and UE power saving.

# 3	Conclusions

In this contribution, we discuss 6G system parameters and make the following proposals.

Observation 1: From initial access point of view, channel raster is not needed in 5G.

Observation 2: In 5G, the number of bits needed to signal the absolute frequency in RRC depends on the granularity of the global frequency raster.

Observation 3: In 5G, because of the adoption of 100kHz, three raster points are specified to ensure that each channel has a corresponding sync raster that lies on the same SCS grid for bands of 100kHz channel raster. However, if the channel raster is specified with a granularity that is the common factor of both 100kHz and SCS (15/30/60/120kHz), such complication can be avoided.

Observation 4: From MRSS perspective, 6G channel raster design at least needs to include the channel raster points of 5G channels, especially those located on the 100kHz raster grid.

Proposal 1: It is proposed to study the need of specifying channel raster in 6G. If a need is identified, we can consider specifying 5kHz raster points instead of 100kHz or SCS (15/30kHz) for FR1 to increase channel placement flexibility and to avoid too many sync raster points.

Proposal 2: It is proposed to study CBW-dependent sync raster design as an option to enable fast cell search and UE power saving.

# 4	References

R4-2514641, "WF for 6GR system parameter," Feature lead (Huawei, HiSilicon)