3GPP TSG-RAN WG4 Meeting #117														R4-2520687

Dallas, USA, Nov. 17-21, 2025

Agenda item:	4.15.1

Source:	Apple

Title:	On remaining issues of FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The core part of NR Radio Resource Management (RRM) Phase 5 has been closed. Meanwhile, there are remaining issues on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor, as captured in the WF at the last meeting [1].

In this contribution, we share our views on how to close those open issues. continue the discussion for

# Discussion

In the WF, the following open issues were captured.

### Issue 1-1: issues related with FBS Issue 1-1-1: Condition of re-entering FSB after exiting <Way Forward>: - Further discuss in next meeting considering the below options: - Option1: Define T2 for re-evaluation after the UE exits FBS mode. - Option2: Not to define T2 for re-evaluation after the UE exits FBS mode. - Option 2.1: Need to update core requirements. - Option 2.2: Not need to update core requirements. Issue 1-1-2: Multi-band supported by FBS <Agreement>: - The serving carrier shall be a FR2-1 carrier. - For inter-frequency measurement, when serving carrier and target carrier are in different bands, use the FBS value reported for the band of the target carrier in the requirements. Issue 1-1-3: Measurement gap types supported for FBS <Agreement>: - FBS applies for inter-frequency measurement with both per-FR measurement gap and per-UE measurement gap. Issue 1-1-4: Applicability of FBS when the target and serving carrier frequency are on different <Way Forward>: - Further discuss the options: - Option 1: When the target and serving carrier frequency are different, FBS applies to target carrier only. - Option 2: When the target and serving carrier frequency are different, FBS applies to the serving carrier and target carrier. - Option 3: Introduce UE capability indicating UE can support FBS on both target and serving carrier frequency.

Issue 1-1-1: Condition of re-entering FSB after exiting

The use of T1 is to help UE save power and thus it is important to have it, especially for the case where the UE is not at the cell edge but is experiencing degraded serving cell quality due to blocking. On the other hand, the use of T2 is less important as anyway it is a corner case where the UE moves to cell edge first, and without handover to a better cell, moves back to cell center. So it is better to leave it to UE implementation. To minimize standards impact considering the core part has been closed, it is OK to take 	Option 2.2: Not need to update core requirements.

Proposal 1: It is proposed to take Option 2.2 regarding Issue 1-1-1: Condition of re-entering FSB after exiting.

Issue 1-1-4: Applicability of FBS when the target and serving carrier frequency are different

There are two cases:

- Intra-band inter-frequency: in this case, UE has the same FBS value for both serving carrier and target carrier measurement. FBS can apply to both carriers.

- Inter-band inter-frequency: in this case, there is the agreement for Issue 1-1-2: Multi-band supported by FBS, that “For inter-frequency measurement, when serving carrier and target carrier are in different bands, use the FBS value reported for the band of the target carrier in the requirements.” If serving carrier FBS is larger than target carrier FBS, obviously the target FBS value cannot apply to serving carrier, and as such the UE cannot support FBS on both carriers with the target FBS value.

Considering the core part has been closed and we are now in the maintenance phase, we propose to take Option 1.

Proposal 2: It is proposed to take Option 1 regarding Issue 1-1-4: Applicability of FBS when the target and serving carrier frequency are different.

In future releases, if the following two cases will be considered, we can revisit the agreement.

- There is more than one target carrier to be measured, and they are in different bands.

- UE is configured with CA or DC.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: It is proposed to take Option 2.2 regarding Issue 1-1-1: Condition of re-entering FSB after exiting.

Proposal 2: It is proposed to take Option 1 regarding Issue 1-1-4: Applicability of FBS when the target and serving carrier frequency are different.

# References

- R4-2514800, WF on RRM requirements for NR_RRM_Ph5_Part1, Moderator (Ericsson)