3GPP TSG RAN WG4 #117	R4-2520695

Dallas, USA, November 17th – 21st, 2025

Agenda item:		4.8.2

Source:				Ofinno

Title:					Analysis of RRM Requirements for Low Band CA via Switching

Document for:		Discussion

# Introduction

The core part of Rel-19 WI on Low Band Carrier Aggregation via Switching was completed in RAN#109 (September) [1]. The Big CR containing the RRM core requirements was agreed in [2], and the corresponding requirements were introduced in TS 38.133. Several RRM core maintenance related issues proposed in the last RAN4 meeting (RAN4#116bis) [3] were not discussed due to lack of them. However, they have been identified and captured in the approved WF for further discussion [4].

In this contribution we provide our views on some of these open issues related to the RRM core requirements.

# Active BWP switching delay requirements

According to the RAN4 WF [4], the impact of the low band CA switching pattern (switchingPattern-r19) on active BWP switching requirements was postponed for further discussion:

Issue 1-3-1: BWP switching requirement for LB CA [Way forward]: - FFS: - Proposal 1 (Apple): - RAN4 assumes that NW can avoid such scenario that active BWP switching is colliding with FDD-SDL carrier switching, and no requirement is needed to consider such colliding case. - Proposal 2 (LGE): - Regarding active BWP switching for FDD PCell and SDL SCell, support the principle as follows: - For a UE supporting LB-CA via switching and configured with switch pattern, during the active BWP switching delay and the first DL or UL slot, the UE remains on the same carrier where the active BWP switching occurs - Proposal 3 (Ofinno): - Clarify the UE behavior for both timer-based and DCI-based BWP switch by adding the following text in clause 8.6.2: - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the timer-based or the DCI-based BWP switch delay requirements refers to the first DL or UL slot, which: - is available after the completion of the BWP switching, and - is assigned to the same serving cell on which the active BWP switching occurs. - Clarify the UE behavior for the RRC-based BWP switch by adding the following text in clause 8.6.3: - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the RRC based BWP switching delay requirements refers to the first DL or UL slot, which: - is available after the completion of the BWP switching, and - is assigned to the FDD PCell.

The requirements on the active BWP switching delay for single CC and multiple CCs are specified in clause 8.6 of TS 38.133.

The requirements on the DCI and timer-based BWP switching delay in clause 8.6.2 and RRC based BWP switch delay in clause 8.6.3 are also applicable to the UE configured in carrier aggregation (CA) as described in the introductory clause 8.6.1. Therefore, the requirements in clause 8.6.2 and clause 8.6.3 are also applicable to the UE configured with a low band CA switching pattern (switchingPattern-r19) in the CA operation.

According to the above WF [4], basically three different approaches/proposals for addressing the BWP switching were identified. Nevertheless, based on the companies’ contribution so far, it has been realized that the BWP switching in one serving cell (e.g., FDD PCell) can potentially overlap with the time slots allocated to the other serving cell (e.g., SDL SCell).

However, the following sections elaborates the third proposal in the WF. In this approach the active BWP switching delay requirements for PCell and SCell are clarified to account for the unavailability of the slots due to low band CA switching pattern (switchingPattern-r19).

- Observation 1: Due to low band CA switching pattern (i.e., switchingPattern-r19), the BWP switching delay on a serving cell may overlap with time slot(s) allocated to the other serving cell.

## DCI and timer-based BWP switching delay

DCI-based BWP switching delay:

Clause 8.6.2 of TS 38.133 defines requirements on the DCI based active BWP switching delay when the BWP switch is performed on a single CC (e.g., on PCell, or SCell) as follows:

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay + Y which starts from the beginning of DL slot n.

In response to the DCI command to switch BWP on a serving cell (e.g., FDD PCell), the UE can perform baseband processing part of the BWP switching (e.g., SCS change, etc.) even if the slots are assigned to the other serving cell (e.g., SDL SCell) during the remaining part of the active BWP switch delay (TBWPswitchDelay).

DCI-based dormant BWP switching delay:

Clause 8.6.2 of TS 38.133 defines requirements on the DCI based dormant BWP switching delay when the BWP switch (to a dormant BWP) is performed on a single CC (e.g., SDL SCell) as follows:

For DCI-based BWP switch, if the new BWP is a dormant BWP, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive CSI-RS (for DL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL slot occurs right after a time duration of TdormantBWPswitchDelay which starts from the beginning of DL slot n.

In response to the DCI command to switch to the dormant BWP on the SDL SCell, the UE can perform baseband processing part of the BWP switching (e.g., SCS change, etc.) even if the slots are assigned to the FDD PCell during the remaining part of the dormant BWP switch delay (TdormantBWPswitchDelay).

Timer-based BWP switching delay:

Clause 8.6.2 of TS 38.133 also defines requirements on the timer-based active BWP switching delay when the BWP switch is performed on a single CC (e.g., on PCell, or SCell) as follows:

For timer-based BWP switch, the UE shall start BWP switch at DL slot n, where slot n is the first slot of a DL subframe (FR1) or DL half-subframe (FR2) immediately after a BWP-inactivity timer bwp-InactivityTimer [2] expires on a serving cell, and the UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n.

In response to bwp-InactivityTimer expiry triggering the switch BWP on a serving cell (e.g., FDD PCell), the UE can perform baseband processing part of the BWP switching (e.g., SCS change, etc.) even if the slots are assigned to the other serving cell (e.g., SDL SCell) during the remaining part of the active BWP switch delay (TBWPswitchDelay).

However, in all the above BWP switching scenario, the UE can only receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on a serving (e.g., PCell) cell provided that the slots are also assigned to that serving cell. When configured with switchingPattern-r19, a situation may arise that the “first slot” after the completion of the BWP in a serving cell is not available in that serving cell and is instead assigned/allocated to the other serving cell. In this case the UE may not be capable of transmitting PDSCH or receiving PUSCH in the first slot on the serving cell whose BWP is switched. To clarify the UE behavior and avoid misinterpretation of the requirements we propose to include the following text in clause 8.6.2:

“If the UE is configured with switchingPattern-r19 then the first DL or UL slot in the DCI or timer-based BWP switching delay requirements defined in this clause refers to the first DL or UL slot, which:

is available after the completion of the BWP switching and

is assigned to the same serving cell on which the active BWP switching occurs.”

- Observation 2: Due to low band CA switching pattern (i.e., switchingPattern-r19), after the BWP switching delay on a serving cell, a first DL or UL slot may not be available in that serving cell.

- Observation 3: If the first DL or UL slot is not available Due to low band CA switching pattern (i.e., switchingPattern-r19), the UE will not be able to transmit PDSCH or receive PUSCH in the first slot on the serving cell whose BWP is switched.

- Proposal 1: Clarify the UE behavior for both timer-based and DCI-based BWP switch by adding the following text in clause 8.6.2:

  - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the timer-based or the DCI-based BWP switch delay requirements refers to the first DL or UL slot, which:

    - is available after the completion of the BWP switching, and

    - is assigned to the same serving cell on which the active BWP switching occurs.

## RRC based BWP switching delay

Clause 8.6.3 of TS 38.133 defines requirements on the RRC based BWP switching delay when the BWP switch is performed on a single CC (applicable to only SpCell) as follows:

For RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWP, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after a time duration of TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length slots which begins from the beginning of DL slot n

In response to RRC message to switch BWP on the FDD PCell, the UE can perform baseband processing part of the BWP switching (e.g., SCS change, etc.) even if the slots are assigned to the SDL SCell during the remaining part of the RRC-based active BWP switch delay.

When the UE is configured with switchingPattern-r19, then a situation similar to the one described in section 3.1 (for DCI and timer-based BWP switching) may arise in that the “first slot” after the completion of the BWP in the FDD PCell is not available in the FDD PCell, i.e., available in the SDL SCell.

Therefore, to clarify the UE behavior and avoid misinterpretation of the requirements we propose to include the following text in clause 8.6.3:

“If the UE is configured with switchingPattern-r19then the first DL or UL slot in the RRC based BWP switching delay requirements defined in this clause refers to the first DL or UL slot, which is available after the completion of the BWP switching and is assigned to the FDD PCell.”

- Observation 4: Due to low band CA switching pattern (i.e., switchingPattern-r19), after the RRC-based BWP switching delay on an FDD PCell, a first DL or UL slot may not be available in the FDD PCell.

- Proposal 2: Clarify the UE behavior for the RRC-based BWP switch by adding the following text in clause 8.6.3:

  - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the RRC based BWP switching delay requirements refers to the first DL or UL slot, which:

    - is available after the completion of the BWP switching, and

    - is assigned to the FDD PCell.

## Analysis of other proposals for BWP switching requirements

In the first proposal (Proposal # 1 in the WF [4]) the idea is to let network perform the BWP switching such that the BWP switching delay does not overlap with the slots allocated to the other serving cell. This approach may be possible for some cases especially when BWP switching delay is small (e.g., Type 1 BWP switching delay capable UE) or when large number of consecutive slots are allocated to the serving cell on which the BWP switching is triggered. However, not all UEs support Type 1 capability. Furthermore, RRC based BWP switching delay can be quite large (16 ms including RRC processing delay of 10 ms and 6 ms switching delay).

The BWP switching and configuration of low band CA switching pattern (switchingPattern-r19) may not always be coordinated in all base station implementations. Therefore, additional drawback is that lack of BWP switching requirements will put restriction on the network implementation.

Due to the above reasons, in our view RAN4 should define BWP switching requirements when the UE is configured low band CA switching pattern (switchingPattern-r19).

It has been assumed in [5], the UE performs the BWP switching processing during the time slots allocated to the same serving cell on which the BWP switching is triggered. In our view, different LBCA capable UEs may have different implementations in terms of baseband processing. For example, the UE with separate baseband capability for each serving cell should be able to perform the BWP switching processing for a serving cell also during the time slots allocated to the other serving cell. One option is to consider an additional processing time in the BWP switching delay to account for different UE implementations since BWP switching delay is minimum requirement.

In the second proposal (Proposal # 2 in the WF [4]) the idea is that the UE remains on a serving cell until the BWP switching on that serving cell is completed. This will require the UE to suspend the low band CA switching pattern (switchingPattern-r19). RAN4 needs to further analyze the impact of this approach on RAN1 procedure on BWP switching.

Observation 5: BWP switching delay can be larger especially when considering Type 2 BWP switching delay capability or RRC based BWP switching delay.

Observation 6: BWP switching and switchingPattern-r19 configuration procedures may not always be well coordinated in all base station implementations.

Observation 7: A UE with separate baseband capability for each serving cell is expected to perform the BWP switching processing for a serving cell also during the time slots allocated to the other serving cell.

Proposal 3: RAN4 to consider additional delay to account for UE implementation which can process BWP switching only during the time slots assigned to the same serving cell on which the BWP switching is triggered.

Observation 8: The approach of UE remaining on a serving cell until the BWP switching on that serving cell is completed will require the UE to suspend the low band CA switching pattern (switchingPattern-r19). This may possibly impact RAN1 procedure on BWP switching.

# RLM and BFD requirements

According to the RAN4 WF [2], the discussion on the applicability of the switching pattern (switchingPattern-r19) when RLM or BFD is triggered to the PCell was deferred to the maintenance stage:

Issue 1-2-1: Switching pattern application after BF(beam failure) or LF(link failure) [Way forward]: - FFS: - Discussion following options: - Option 1: No optimization for switching pattern applicability is needed related to beam or link recovery (BFD, CBD, RLM) in Rel-19. - Option 2: - No optimization for switching pattern applicability is needed related to CBD in Rel-19. - The switching pattern is deactivated upon triggering radio link failure (RLF) (i.e., upon starting T310). - The switching pattern is activated upon stopping RLF timer (i.e., upon stopping T310). - Option 3: - The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering radio link failure (RLF) (i.e., upon starting T310). (Ofinno, vivo) - The switching pattern (i.e., switchingPattern-r19) is activated upon stopping RLF timer (i.e., upon stopping T310). (Ofinno, vivo) - The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering beam failure detection (BFD). - The switching pattern (i.e., switchingPattern-r19) is activated upon detecting a new beam on the PCell during the beam failure recovery procedure. Issue 1-2-2: BFR requirements for SDL SCell [Way forward]: - FFS: - Proposal 1 (LGE): - RAN4 to consider BFR requirements for SCell in 8.5.9 as follows: - The UE shall be capable of transmit PUCCH with an LRR at the first available PUCCH occasion that is not overlapped with SDL ON duration corresponding to the LB CA switching pattern after T2+D - where, T2 is CBD-RS evaluation period, and D is 2ms UE processing time

The two issues (RLM and BFD) are separately analyzed below:

## RLM

The UE triggers radio link failure (RLF) and start RLF timer (T310) upon receiving N310 number of consecutive out sync (OOS) indications from the lower layers. The UE stops the RLF timer (T310) upon receiving N311 number of consecutive in-sync (IS) indications from the lower layers.

Upon expiry of the RLF timer (T310), the UE initiates the RRC connection re-establishment procedure. During the RRC connection re-establishment procedure, there is typically long disruption of connection. Therefore, RLF is a critical operation and should be avoided as much as possible to prevent communication disruption.

Due to the switching pattern, the OOS and IS evaluation periods are extended depending on the periodicity of the switching pattern since the UE may some of the RLM-RS occasions (e.g., SSB), which overlap with time slots of the SDL. To reduce the IS evaluation period during the RLF (i.e., when T310 is running), it is desirable that the UE does not miss the RLM-RS of the PCell overlapping with time slots assigned to the SDL. To achieve this objective, the switching pattern can be suspended allowing the UE to stay on the PCell and receive the RLM-RS for fast recovery from the RLF. If the UE stops the RLF timer then the UE can resume the switching pattern.

- Observation 9: The radio link failure can cause long disruption of connection and should therefore be avoided as much as possible.

- Proposal 4: The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering radio link failure (RLF) (i.e., upon starting T310).

- Proposal 5: The switching pattern (i.e., switchingPattern-r19) is activated upon stopping RLF timer (i.e., upon stopping T310).

## BFD

The UE triggers the beam failure detection (BFD) when the downlink radio link quality exceeds a threshold corresponding to 10% block error rate (BLER) of a hypothetical PDCCH transmission. The UE estimates the downlink radio link quality over a BFD evaluation period.

Due to the switching pattern, the BFD evaluation period is extended depending on the periodicity of the switching pattern. To initiate the beam failure recovery the UE may transmit a random access. During the beam failure recovery procedure, the UE monitors new beam on the PCell. Therefore, to speed up the beam failure recovery, the UE should not miss the reference signal (RS) overlapping with time slots assigned to the SDL. To achieve this objective, upon triggering the BFD the UE can suspend the switching pattern enabling the UE to stay on the PCell and receive the RS for fast beam failure recovery. Upon detecting the new beam on the PCell, the UE can resume the switching pattern PCell. However, the details of the time when the UE shall resume the switching pattern can be further discussed.

- Proposal 6: The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering beam failure detection (BFD).

- Proposal 7: The switching pattern (i.e., switchingPattern-r19) is activated upon detecting a new beam on the PCell during the beam failure recovery procedure. Exact timeline is FFS.

# Conclusion

In this contribution we have analyzed RRM requirements related to the active BWP switching delay when the UE is configured with low band CA switching pattern (switchingPattern-r19). Based on this analysis our observations and proposals are as follows:

BWP switching:

- Observation 1: Due to low band CA switching pattern (i.e., switchingPattern-r19), the BWP switching delay on a serving cell may overlap with time slot(s) allocated to the other serving cell.

DCI or timer-based BWP switching:

- Observation 2: Due to low band CA switching pattern (i.e., switchingPattern-r19), after the BWP switching delay on a serving cell, a first DL or UL slot may not be available in that serving cell.

- Observation 3: If the first DL or UL slot is not available Due to low band CA switching pattern (i.e., switchingPattern-r19), the UE will not be able to transmit PDSCH or receive PUSCH in the first slot on the serving cell whose BWP is switched.

- Proposal 1: Clarify the UE behavior for both timer-based and DCI-based BWP switch by adding the following text in clause 8.6.2:

  - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the timer-based or the DCI-based BWP switch delay requirements refers to the first DL or UL slot, which:

    - is available after the completion of the BWP switching, and

    - is assigned to the same serving cell on which the active BWP switching occurs.

RRC based BWP switching:

- Observation 4: Due to low band CA switching pattern (i.e., switchingPattern-r19), after the RRC-based BWP switching delay on an FDD PCell, a first DL or UL slot may not be available in the FDD PCell.

- Proposal 2: Clarify the UE behavior for the RRC-based BWP switch by adding the following text in clause 8.6.3:

  - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the RRC based BWP switching delay requirements refers to the first DL or UL slot, which:

    - is available after the completion of the BWP switching, and

    - is assigned to the FDD PCell.

Other BWP switching approaches:

- Observation 5: BWP switching delay can be larger especially when considering Type 2 BWP switching delay capability or RRC based BWP switching delay.

- Observation 6: BWP switching and switchingPattern-r19 configuration procedures may not always be well coordinated in all base station implementations.

- Observation 7: A UE with separate baseband capability for each serving cell is expected to perform the BWP switching processing for a serving cell also during the time slots allocated to the other serving cell.

- Proposal 3: RAN4 to consider additional delay to account for UE implementation which can process BWP switching only during the time slots assigned to the same serving cell on which the BWP switching is triggered.

- Observation 8: The approach of UE remaining on a serving cell until the BWP switching on that serving cell is completed will require the UE to suspend the low band CA switching pattern (switchingPattern-r19). This may possibly impact RAN1 procedure on BWP switching.

RLM and BFD requirements:

- Observation 9: The radio link failure can cause long disruption of connection and should therefore be avoided as much as possible.

- Proposal 4: The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering radio link failure (RLF) (i.e., upon starting T310).

- Proposal 5: The switching pattern (i.e., switchingPattern-r19) is activated upon stopping RLF timer (i.e., upon stopping T310).

- Proposal 6: The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering beam failure detection (BFD).

- Proposal 7: The switching pattern (i.e., switchingPattern-r19) is activated upon detecting a new beam on the PCell during the beam failure recovery procedure. Exact timeline is FFS.

# References

- RP-251315, “Revised WID on Low NR band carrier aggregation via switching”, TELUS (Rapporteur).

- R4-2509499, “Big CR for R19 NR_LBCA_Sw RRM”, Apple.

- R4-2513508, “Topic summary for [116bis][202] NR_LBCA_Sw”, Moderator (Apple).

- R4-2514870, “WF on RRM requirements for NR_LBCA_Sw”, Moderator (Apple).

- R4-2513605, “On RRM core requirement for R19 LB CA”, Apple.