3GPP TSG-RAN WG4 Meeting # 117 	R4-2520724

Dallas, USA, Nov. 17-21, 2025

Source: 	vivo

Title: 	Discussion on 6G general RF and UE RF

Agenda Item:	8.3

Document for:	Approval

- Introduction

In the last meeting, the 6G RF was discussed first time and several high-level study points are identified [1]. We further share our views on each dedicated topic.

- Discussion

  - Single carrier output power requirements

- Power class framework design

In [1], we agreed that:

- Study 6G output power requirements to enable more flexibility to different UE implementations to reflect the RF front-end capability for single band

  - Encourage inputs on how to allow UE to deliver more output power and how power class framework includes e.g. Tx diversity and UL-MIMO

Based on the experience in NR, several pain points are listed below:

- Power class for single carrier initially consider 1Tx while TxD was introduced in later stage, which bring ambiguity when UE use multiple Tx chain via TxD to transmit higher power

- RAN4 define the power class of UL MIMO in the spec without explicit indication, while RAN1 initially assume the power of each layer is shared based on power class of single carrier.

Observation 1: The LTE/NR power class framework is originally designed for 1Tx, NW cannot totally understand the power ability of each RF chain when UE equip with multiple RF chain.

- Single power amplifier in NR normally has higher power than nominal power class, which is not fully utilized

- For certain UE implementation whose maximum power is between power classes, e.g., 23+26, power loss is unavoidable due to 3 dB granularity.

Observation 2: Power loss exist in NR due to fixed power class framework.

- In NR NTN, power margin due to HD-FDD, PA characteristic was discussed in R19 and the final conclusion is to use power boosting to utilize these margins.

Observation 3: Power boosting in NR is already used to utilize implementation margin.

To make 6G power class clearer, there could be two ways to go. One way is to follow the original design logic which is that the power class in legacy framework is for 1Tx only, and RAN4 can further study how to reflect the power ability due to implementation margin or multiple Tx chain, e.g., power boosting mentioned above. Another way is to redesign the power class framework in 6G, to accommodate the power ability for both single Tx and multiple Tx.

Proposal 1: RAN4 to study the power class framework with the clarification that the power class is the maximum power via 1Tx, FFS how to reflect the power ability due to UE equip with multiple Tx chain and implementation margin, e.g., using power boosting.

- Pcmax simplification

In [1], we agreed that:

- Study improvements on configured maximum output power requirements and associated parameters

We summarize the parameters of current Pcmax and analysis in Table I below

| Parameter | Analysis | Recommendation |
| --- | --- | --- |
| PEMAX,c | Maximum power that controlled by NW | Keep |
| ∆TC,c | Performance degradation at channel edge due to the frequency response of filter, including RF band filter, duplexer, BB filter. | Keep. since it is nature phenomenon in filter design, FFS whether the value can be improved in 6G |
| ΔPPowerClass | Related to duty-cycle based SAR solution, and almost no commercial use in NR. | Remove, FFS on the concept of duty-cycle |
| ΔPPowerBoost | Introduced in NR coverage enhancement in R18, indicating the power boosting due to FDSS or other enhancement | Keep as a general requirement for all 6G power enhancement feature. |
| ∆MPRc | MPR relaxation due to large relative channel bandwidth considering PA memory effect, difficulty of impedance match, etc. | Keep, especially if 6G needs to support UL 400MHz, the relative channel bandwidth will be around 6% at 7GHz, which is significantly larger than 4%. |
| MPRc/ A-MPRc | Nominal requirement to comply with emission requirements. | Keep |
| ΔTIB,c | Power degradation due to UE support band combination | Keep, simplification can be studied. |
| ∆TRxSRS | Power loss due the additional insertion loss for SRS antenna switching. | Remove, but further discuss whether there is better place to capture this requirement, e.g., treat as a dedicated RF requirement. |
| P-MPRc | Power reduction due to SAR | Keep |

Based on the analysis above, the simplification of Pcmax can focus on ΔPPowerClass and ∆TRxSRS. The duty cycle is a valid approach to mitigate the challenge in SAR, but the ΔPPowerClass is a complicated solution which require a lot clarification especially when the power class is changed. The ∆TRxSRS only apply to SRS transmission occasion but captured in common Pcmax formula which is quite confused. RAN5 test case is based RAN4 requirement list, and accommodating SRS antenna switching insertion loss in Pcmax is not treated as a dedicated RAN4 requirement, which could be one of reason why the RMC is absent even at the later release of NR.

Observation 4: ΔPPowerClass in NR is too complicated which require a lot clarification in the spec especially for the requirement applicability when the power class is changed.

Observation 5: ∆TRxSRS only apply to SRS transmission occasion but captured in common Pcmax formula, which from RAN5 perspective couldn’t be identified as dedicated RF requirement for verification.

TIB and RIB was introduced into spec since LTE Rel-10. And the original intention was to consider that the introduction of the CA combination would require a certain degree of relaxation of the output power and REFSENS settings of the equipment. From some chip vendors’ side, with the maturation of chip technology, the significance of these compensation values will diminish, and their importance in link budget calculations will also be reduced.

However, for terminal implementation, TIB and RIB are parameters strongly correlated with band combinations. There are still certain differences in TIB and RIB across different band combinations, with some combinations potentially requiring TIB/RIB values approximately 0.5 dB higher than those for band combinations with less interference.

Therefore, it is not advisable to directly remove these two requirements. Instead, band combinations can be classified based on their characteristics, such as frequency range, to achieve a certain degree of categorization and simplification for TIB and RIB. This would greatly facilitate the formulation of these metrics for newly introduced band combinations in the future. The RAN4 PRD has preliminarily implemented a simplification of TIB and RIB for two-band combinations based on the concept of band classification, which can serve as a starting point for further research. Specific details are expected to be discussed under the 6G spectrum project.

Proposal 2: For Pcmax simplification, the following two points can be further studied

  - Remove ΔPPowerClass, and FFS how to utilize duty-cycle in 6G

  - Remove ∆TRxSRS, and consider treat it as a dedicated RF requirement

  - Retain the TIB and RIB, and FFS the simplification method under the 6G spectrum SI.

- Clarification on UE conditions

In [1], we agreed that:

- Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements

As we mentioned in the last meeting, for CA, a UE may in following status:

- UE is configured with DL CA, but no UL CA

- UE is configured with UL CA, but the Scell is deactivated.

- UE is configured with UL CA and Scell is activated, but no scheduling in Scell

- UE is configured with UL CA and Scell is activated, and scheduling exists in Scell

The main clarification is required for condition b and c, and need to be discussed based on dedicated scenario.

Inter-band CA

higherPowerLimit-r17 was introduce in NR which means that for inter-band CA, the upper limit for total power of all CCs can be removed, so for b and c, the UE power can be same as the maximum output power of each band.

Intra-band contiguous CA

Typically, the intra-band contiguous CA will use shared RF chain for CCs, so the power will be split to each CC. However, from power amplifier operation perspective, since the MPR is different for single carrier and CA, the operation status could be changed, e.g., supply voltage, it would be challenge for UE to adjust the power with DCI based scheduling (condition c). For condition b, the time of Scell activation is in milliseconds, which could be enough for UE to change its power.

Intra-band non-contiguous CA

In NR, both shared RF chain and separate chain architecture are possible for NCCA. For separate RF chain, similar status as the shared RF chain, there is a big power backoff when UE transmit non-contiguous resource allocation, it would be also hard for UE to fast change its status for DCI based scheduling. Except the RF component adjustment, if UE support higher power for a band via TxD, the power fallback still exist since each PA could not transmit the maximum power as indicated.

Observation 6: For intra-band CA, there is gap of MPR between single CC and CA, which may require adjustment of RF component, e.g., PA, which make it would be hard to perform fast change and the characteristic could be different for each UE.

Proposal 3: The following table can be considered as starting point for discussion to clarify the UE power ability under different conditions, further study how to accommodate different UE implementations.

|  | CA with Scell is deactivated. | CA without scheduling on Scell |
| --- | --- | --- |
| Inter-band CA | Follow single carrier power class | Follow single carrier power class |
| Intra-band CCA | Follow single carrier power class when TxD is not indicated | Follow CA power class |
| Intra-band NCCA | Follow single carrier power class when TxD is not indicated | Follow CA power class |

- Default power class

In [1], we agreed that:

- Study the default power class for FDD and TDD bands for 6G

HPUE (e.g., PC2) is already supported in both NR FDD and TDD bands. However, there is two frequency region we want to discuss. First is the FDD low band where frequency <1GHz. From chip design perspective, it is not a problem to support higher power, but this will further increase the chip size which make the 6G UE design is harder considering support of new bands and limited room while the power consumption will also become larger.

In R17, the FDD HPUE was studied for the frequency around 2GHz. The following are the simulation results we provided before.

Table I FDD HPUE with P0 = -76/alpha =0.6, FTP3 model

| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Duty cycle | Cell avg. UPT/Mbps | 5% UPT/Mbps | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 100k Byte | 5 file/s | 100% | 190.2 | 3.58 | 0%, baseline | 0%, baseline |
| BS: 4R UE: 1T | 26 | 100k Byte | 5 file/s | 100% | 191.9 | 4.72 | 0.9% | 32.0% |
| BS: 4R UE: 1T | 26 | 100k Byte | 5 file/s | 50% | 190.3 | 3.31 | 0.07% | -7.4% |

Table II FDD HPUE with P0 = -76/alpha =0.6, Full buffer model

| Antenna | Max Tx power/dBm | Duty cycle | Cell avg. UPT/Mbps | 5% UPT/Mbps | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 100% | 41.5 | 0.7 | 0%, baseline | 0%, baseline |
| BS: 4R UE: 1T | 26 | 100% | 40.9 | 0.7 | -1.37% | 0 % |

The results shows that performance gain of FDD HPUE could be reduced if 50% duty-cycle constrain apply, and with the increased RU (resource utilization), e.g., full buffer, the performance of HPUE is even worse than PC3 since the intra-system interference is higher.

Observation 7: The chip size and power consumption to support HPUE in low band (<1GHz) will increase and the system performance gain could be diluted due to the increased intra-system interference and duty-cycle constrain.

Another range we want to discuss is the frequency larger than 6GHz. Although this range is for TDD bands, the problem is that whether the 400MHz needs to be supported in UL. The figure below shows a output power performance from a 7GHz PA demo measurement.

Figure I Measurement results of 7GHz demo PA

The P1dB is around 29 dBm. Considering at 7GHz, the post PA loss could be larger and PA suffer from severe memory effect due to 400MHz UL CBW (relative channel bandwidth is around 6%) which will lead to worse out of band emission, it will be very challenge for UE to take PC2 as default power class.

Observation 8: To support 400MHz UL CBW at 7GHz, it could be challenge for UE design to transmit with 26 dBm due to the severer memory effect, higher post-PA loss, worse out of band emission.

In addition to FDD and TDD, we suggest to divide FR1 into more sub frequency range to discuss the feasibility of PC2 default power class.

Proposal 4: For the default power class in 6G, it is suggested to further divide FR1 into 3 sub range for further discussion:

- Range 1(f<1GHz)：Study whether the system performance gain is still valid, considering high RU, 50% duty cycle constrain and intra-system interference

- Range 2(1GHz<f<6GHz): PC2 default power class as starting point.

- Range 3(>6GHz): Study the characteristic and feasibility of PA to support PC2 as default power class, especially for supporting large UL CBW, e.g., 400MHz

- SAR/MPE solution

In [1], we agreed that:

- Study how to handle SAR/MPE for 6GR

The concept of duty cycle was used in NR as one of the SAR solutions. However, due to the complicated UE behavior and rule of ΔPPowerClass, such solution is not attractive. In SAR regulation, no matter for short distance communication, e.g., WIFI, Bluetooth, or cellular, the duty-cycle is already used a common configuration. For example, in FCC guidance, one of the descriptions is listed below:

Unless it is specified differently in the published RF exposure KDB procedures, these requirements also apply to test reduction and test exclusion considerations. Time-averaged maximum conducted output power applies to SAR and, as required by § 2.1091(c), time-averaged effective radiated power applies to MPE.

Observation 9: The time averaged SAR is already universally used in SAR regulation.

In 6G, we prefer still keep this concept but further study whether there is better way to use this concept to mitigate the SAR issue.

Proposal 5: RAN4 further study how to better utilize duty cycle concept to mitigate the SAR related issue in HPUE.

  - Tx requirements

In [1], we agreed that

- Study the UE Tx impairment requirements/assumptions (e.g. carrier leakage, image rejection, CIM3 and CIM5)

- Spurious emissions requirements, SEM and ACLR will be defined for 6GR

  - FFS for

    - OOB boundary

      - potential accommodations for different frequency sub-ranges

- In RAN4#117, prioritize discussion for UE Tx impairments and unwanted emission framework

- Tx impairment

In [2] link budget for 256QAM is collected and summarized, as show in Table II

Table III 256QAM EVM link budget in [2]

| Tx EVM contributor | EVM | SNR (dB) |
| --- | --- | --- |
| PA | 1.85% | 34.7 |
| Transmitter | 1.19% | 38.5 |
| Phase noise | 1.78% | 35 |
| IQ imbalance | 2.06% | 33.7 |
| Total | 3.5% | 29.1 |

NR UE is already support 256QAM which means at least the impairment assumption above should be achievable. The main concern here is the impairment of PA since 6G need to work at higher frequency and larger channel bandwidth. If larger impairment of PA is allowed, further tighten other parameters may be required.

Proposal 6：Take the Tx impairment assumption from 256QAM in NR as the starting point for 6G, further study whether need to relax the impairment for PA considering the support of larger channel bandwidth.

- Tx Emission requirement framework

In LTE/NR, the spurious emission requirement is mainly based on the category B limit in ITU-R SM.329. The OOB boundary is also clearly defined:

However, the OOB boundary is more stringent than ITU informed for larger channel bandwidth.

| Channel bandwidth | OOB boundary FOOB (MHz) | % |
| --- | --- | --- |
| 3 | 6 | 250% |
| 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | BWChannel + 5 | 155%-250% |

Observation 10: The spurious emission requirement in NR follows the category B limit in ITU-R SM.329 while the OOB boundary is more stringent than ITU suggested

For 6G, we suggested the OOB boundary follow the ITU-R at least for new bands.

Proposal 7: The OOB boundary in 6G can consider follow the definition in SM.329 at least for 6G new bands, i.e., 250% from carrier center.

For SEM, we notice that WP5D already has some initial discussion and a draft version for SEM is recorded in [3]

Table IV Mobile station Spectral mask for 6 425-7 125 MHz and 10-10.5 GHz operation in [3]

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 | −10 | −10 | −10 | −10 |  |  |  |  |  |  | 1% channel bandwidth |
| ± 0-1 |  |  |  |  | −21 | −21 | −21 | −21 | −21 | −21 | 30 kHz |
| ± 1-5 | −7 | −7 | −7 | −7 | −7 | −7 | −7 | −7 | −7 | −7 | 1 MHz |
| ± 5-105 | See (2), § 6.5.2.2, Table 6.5.2.2-1 |  |  |  |  |  |  |  |  |  |  |

It is also noticed that this draft SEM is already used in some regulation study [4], which means the it is possible to relax the SEM in new 6G band. This will be helpful for release UE power in higher frequency and alleviate the difficulty for chip design.

Observation 11: A relaxed SEM was discussed in WP5D and also already used in some regulation study.

Proposal 8: RAN4 to study the feasibility to relax SEM in new 6G band, and the following table can be the starting point.

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 | −10 | −10 | −10 | −10 |  |  |  |  |  |  | 1% channel bandwidth |
| ± 0-1 |  |  |  |  | −21 | −21 | −21 | −21 | −21 | −21 | 30 kHz |
| ± 1-5 | −7 | −7 | −7 | −7 | −7 | −7 | −7 | −7 | −7 | −7 | 1 MHz |
| ± 5-105 | Same as NR |  |  |  |  |  |  |  |  |  |  |

  - Rx requirement

- MSD Simplification

According to the assumption of band simplification, such as band group concept, lots of band combination could be simplified and categorized into several ‘group combinations’. For example, when defining band groups based on division of the specific frequency range and categorizing the current FR1 frequencies roughly into three groups as illustrated in Fig. 1, the bands of FR1 would be classified as follows:

Table V. Band grouping directory based on division of FR1

| Low Group | （sub1G~1.52G） | n5, n8, n12, n13, n14, n18, n20, n26, n28, n29, n31, n50, n51, n67, n68, n71, n72, n74, n75, n76, n81, n82, n83, n85, n87, n88, n89, n91, n92, n93, n94, n100, n105, n106, n109, n110 |
| --- | --- | --- |
| Mid Group | （1.52~3G） | n1, n2, n3, n7, n24, n25, n30, n34, n38, n39, n40, n41, n53, n54, n65, n66, n70, n80, n84, n86, n90, n95, n97, n98, n99, n101 |
| High Group | （ >3G） | n46, n47, n48, n77, n78, n79, n96, n102, n104 |

Then the simplification of the band combination can be further discussed. There are two types: one is CA within a group, and the other is CA between groups. Based on the latter, the simplification of some CA combination requirements can be based on the concept of group combination, such as the simplification of the MSD part.

- For MSD due to UL harmonic:

Table VI. The n-th harmonic interference of UL falling within DL

| DL UL with n order | Low Group | Mid Group | High Group |
| --- | --- | --- | --- |
| Low Group | 2nd, (3rd) | 2nd, 3rd, 4th | 2nd, (3rd), 4th, 5th |
| Mid Group |  | (2nd) | 2nd, (3rd), 4th |
| High Group |  |  | 2nd, (…） |
| Note: () represents there is no band combinations with corresponding harmonic interference issues in the spec currently. |  |  |  |

From the above table, it can be observed that most of the uplink harmonic interference originates from low-frequency groups, which is consistent with the concept of uplink harmonics that the n-th harmonic of the uplink frequency band interferes with the 1st harmonic of the downlink frequency band. Moreover, it could also be seen that some UL interference falls into the DL band of the same band group, such as CA_n28-n74/n75/n94 (2nd harmonic interference, Low-Low group combination), CA_n78-n104 (2nd harmonic interference, High-High group combination).

Furthermore, for the inter-group interference, the MSD could be simplified according to the group combination categories. Some example group combs with the same or similar MSD values are listed as follows, which are excerpted and inducted from the current spec. From Table 5 it could be seen that, the MSD of inter-group combs are nearly identical for the same group combinations, the same order, and identical allocations. Also, some exception cases also exist and not excerpted here.

Observation 12: Based on low, mid, high group definition for FR1, for UL harmonic the MSD of inter-group combs could be classified according to the harmonic order and allocation. Some exception cases also exist.

Table VII. Some PC3 harmonic MSD classification examples based on group combination

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |
| n1/n2/n3/n25/66/n70 | n77 | 5 | 15 | 12 | 10 | 23.9 | UL2/DL1 direct-hit |
| n2/n3/n25/n66/n70 | n78 |  |  |  |  |  |  |
| n1/n2/n3/n25/n66/n70 | n77 | 5 | 15 | 12 | 100 | 13.8 |  |
| n2/n3/n25/n66/n70 | n78 |  |  |  |  |  |  |
| n66 | n48 |  |  |  |  |  |  |
| n8 | n7 | 5 | 15 | 8 | 5 | 10 | UL3/DL1 direct-hit |
| n12/n85 | n66 |  |  |  |  |  |  |
| n28 | n1 |  |  |  |  | 10.2 |  |
| n5 | n77/n78 | 5 | 15 | 6 | 10 | 10.5 | UL4/DL1 direct-hit |
| n8/n20 | n77/n78 |  |  |  |  | 10.8 |  |
| n5 | n77/n78 | 5 | 15 | 6 | 100 | 2.9 |  |
| n8/n20 | n77/n78 |  |  |  |  | 3.1 |  |
| n5/n12/n13/n14/n18 | n77 | 5 | 15 | 6 | 10 | 10.4 | UL5/DL1 direct-hit |
| n12/n18/n28 | n78 | 5 | 15 | 5 | 10 | 10.4 |  |
| n8 | n79 |  |  |  |  | 12.0 |  |
| n5/n12/n13/n14/n18/n28 | n77 | 5 | 15 | 5 | 100 | 2.9 |  |
| n8 | n79 |  |  |  |  | 3.8 |  |

- For MSD due to harmonic mixing:

For comb groups with the same order and identical UL/DL allocations, the dispersion in MSD values resulting from harmonic mixing is generally greater than that caused by UL harmonics. Although there are more exception cases compared to those in the UL harmonic table, a significant number of MSD values remain similar and can be effectively classified.

Table VIII. Some PC3 harmonic mixing MSD classification examples based on group combination

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW |  | MSD | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |
| n1 | n105 | 5 | 15 | 25 | 5 |  | 26.8 | UL1/DL3 |
| n1/n2/n25 | n71 |  |  |  |  |  |  |  |
| n2/n25 | n71 | 5 | 15 | 25 | 20 |  | 15.6 |  |
| n40 | n20 | 10 | 15 | 25 | 5 |  | 27.8 |  |
| n41 | n5 |  |  |  |  |  | 24.3 |  |
| n41 | n18 |  |  |  |  |  | 26.3 |  |
| n48/n77/n78 | n5 | 10 | 15 | 25 |  | 5 | 5.7 | UL1/DL4 |
| n48/n77/n78 | n26 |  |  |  |  |  | 5.4 |  |
| n48 | n26 | 10 | 15 | 25 |  | 20 | 1 |  |
| n77 | n5/n8 |  |  |  |  |  | 0.8 |  |
| n77/n78 | n2 | 10 | 15 | 25 |  | 5 | 6.7 | UL1/DL2 |
| n77/n78 | n3 |  |  |  |  |  | 5.7 |  |
| n77/n78 | n2 | 10 | 15 | 25 |  | 20 | 2.8 |  |
| n77/n78 | n3 |  |  |  |  |  | 2.2 |  |
| n79 | n5 | 10 | 15 | 25 |  | 5 | 27.5 | UL1/DL5 |
| n79 | n8 |  |  |  |  |  | 25 |  |
| n77/n78 | n12/n28 |  |  |  |  |  | 31 |  |
| n78 | n67 |  |  |  |  |  |  |  |
| n77/n78 | n40/n41 | 10 | 15 | 12 |  | 10 | 11.7 | UL2/DL3 |
| n77/n78 | n40/n41 | 10 | 15 | 12 |  | 100 | 3.6 |  |
| n41 | n77 | 10 | 15 | 6 |  | 10 | 8.9 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 |  | 10 | 9.3 |  |
| n41 | n48/n78 | 10 | 15 | 6 |  | 100 | 2.3 |  |
| n41 | n77 | 10 | 15 | 6 |  | 100 | 2.1 |  |

Observation 13: Based on low, mid, high group definition for FR1, for harmonic mixing the MSD of inter-group combs exhibits greater dispersion compared to UL harmonic, but to some extent, it can still be classified based on harmonic order and allocation.

- For MSD due to crossband:

For cross band interference, it is caused by “ACLR1/2, or >ACLR” which separately indicates that the first/second, or neither the first, nor the second adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

The following table includes all the inter-group combos for PC3, and despite different band combination has different UL & DL Fc and BW, the MSD values are similar inside the same group combination. Some of the MSD values may need further check and revision, but fewer exception cases could be expected.

Table IX. Some PC3 crossband MSD classification examples based on group combination

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 2.6 | >ACLR2 |
| n3 | n75 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 4.3 | >ACLR2 |
| n41 | n48 | 2680 | 100 | 30 | 270 (RBstart=3) | 3552.5 | 5 | 8.3 | >ACLR2 |
| n38 | n78 | 2600 | 40 | 15 | 216 (RBstart=0) | 3305 | 10 | 8.3 | >ACLR2 |
| n41 | n77 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | 8.3 | >ACLR2 |
| n41 | n78 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | 8.3 | >ACLR2 |
| n48 | n411 | 3570 | 40 | 15 | 216 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n48 | n411 | 3570 | 40 | 15 | 216 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |
| n77 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 4.5 | >ACLR2 |
| n78 | n71 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 4.5 | >ACLR2 |
| n78 | n38 | 3350 | 100 | 30 | 270 (RBstart=0) | 2617.5 | 5 | 3.3 | >ACLR2 |
| n78 | n38 | 3350 | 100 | 30 | 270 (RBstart=0) | 2600 | 40 | 3.3 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 4.5 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2395 | 10 | 4.5 | >ACLR2 |
| n78 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 4.5 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 4.5 | >ACLR2 |
| n78 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |

Observation 14: Based on low, mid, high group definition for FR1, for MSD due to crossband isolation the inter-group combs are finite and MSD values could be effectively classified, fewer exception cases are expected.

- For MSD due to IMD:

The following two tables show some example group combos for MSD caused by IMD2/IMD3, it could be observed that for IMD product, the dispersion in MSD values is much greater than the other three interference source. For example, for IMD2 of mid-high group combos, most of the MSD is 26dB while some of them is as high as 31dB. For IMD3, some of the low-mid and mid-high combos has the MSD value around 12dB while some others around 15~16dB. Further classification can be carried out based on the respective UL and DL groups.

At the same time, from the relevant discussion of MSD simplification of HPUE at R19 it could be found that the simplification of IMD MSD value can allow up and down to a certain degree of fluctuation and coordination due to different RF architectures. While for calculation of PC3 MSD, some RF device parameters such as filter rejection and duplexer/triplexer rejection could be shared among different adjacent/overlapping bands within the same group, e.g., n2 and n25, n77 and n78. In all, for the same group combo with the same order of IMD product, there is still some room for MSD simplification.

Table X. Some PC3 IMD2 MSD classification examples based on group combination

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n2-n78 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n48-n70 | n70 | 1697.5 | 25/15 | 25 | 1997.5 | 26 | FDD | IMD24 |
|  | n48 | 3695 | 10 | 50 | 3695 | N/A | TDD | N/A |
| CA_n7-n46 | n7 | 2550 | 10 | 50 | 2670 | 26.8 | FDD | IMD24 |
|  | n46 | 5220 | 20 | 100 | 5220 | N/A | TDD | N/A |
| CA_n1-n77 | n1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD24 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 31 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
| CA_n70-n77 | n70 | 1702.5 | 5 | 25 | 2002.5 | 31 | FDD | IMD2 |
|  | n77 | 3705 | 10 | 50 | 3705 | N/A | TDD | N/A |
| CA_n70-n78 | n70 | 1705 | 5 | 25 | 2005 | 31 | FDD | IMD2 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n3-n5 | n3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | FDD | IMD23 |
| CA_n3-n26 | n3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | n26 | 838 | 5 | 25 | 883 | 26 | FDD | IMD24 |
| CA_n5-n66 | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| CA_n26-n66 | n26 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| CA_n26-n70 | n26 | 831 | 5 | 25 | 876 | 30 | FDD | IMD24 |
|  | n70 | 1707.5 | 5 | 25 | 2007.5 | N/A | FDD | N/A |

Table XI. Some PC3 IMD3 MSD classification examples based on group combination

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_n1-n102 | n1 | 1922.5 | 5 | 25 | 2112.5 | 13 | FDD | IMD3 |
|  | n102 | 5957.5 | 20 | 100 | 5957.5 | N/A | TDD | N/A |
| CA_n5-n7 | n5 | 834 | 5 | 25 | 879 | 12 | FDD | IMD34 |
|  | n7 | 2547 | 10 | 50 | 2667 | N/A | FDD | N/A |
| CA_n8-n41 | n8 | 882.5 | 5 | 25 | 927.5 | 12.1 | FDD | IMD34 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_n20-n41 | n20 | 851 | 5 | 25 | 810 | 12.1 | FDD | IMD3 |
|  | n41 | 2512 | 10 | 50 | N/A | N/A | TDD | N/A |
| CA_n5_n41 | n5 | 839 | 5 | 25 | 884 | 15.6 | FDD | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | TDD | N/A |
| CA_n7-n20 | n20 | 851 | 5 | 25 | 810 | 12 | FDD | IMD3 |
|  | n7 | 2556 | 5 | 25 | 2676 | N/A | FDD | N/A |
| CA_n7-n26 | n7 | 2556 | 5 | 25 | 2676 | N/A | FDD | N/A |
|  | n26 | 837 | 5 | 25 | 882 | 16.0 | FDD | IMD34 |
| CA_n18-n41 | n18 | 820 | 5 | 25 | 865 | 15.6 | FDD | IMD3 |
|  | n41 | 2505 | 5 | 25 | 2505 | N/A | TDD | N/A |
| CA_n1-n71 | n1 | 1958 | 5 | 25 | 2148 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | 15.1 | FDD | IMD3 |
| CA_n7-n26 | n7 | 2512 | 5 | 50 | 2632 | N/A | FDD | N/A |
|  | n26 | 837 | 5 | 25 | 882 | 16.0 | FDD | IMD34 |
| CA_n1-n105 | n1 | 1958 | 5 | 25 | 2148 | N/A | FDD | N/A |
|  | n105 | 673 | 5 | 25 | 622 | 15.1 | FDD | IMD3 |

One special issue for IMD is that there are some complex combinations composed of three bands, if only restricting the CA implementation between different groups then some combinations would be deleted. Otherwise, the classification of MSD could also be effective to some extent as shown in Table 10, while more exceptions should be considered.

Table XII. Some PC3 IMD3 MSD classification examples based on group combination

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3700 | 28.4 | TDD | IMD22 |
| CA_n1-n3-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1807.5 | 31.5 | FDD | IMD21,2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |
| CA_n1-n3-n77 | n1 | N/A | 5 | N/A | 2140 | 31.0 | FDD | IMD21 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A |  | N/A |
|  | n78 | N/A | 10 | N/A | 3700 | 28.4 | TDD | IMD2 |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1830 | 27.9 |  | IMD2 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
| CA_n1-n3-n71 | n1 | N/A | 5 | N/A | 2160 | 5 | FDD | IMD4 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n71 | 695 | 5 | 25 | 649 | N/A | FDD | N/A |
| CA_n1-n3-n105 | n1 | N/A | 5 | N/A | 2160 | 5 | FDD | IMD4 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n105 | 695 | 5 | 25 | 644 | N/A | FDD | N/A |
| CA_n1-n3-n28 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1818.5 | 4.0 | FDD | IMD5 |
| CA_n1-n3-n71 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1855 | 4 | FDD | IMD5 |
|  | n71 | 695 | 5 | 25 | 649 | N/A | FDD | N/A |
| CA_n1-n3-n105 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1855 | 4 | FDD | IMD5 |
|  | n105 | 695 | 5 | 25 | 644 | N/A | FDD | N/A |

Observation 15: Based on low, mid, high group definition for FR1, the MSD of inter-group combs due to IMD exhibits the greatest dispersion but there is still some room for simplification.

Proposal 9: Further study MSD simplification based on the band group concept discussed under 6G spectrum SI.

And for each group combination several typical band combinations and test points could be selected as the starting point for requirement simplification and related study could also be used to guide the definition of band group.

Proposal 10: For each group combination, select several typical band combinations as the starting point for requirements simplification and the related study could also be used to guide the definition of band group.

  - Spectrum aggregation

In [1], we agreed that

  - Below spectrum aggregation topics can be discussed as starting point.

  - RF switch-time performance

  - Need for intra-band support of non-contiguous UL CA and RB allocations

  - Study benefits-drawbacks of using CA vs. using wider single carrier BW

  - Study how to account for simultaneous TX/RX between the bands properly from the beginning

- Intra-band non-contiguous UL CA and RB allocations

The requirement for intra-band contiguous CA with non-contiguous RB and intra-band NCCA was introduced in NR, but one of the problems for these features is large MPR, and the minimum MPR of different cases are summarized below:

Table IV Minimum required MPR for NCCA and NC RB in NR

|  | Class B | Class C |
| --- | --- | --- |
| CCA with NC RB PC3 | 2 dB | 2.5 dB |
| CCA with NC RB PC2 | 3 dB | 3 dB |
| CCA with NC RB PC2 2Tx | 4 dB | 4 dB |
| CCA with NC RB PC1.5 2Tx | 5 dB | 5 dB |
|  | Meet -30dBm/MHz | Meet -13dBm/MHz |
| NCCA PC3 2Tx | 9 dB | 4 dB |
| NCCA PC3 1Tx | 9 dB | 7 dB |
| NCCA PC2 2Tx | 9 dB | 4 dB |
| NCCA PC2 1Tx | 10.5 dB | 6.5 dB |
| NCCA PC1.5 2Tx | 11 dB | 6 dB |

Transmit non-contiguous resource simultaneously require large power backoff to comply with emission requirement, which lead to such feature can only be use in limited conditions otherwise the benefit of larger CBW will be cancelled by poor UL coverage.

Observation 16: The intra-band non-contiguous UL CA and RB allocations suffer from high MPR which can only be used in limited condition.

DL CA via switching was finished in NR R19 due to the difficulty of duplexer design for certain band combination. It is a possible to use Tx switching to support NCCA or non-contiguous RB to avoid the large power reduction.

Proposal 11: RAN4 to enable Tx chain switching to utilizing intra-band non-contiguous resource (e.g., CCA with non-contiguous RB allocation, NCCA) in 6G.

- CA vs wider single carrier

From NW perspective, single carrier is always a better way to utilize the spectrum, but from UE perspective, it could be hard for UE to support 400MHz single carrier. As stated in SID, the 7GHz should has similar coverage as NR 3.5GHz. Even though the single PA can transmit such wide bandwidth, it still could suffer from large MPR to comply with the emission requirement, which make the coverage target fail.

Observation 17: UL 400MHz single carrier may require large MPR which make the coverage target in 7GHz is hard to achieve.

Another solution is to use intra-band UL CA to support 400MHz with each PA only contain 200MHz signal. This architecture also was discussed in NR but only 1Tx choose as the reference architecture for requirement evaluation. Considering the challenge in 6G, it is better to use 2x200MHz as the reference architecture for requirement evaluation.

Proposal 12: RAN4 consider use intra-band CA to support UL 400MHz rather than single carrier in 6G, with the 2x200MHz as reference architecture.

  - Joint UE and BS RF

In [1], we agreed that

- For Tx EVM relaxation

  - Discuss non-AI based UL demod in UE RF thread

    - The impact on BS receiver could be considered.

  - Do not discuss AI based solution(s) in UE RF thread, except the potential common part.

  - The potential common part between AI and non-AI solutions will be discussed in this thread. AI solutions, which are discussed under AI agenda, should leverage the agreements in this thread as much as possible.

    - Strive to study TX non-linearity sources, including PA, in this thread

    - It is FFS whether and how, if agreed, to take into consideration of the existing 5G requirements and RF impairment assumptions for both 5GA and 6G.

  - Strive to avoid overlapping work between different threads

  - In RAN4#117, prioritize discussion on study scope and necessary details aligned with RAN guidance, e.g.

    - Which aspects need to be considered to conclude on feasibility

    - Need for network control

Traditional link level simulation to evaluation EVM normally use adding white noise to mimic the EVM distortion. For Tx EVM, the DPoD algorithm can only compensate the distortion due to the nonlinearity. If the evaluations still use white noise to model the EVM distortion, the algorithm actually cannot do any compensation, since there no trend can be estimated by DPoD. It is suggested to use real PA model to generate the distortion in the Net gain evaluation.

Proposal 13: For net gain evaluation, the EVM distortion should be generated from real PA model rather than adding white noise.

As we mentioned in the last meeting, the performance gain under same DPoD algorithm would be changed significantly when PA model is different. From UE perspective, such unstable gain is not preferred since if the NW cannot compensate the non-linearity as expected, UE will suffer from throughput loss and UE can do nothing for this condition. From feasibility perspective, how to validate that a stable performance gain can be achieved should be an important aspect to be considered.

Proposal 14: RAN4 need to study how to validate that a stable performance gain can be achieved across all different PA models, which should be one of aspect for feasibility conclusion.

# Conclusion

In this contribution, we share our views on 6G RF.

Single carrier output power requirements

Observation 1: The LTE/NR power class framework is originally designed for 1Tx, NW cannot totally understand the power ability of each RF chain when UE equip with multiple RF chain.

Observation 2: Power loss exist in NR due to fixed power class framework.

Observation 3: Power boosting in NR is already used to utilize implementation margin.

Proposal 1: RAN4 to study the power class framework with the clarification that the power class is the maximum power via 1Tx, FFS how to reflect the power ability due to UE equip with multiple Tx chain and implementation margin, e.g., using power boosting.

Observation 4: ΔPPowerClass in NR is too complicated which require a lot clarification in the spec especially for the requirement applicability when the power class is changed.

Observation 5: ∆TRxSRS only apply to SRS transmission occasion but captured in common Pcmax formula, which from RAN5 perspective couldn’t be identified as dedicated RF requirement for verification.

Proposal 2: For Pcmax simplification, the following two points can be further studied

  - Remove ΔPPowerClass, and FFS how to utilize duty-cycle in 6G

  - Remove ∆TRxSRS, and consider treat it as a dedicated RF requirement

  - Retain the TIB and RIB, and FFS the simplification method under the 6G spectrum SI.

Observation 6: For intra-band CA, there is gap of MPR between single CC and CA, which may require adjustment of RF component, e.g., PA, which make it would be hard to perform fast change and the characteristic could be different for each UE.

Proposal 3: The following table can be considered as starting point for discussion to clarify the UE power ability under different conditions, further study how to accommodate different UE implementations.

|  | CA with Scell is deactivated. | CA without scheduling on Scell |
| --- | --- | --- |
| Inter-band CA | Follow single carrier power class | Follow single carrier power class |
| Intra-band CCA | Follow single carrier power class when TxD is not indicated | Follow CA power class |
| Intra-band NCCA | Follow single carrier power class when TxD is not indicated | Follow CA power class |

Observation 7: The chip size and power consumption to support HPUE in low band (<1GHz) will increase and the system performance gain could be diluted due to the increased intra-system interference and duty-cycle constrain.

Observation 8: To support 400MHz UL CBW at 7GHz, it could be challenge for UE design to transmit with 26 dBm due to the severer memory effect, higher post-PA loss, worse out of band emission.

Proposal 4: For the default power class in 6G, it is suggested to further divide FR1 into 3 sub range for further discussion:

- Range 1(f<1GHz)：Study whether the system performance gain is still valid, considering high RU, 50% duty cycle constrain and intra-system interference

- Range 2(1GHz<f<6GHz): PC2 default power class as starting point.

- Range 3(>6GHz): Study the characteristic and feasibility of PA to support PC2 as default power class, especially for supporting large UL CBW, e.g., 400MHz

Observation 9: The time averaged SAR is already universally used in SAR regulation.

Proposal 5: RAN4 further study how to better utilize duty cycle concept to mitigate the SAR related issue in HPUE.

Tx requirements

Proposal 6：Take the Tx impairment assumption from 256QAM in NR as the starting point for 6G, further study whether need to relax the impairment for PA considering the support of larger channel bandwidth.

Observation 10: The spurious emission requirement in NR follows the category B limit in ITU-R SM.329 while the OOB boundary is more stringent than ITU suggested

Proposal 7: The OOB boundary in 6G can consider follow the definition in SM.329 at least for 6G new bands. i.e., 250% from carrier center.

Observation 11: A relaxed SEM was discussed in WP5D and also already used in some regulation study.

Proposal 8: RAN4 to study the feasibility to relax SEM in new 6G band, and the following table can be the starting point.

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 | −10 | −10 | −10 | −10 |  |  |  |  |  |  | 1% channel bandwidth |
| ± 0-1 |  |  |  |  | −21 | −21 | −21 | −21 | −21 | −21 | 30 kHz |
| ± 1-5 | −7 | −7 | −7 | −7 | −7 | −7 | −7 | −7 | −7 | −7 | 1 MHz |
| ± 5-105 | Same as NR |  |  |  |  |  |  |  |  |  |  |

Rx requirement

Observation 12: Based on low, mid, high group definition for FR1, for UL harmonic the MSD of inter-group combs could be classified according to the harmonic order and allocation. Some exception cases also exist.

Observation 13: Based on low, mid, high group definition for FR1, for harmonic mixing the MSD of inter-group combs exhibits greater dispersion compared to UL harmonic, but to some extent, it can still be classified based on harmonic order and allocation.

Observation 14: Based on low, mid, high group definition for FR1, for MSD due to crossband isolation the inter-group combs are finite and MSD values could be effectively classified, fewer exception cases are expected.

Observation 15: Based on low, mid, high group definition for FR1, the MSD of inter-group combs due to IMD exhibits the greatest dispersion but there is still some room for simplification.

Proposal 9: Further study MSD simplification based on the band group concept discussed under 6G spectrum SI.

Proposal 10: For each group combination, select several typical band combinations as the starting point for requirements simplification and the related study could also be used to guide the definition of band group.

Spectrum aggregation

Observation 16: The intra-band non-contiguous UL CA and RB allocations suffer from high MPR which can only be used in limited condition.

Proposal 11: RAN4 to enable Tx chain switching to utilizing intra-band non-contiguous resource (e.g., CCA with non-contiguous RB allocation, NCCA) in 6G.

Observation 17: UL 400MHz single carrier may require large MPR which make the coverage target in 7GHz is hard to achieve.

Proposal 12: RAN4 consider use intra-band CA to support UL 400MHz rather than single carrier in 6G, with the 2x200MHz as reference architecture.

Joint UE and BS RF

Proposal 13: For net gain evaluation, the EVM distortion should be generated from real PA model rather than adding white noise.

Proposal 14: RAN4 need to study how to validate that a stable performance gain can be achieved across all different PA models, which should be one of aspect for feasibility conclusion.

- References

- R4-2514642, WF for [116bis][102] 6G general RF and UE RF, Qualcomm, RAN4#116bis

- R4-166954，WF on MPR\AMPR simulation assumptions for UL 256QAM

- Annex 4.4 to Working Party 5D Chairman’s Report Document 5D/716 (June 2021)

- ECC Report 366, Feasibility of a potential shared use of the 6425-7125 MHz frequency band between Mobile/Fixed Communications Networks (MFCN) and Wireless Access Systems including Radio Local Area Networks (WAS/RLAN), June 2025