3GPP TSG-RAN WG4 Meeting#116bis 	R4-2520729

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Further discussions on 6G testability and OTA

Agenda Item:	8.12

Document for:	Approval

- Introduction

RAN4#116bis meeting started the 6G discussions and made good progress on 6G testability and OTA topics [1].

In this contribution, we provide further views on some of 6G OTA and testability topics.

- Discussion

  - Improve traditional conducted test to OTA

- Radiated MSD

Last RAN4 meeting, the following agreements were reached:

Issue 1-2-1: Whether it is valuable to improve some conducted requirements to be verified via OTA approach

Agreements:

- RAN4 can further study the testability limitations of some conducted test cases, consider the following aspects as starting point, e.g.,

  - FR1 conducted test cases already consider antenna-performance impacts but not verified via radiated approach in a case-by-case manner, e.g. MSD

  - FR1 test case simplification with both conducted and radiated considered, e.g., spurious emission

  - Discuss which conducted conformance testing could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern.

RAN4 agreed to further study the limitations of some conducted test cases, MSD was one of the hottest topics collecting many interests from companies. According to the discussions, the limitations of conducted MSD can be briefly summarized as following:

- The requirement analysis needs lots of efforts, but the final conducted requirements is far from the real UE performance verified via conductive approach.

- Conformance testing is pass/fail testing, but the real performance/value is not reflected.

- The requirement does not provide much useful information to operators for network configuration.

If the MSD is measured by radiated approach in 6G, there would be a chance that the value could be optionally reported to network to assist the CA dynamic configurations.

However, one challenge is the testing efficiency, the MSD OTA testing would be time consuming given many MSD test cases would be defined in RAN4. In 6G, study on how to efficiently verify this performance would be a key aspect in RAN4, which is also related to the discussions of simplified MSD.

Proposal 1: RAN4 further discuss a reasonable testing efficiency or skipping rule to reduce MSD OTA test burden, e.g., a general framework on how to select MSD configurations for OTA testing.

Proposal 2:  RAN4 further discuss whether the radiated MSD performance could be helpful for network configuration or UE design.

  - New methods to cover new 6G frequencies

- Enhanced OTA method for new frequency range

Last RAN4 meeting, the following agreements were reached:

Issue 2-1-1: Testability for New Frequencies between FR1 and FR2

Agreements:

RAN4 study the testability including conducted testing and radiated testing for the new frequencies. The following aspects can be considered:

- The frequency range definition will be discussed under the spectrum agenda.

- For conducted test, use existing test method as a starting point

- For OTA test

  - Further study potential test methodologies including the full-package, i.e., supported frequency range, test setup, configuration, positioning, procedure, validation/calibration, quiet-zone/test zone, MU, testing time reduction

The new frequency range is studied in 6G spectrum agenda, which may need more meeting cycles discussion to reach consensus. Besides, this aspect also belongs to RAN plenary level discussion and decision. What we can do now in this testability agenda is the study and technical analysis of current limitation of AC system and IFF system, and discuss how to extend the system capability to cover potential new frequencies between ~7GHz and 24GHz.

Proposal 3: RAN4 should further study details of the following solutions for new frequencies of 6GR:

- Study the limitations and feasibility to broaden FR1 AC system frequency range to cover FR1 to new 6G frequencies and discuss potential upper limit.

- Study the limitations and feasibility to broaden FR2 IFF system frequency range to cover new 6G frequencies to FR2 and discuss potential lower limit.

- Leverage existing FR1 and FR2 OTA system, and study a dedicate/new system only support the new frequency range and analyze the feasible QZ size and measurement distance.

  - Test method to support variety of device types

﻿5G starts from smartphone, and further extend to RedCap, XR, NTN, and A-IoT UE types, which introduce difficulty to enhance the test system after it is lunched to market. Currently, in RAN1 and RAN plenary discussions, majority companies believe 6G day-1 should support variety of device types. The testability issue of different device type should also be considered and RAN4 try to resolve that by a generic solution.

Issue 4-1-1: OTA testability applicability for different UE types in 6GR day-1

Agreements:

- RAN4 consider the study of OTA test system to better accommodate different UE types, new feature/functionalities, new performance metrics and test cases.

- The UE types and form factor considered in testability can be further discussed in this SI.

- RAN4 study the feasibility of developing a single system to cover above aspects.

In RAN#109 meeting, in the moderator summary [RP‑252873], the following device types are mentioned:

RAN plenary meeting still needs more time to discuss whether 6G device type should be categorized and how to define the UE types. But in RAN4 testability study, no matter how the devices are classified, the goal of test methodologies is to be generic enough to cover different device types to minimize the overall cost and complexity of OTA test system.

Proposal 4: RAN4 further discuss the feasibility and limitation of test system to cover different UE types and identify the key aspects of different UE type for test system discussion, e.g., form factor, positioning, phantoms.

Proposal 5: The following 6G device types could be considered as starting point for testability discussions: smartphone, CPE, XR, VUEs, wearables, sensing, NTN, AI agents, and robots.

  - Testability for 6G AI features

Last meeting, we agreed that

Issue 5-1-1: RAN4 study on enhancement of AI/ML testability for 6GR

Agreements:

- The corresponding test method including both OTA and conducted, to verify the AI/ML features (FFS details) should be studied in RAN4 in 6G SI.

  - The methodologies identified in 5G will be considered as the starting point for both conducted and OTA.

The detailed test method (including the channel model, validation, test procedure) of AI/ML based BM is not finalized according to the extension of WID [RP-252930], the outcome of the study can be a baseline for 6G. However, to meet the real requirement of AI/ML-based BM, RAN4 still needs to study how to overcome of the challenges of generating a complex scenario with multiple BS beams towards a CDL model in the chamber by limited number of probes rather than a massive number of probes and RF channels in the channel emulator.

For 6G AI use cases, the following general scope has been agreed last meeting.

Issue 1-1: RAN4 6G AI/ML study scope

Agreement:

- In 6G AI/ML study, the detailed RAN4 scope shall include

  - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

    - RAN4-driven use cases: RAN4 shall identify, select and study only those AI/ML use cases that fall within RAN4 core responsibility. The detailed principles are captured under 3-1.

    - Other WG-driven use cases: RAN4 shall assist other working groups in these use cases by studying RAN4 impacts with RAN4 analysis and evaluation relevant to implementation and performance.

  - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

    - LCM procedures: RAN4 can study relevant RAN4 requirements for LCM procedures. If the use cases are identified by other WG, RAN4 related study should be based on the agreements and/or requests by other WG. The exact LCM context will be separately discussed and decided in the responding WGs.

    - RAN4 may study the feasibility and testability of the AI/ML functionalities. The study may include different aspects. It will be further decided how to manage the discussion under different agenda items, e.g. AI or testability.

According to the experience from Rel-19 AI/ML test methodology discussions, the performance metric should be discussed and decided first and then the corresponding test framework and setup could be developed accordingly. In addition, the generation of channel model is a key aspect of test setup configuration, which is also highly dependent on the test system complexity and cost. Therefore, the generation of channel model, test setup and potential test framework could be discussed in Testability agenda. Detailed management of test related discussions can be further refined based on the progress in RAN4.

Proposal 6: For 6G AI, the test methodology will be developed based on use cases and performance metric. The proper channel model, test framework and corresponding test setup could be discussed in testability agenda.

  - Test efficiency improvement

Last meeting, we have the general agreements for test efficiency improvement:

Issue 9-1-1: Improve OTA test efficiency in 6GR

Agreements:

6G OTA testability study should consider system complexity, cost and test time reduction.

Typically, the test methodologies are introduced by a baseline method with few follow-up alternatives as optional method but with applicability restrictions. This approach has been used for many years, which is helpful to improve the test efficiency but lead to a situation that the alternatives might not be widely used given the later development and strict restriction of applicability.

For 6G, it is proposed to develop alternatives together with baseline method, the MU assessment of different solutions can be developed simultaneously. Then the test bodies have a chance to select the proper methodology based on different considerations and purpose, e.g., smaller MU for conformance testing or faster measurement just for performance verification.

The following agreed 5G approaches can be considered as basis in the beginning of 6G as alternative solutions, e.g.,

- Single point offset approach

- Coarse measurement grid

- Spiral Scan procedure for TRP

- Reverberation chamber

- Fast Tx/Rx Spherical Coverage Method

Proposal 7: RAN4 could consider the following alternatives for 6G test method consideration:

- Single point offset approach

- Coarse measurement grid

- Spiral Scan procedure for TRP

- Reverberation chamber

- Fast Tx/Rx Spherical Coverage Method

- Conclusion

In this contribution, we further discuss the testability and OTA topics and share the following proposals:

Proposal 1: RAN4 further discuss a reasonable testing efficiency or skipping rule to reduce MSD OTA test burden, e.g., a general framework on how to select MSD configurations for OTA testing.

Proposal 2:  RAN4 further discuss whether the radiated MSD performance could be helpful for network configuration or UE design.

Proposal 3: RAN4 should further study details of the following solutions for new frequencies of 6GR:

- Study the limitations and feasibility to broaden FR1 AC system frequency range to cover FR1 to new 6G frequencies and discuss potential upper limit.

- Study the limitations and feasibility to broaden FR2 IFF system frequency range to cover new 6G frequencies to FR2 and discuss potential lower limit.

- Leverage existing FR1 and FR2 OTA system, and study a dedicate/new system only support the new frequency range and analyze the feasible QZ size and measurement distance.

Proposal 4: RAN4 further discuss the feasibility and limitation of test system to cover different UE types and identify the key aspects of different UE type for test system discussion, e.g., form factor, positioning, phantoms.

Proposal 5: The following 6G device types could be considered as starting point for testability discussions: smartphone, CPE, XR, VUEs, wearables, sensing, NTN, AI agents, and robots.

Proposal 6: For 6G AI, the test methodology will be developed based on use cases and performance metric. The proper channel model, test framework and corresponding test setup could be discussed in testability agenda.

Proposal 7: RAN4 could consider the following alternatives for 6G test method consideration:

- Single point offset approach

- Coarse measurement grid

- Spiral Scan procedure for TRP

- Reverberation chamber

- Fast Tx/Rx Spherical Coverage Method

- References

- R4-2514647 WF for [116bis][110] 6G testability and OTA, Feature lead (vivo), RAN4#116bis, Oct 2025.

- R4-2514517 Topic summary for [116bis][110] 6G testability and OTA, Feature lead (vivo), RAN4#116bis, Oct 2025.

- R4-2520731 Running Summary of 6G Testability and OTA_after_RAN4#116bis, Feature lead (vivo), AN4#117, Nov 2025.