# 3GPP TSG-RAN WG4 Meeting #117	R4-2520738

# Dallas, US, 17th – 21th Octobor, 2025

Source: 	vivo

Title: 	Discussion on 6G waveform

Agenda Item:	8.2.1

Document for:	Approval

Introduction

In this contribution, some views on 6G system parameters are provided, based on WF[1] of #116bis meeting.

Discussion

Waveform

The agreement of RAN4 #116bis meeting regarding waveform for 6G is as follows:

Agreement of RAN4 #116bis: - Agreement - The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1 - To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects - Tx assumption including PA model - Related RF requirements which should be taken into consideration - Both UL and DL are considered - Take the candidate waveforms identified/discussed in RAN1 as the baseline - Identify the evaluation metric, e.g. net gain - Implementation constraints - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs - Evaluation cases could be selected step-by-step, based on RAN1 progress - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement - Agreement - Consider PA modelling at least for the following aspects for RAN4 discussion - Memory effects for UE supporting larger CBW - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3) - Calibration conditions - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc. - PA models for different device types, handheld UE is prioritized - Applicable requirements - 5G-A requirements as starting point - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

As can be seen, CP-OFDM and DFT-s-OFDM waveforms in 5G NR would be considered as benchmark in 6G stage. And based on the assumption, some remaining issues about the waveforms should be studied further in 6G, such as coverage enhancement. One of the effective approaches is to design the low-PAPR waveform.

The waveform with high PAPR necessitates significant power back-off to avoid operating in the nonlinear region of the PA, which would otherwise lead to saturation. This reduction in power decreases PA efficiency and limits output power. To comply with requirements such IBE, EVM, ACLR, and SEM, a higher MPR is inevitably required.

Observation 1: Waveform with high PAPR would decrease PA efficiency and further reduce maximum output power, which is a key requirement in RAN4.

Low-PAPR scheme specified in NR R15 and R18: FDSS

From the discussion details in RAN1, in addition to the baseline waveform discussion, the 6G waveform enhancements: low-PAPR waveform for coverage extension in UL has also been discussed extensively in RAN1. In 5G Rel-18 coverage enhancement WI, due to limited TU and consideration for avoiding complex impact on RAN1, similar discussion is converged to a transparent spectrum shaping scheme: FDSS. PAPR is reduced by using a certain shaping filter to attenuate specific high frequency components, which are the source of peaks in the signal. FDSS was specified earlier in R15 for pi/2 BPSK and expanded the utilization to QPSK in R18. FDSS doesn’t have any impact on RAN1 and is considered as a transparent spectrum shaping scheme, which is much simpler but has limited performance improvement for PAPR reduction. In RAN4 spec, finally there is 1/0.5dB power boost for inner region.

Observation 2: Transparent spectrum shaping schemes to reduce PAPR such as FDSS have been discussed extensively in 5G R15 and R18 stage.

Remaining scheme in NR R18: FDSS-SE

In R18-stage, some non-transparent approaches have also been discussed, the most typical one is FDSS-SE: FDSS with spectrum extension. Compared to FDSS, FDSS-SE first performs symmetrical expansion on both sides of the spectrum to place the copied in-band data at the edge of the allocated RBs, followed by frequency-domain shaping of the expanded spectrum as a whole. Although the expanded spectrum portion would be attenuated by filtering, the receiver can still improve reception performance through combining.

Comparing transparent and non-transparent approaches, it could be seen that in the transparent scheme, the specific waveform shaping used by the transmitter is not explicitly signalled to the network side, preventing the receiver from performing adaptive equalization. It would in turn restricts the choice of shaping parameters at the transmitter, making it hard to achieve optimal shaping. In contrast, the non-transparent scheme, with its superior coordination, enables the receiver to configure appropriate equalization parameters. As a result, constraints on filtering parameters at the transmitter can be relaxed, allowing better waveform designs to further reduce PAPR.

It should be pointed out that the performance comparation of no filtering, FDSS, and FDSS-SE in R18 was based on the assumption of keeping the total allocated bandwidth, the spectral efficiency and resource in time domain the same for all compared cases. Also, for FDSS-SE these expanded frequency-domain resources could not be allocated to other users.

Figure 1. Illustration of FDSS and FDSS-SE

In RAN1 #122 meeting, there are more advanced schemes to reduce PAPR proposed by different companies, including some transparent schemes and non-transparent schemes, such as FDSS (approximating GMSK) [2], FDSS-CE[3], pre-coded DFT-s-OFDM[4], CFR-SE[5] and so on.

Recommended scheme for 6G: CFR-SE

Clipping, i.e., Crest Factor Reduction (CFR), is typically implemented in UEs to reduce the PAPR of uplink signals, thereby enabling lower PA power back-off. A common CFR approach involves iterative clipping and filtering operations: Clipping removes signal peaks to achieve lower PAPR, while subsequent filtering mitigates the out-of-band leakage caused by the clipping process. Given filtering will introduce new peaks, usually multiple iterations of this clipping-filtering operations are needed. One way to reduce the number of iterations and further improve PAPR is CFR with spectrum extension (CFR-SE), where filtering is carried out based on extend spectrum to retain more sidelobes (in frequency) for clipping signals. These sidelobes can help to further reduce PAPR. As shown in Figure 2, actual PRBs are used to carry UL transmission block while indicated PRBs are used to determine filter. In addition to better PAPR performance, the extended PRBs can be shared by other UEs because no actual data signal is transmitted using this part. Further, as clipping algorithm is sufficiently mature and implemented in commercial UEs, and CFR-SE only extends the frequency band of the filtering, the additional complexity and effort to implement CFR-SE is marginal. Moreover, it can be applicable to both CP-OFDM and DFT-s-OFDM.

Figure 2. Schematic of CFR-SE[7]

Evaluation Metric discussion

In R18 power boosting topic, the net gain metric is considered as high-comprehensive requirement to evaluate the performance of a low-PAPR waveform. The formula is excerpted as follows, PoutFilter i-PoutRef. filter represents the coverage enhancement gain, i.e., power back-off gain caused by PAPR reduction. While SNR10%Ref. filter-SNR10%Filter i is the SNR gain to achieve 10% BLER compared against the reference. A high-performance waveform should achieve good power efficiency without much degradation of the spectral efficiency, thus it is recommended to reuse the net gain metric to evaluate the low-PAPR schemes in 6G as a starting point.

Net Gain dB=PoutFilter i-PoutRef. filter+SNR10%Ref. filter-SNR10%Filter i

In RAN1#122 meeting, regarding the evaluation metric of low-PAPR waveform the agreement is shown as follows:

Agreement - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

Therefore, net gain could also be considered as the evaluation metric for low-PAPR waveform from RAN4 side and carry on the tradition of R18. And at the same time, spectrum utilization optimization could also be considered as another target for waveform evaluation.

Generally, these two metrics are in a opposite relationship, evaluating waveforms with the goal of achieving a positive net gain requires ensuring the same spectrum utilization, as indicated in the RAN1 agreement. On the other hand, evaluating waveforms with the goal of improving spectrum utilization cannot achieve MPR enhancement, meaning it cannot realize a positive net gain. Therefore, when evaluating different waveform schemes, these two metrics need to be considered separately. Moreover, since the RAN1 side has not considered the research direction of improving SU, the SU enhancement in RAN4 should be evaluated using waveforms that do not affect the RAN1 side, i.e., transparent waveforms.

Observation 3: Net gain and SU improvement are opposite objectives in waveform enhancement research: evaluating waveforms with the goal of achieving a positive net gain requires maintaining the same spectrum utilization, while evaluating waveforms with the goal of improving spectrum utilization cannot achieve MPR enhancement, meaning it cannot realize a positive net gain.

Proposal 1: When evaluating different waveform schemes, the benefits of net gain and spectrum utilization should be considered separately, meaning the performance of each waveform should be evaluated based on these two metrics individually:

- Evaluating waveforms with the goal of achieving a positive net gain should maintain the same spectrum utilization.

- Evaluating waveforms with the goal of improving spectrum utilization should maintain the same MPR.

Proposal 2: The SU enhancement in RAN4 should be evaluated on the waveforms that do not affect the RAN1 side, i.e., transparent waveforms.

Proposal 3: For waveform enhancement schemes aimed at reducing PAPR, net gain should be used as the evaluation metric.

The baseline waveform

To better compare the performance of different waveform enhancement schemes, it is necessary to establish a baseline waveform. There are two recommended options: one is a baseline scheme without any spectral shaping, such as raw DFT/CP-OFDM; the other is an already implemented low-PAPR waveform, such as the FDSS scheme from R18.

The raw DFT/CP-OFDM waveform is suitable for performance studies on SU enhancement, i.e., observing the improvement in SU achieved by different waveform enhancement schemes. It is also applicable to low-PAPR waveform studies. For example, the net gain metric for low-PAPR waveforms during the R18 period was essentially comparing the MPR gain and BLER loss against the raw waveform.

Meanwhile, the R18 FDSS scheme should also serve as one of the comparative schemes for evaluating the net gain of new enhanced waveforms in 6G. That is, any new 6G waveform must demonstrate performance superior to that of FDSS to be considered a valid enhancement; otherwise, the FDSS scheme could continue to be adopted.

Proposal 4: The baseline for 6G waveform research should consider both raw DFT/CP-OFDM and the already implemented enhanced waveform schemes in the spec:

- Raw DFT/CP-OFDM can serve as a baseline for both PAPR reduction and SU enhancement.

- Implemented waveform enhancement schemes (such as FDSS) should serve as another baseline for low-PAPR waveforms, i.e., any new waveform must demonstrate superior net gain performance compared to schemes like FDSS to be considered a valid enhancement.

Initial simulation results of FDSS, CFR, CFR-SE, FDSS-SE and TR

Some initial simulation results could be seen as follows, the CCDF of PAPR is illustrated to show the performance of different schemes. The relative simulation assumptions are as shown in Annex. Comparing the baseline and five different spectrum shaping schemes, CFR-SE demonstrates better PAPR performance than FDSS, FDSS-SE, CFR and TR.

The net gain (calculated as MPR gain minus SNR loss at BLER=0.1) for π/2-BPSK and QPSK is also evaluated as follows:

For π/2-BPSK: Compared to the baseline of raw DFT-S-OFDM, transparent FDSS yields a net gain of 0.8 dB for both MCS0 and MCS1. Another transparent solution, CFR, achieves a net gain of 1dB. The CFR-SE solution provides an additional 0.5dB gain over transparent CFR for MCS0. For QPSK: The CFR-SE solution demonstrates superior performance, securing an additional 1dB net gain relative to transparent CFR. This result is primarily driven by significant MPR gain, a consequence of low PAPR and OOBE, that effectively compensate for a minor BLER performance penalty. Meanwhile, CFR-SE also shows better net gain performance than FDSS-SE and TR, not only for π/2-BPSK but also for QPSK.

Observation 4: CFR-SE demonstrates superior performance in terms of PAPR and net gain compared to FDSS, FDSS-SE, CFR and TR.

From RAN4 side, some evaluation regarding diverse low-PAPR waveform schemes could also be started, including the impact of spectrum shaping on RF requirements such as ACLR and EVM. And the requirements in the extended RBs may also need further analysis as in some schemes these extended RBs don’t transmit the actual data and are not part of the allocated RBs. Finally, the value of MPR reduction or power boosting should be simulated as the key point of PAPR reduction. And after the final scheme is specified, EVM spectrum flatness also need re-evaluation to accommodate the shaping.

Proposal 5: Study transparent and non-transparent techniques to further reduce PAPR, including CFR-SE. RAN 4 could start the evaluation of affected RF requirements, such as EVM, ACLR, MPR and applicable requirements in the extended RBs if needed. Other related spec impact needs further identification.

The alignment of evaluation assumption

RAN4 should align the simulation assumption for MPR evaluation of low PAPR waveform, e.g., modulation order, prioritized CBW, power class, and so on. The following table is provided as a reference.

Simulation Assumption for MPR evaluation

| Parameter | Value |
| --- | --- |
| Carrier frequency and scenario | 7GHz (Urban), 3.5GHz |
| Channel BW | 200MHz (for 7GHz band), 100MHz, 20MHz |
| UE power class | 23dBm or 26dBm |
| SCS | 30 kHz, 15kHz |
| Waveform | DFT-s-OFDM, CP-OFDM |
| Modulation | At least π/2-BPSK、QPSK |
| Extension factor [FDSS-SE] / sideband size [TR][CFR-SE] (α), apply for non-transparent schemes | 0, 0.111(1/9), 0.125, 0.25, 0.375, 0.5 as starting point, other values not precluded |

Proposal 6: RAN4 should align the simulation assumption for MPR evaluation for low PAPR waveform, the following table could be considered as a reference:

Simulation Assumption for MPR evaluation

| Parameter | Value |
| --- | --- |
| Carrier frequency and scenario | 7GHz (Urban), 3.5GHz |
| Channel BW | 200MHz (for 7GHz band), 100MHz, 20MHz |
| UE power class | 23dBm or 26dBm |
| SCS | 30 kHz, 15kHz |
| Waveform | DFT-s-OFDM, CP-OFDM |
| Modulation | At least π/2-BPSK、QPSK |
| Extension factor [FDSS-SE] / sideband size [TR][CFR-SE] (α), apply for non-transparent schemes | 0, 0.111(1/9), 0.125, 0.25, 0.375, 0.5 as starting point, other values not precluded |

Clarify the scope of RAN4’s responsibilities

Regarding low-PAPR waveform schemes, RAN4 would like to engage early in the evaluation of waveform schemes, allowing us to conduct MPR analysis on potential waveform candidates at an earlier stage. Meanwhile, the waveform selection process in RAN1 also requires MPR metrics for net gain calculation, meaning RAN1 will simultaneously perform MPR simulations. Therefore, it is necessary to discuss how to avoid duplicated evaluation efforts between RAN1 and RAN4, as well as how to delineate the responsibilities between the two groups, such as R4 should provide the MPR and other related information to R1 or R4 collect the SNR loss and maybe throughput information from R1 and further calculate the net gain. At the very least, this discussion should be initiated within RAN4, since our MPR evaluation work often occur later than those in RAN1. Also, when and how to organize the joint discussion from RAN1 and RAN4 could also be discussed if necessary.

Proposal 7: For the MPR evaluation of low-PAPR waveforms, RAN4 should internally discuss how to avoid duplicating the evaluation work with RAN1 and define the scope of RAN4's responsibilities. Also, when and how to organize the joint discussion from RAN1 and RAN4 could also be discussed if necessary.

PA model

Another important issue is the selection of PA model. The evaluation of MPR needs a certain PA model to simulate the power back-off value, and different PA model would bring about diverse output performance and different saturation point. There is an agreement to align a consensus on the PA model assumption for more accurate simulation.

Deadline for aligning the PA model for RAN1

In the 116bis meeting, RAN1 sent a LS to RAN4 for the feedback of an appropriate PA model and other relevant RF parameters for UL and DL evaluations.

1. Overall Description: For waveform evaluations, e.g. evaluations of PAPR reduction/MPR, PA models are needed. RAN1 respectfully asks RAN4 to provide appropriate PA model(s) and other relevant RF parameters for both downlink and uplink evaluations, at least for around 7GHz. Any timely feedback is appreciated, before which RAN1 continues waveform evaluations with PA model assumptions that might be based on companies’ inputs. 2. Actions: To RAN4 ACTION: RAN1 would like to respectfully ask RAN4 to provide feedback in a timely manner. 3. Date of Next RAN WG1 Meetings: RAN WG1 Meeting#123 November 17th – 21st 2025 Dallas, USA RAN WG1 Meeting#124 February 9th – 13rd 2026 Gothenburg, Sweden

Considering the limited TU arrangements and the timeliness requirements for low-PAPR waveform evaluation from RAN1, RAN4 should establish a deadline for finalizing the PA model and make every effort to unify a PA model that is widely recognized by all parties and suitable for waveform evaluation. This model should, for instance, support larger bandwidths and account for memory effects, prioritizing the 7 GHz and sub-6 GHz frequency bands, among other criteria. Simultaneously, new calibration point assumptions need to be adapted for the new PA model. Taking into account the above factors as well as the check point of RAN1, it is appropriate to set RAN4 #118 in February or #118bis in April as the deadline for providing RAN1 with a unified PA model.

Proposal 8: Set RAN4 #118 (February) or 118bis meeting (April) as the deadline for RAN4 to reach consensus on the PA model to be provided to RAN1.

Decouple the discussion of PA model for RAN1 and for RAN4

Meanwhile, RAN4 needs to continue conducting internal discussions on the PA model for 6G. This means that after providing a PA model to RAN1, RAN4 will continue optimizing the PA model for 6G usage. Therefore, the model provided to RAN1 will primarily be used to compare the performance of different waveform enhancement schemes, while the final RAN4 RF requirements evaluation for the selected waveform must be based on the ultimately unified 6G PA model. Consequently, this final model may differ from the one provided to RAN1.

Proposal 9: The ultimate MPR specification formulation for low-PAPR waveforms by RAN4 should be based on the final converged PA model, which may differ from the PA model fed back to RAN1.

At the same time, the simple and traditional AM-AM/AM-PM model should also be considered as the baseline, and we should try to identify what are the most important and needed parameters that should be updated in this model given to R1. From one aspect, the memoryless models below sub6G still have certain value for the initial assessment of RAN1 as they only need to compare the performance difference between different waveform schemes and the baseline, and it is sufficient as long as the linearity of the PA model can guarantee sufficient power gain. And only focusing on handheld UE is recommended at this stage.

Some preliminary fitted PA models

We did some test work of two actual PAs, one is a PA model, another is a demo FEM PA, both PAs are work at 7GHz while can support 200MHz signal bandwidth. The input data and the output data obtained after passing through an actual PA were fitted in MATLAB, resulting in a memory-based PA model with significant practical reference value. This model can operate around 7 GHz and support 200 MHz bandwidth.

The process of obtaining test input and output data using the actual PA model generally follows these steps: A 200 MHz baseband signal is generated in MATLAB and fed into a signal generator to produce the input data. With appropriate PA supply voltage, the signal generated by the signal generator is input to the PA. The PA output then passes through an attenuator before being fed into a spectrum analyzer to obtain the output sampled data.

Fig 3. Test environment diagram for 7GHz PA module

The reference PA model is an improved version of the Volterra model, i.e., Memory Polynomial (MP) model. This model retains only the diagonal terms while setting the cross-terms to zero. For the nonlinear order K, we only preserve the odd-order nonlinear terms. Even-order nonlinear terms are disregarded since they generate harmonics at DC and out-of-band frequencies, which can be effectively filtered out by RF bandpass filters in practical applications. This approach is also fully consistent with the method described in reference [6].

Using the synchronized input and output data, fitting was performed in MATLAB with a maximum nonlinear order of K=15 (where K only takes odd values: 1, 3, 5, ..., 15) and a memory depth of M=8. The normalized AM-AM plot between the input data and both the measured output data and the fitted output data is shown in the figure below. Due to memory effects, the AM-AM relationship between input and output data appears not as a single curve but as a cluster of discrete points.

The high degree of overlap between the blue measured points and orange simulated points indicates a relatively ideal AM-AM and AM-PM fitting performance. This suggests good overall fitting between the input and output data. The NMSE is approximately -18.5 dB, though the strong memory effects inherent in wideband PA models naturally contribute to some degradation in NMSE performance.

Fig 4. Fitting results of PA model

The final fitted PA model takes the following form.

yn= k=1,3,5…Km=0Makm*xn-m*|x(n-m)|k-1

y(n): The output signal of the nth sampling point

x(n): The input signal of the nth sampling point

K: Nonlinear order, take value of 11.

M: Memory depth, take values of 8.

akm​: Model coefficient matrix:

akm=

| -1.910711 + 0.1504872i | 2.551149 + 1.343539i | -6.569992 - 3.966481i | 8.958688 + 4.100210i | -4.253517 - 0.5814711i | 0.07352530 - 0.7917325i |
| --- | --- | --- | --- | --- | --- |
| 0.5216171 - 0.07906193i | 0.004630474 - 0.1178719i | -1.258002 + 0.3309597i | 4.836217 - 0.001656875i | -7.035542 - 0.6939270i | 3.376813 + 0.4725811i |
| -0.3276958 + 0.05705146i | 0.1707057 - 0.3582986i | -0.1481792 + 2.984477i | -0.8889450 - 10.30096i | 2.039757 + 14.17936i | -1.074367 - 6.548604i |
| 0.4035459 - 0.08481854i | -0.1609275 - 0.1712265i | 0.2573957 - 0.06138287i | 0.002127106 + 2.652275i | -0.08159365 - 5.754862i | -0.1288747 + 3.343694i |
| -0.3107937 + 0.07952406i | -0.5091758 + 0.1209539i | 4.240531 - 0.8015754i | -12.48951 + 2.499498i | 14.91887 - 3.473583i | -6.166041 + 1.672799i |
| 0.2320184 - 0.04514880i | 0.4401533 - 0.5734600i | -3.269523 + 5.081731i | 8.569656 - 16.08264i | -9.499244 + 20.27052i | 3.768901 - 8.677537i |
| -0.2048200 + 0.01689607i | 0.3415792 + 0.4402473i | -1.643197 - 2.003788i | 3.164426 + 3.922279i | -1.888563 - 3.167664i | 0.07308199 + 0.8418229i |
| 0.1082316 - 0.05234183i | 0.06460105 + 0.8307763i | -0.4203463 - 5.475287i | 1.459330 + 14.72318i | -2.398578 - 17.02229i | 1.285395 + 6.998043i |
| -0.05387904 - 0.003554505i | 0.1190551 + 0.07313019i | -1.039503 - 0.3111439i | 3.896023 + 0.05709343i | -5.674473 + 0.5692979i | 2.701834 - 0.3934415i |

For the 7GHz demo FEM PA, due to the limited time, we only get the measurement resulted of am-am and am-pm, as shown in below:

Figure 5. AM-AM/AM-PM of 7GHz FEM PA

A memory-less model can be extracted from the data above but it could be meaning less for PA operate in such larger channel bandwidth. We will update the memory model parameter for this more practical FEM PA.

Proposal 10: Recommend to adopt this memory effect model for 7GHz with a 200M bandwidth as the unified PA model for 6G.

yn= k=1,3,5…11m=08akm*xn-m*|x(n-m)|k-1

Where

y(n): The output signal of the nth sampling point

x(n): The input signal of the nth sampling point

akm=

| -1.910711 + 0.1504872i | 2.551149 + 1.343539i | -6.569992 - 3.966481i | 8.958688 + 4.100210i | -4.253517 - 0.5814711i | 0.07352530 - 0.7917325i |
| --- | --- | --- | --- | --- | --- |
| 0.5216171 - 0.07906193i | 0.004630474 - 0.1178719i | -1.258002 + 0.3309597i | 4.836217 - 0.001656875i | -7.035542 - 0.6939270i | 3.376813 + 0.4725811i |
| -0.3276958 + 0.05705146i | 0.1707057 - 0.3582986i | -0.1481792 + 2.984477i | -0.8889450 - 10.30096i | 2.039757 + 14.17936i | -1.074367 - 6.548604i |
| 0.4035459 - 0.08481854i | -0.1609275 - 0.1712265i | 0.2573957 - 0.06138287i | 0.002127106 + 2.652275i | -0.08159365 - 5.754862i | -0.1288747 + 3.343694i |
| -0.3107937 + 0.07952406i | -0.5091758 + 0.1209539i | 4.240531 - 0.8015754i | -12.48951 + 2.499498i | 14.91887 - 3.473583i | -6.166041 + 1.672799i |
| 0.2320184 - 0.04514880i | 0.4401533 - 0.5734600i | -3.269523 + 5.081731i | 8.569656 - 16.08264i | -9.499244 + 20.27052i | 3.768901 - 8.677537i |
| -0.2048200 + 0.01689607i | 0.3415792 + 0.4402473i | -1.643197 - 2.003788i | 3.164426 + 3.922279i | -1.888563 - 3.167664i | 0.07308199 + 0.8418229i |
| 0.1082316 - 0.05234183i | 0.06460105 + 0.8307763i | -0.4203463 - 5.475287i | 1.459330 + 14.72318i | -2.398578 - 17.02229i | 1.285395 + 6.998043i |
| -0.05387904 - 0.003554505i | 0.1190551 + 0.07313019i | -1.039503 - 0.3111439i | 3.896023 + 0.05709343i | -5.674473 + 0.5692979i | 2.701834 - 0.3934415i |

- Conclusion

Observation 1: Waveform with high PAPR would decrease PA efficiency and further reduce maximum output power, which is a key requirement in RAN4.

Observation 2: Transparent spectrum shaping schemes to reduce PAPR such as FDSS have been discussed extensively in 5G R15 and R18 stage.

Observation 3: Net gain and SU improvement are opposite objectives in waveform enhancement research: evaluating waveforms with the goal of achieving a positive net gain requires maintaining the same spectrum utilization, while evaluating waveforms with the goal of improving spectrum utilization cannot achieve MPR enhancement, meaning it cannot realize a positive net gain.

Proposal 1: When evaluating different waveform schemes, the benefits of net gain and spectrum utilization should be considered separately, meaning the performance of each waveform should be evaluated based on these two metrics individually:

- Evaluating waveforms with the goal of achieving a positive net gain should maintain the same spectrum utilization.

- Evaluating waveforms with the goal of improving spectrum utilization should maintain the same MPR.

Proposal 2: The SU enhancement in RAN4 should be evaluated on the waveforms that do not affect the RAN1 side, i.e., transparent waveforms.

Proposal 3: For waveform enhancement schemes aimed at reducing PAPR, net gain should be used as the evaluation metric.

Proposal 4: The baseline for 6G waveform research should consider both raw DFT/CP-OFDM and the already implemented enhanced waveform schemes in the spec:

- Raw DFT/CP-OFDM can serve as a baseline for both PAPR reduction and SU enhancement.

- Implemented waveform enhancement schemes (such as FDSS) should serve as another baseline for low-PAPR waveforms, i.e., any new waveform must demonstrate superior net gain performance compared to schemes like FDSS to be considered a valid enhancement.

Observation 4: CFR-SE demonstrates superior performance in terms of PAPR and net gain compared to FDSS, FDSS-SE, CFR and TR.

Proposal 5: Study transparent and non-transparent techniques to further reduce PAPR, including CFR-SE. RAN 4 could start the evaluation of affected RF requirements, such as EVM, ACLR, MPR and applicable requirements in the extended RBs if needed. Other related spec impact needs further identification.

Proposal 6: RAN4 should align the simulation assumption for MPR evaluation for low PAPR waveform, the following table could be considered as a reference:

Simulation Assumption for MPR evaluation

| Parameter | Value |
| --- | --- |
| Carrier frequency and scenario | 7GHz (Urban), 3.5GHz |
| Channel BW | 200MHz (for 7GHz band), 100MHz, 20MHz |
| UE power class | 23dBm or 26dBm |
| SCS | 30 kHz, 15kHz |
| Waveform | DFT-s-OFDM, CP-OFDM |
| Modulation | At least π/2-BPSK、QPSK |
| Extension factor [FDSS-SE] / sideband size [TR][CFR-SE] (α), apply for non-transparent schemes | 0, 0.111(1/9), 0.125, 0.25, 0.375, 0.5 as starting point, other values not precluded |

Proposal 7: For the MPR evaluation of low-PAPR waveforms, RAN4 should internally discuss how to avoid duplicating the evaluation work with RAN1 and define the scope of RAN4's responsibilities. Also, when and how to organize the joint discussion from RAN1 and RAN4 could also be discussed if necessary.

Proposal 8: Set RAN4 #118 (February) or 118bis meeting (April) as the deadline for RAN4 to reach consensus on the PA model to be provided to RAN1.

Proposal 9: The ultimate MPR specification formulation for low-PAPR waveforms by RAN4 should be based on the final converged PA model, which may differ from the PA model fed back to RAN1.

Proposal 10: Recommend to adopt this memory effect model for 7GHz with a 200M bandwidth as the unified PA model for 6G.

yn= k=1,3,5…11m=08akm*xn-m*|x(n-m)|k-1

Where

y(n): The output signal of the nth sampling point

x(n): The input signal of the nth sampling point

akm=

| -1.910711 + 0.1504872i | 2.551149 + 1.343539i | -6.569992 - 3.966481i | 8.958688 + 4.100210i | -4.253517 - 0.5814711i | 0.07352530 - 0.7917325i |
| --- | --- | --- | --- | --- | --- |
| 0.5216171 - 0.07906193i | 0.004630474 - 0.1178719i | -1.258002 + 0.3309597i | 4.836217 - 0.001656875i | -7.035542 - 0.6939270i | 3.376813 + 0.4725811i |
| -0.3276958 + 0.05705146i | 0.1707057 - 0.3582986i | -0.1481792 + 2.984477i | -0.8889450 - 10.30096i | 2.039757 + 14.17936i | -1.074367 - 6.548604i |
| 0.4035459 - 0.08481854i | -0.1609275 - 0.1712265i | 0.2573957 - 0.06138287i | 0.002127106 + 2.652275i | -0.08159365 - 5.754862i | -0.1288747 + 3.343694i |
| -0.3107937 + 0.07952406i | -0.5091758 + 0.1209539i | 4.240531 - 0.8015754i | -12.48951 + 2.499498i | 14.91887 - 3.473583i | -6.166041 + 1.672799i |
| 0.2320184 - 0.04514880i | 0.4401533 - 0.5734600i | -3.269523 + 5.081731i | 8.569656 - 16.08264i | -9.499244 + 20.27052i | 3.768901 - 8.677537i |
| -0.2048200 + 0.01689607i | 0.3415792 + 0.4402473i | -1.643197 - 2.003788i | 3.164426 + 3.922279i | -1.888563 - 3.167664i | 0.07308199 + 0.8418229i |
| 0.1082316 - 0.05234183i | 0.06460105 + 0.8307763i | -0.4203463 - 5.475287i | 1.459330 + 14.72318i | -2.398578 - 17.02229i | 1.285395 + 6.998043i |
| -0.05387904 - 0.003554505i | 0.1190551 + 0.07313019i | -1.039503 - 0.3111439i | 3.896023 + 0.05709343i | -5.674473 + 0.5692979i | 2.701834 - 0.3934415i |

- References

[1] R4-2514641, WF for 6GR system parameter, Huawei, HiSilicon, RAN4#116bis.

[2] R1-2505913, Waveforms for 6GR air interface, Apple, RAN1#122.

[3] R1-2506306, Discussion on Waveform, NTT DOCOMO, INC., RAN1#122.

[4] R1-2506020, Waveform for 6GR air interface, MediaTek Inc., RAN1#122.

[5] R1-2505416, Discussion on waveform for 6GR air interface, vivo, RAN1#122.

[6] M. Abdi Abyaneh and Y. Liu, “Evaluation of PA Memory Effects from 5G-NR to 6G Standards,” presented at the IEEE Conference on Standards for Communications and Networking (CSCN) 2025, Bologna, Italy, Sep. 2025.

- Annex

Simulation Assumption

| Parameter | Value |
| --- | --- |
| Channel | PUSCH, 14 symbols |
| Carrier frequency and scenario | 7GHz (Urban) |
| Channel BW | 100MHz (for 7GHz band) |
| UE power class | 23dBm |
| SCS | 30 kHz |
| Channel model | TDL-C 300ns for FR1 Urban (7GHz) |
| UE speed | 3km/h |
| Waveform | DFT-s-OFDM |
| Modulation | π/2-BPSK、QPSK |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 8 |
| Number of DMRS symbols | 2 |
| Number of PUSCH data symbols | 12 |
| HARQ configuration | No retransmissions |
| Frequency hopping | Disabled |
| Number of PRBs | BLER evaluation: 40 PRBs |
| Extension factor [FDSS-SE] / sideband size [TR][CFR-SE] (α) | [1/4] as starting point |
| BLER | 10% |