3GPP TSG-RAN WG4 Meeting #117															R4-2520750

Dallas, USA, 17th – 21st November, 2025

Agenda item:			8.2.2

Source:	Samsung

Title:	Discussion on modulation for 6GR

Document for:	Discussion

1	Introduction

In RAN4#116bis, the latest meeting, RAN4 got started a SI for a single technology framework based on a standalone architecture needs to be developed across WGs based on the 6G Radio SID [1]. In regards to 6GR modulation, relevant proposals and agreements were captured in the WF as follows [2]:

- Agreement - RAN4 evaluation work focus on the feasibility study and RF requirements impact - To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1 - Identify the main affected requirements for modulation evaluations - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions. - Align on the evaluation assumptions. - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation. - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed. - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1. - Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies. - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

In this contribution, we provide our views on the following aspects keeping in mind the previous agreements of RAN4 and RAN1:

- Evaluation methodology and common assumptions

- Higher-order modulations (DL 4096-QAM, UL 1024-QAM)

- Non-uniform constellations and probabilistic shaping

2	Discussion

2.1	Evaluation methodology and common assumptions

At its last meeting, RAN4 confirmed that the modulation study in RAN4 should stay aligned with RAN1 inputs, while reusing the well-known 5G NR RF requirement structure wherever this is reasonable [2]. Higher-order QAM is considered worth an early feasibility check, but expanding the modulation option would mainly increase test combinations and editorial overhead. For non-uniform items such as probabilistic shaping, most design choices lie in the coding and mapping chain, so it is natural that RAN1 leads the definition and timing. Within this context, the intention of this contribution is to suggest a compact way for RAN4 to evaluate cases in a consistent and comparable manner.

Observation 1:	RAN4 discussed higher-order QAM and non-uniform constellation such as probabilistic shaping for 6G modulation evaluation.

In practice, the net benefit of higher-order modulation depends strongly on the required SNR at a target BLER, the EVM headroom at the transmitter, and realistic transmitter operation, in particular PA back-off linked to MPR, memory effects for wider bandwidths, and the use (or not) of DPD. These aspects interact with spectrum or regulatory compliance (ACLR, SEM and out-of-band emissions) and can offset nominal throughput gains. By contrast, probabilistic shaping changes only the symbol probabilities, while the constellation coordinates remain unchanged. If this probabilistic sharing is introduced for modulation, RAN4 shall evaluate the existing RF requirements (such as EVM, ACLR and SEM/OOBE) under this new modulation, by comparing the legacy modulation with the average power being maintained in the same level.

Observation 2:	Higher-order modulation discussion depends strongly on the realistic transmitter operation and spectrum or regulatory compliance (ACLR and SEM/OOBE).

Observation 3:	Probabilistic shaping changes only the symbol probabilities, while RAN4 shall still evaluate its impact on relevant RF requirements (such as EVM, ACLR and SEM/OOBE) by assuming the probability sharing agreed in RAN1 and resultant constellation expansion after power normalization.

For each modulation case, it may be useful that RAN4 adopts a single, harmonized methodology so that results across different modulation orders and constellation types remain directly comparable. In doing so, RAN4 could refer to the same transmitter-chain assumptions as in other 6G studies (for example, the waveform study), such as a PA back-off policy linked to MPR, the enablement of memory effects when the channel bandwidth makes them relevant, and an explicit indication of whether DPD is active. The intention is not to introduce a new family of transmitter models in the modulation context, but rather to let the evaluation inherit one coherent set of assumptions, thereby avoiding artificial gains or penalties that might arise from inconsistent modeling. On the receiver side, it may be sufficient to check the baseline decoder and interleaver for transparency; unless RAN1 explicitly requests further work, RAN4 would not be expected to optimize or modify the coding/interleaving chain, beyond documenting the effective code rate and the BLER target used in the SNR and EVM evaluations.

Proposal 1:		RAN4 should adopt a single, harmonized methodology so that results across different modulation orders and constellation types remain directly comparable.

On top of these items, it may also be helpful to add a simple indication of whether any requirement change is suggested for that case, together with a short explanation. In the absence of strong and repeatable gains that do not require disproportionate back-off or test effort, it would be natural for RAN4 to reuse the 5G requirement values as much as possible and to focus on the deltas caused by 6G-specific assumptions.

Proposal 2:		RAN4 should reuse the 5G requirement values as much as possible and to focus on the deltas caused by 6G-specific assumptions.

2.2	Higher-order modulations (DL 4096-QAM, UL 1024-QAM)

According to [2], QPSK up to 1024QAM in the downlink and π/2-BPSK/QPSK up to 256QAM in the uplink are supported as the baseline modulation orders for “Day-1” 6G specifications, while the feasibility of UL 1024QAM and DL 4096QAM is identified as a study topic for optional higher-order modulations. In parallel, RAN1 discussion on 6GR modulation also notes that current studies are already considering constellations up to 1024QAM (UL), and even up to 4096QAM (DL), as a way to increase spectral efficiency at high SNR, while also observing that for uniform 1024QAM the gap between BICM capacity and unconstrained Shannon capacity can reach around 1.5 dB. These points together suggest that higher-order QAM is relevant for 6GR, but that its practical use must be examined carefully from an RF feasibility and requirement-impact perspective.

Observation 4:	Both RAN4 and RAN1 identified UL 1024QAM and DL 4096QAM as a study topic for optional higher-order modulations.

On the downlink side, the main motivation for 4096QAM lies in the potential spectral-efficiency gain when the link operates in high-SNR conditions. At the same time, DL operation must respect the existing EVM targets, available PA headroom in practical gNB deployments, and the multiplexing behavior under multi-user scheduling. As indicated in the WF, RAN4 is expected to focus on feasibility studies and RF-requirement impact, using the existing 5G NR requirements as a starting point and refining them only when justified by 6G-specific findings. In such a setting, it appears reasonable that any study on DL 4096QAM would (i) take CP-OFDM with 1024QAM as a reference, (ii) reuse the same measurement bandwidths and filters as in NR for transmitter-quality and emission checks, and (iii) examine whether the present EVM limits can still be met without requiring excessive back-off that would effectively remove the expected throughput gain. When additional insight is needed for particular high-throughput use cases, RAN4 could, if desired, perform sensitivity checks on how much extra back-off is needed to preserve ACLR and SEM/OBE margins at the intended operating point. Such checks would be informative in nature and would not, by themselves, imply a change of requirements.

Observation 5:	DL 4096QAM operation must respect the existing EVM targets, available PA headroom in practical gNB deployments, and the multiplexing behavior under multi-user scheduling.

Proposal 3:		Any study on DL 4096QAM would (i) take CP-OFDM with 1024QAM as a reference, (ii) reuse the same measurement bandwidths and filters as in NR for transmitter-quality and emission checks, and (iii) examine whether the present EVM limits can still be met without requiring excessive back-off that would effectively remove the expected throughput gain.

For the uplink, 1024QAM is further constrained by the UE transmitter chain, especially by back-off practices associated with MPR, the need to support larger CBWs with PA memory effects, and the limited form factor and power capability of handheld devices. The WF suggests that studies for enabling higher-order modulations should include MPR, required SNR, throughput and coverage, and could also look at advanced UE techniques such as DPD, together with base-station receiver techniques, as possible enablers for UL 1024QAM. In this light, it may be useful that RAN4, when assessing UL 1024QAM as an optional feature, compares it directly against UL 256QAM under identical transmitter assumptions and documents the incremental differences in required SNR and EVM, together with the additional back-off needed to maintain spectrum-emission compliance. If the combined penalty in SNR and back-off tends to cancel the nominal throughput advantage in realistic high-SNR deployment scenarios, it would be natural to record such cases as not leading to any requirement change, while still keeping UL 1024QAM on the table as an optional feature for specific device types or operating points.

Observation 6:	UL 1024QAM is further constrained by the UE transmitter chain, especially by back-off practices associated with MPR, and the limited form factor and power capability of handheld devices.

Proposal 4:		RAN4 should compare it directly against UL 256QAM under identical transmitter assumptions and check the incremental differences in required SNR and EVM, together with the additional back-off needed to maintain regulatory compliance.

The main aspects that RAN4 might have to consider and check for optional higher-order QAM could be summarized as follows, for example:

- the link direction (DL 4096QAM vs. DL 1024QAM reference; UL 1024QAM vs. UL 256QAM reference);

- the assumed code rate and BLER target used to define the required-SNR point;

- the PA back-off and transmitter-chain assumptions (including any use of DPD and memory effects);

- the resulting required SNR at the BLER target and the EVM at the nominal operating point;

- the ACLR and SEM/OBE behavior at the chosen back-off level; and

- a short note on coverage or throughput impact and on implementation/test burden.

Based on these observations, a practical way forward could be that RAN4 treats DL 4096QAM and UL 1024QAM explicitly as optional features for which feasibility is studied under realistic transmitter and receiver assumptions, with the baseline remaining DL up to 1024QAM and UL up to 256QAM. In the initial phase, only a small number of representative cases may need to be considered, and the outcome would mainly be to clarify whether any consistent gain can be achieved without disproportionate back-off or complexity. Unless such gains are clearly demonstrated, it would seem reasonable that no change is proposed to the existing NR-based requirements, and that the optional higher-order cases are simply documented for future reference in line with the agreed WF scope.

Proposal 5:		RAN4 can treat DL 4096QAM and UL 1024QAM explicitly as optional features for which feasibility is studied under realistic transmitter and receiver assumptions, with the baseline remaining DL up to 1024QAM and UL up to 256QAM.

2.3	Non-uniform constellations and probabilistic shaping

In [2], it is stated that, for new non-uniform constellations, RAN4 evaluation should depend on RAN1 progress and explicit request. In RAN1, constellation shaping is broadly divided into geometric shaping (GS), which changes the constellation geometry, and probabilistic shaping (PS), which changes only the symbol probabilities while keeping the geometry fixed. RAN1 has identified to treat geometrically shaped 1D non-uniform constellations (1D-NUC) as candidate schemes for 6GR, noting that 1D-NUC can outperform uniform QAM over a wide SNR range, with performance gains increasing with modulation order, and that the PAPR increase and receiver-complexity impact remain limited. For PS, it describes as a technique that adjusts symbol probabilities via distribution matching (DM), leading to a higher effective LDPC code rate that must be taken into account when assessing performance.

Observation 7:	In RAN1, constellation shaping is broadly divided into geometric shaping (GS), which changes the constellation geometry, and probabilistic shaping (PS), which changes only the symbol probabilities while keeping the geometry fixed.

From an RF-evaluation perspective, these observations suggest that the main design work for non-uniform constellations lies in RAN1, and that the incremental impact on RAN4 activities is expected to be small. In the case of PS, as shown in Figure 1, the constellation coordinates remain unchanged and only the symbol occurrence probabilities are modified by the DM block; as long as average power, measurement bandwidth and filtering are kept the same, the existing RF measurements and definitions, such as EVM, ACLR and SEM/OBE, remain applicable without change.

Figure 1: Coding and mapping chain with PS/DM annotated

For such schemes, RAN4 would mainly need to be aware that the effective code rate is increased by DM and that the BLER target and code parameters used in RAN1 evaluations are clearly clarified, while the RF test procedures themselves can stay aligned with those for the corresponding uniform constellations.

Proposal 6:		If PS is shown to cause a significant and consistent change in amplitude statistics, RAN4 shall consider its impact on back-off policy, and relevant RF requirements.

For geometric shaping such as 1D-NUC, proposals to RAN1 indicate that 1D-NUC can provide performance gains over uniform QAM at similar or even lower receiver-side complexity in the SNR range of interest, with no significant PAPR increase compared to uniform QAM. When RAN1 identifies a specific GS candidate whose point locations depart from the uniform grid, RAN4 could evaluate it using the same EVM, ACLR and SEM/OOBE procedures, provided that the spectral shape and nominal power remain comparable to the baseline. Any discussion on detector type or receiver complexity may be checked for information, but would not typically fall within the core RF requirement-setting scope. On the uplink, if geometric shaping is combined with DFT-s-OFDM, the small PAPR differences reported by RAN1 can be treated within the existing transmitter-assumption framework by adjusting back-off in a consistent manner. Only if these differences were found to systematically require a materially different back-off across realistic operating points would RAN4 need to consider a specific note; otherwise, it would be natural to keep the current NR-based RF requirements unchanged.

Observation 8:	For geometric shaping such as 1D-NUC, proposals to RAN1 indicate that 1D-NUC can provide performance gains over uniform QAM at similar or even lower receiver-side complexity in the SNR range of interest, with no significant PAPR increase compared to uniform QAM.

The key points that RAN4 might have to confirm when non-uniform constellations are studied could, for example, be summarized as follows:

- whether the geometry is changed (GS/NUC) and whether the symbol probabilities are changed (PS/DM);

- the effective code rate and BLER target used in the underlying RAN1 evaluation;

- any observed EVM difference at the nominal operating point compared to a uniform-QAM reference;

- any required change in PA back-off in order to maintain ACLR and SEM/OBE margins; and

- a simple recommendation on whether the case suggests no requirement change, an informative note only, or a need for further input from RAN1.

Taken together, the current understanding from RAN1 studies and the WF agreements indicates that non-uniform constellations can be investigated when needed, but that RAN4’s work for such schemes is expected to remain limited. In particular, probabilistic shaping, as a probability-based method that keeps the constellation geometry unchanged before normalization and primarily affects the coding chain, it mainly requires clear identification of the associated code-rate and BLER assumptions. For geometric shaping, existing RF procedures appear sufficient as long as the overall spectral and power characteristics remain similar to those of the corresponding uniform constellations. Unless future RAN1 inputs reveal a clear and systematic RF impact that cannot be captured within the current framework, it would therefore seem reasonable for RAN4 to continue using the existing evaluation methods and NR-based RF requirements also for non-uniform constellations.

Observation 9:	For probabilistic shaping, as a probability-based method that keeps the constellation geometry unchanged before normalization and primarily affects the coding chain.

Observation 10:	For geometric shaping, existing RF procedures appear sufficient as long as the overall spectral and power characteristics remain similar to those of the corresponding uniform constellations.

Proposal 7:		Unless future RAN1 inputs reveal a clear and systematic RF impact that cannot be captured within the current framework, RAN4 should continue using the existing evaluation methods and 5G-based RF requirements also for non-uniform constellations

3	Conclusion

In this contribution, we provide our views on evaluation methodology, higher-order modulations (DL 4096-QAM, UL 1024-QAM), non-uniform constellations for 6GR modulation in RAN4. Following observations and proposals were derived to be considered by this meeting.

Evaluation methodology

Observation 1:	RAN4 discussed higher-order QAM and non-uniform constellation such as probabilistic shaping for 6G modulation evaluation.

Observation 2:	Higher-order modulation discussion depends strongly on the realistic transmitter operation and spectrum or regulatory compliance (ACLR and SEM/OOBE).

Observation 3:	Probabilistic shaping changes only the symbol probabilities, while RAN4 shall still evaluate its impact on relevant RF requirements (such as EVM, ACLR and SEM/OOBE) by assuming the probability sharing agreed in RAN1 and resultant constellation expansion after power normalization.

Proposal 1:		RAN4 should adopt a single, harmonized methodology so that results across different modulation orders and constellation types remain directly comparable.

Proposal 2:		RAN4 should reuse the 5G requirement values as much as possible and to focus on the deltas caused by 6G-specific assumptions.

Higher-order modulations

Observation 4:	Both RAN4 and RAN1 identified UL 1024QAM and DL 4096QAM as a study topic for optional higher-order modulations.

Observation 5:	DL 4096QAM operation must respect the existing EVM targets, available PA headroom in practical gNB deployments, and the multiplexing behavior under multi-user scheduling.

Proposal 3:		Any study on DL 4096QAM would (i) take CP-OFDM with 1024QAM as a reference, (ii) reuse the same measurement bandwidths and filters as in NR for transmitter-quality and emission checks, and (iii) examine whether the present EVM limits can still be met without requiring excessive back-off that would effectively remove the expected throughput gain.

Observation 6:	UL 1024QAM is further constrained by the UE transmitter chain, especially by back-off practices associated with MPR, and the limited form factor and power capability of handheld devices.

Proposal 4:		RAN4 should compare it directly against UL 256QAM under identical transmitter assumptions and check the incremental differences in required SNR and EVM, together with the additional back-off needed to maintain regulatory compliance.

Proposal 5:		RAN4 can treat DL 4096QAM and UL 1024QAM explicitly as optional features for which feasibility is studied under realistic transmitter and receiver assumptions, with the baseline remaining DL up to 1024QAM and UL up to 256QAM.

Non-uniform constellations

Observation 7:	In RAN1, constellation shaping is broadly divided into geometric shaping (GS), which changes the constellation geometry, and probabilistic shaping (PS), which changes only the symbol probabilities while keeping the geometry fixed.

Proposal 6:		If PS is shown to cause a significant and consistent change in amplitude statistics, RAN4 shall consider its impact on back-off policy, and relevant RF requirements.

Observation 8:	For geometric shaping such as 1D-NUC, proposals to RAN1 indicate that 1D-NUC can provide performance gains over uniform QAM at similar or even lower receiver-side complexity in the SNR range of interest, with no significant PAPR increase compared to uniform QAM.

Observation 9:	For probabilistic shaping, as a probability-based method that keeps the constellation geometry unchanged before normalization and primarily affects the coding chain.

Observation 10:	For geometric shaping, existing RF procedures appear sufficient as long as the overall spectral and power characteristics remain similar to those of the corresponding uniform constellations.

Proposal 7:		Unless future RAN1 inputs reveal a clear and systematic RF impact that cannot be captured within the current framework, RAN4 should continue using the existing evaluation methods and 5G-based RF requirements also for non-uniform constellations

Reference

[1] RP-252912	Revised SID: Study on 6G Radio		RAN#109

[2]	R4-2511652	Plan on RAN4 6G discussion and meeting arrangement in Rel-20	RAN4#116