3GPP TSG-RAN WG4 Meeting#117		R4-2520767

Dallas, USA, Nov 17th - 21th, 2025

Agenda item:	8.5

Source: 	Spreadtrum, UNISOC

Title: 	Views on 6G spectrum

Document for:	Discussion

Introduction

In RAN4#116bis meeting, a WF [1] about 6G spectrum was approved. In this contribution, we continue to provide some views on 6G spectrum.

Discussion

Frequency ranges

In 5G, spectrum was divided into FR1 and FR2 for TN: FR1 (from 410MHz to 7125MHz) and FR2-1(from 24.25GHz to 52.6GHz). For NTN, spectrum was divided into FR1-NTN and FR2-NTN: FR1-NTN (from 410MHz to 14.5GHz) and FR2-NTN (from 10.7GHz to 30GHz).

The most important principle of frequency range is numerology, now the numerology is under discussion in RAN1 and RAN plenary. In addition, a single SCS for sub frequency range should be follow in 6GR.

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

- WF - RAN4 acknowledge that a definition of frequency ranges depends on multiple criteria’s which may have different importance dependent on individual company views. As a starting point the following can be discussed, not precluding companies to also consider others: - Regulatory framework - UE RF front-end architecture (e.g. whether requirements can be verified conducted or not) - Commonality of UE RF/RRM requirements - Numerology dependency - Unification of TN and NTN operation - BS types and requirements - Companies are encouraged to share information on the criteria they are considering for their proposal on frequency range definition(s)

According to the TR 38.822, the UE architecture for from 7-10GHz range can be homodyne architecture. For 10-24GHz, heterodyne architecture can be applied. According to the TR 38.922, the antenna size for array antenna can be similar with sub-6G antenna in 12-15GHz. In addition, pathloss in free space will increase with the frequency increase. However, according to the SID, at least 7GHz has the same coverage target with 3.5GHz, the pathloss in free space in 7GHz is similar with 8GHz.Regarding to the antenna technology, array antenna can be applied to around 15GHz, 7-8GHz we can still use the patch antenna to compensate the pathloss by higher UL/DL MIMO. For NTN, Ka band from 10GHz to 20GHz can be applied to FR1 and FR2. In order to avoid multi SCS for one band, in our understanding, based on the above analysis, we can extend FR1 to 8.4GHz to use same SCS, UE architecture, antenna and so on. From 8.4GHz to 15GHz, these frequency have higher pathloss, antenna can be patch or array antenna. From 15GHz to 24GHz, array antenna, SCS, UE architecture can be same. Therefore, we can define FR1 extend to 8.4GHz, FR2 extend to 15GHz, 8.4GHz to 15GHz can be FR3. The boundary between FR3 and FR2-1 can be further studied. Our proposal about Frequency ranges in Figure1.

Figure1: proposed Frequency ranges in 6GR

Proposal 1: Proposed Frequency ranges in Fiugre1 in 6GR can be a starting point, the boundary (e.g., 15GHz) between FR3 and FR2-1 can be further studied.

Band/band combination definition and simplification

Band group concept

In 5G NR, for FR1 range, there are a total of 51 bands, for FR2, there are 7 bands. 5G NR FR1 and FR2-1 bands will be re-farming and reused in 6G, and FR3 will be introduced in 6GR. With regard to operators’ requests, there are so many bands and band combinations defined. However, most of the band combinations are not commercial. In order to support these band combinations, the complexity of UE implementation will obviously be increased. Thus, it is necessary to simplify the band combinations.

How to simplify the band combinations, a concept of band group is popular in 6GR. Based on the concept, the number of band combinations will obviously decrease. The concept of band group is that bands with different frequency bands are combined, switched within groups and aggregated between groups. Such as sub-1GHz can be group1, n1, n3, n39, etc. can be group2, n7, n41, etc. can be group3. Within one band group, we can choose one band by switching. In this one band group, requirements for these bands can be same. Inter band groups can be band combinations. In our understanding, the concept of band group can be studied in 6G.

Proposal 2: To simplify band combinations definition, band group concept can be studied in 6G.

Bandwidth combination Sets in 6G

In 5G, the bandwidth combination sets for NR CA and asymmetric channel bandwidth and intra-band contiguous ENDC.BCS can set to be BCS0/1/2/3/4/5. BCS 0/1/2/3 can support partial bandwidth, BCS4 can support all of bandwidth, and BCS 5 can support flexible bandwidth. Generally speaking, for single carrier or single band, which should be to support the maximum bandwidth. It is easier to achieve for a single carrier. However, it is difficult to support all the bandwidth which are defined in spec. Because the baseband processing capability is limited, especially for multi carriers. With the development of 6G, the number of CA may be more and more, and the requirement for baseband capability will be gradually improved. Therefore, UE implementation shouldn’t be limited to supporting all bandwidth, especially for different device types. In order to reduce the signalling overhead and keep the flexibility, BCS 5 can be kept, if UE can support the all bandwidth for CA/asymmetric channel bandwidth, and the value of BCS can be absent, which means support the all bandwidths for UE.

Proposal 3: BCS concept should be kept in 6GR. The number of BCS can be considered to be one in 6GR.

∆TIB and ∆RIB in 6G

∆TIB and ∆RIB are the additional tolerance for uplink and downlink in NR CA/SUL/ENDC. Because the insertion loss will increase when the carrier is in a band combination. It is observed that ∆TIB and ∆RIB are all under 1dB. On one side, if the band group will be introduced in 6GR, the number of band combinations will decrease, thus the analysis of ∆TIB and ∆RIB will be easier. On the other side, in order to reduce the workload for ∆TIB and ∆RIB, we can define a simple approach, such as set two gears of value of ∆TIB and ∆RIB based on some rules. In summary, we can improve the ∆TIB and ∆RIB rather than remove it.

Proposal 4: The approach to derive the ∆TIB and ∆RIB can be improved in 6GR.

Spectrum related regulatory survey

New spectrum

According the WRC-23 outcomes, studies on frequency-related matters for the terrestrial component of International Mobile Telecommunications identification in the frequency bands 3 300-3 400 MHz, 3 600-3 800 MHz, 6 425-7 025 MHz, 7 025-7 125 MHz and 10.0-10.5 GHz. And to invites the WRC-27 to consider, based on the results of studies, the identification of frequency bands:

– 4 400-4 800 MHz, or parts thereof, in Region 1 and Region 3;

– 7 125-8 400 MHz, or part thereof, in Region 2 and Region 3;

– 7 125-7 250 MHz and 7 750-8 400, or part thereof, in Region 1;

– 14.8-15.35 GHz,

For the terrestrial component of IMT. Based on the above information, we can focus on 6.425-7.125GHz, 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35GHz as candidate spectrum in 6G.

Proposal 5: 6.425-7.125GHz, 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35GHz as candidate spectrum to focus on studying in 6GR.

SAR/PD regulatory survey

ICNIRP

According to the ICNIRP, within the above 6GHz, EMF energy is deposited predominantly in superficial tissues; this makes SAR10g, which includes deeper tissues, less relevant to this frequency range.Conversely, absorbed power density (Sab) provides a measure of the power absorbed in tissue that closely approximates the superficial temperature rise. From 6 to 10 GHz there may still be significant absorption in the subcutaneous tissue. However, the maximum and thus worst-case temperature rise from 6 to 10GHz there may still be significant absorption in the subcutaneous tissue. However, the maximum and thus worst-case temperature rise from 6 to 300 GHz is close to the skin surface. It shows the basic restrictions for electromagnetic field exposure from 100 kHz to 300GHz, for averaging intervals ≥ 6min. Based on the ICNIRP, SAR can be applied in below 6GHz, MPE (PD) can be applied in above 6GHz in 6GR.

IEEE Standard C951

According to the IEEE Standards C951 in Annex B.1.3.1 General as follows, below 6GHz, SAR or MPE is applied metric, above 6GHz, MPE is applied metric. Therefore, SAR can be applied below 6GHz, MPE can be applied above 6GHz in 6GR. DRLs to protect against adverse health effects associated with surface heating are established for epithelial power density for local exposures at frequencies between 6 GHz and 300 GHz. Based on the IEEE Standard C951, SAR can be applied in below 6GHz, MPE (PD) can be applied in above 6GHz in 6GR.

FCC

According to the 47 CFR 1.1307(b) and 1.1310 and [7], frequencies from 100kHz to 6GHz, SAR limits are defined as follow.

- Extremities (hands, wrists, ankles, feet, pinnae) 4.0 W/Kg (10-gram averaging)

- Body/Head: 1.6 W/Kg (1-gram averaging)

Above 6GHz, Maximum Permissible Exposure (MPE) limits are applicable.

Observation1: Based on the IEEE Standard C951 and ICNIRP and FCC, SAR can be applied in below 6GHz, MPE (PD) can be applied in above 6GHz in 6GR.

Observation2: Based on the IEEE Standard C951 and ICNIRP, Epithelial power density is 20W/m2 for persons in unrestricted environments (General public) and 100W/m2 for persons in restricted environments (Occupational) for above 6GHz.

Observation3: Based on the FCC, for occupational in above 6GHz, MPE is 5mW/cm2; for occupational in above 6GHz MPE is 1mW/cm2.

Conclusions

In this paper, we provide our initial views on 6G spectrum, we have made the following proposals and observations.

Proposal 1: Proposed Frequency ranges in Fiugre1 in 6GR can be a starting point, the boundary (e.g., 15GHz) between FR3 and FR2-1 can be further studied.

Proposal 2: To simplify band combinations definition, band group concept can be studied in 6G.

Proposal 3: BCS concept should be kept in 6GR. The number of BCS can be considered to be one in 6GR.

Proposal 4: The approach to derive the ∆TIB and ∆RIB can be improved in 6GR.

Proposal 5: 6.425-7.125GHz, 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35GHz as candidate spectrum to focus on studying in 6GR.

Observation1: Based on the IEEE Standard C951 and ICNIRP and FCC, SAR can be applied in below 6GHz, MPE (PD) can be applied in above 6GHz in 6GR.

Observation2: Based on the IEEE Standard C951 and ICNIRP, Epithelial power density is 20W/m2 for persons in unrestricted environments (General public) and 100W/m2 for persons in restricted environments (Occupational).

Observation3: Based on the FCC, for occupational in above 6GHz, MPE is 5mW/cm2; for occupational in above 6GHz MPE is 1mW/cm2.

- Reference

- R4-2514650, WF on 6G Spectrum, Nokia. TSG-RAN4#116bis meeting

- TR 38.922

- TR 38.820

- ICNIRP GUIDELINES

- IEEE Standards for Safety Levels with Respect to Human Exposure to Electric, Magnetic, and Electromagnetic Fields, 0 Hz to 300 GHz

- FCC 47 CFR 1.1310/1307

- New FCC RF Exposure Rules