3GPP TSG-WG4 Meeting #117	R4-2520792

# Dallas, Texas, USA, 17-21 Nov. 2025

# Title:			Intra-Band Contiguous CA Guardband Issue

# Agenda Item:	10.2.1

# Source:		T-Mobile USA

# Document for:	Approval

## 1 Introduction

We previously raised the issue of the Foffset,high equation for 38.101-1 not providing sufficient guardband to meet the minimum guardband for the local carrier in [1] and [2]. In [3] Skyworks shared thoughts on this topic and proposed two options for resolving the issue.

In this contribution we show how Proposal 2 from [3] to increase the Foffset,high by one largest subcarrier is unnecessary in many cases because the current equations already provide sufficient guardband, and excessive in others according to our calculations because less than a maximum subcarrier of additional guardband is needed to meet the minimum guardband.

We propose an alternative solution to the ones proposed in [3] that would ensure the minimum guardband is met without requiring excessive base station retunes.

## 2 Discussion

We tried to recreate the calculations in R4-2514496, but we got slightly different numbers.

It seems like in [3] the author is using the nominal upper guardband for the high carrier calculations instead of the minimum guardband for the upper carrier. We believe the minimum guardband is appropriate.

Observation 1: Minimum guardband is the appropriate metric for the high carrier to ensure sufficient guardband rather than nominal guardband.

We also did calculations where both carriers had the 30 kHz SCS. In many cases there is 30 kHz insufficient guardband for the high carrier. Here are examples with 100 MHz paired with other bandwidths:

Observation 2: The low guardband seems to always be sufficient for the minimum guardband of the low carrier.

Observation 3: The high guardband is sometimes sufficient and sometimes insufficient for the minimum guardband of the high carrier. When it is insufficient, the magnitude is equal to the largest SCS supported by both bandwidths minus the SCS of the high CC.

Observation 4: Changing the minus to a plus in the equation for Foffset,high as proposed  [3] in always increases Foffset,high by the maximum SCS, which is more than the minimum required to meet the minimum high carrier guardband. Observation 5: For cases where the current Foffset,high equation provides sufficient guardband, always adding 60 kHz would require an unnecessary re-tune of existing base stations.

Out of curiosity, we checked if we check the max of the two guardbands for the deployed SCS were met. In some cases, Foffset,low provides insufficient guardband for the max of two carriers with the 30 kHz SCS even if 60 kHz were added:

# Also, in some cases Foffset,high provides insufficient guardband for the max of the 30 kHz SCS guardbands, even if 60 kHz were added.

# Observation 6: Calculating Foffset,low and Foffset,high with the maximum SCS, as specified in 38.101-1 does not always provide sufficient guardband for the Max of the guardbands of the deployed SCS, even if adjusted as proposed in R4-2514496.

# If we wanted to ensure that there was always sufficient guardband for the max to the deployed guardband when both CCs use the same SCS, then we should calculate Foffset,low,scs and Foffset,high,scs for each SCS, and then Foffset,low=Max(Foffset,low,scs) and Foffset,high=Max(Foffset,high,scs). It is probably too late for this. Maybe something to consider in 6G.

# Observation 7: For 6G we should consider calculating Foffset,low,scs and Foffset,high,scs for each SCS, and then Foffset,low=Max(Foffset,low,scs) and Foffset,high=Max(Foffset,high,scs).

# What we really need to ensure that the single carrier guardband for the high CC is met is to add a check for that.

# Observation 8: A simple way to ensure that the minimum single carrier guardband is met for the closest carrier is always met would be to add a check that the guardband of the closest carrier at the SCS is met.

# Proposal 1: Add a check that in addition to calculating Foffset,high, the the guardband of the high CC shall also be met.

# Also, because 38.101-1 allows contiguous intra-band CCs to be moved together closer than the nominal spacing, The equations for Foffset,low and Foffset,high should have a  “≥” sign rather than an “=” sign in order to make it clear that they are minimums.

# Proposal 2: To make it clear that Foffset,low and Foffset,high are minimums, the equations should be changed as follows:

# From:

# Foffset,low = (NRB,low*12 + 1)*SCSlow/2 + BWGB (MHz)

# Foffset,high = (NRB,high*12 - 1)*SCShigh/2 + BWGB (MHz)

# To:

# Foffset,low ≥ (NRB,low*12 + 1)*SCSlow/2 + BWGB (MHz)

# Foffset,high ≥ (NRB,high*12 - 1)*SCShigh/2 + BWGB (MHz)

# 3 Conclusions

# Observation 1: Minimum guardband should be the appropriate metric for the high carrier to ensure sufficient guardband rather than nominal guardband.

# Observation 2: The low guardband seems to always be sufficient for the minimum guardband of the low carrier.

# Observation 3: The high guardband is sometimes sufficient and sometimes insufficient for the minimum guardband of the high carrier. When it is insufficient, the magnitude is equal to the largest SCS supported by both bandwidths minus the SCS of the high CC.

# Observation 4: Changing the minus to a plus in the equation for Foffset,high as proposed  [3] in always increases Foffset,high by the maximum SCS, which is more than the minimum required to meet the minimum high carrier guardband.

# Observation 5: For cases where the current Foffset,high equation provides sufficient guardband, always adding 60 kHz would require an unnecessary re-tune of existing base stations.

# Observation 6: Calculating Foffset,low and Foffset,high with the maximum SCS, as specified in 38.101-1 does not always provide sufficient guardband for the Max of the guardbands of the deployed SCS, even if adjusted as proposed in R4-2514496.

# Observation 7: For 6G we should consider calculating Foffset,low,scs and Foffset,high,scs for each SCS, and then Foffset,low=Max(Foffset,low,scs) and Foffset,high=Max(Foffset,high,scs).

# Observation 8: A simple way to ensure that the minimum single carrier guardband is met for the closest carrier is always met would be to add a check that the guardband of the closest carrier at the SCS is met.

# Proposal 1: Add a check that in addition to calculating Foffset,high, the the guardband of the high CC shall also be met.

# Proposal 2: To make it clear that Foffset,low and Foffset,high are minimums, the equations should be changed as follows:

# From:

# Foffset,low = (NRB,low*12 + 1)*SCSlow/2 + BWGB (MHz)

# Foffset,high = (NRB,high*12 - 1)*SCShigh/2 + BWGB (MHz)

# To:

# Foffset,low ≥ (NRB,low*12 + 1)*SCSlow/2 + BWGB (MHz)

# Foffset,high ≥ (NRB,high*12 - 1)*SCShigh/2 + BWGB (MHz)

# 4 References

- R4-2506183, “Intra-band Contiguous CA Issue and Proposed Resolution,” 3GPP RAN4 Meeting #115, St. Julian’s, Malta, T-Mobile USA

- R4-2509585, “Intra-band Contiguous CA Issue and Proposed Resolution,” 3GPP RAN4 Meeting # 116, Bengaluru, India, T-Mobile USA.

- R4-2514496, “On Intra-Band Contiguous CA GB and Outer1,2,” 3GPP RAN4 Meeting #116bis, Prague, Czech Republic, Skyworks Solutions, Inc.