3GPP TSG-RAN WG4 Meeting #117 	R4-2520881

Dallas, USA, Nov. 17-21, 2025

Source: 	Xiaomi

Title: 	Discussion on the modified MPR remaining issues

Agenda item:	10.2.1

Document for:	Discussion

- Introduction

After several meetings discussion, there’re still some remaining issues which needs more thinking [1]. This contribution provides our views on these issues.

- Discussion

- For spec version number, the followings are some candidates for further discussion in next meeting.

  - Keep the version number and add the following note outside the table.

    - NOTE 2: Specification version in the “Definition” column in Table L.1-1 refer to the version N in which the requirements were modified. If UE supports release P which is later than release N then UE supports requirements defined in that release P.

  - Remove the version number following the approach similar with 38.101-2.

  - Remove the version number and using wording like “MPR/AMPR defined in clause xxx of present specification”.

  - Other solutions are not excluded.

For the following open issue:

Generally, we support the approach similar with 38.101-2. But when the requirements are changed more than once, version number may be needed for the changes which is not the last change.

Proposal 1: If the requirement is unchanged compared with the latest spec, the spec version number can be removed. Otherwise, the spec version number is kept.

- If the requirements are changed (tighten or relaxation) more than once, further discuss the following Options:

  - Option 1: Use separate ModifiedMPR-Behavior bits for the second changed MPR/AMPR

    - Note: Error correction can be made but the modifiedMPR-Behaviour bit is kept unchanged

  - Other options are not excluded

For the following open issue:

For this issue, we support option 1 but would not support unnecessary MPR changes.

Proposal 2: For the requirements which are changed several times, a separate ModifiedMPR-Behavior bit is used for a different requirement.

n41 correction: Remove MPR in bit 2 can be a candidate for further check, others follow the above agreements.

| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
| --- | --- | --- | --- |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |

For the correction n41, we support the following proposal.

- Conclusion

For the open issues in the WF [1], we have the following observations and proposals,

Proposal 1: If the requirement is unchanged compared with the latest spec, the spec version number can be removed. Otherwise, the spec version number is kept.

Proposal 2: For the requirements which are changed several times, a separate ModifiedMPR-Behavior bit is used for a different requirement.

n41 correction: Remove MPR in bit 2 can be a candidate for further check, others follow the above agreements.

| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
| --- | --- | --- | --- |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |

For the correction n41, we support the following proposal.

- References

[1] R4-2511776, “WF on Modified MPR Behavior”, Xiaomi, MediaTek Inc., OPPO, Nokia, RAN4#116