3GPP TSG-RAN WG4 Meeting #117	R4-2520887

# Dallas, USA, Nov. 17-21, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for enhanced Low Band CA operation

Agenda item:	7.8.4

Document for:	Discussion

Introduction

In this contribution, we provide our initial views on RRM issues for the enhanced LB CA via switching based on approved WF [1].

Discussion

General

MRTD assumption

In Rel-20 LB CA, the deployment constraints are the same as Rel-19 LB CA deployments. So, MRTD considered in Rel-19 LB CA can be reused.

- Proposal 1: The 3us maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching in Rel-20 LB CA.

SSB assumption

In Rel-19 LB CA via switching, RAN4 only considered that SDL SCell is transmitting SSB, i.e., no requirements for SSB-less scenarios. The deploymenet of LB CA via switching is co-located and synchronized network for both FDD and SDL carriers, so SSB-less scenario could be useful. The SSB based requirements for LB CA would be baseline. However, as agreed in the last meeting, most RRM requirements for Rel-19 LB CA via switching could be resued for Rel-20 enhanced LB CA via switching, so RAN4 needs to discuss and introduce SSB-less based requirements.

- Proposal 2: SSB based SDL SCell operation is baseline to define RRM requirements, and RAN4 to deiscuss and introduce RRM requirements for SSB-less SCell in enhanced LB CA via switching.

Carrier switching between CC/Switching pattern applicability

Interruption or scheduling restriction requirement for CC switching

In Rel-20 LB CA, the semi-static switching mechanisms based on RRC configuration is reused. Therefore, switching gap could be reused for enhanced LB CA via switching. Since no interruption and scheduling restriction deu to switching was required in Rel-19 LB CA via switching, it is also not to introduce interruption and scheduling restriction in Rel-20 due to switching.

- Proposal 3: For Rel‑20 LB CA via switching, it is proposed to maintain the same approach in Rel-19, i.e., no interruption or scheduling restriction shall be considered due to carrier switching.

Applicability of Rel-19 LB-CA RRM requirements

Requirement design for gap-less based L3 measurement and L1 measurement of activated SDL SCC - RRM requirement applicability

In Rel-19, RRM requirement applicability has been agreed

  - for gap-less based L3 measurement and L1 measurement of a serving carrier which can be either FDD PCC or activated SDL SCC:

    - a.   If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed.

    - b.   If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

    - c.   If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

In Rel-20, durting UE staty SDL carrier, UE can receive signal from both FDD DL and SDL carriers simultaneously. Therefore, the RRM requirement applicability for Rel-20 enhanced LB CA should be applied to only SDL carriers.

  - For gap-less based L3 measurement and L1 measurement of activated SDL SCC:

    - a.   If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed.

    - b.   If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

    - c.   If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Figure 1 LB CA via switching with dual DL between FDD and SDL band

- Proposal 4: RRM requirement applicability for Rel-20 enhanced LB CA via switching

  - For gap-less based L3 measurement and L1 measurement of activated SDL SCC:

    - a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed.

    - b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

    - c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Potential RRM requirement impact for Rel-20 LB-CA

Interruption requirement for deactivated SDL SCell measurement

RAN4 has defined the missed ACK/NACK rate due to deactivated SDL SCell measurement in Rel-19 below

  - Interruptions on PCell due to the measurement on deactivated SDL SCell are allowed with up to Y % probability of missed ACK/NACK. The value of Y is defined as 2*Length_intmeasCycleSCell ;where, Length_int is equal to  (Tuning time from PCell to SCell + TSMTC_duation + Retuning time from SCell to PCell)

Since enhanced LB CA via switching consider dual DL between FDD DL and SDL, the interruption requirements for deactivated SCell measurement cannot be reused. The interruption length for PCell UL is the same as the interruption length for Rel-19 LB CA, but the interruption length for PCell DL is 2 x retuning time due to dual DL operation. The interruption on PCell due to the measurement on deactivated SCell are allowed with up to [(2 x retuning time + SMTC duration)/measCycleSCell] probability of missed ACK/NACK.

- Proposal 5: For interruption length and missed ACK/NACK rete,

  - Interruption length for PCell UL: 2 x retuning time + SMTC duration

  - Interruption length for PCell DL: 2 x retuning time

  - Missed ACK/NACK rate : [(2 x retuning time + SMTC duration)/measCycleSCell

Conclusion

In this contribution, we provide our initial views on Rel-20 enhanced low band CA via switching with dual DL, and we propose

- Proposal 1: The 3us maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching in Rel-20 LB CA.

- Proposal 2: SSB based SDL SCell operation is baseline to define RRM requirements, and RAN4 to deiscuss and introduce RRM requirements for SSB-less SCell in enhanced LB CA via switching.

- Proposal 3: For Rel‑20 LB CA via switching, it is proposed to maintain the same approach in Rel-19, i.e., no interruption or scheduling restriction shall be considered due to carrier switching.

- Proposal 4: RRM requirement applicability for Rel-20 enhanced LB CA via switching

  - For gap-less based L3 measurement and L1 measurement of activated SDL SCC:

    - a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed.

    - b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

    - c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

- Proposal 5: For interruption length and missed ACK/NACK rete,

  - Interruption length for PCell UL: 2 x retuning time + SMTC duration

  - Interruption length for PCell DL: 2 x retuning time

  - Missed ACK/NACK rate : [(2 x retuning time + SMTC duration)/measCycleSCell

Reference

[1] R4-2514824, “WF for [116bis][224] NR_LBCA_Sw_enh_RRM” Moderator (Samsung)