3GPP TSG-RAN WG4 Meeting #117		R4-2520981

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.7.3

Source: 	CMCC

Title: 			 Discussion on RRM test cases for R19 ATG

Document for:	Discussion

- Introduction

After RAN4#116bis meeting, the only remaining issue is whether and how to introduce Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms or 160ms SCell measurement cycle (with signalling for neighbour cell skipping).

Issue 2-1-1: Test scope - separate tests for neighbour cell skipping/not skipping Agreement: - Introduce Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle (no signalling for neighbour cell skipping) - FFS whether and how to introduce Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms or 160ms SCell measurement cycle (with signalling for neighbour cell skipping)

In this contribution, we further discuss the test feasibility of Event triggered reporting on SCC with deactivated SCell test with signalling for neighbour cell skipping.

- Discussion

Issue 2-1-1: Test scope - separate tests for neighbour cell skipping/not skipping

Agreement:

- Introduce Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle (no signalling for neighbour cell skipping)

- FFS whether and how to introduce Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms or 160ms SCell measurement cycle (with signalling for neighbour cell skipping)

For an ATG UE capable of antenna array on SCC configured with neighSCellMeasSkipping signalling, if UE also capable of two simultaneous separate Rx beams, then no scheduling restriction will be applied on SMTC of SCC MO, in other words, network could schedule DL data on SMTC occasions of SCC MO. Therefore, it is necessary to verify that the UE skips the neighbour cell measurement correctly.

How to verify the UE skips the neighbour cell measurement is one of the key issue. One approach is to configure UE to report the measurement result of neighbour cell if any, and then check whether neighbour cell measurement results exist in measurement report or not.

Hence, next step is to investigate how to configure UE to report the measurement result of neighbour cell if any, we found to ways to go:

- Configure periodicity measurement report on SCC MO

- Configure event triggering based measurement report with Event A2 (Serving becomes worse than absolute threshold) and reportAddNeighMeas-r16 configured as ‘setup’

- Considering the configuration of second way is not realistic, i.e, network configure neighbour cell measurement skipping and best neighbour cell measurement report simultaneously, not sure whether it will be considered as error case from both UE and network side, we prefer to test with the first way, configure periodicity measurement report on SCC MO for the deactivated SCell measurement test case with neighSCellMeasSkipping signalling.

Proposal 1: Introduce the Periodic measurement reporting on SCC with deactivated SCell test case with neighSCellMeasSkipping signalling.

Regarding the test configuration, we prefer to use non-DRX and 160ms SCell measurement cycle to reduce the test time, for the others, the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except that:

- No highSpeedMeasCA-Scell-r17 indication configured

- the DRX configuration shall be OFF

- the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A

- the measCycleSCell shall be configured as 160ms

- the neighbour cell measurement skipping indication shall be configured

- the propagation condition shall follow R18 ATG test cases

- The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2.

- Periodic measurement report configuration

Proposal 2: For the test configuration, the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except that:

- No highSpeedMeasCA-Scell-r17 indication configured

- the DRX configuration shall be OFF

- the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A

- the measCycleSCell shall be configured as 160ms

- the neighbour cell measurement skipping indication shall be configured

- the propagation condition shall follow R18 ATG test cases

- The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2 and PUSCH resource configuration.

- Periodic measurement report configuration

Regarding the test requirement, both delay requirement and neighbour cell skipping functionality shall be verified. Therefore, UE shall report the measurement reporting within the delay requirement and no neighbour cell measurement results included.

Proposal 3: For the test requirement, UE shall report the measurement reporting within the delay requirement and no neighbour cell measurement results included.

- Conclusion

Based on the discussion above, the following observation and proposals are concluded.

Proposal 1: Introduce the Periodic measurement reporting on SCC with deactivated SCell test case with neighSCellMeasSkipping signalling.

Proposal 2: For the test configuration, the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except that:

- No highSpeedMeasCA-Scell-r17 indication configured

- the DRX configuration shall be OFF

- the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A

- the measCycleSCell shall be configured as 160ms

- the neighbour cell measurement skipping indication shall be configured

- the propagation condition shall follow R18 ATG test cases

- The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2 and PUSCH resource configuration.

- Periodic measurement report configuration

Proposal 3: For the test requirement, UE shall report the measurement reporting within the delay requirement and no neighbour cell measurement results included.

- Reference

- R4-2514795, WF on NR_ATG_enh_RRM, CMCC