3GPP TSG RAN WG4 Meeting #117	R4-2521008

Dallas, USA, 17 – 21 November, 2025

Agenda item:			6.3.2

Source:	vivo

Title:	On RRM performance requirements for LB CA via switching

Document for:	Discussion

# Introduction

In the last RAN4 meetings, RAN4 started discussions on RRM performance requirements for Rel-19 LB CA via switching. Following agreements were made and captured in the WF [1].

### Issue 2-1: General ### Issue 2-2: TC configuration Issue 2-2-1: TC configuration - General configuration: - For test configuration and parameters, existing FR1 with 15kHz SCS test configuration and parameters could be baseline - CA between FDD PCell and SDL SCell is configured - PCell and SCell are co-located deployed and keep sync between them (Max RTD is 3us) - The SCS is 15kHz and the frequencies of PCell and SCell are lower than 1GHz - As baseline, test partially overlapped scenario for LB CA RRM, i.e., SSB/SMTC is partially overlapped with LB-CA switching pattern. - Working assumption for TC: SSB/SMTC periodicity is 20ms and switching pattern periodicity is 40ms; - The details of switching pattern can be FFS. - RAN4 will pick one TC in the list to cover fully overlapping and partially overlapping cases ### Issue 2-3: TC list Issue 2-3-1: detailed TC list TC Index; Requirement; RS used for test; Target Cell for testing; Support this TC; Leading Company. 1; Intra-frequency L3 measurement for activated SDL SCell or PCell; SSB; Both PCell and SCell in the same TC PCell and SCell can be configured with different events, details can be FFS. PCell is in partially overlapped case. SCell is in fully overlapped case.; Ericsson. 2; Deactivated SDL SCell measurement and interruption requirement; SSB; Verify measurement requirement on SDL SCell. And in the same TC verify interruption requirement to the PCell.; MTK. 3; SDL SCell activation/deactivation; SSB(for sync and L1-RSRP) and CSI-RS for CQI; SDL SCell. Only MAC CE based SCell activation is verified.; Nokia. 4; BFD/CBD for SDL SCell; Option 1: SSB (Apple) Option 2: CSI-RS Option 3: both; Option 1: SCell only (Apple) Option 2: Both PCell and SCell (OPPO, HW, ZTE, Ericsson) [Ericsson]: If option 2 is adopted, test PCell/SCell in the different test.; FFS whether this TC shall be introduced.. 6; PL-RS switch delay for PCell; Option 1: SSB (Apple) Option 2: CSI-RS Option 3: both; FDD PCell (Apple, OPPO, ZTE, Ericsson); FFS whether this TC shall be introduced.. 7; L1-RSRP for SDL SCell; SSB; SCell only; Huawei. 8; RLM requirement for PCell; SSB; FDD PCell; OPPO.

In this contribution, we provide our views on the open issues for RRM performance requirements for Rel-19 LB CA via switching.

# Discussion

## 2.1 Test configuration

For the test configuration, it was assumed that SSB/SMTC periodicity is 20ms and switching pattern periodicity is 40ms. The details for switching pattern configuration are not decided yet.

The number of slots for SDL SCell can be decided firstly. Considering the ACK/NACK feedback needs to be on PCell and HARQ timeline, 10 slots for SDL SCell seem to be a reasonable configuration. The switching gap from PCell to SCell can be configured with 3 symbols and it can be configured with 1 slot (14 symbols) from SCell to PCell considering TA.

Proposal 1: The switching pattern can be configured as 10 slots for SDL SCell and 30 slots for FDD PCell.

Proposal 2: Switching gap is 3 symbols from FDD PCell to SDL SCell and 14 symbols from SDL SCell to FDD PCell.

## 2.2 Test case list

For L1 measurement/procedure, it was agreed to introduce test cases for L1-RSRP measurement for SDL SCell operation and RLM for FDD PCell operation. It is FFS for BFD/CBD measurement requirements and PL-RS switch delay requirements.

For BFD/CBD procedure, it was agreed to define requirement for BFD/CBD for SDL SCell in LB CA via switching. In our view, it is not necessary features for FR1 operation below 1GHz. It may not be used in practical deployment. Furthermore, the UE requirements for BFD/CBD due to switching between carriers are similar to RLM requirements. Therefore, there is no strong need to introduce corresponding tests.

For PL-RS switch delay, though it may not be used for FR1 operation below 1GHz, a test can be considered.

Proposal 3: No need to introduce test to verify BFD/CBD measurement requirements when switching pattern is configured for both FDD PCell and SDL SCell.

Proposal 4: Introduce a test to verify PL-RS switch delay requirements when switching pattern is configured for FDD PCell operation.

# Summary

In this contribution, we further provided views on RRM performance requirements for Rel-19 LB-CA via switching. Following proposals and observations are provided.

Proposal 1: The switching pattern can be configured as 10 slots for SDL SCell and 30 slots for FDD PCell.

Proposal 2: Switching gap is 3 symbols from FDD PCell to SDL SCell and 14 symbols from SDL SCell to FDD PCell.

Proposal 3: No need to introduce test to verify BFD/CBD measurement requirements when switching pattern is configured for both FDD PCell and SDL SCell.

Proposal 4: Introduce a test to verify PL-RS switch delay requirements when switching pattern is configured for FDD PCell operation.

- References

- R4-2514870 WF on NR_LBCA_Sw_RRM, Apple