3GPP TSG-RAN WG4 Meeting #117	R4-2521035

Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Applicability rules for 6Rx CQI requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Perf |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New CQI performance requirements for 6Rx UE agreed to be introduced, hence related descriptions of applicability rules for 6Rx CQI requirements should be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of changes in this CR as below: - Add descriptions of applicability rules for 6Rx CQI requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New CQI performance requirements for 6Rx will not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

6.1.1.2	Applicability of requirements for different number of RX antenna ports

The number of RX antenna ports for different RF operating bands is up to UE declaration.

The UE shall support 2, 4, 6 or 8RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in clause 7.2 of TS 38.101-1 [6]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 6.1.1.2-1.

Table 6.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports only 2RX | CQI | All tests in Clause 6.2.2 |
|  | PMI | All tests in Clause 6.3.2 |
|  | RI | All tests in Clause 6.4.2 |
| UE supports only 4RX or both 2RX and 4RX | CQI | All tests in Clause 6.2.3 |
|  | PMI | All tests in Clause 6.3.3 |
|  | RI | All tests in Clause 6.4.3 |
| UE supports 2Rx, 4Rx and 6Rx, or UE supports 4Rx and 6Rx | CQI | All tests in Clause 6.2.3(Note 3) All tests in Clause 6.2.5 |
|  | PMI | All tests in Clause 6.3.3(Note 3) |
|  | RI | All tests in Clause 6.4.3(Note 3) |
| UE supports 2Rx, 4Rx and 8Rx, or UE supports 4Rx and 8Rx | CQI | Tests in Clause 6.2.3(Note 1) All tests in Clause 6.2.4 |
|  | PMI | All tests in Clause 6.3.3(Note 1) |
|  | RI | All tests in Clause 6.4.3(Note 1) |
| UE supports 2Rx, 4Rx, 6Rx and 8Rx, or UE supports 4Rx, 6Rx and 8Rx | CQI | All tests in Clause 6.2.3(Note 1) All tests in Clause 6.2.4 All tests in Clause 6.2.5(Note 5) |
|  | PMI | All tests in Clause 6.3.3(Note 1) |
|  | RI | All tests in Clause 6.4.3(Note 1) |
| UE supports 2Rx and 6Rx | CQI | All tests in Clause 6.2.2(Note 4) All tests in Clause 6.2.5 |
|  | PMI | All tests in Clause 6.3.2(Note 4) |
|  | RI | All tests in Clause 6.4.2(Note 4) |
| UE supports 2Rx and 8Rx | CQI | Tests in Clause 6.2.2(Note 2) All tests in Clause 6.2.4 |
|  | PMI | All tests in Clause 6.3.2(Note 2) |
|  | RI | All tests in Clause 6.4.2(Note 2) |
| UE supports 2Rx, 6Rx and 8Rx | CQI | Tests in Clause 6.2.2(Note 2) All tests in Clause 6.2.4 All tests in Clause 6.2.5(Note 5) |
|  | PMI | All tests in Clause 6.3.2(Note 2) |
|  | RI | All tests in Clause 6.4.2(Note 2) |
| UE supports 6Rx and 8Rx | CQI | All tests in Clause 6.2.4 All tests in Clause 6.2.5(Note 5) |
|  | PMI | N/A |
|  | RI | N/A |
| UE supports only 6Rx | CQI | All tests in Clause 6.2.5 |
|  | PMI | N/A |
|  | RI | N/A |
| UE supports only 8Rx | CQI | All tests in Clause 6.2.4 |
|  | PMI | N/A |
|  | RI | N/A |
| Note 1: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 2: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. Note 3: 6Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 6 Rx with data source from system simulator, and the other 2 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 6Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 2 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. Note 5: 8Rx capable UEs are tested on any of the 6Rx supported RF bands by connecting 6 out of 8 Rx with data source from system simulator, and the other 2 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 6Rx should be applied. |  |  |

==============End of change==============