3GPP TSG-RAN WG4 Meeting #117	R4-2521042

Dallas, USA, Nov. 17-21, 2025

Agenda item:	7.5.3

Source:	Samsung

Title:	Discussion on Rel-20 performance requirements for FR1 SU-MIMO with spatial channel model

Document for:	Discussion

- Introduction

In the RAN#109 meeting, a new work item for “NR demodulation performance: Phase 6” has been approved [1], in which specifying optional performance requirements under spatial channel model (SCM) for SU-MIMO in FR1 is an important objective for the enhancement of UE demodulation performance parts. The detailed descriptions of this objective are as below:

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

Generally, the UE demodulation performance requirements for SCM in this WI could be summarized as two parts. The first one is to specify PDSCH demodulation performance requirements with SCM, the second one is to study and specify PMI reporting performance requirements with SCM if performance alignment on PMI reporting is achieved.

For the SCM channel model which will be used for specifying performance requirements, rCDL-C1 channel model in TR38.753 has been agreed as the starting point [1]. rCDL-C1 [2] is a Clustered Delay Line (CDL) based channel model focusing on 3.5GHz carrier frequency, which starts from TR38.901 CDL-C constructed to represent one channel profile for NLOS, with angle scaling, randomness reduction (fixed coupling pattern for ray angles and fixed initial phases for polarization matrices in the channel coefficient generation), and cluster reduction (reduced to 12 clusters from 24 clusters).

Given that the study in TR38.753 on rCDL-C1 channel model is based on Rel-18 TR38.901, and only focus on 3.5GHz carrier frequency, further update to align with latest TR38.901 and further enhancement to implement frequency generalization are necessary. However, in last RAN4#116bis meeting [3], the maximum Doppler was decided to be fixed as 10Hz to avoid further enhancement on frequency generalization aspect.

In addition, the evaluation results of rCDL-C1 are not aligned for PMI performance testing with eType-II codebook, three clusters of results can be observed [2], and current rCDL-C1 has fixed angular properties with non-uniform PMI statistics. Consequently, the PMI bias problem need to be studied and solved firstly, then check if the performance alignment on PMI reporting could be achieved.

In this contribution, we share our views and analysis on open issues for FR1 SU-MIMO normative work under SCM channel model.

- Discussion

  - Potential updates to rCDL channel model

Angle scaling for rCDL

In last RAN4#116bis meeting, agreement [3] reached to update the rCDL channel model based on the latest angular scaling procedure as in TR38.901 v19.1.0 and further discuss the details, and further agreement for obtaining scale factor ‘s’ using desired AS: ASD – 25; ASA – 60; ZSD – 5, ZSA – 15 as baseline. Based on these agreements, the 20 clusters table could be updated as below Table 2-1.

Table 2-1: Updated channel model parameters

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.4 | -117.2 | 95.4 | 101.1 |
| 2 | 76 | -1.2 | -22.8 | 123.5 | 98 | 70.9 |
| 3 | 80 | -5.5 | -34.5 | -140 | 101.6 | 66.9 |
| 4 | 232 | 0 | -7.8 | 166.7 | 99.1 | 77.1 |
| 5 | 240 | -10.4 | 39.6 | 68 | 110 | 61.7 |
| 6 | 290 | -10.1 | -50 | 77.5 | 92 | 54.7 |
| 7 | 300 | -13.7 | 44.2 | -71.8 | 111.7 | 69.5 |
| 8 | 340 | -14.1 | -71.2 | 60.7 | 88.7 | 48.2 |
| 9 | 448 | -8.1 | -44 | 78.9 | 107.3 | 106.9 |
| 10 | 476 | -9.8 | -49.9 | -89.5 | 108.2 | 47.4 |
| 11 | 790 | -11.7 | -59.1 | 90.4 | 87.8 | 49.2 |
| 12 | 988 | -16.2 | 58.9 | 46.8 | 108.2 | 126.6 |
| 13 | 1552 | -16.9 | 56.6 | 6.4 | 91.3 | 51.7 |
| 14 | 1676 | -16.9 | 49.8 | 23.7 | 88 | 60.3 |
| 15 | 2000 | -18.8 | -74.4 | 8.7 | 86.3 | 33.4 |
| 16 | 2042 | -20.1 | 52.1 | 28.8 | 112.8 | 50.8 |
| 17 | 2298 | -19 | 52.8 | 25.3 | 87.8 | 31.5 |
| 18 | 2418 | -18.7 | 61.4 | 21 | 87.6 | 50.6 |
| 19 | 2566 | -24.6 | 69.8 | 1.6 | 110 | 43.4 |
| 20 | 3152 | -25.8 | -88.6 | 49.3 | 114.8 | 41.4 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 2 | 15 | 3 | 7 | 7 |

Proposal 1: Updated rCDL channel model parameters based on the latest angular scaling procedure as in TR38.901 v19.1.0 and scale factor ‘s’ using desired AS: ASD – 25; ASA – 60; ZSD – 5, ZSA – 15 as below.

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.4 | -117.2 | 95.4 | 101.1 |
| 2 | 76 | -1.2 | -22.8 | 123.5 | 98 | 70.9 |
| 3 | 80 | -5.5 | -34.5 | -140 | 101.6 | 66.9 |
| 4 | 232 | 0 | -7.8 | 166.7 | 99.1 | 77.1 |
| 5 | 240 | -10.4 | 39.6 | 68 | 110 | 61.7 |
| 6 | 290 | -10.1 | -50 | 77.5 | 92 | 54.7 |
| 7 | 300 | -13.7 | 44.2 | -71.8 | 111.7 | 69.5 |
| 8 | 340 | -14.1 | -71.2 | 60.7 | 88.7 | 48.2 |
| 9 | 448 | -8.1 | -44 | 78.9 | 107.3 | 106.9 |
| 10 | 476 | -9.8 | -49.9 | -89.5 | 108.2 | 47.4 |
| 11 | 790 | -11.7 | -59.1 | 90.4 | 87.8 | 49.2 |
| 12 | 988 | -16.2 | 58.9 | 46.8 | 108.2 | 126.6 |
| 13 | 1552 | -16.9 | 56.6 | 6.4 | 91.3 | 51.7 |
| 14 | 1676 | -16.9 | 49.8 | 23.7 | 88 | 60.3 |
| 15 | 2000 | -18.8 | -74.4 | 8.7 | 86.3 | 33.4 |
| 16 | 2042 | -20.1 | 52.1 | 28.8 | 112.8 | 50.8 |
| 17 | 2298 | -19 | 52.8 | 25.3 | 87.8 | 31.5 |
| 18 | 2418 | -18.7 | 61.4 | 21 | 87.6 | 50.6 |
| 19 | 2566 | -24.6 | 69.8 | 1.6 | 110 | 43.4 |
| 20 | 3152 | -25.8 | -88.6 | 49.3 | 114.8 | 41.4 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 2 | 15 | 3 | 7 | 7 |

CDL cluster truncation

In addition, when we review the evaluation results of the CDL in RAN4#116 meeting, truncation appears to have a considerable effect which is non negligible, especially on 8-layer and PMI test cases. It may cause RAN4 get different conclusions as RAN1 for some features, since we will use truncated CDL models to test RAN1 features evaluated based on 38.901 CDL channel model without truncation. Therefore, we’d better to clarify whether the truncation is actually needed or not and the exact value for the number of clusters.

Observation 1: For evaluation results of the CDL in RAN4#116 meeting, truncation appears to have a considerable effect which is non negligible, especially on 8-layer and PMI test cases.

Proposal 2: RAN4 to clarify whether the truncation for 38.901 CDL model is actually needed or not and clarify the exact value for the number of clusters RAN4 could support, e.g., 12, 16, 20.

UE antenna modelling

As outcomes of RAN1 study item FS_NR_7_24GHz_Chmod, UT radiation power pattern, UT antenna placements and UT polarization are introduced in Rel-19 TR38.901 and copied as following, which are noteworthy for higher number of UE antennas modelling.

UT Radiation power pattern in TR38.901

Table 7.3-2: Radiation power pattern of a single antenna element for handheld UT Parameter; Values. Vertical cut of the radiation power pattern (dB); AdB″θ″, ϕ″=0°=-min12θ″-90°θ3dB2,SLAV with θ3dB=125°,SLAV=22.5dB and θ″∈0°, 180°. Horizontal cut of the radiation power pattern (dB); AdB″θ″=90°, ϕ″=-min12ϕ″ϕ3dB2,Amax with ϕ3dB=125°, Amax=22.5 dB and ϕ″∈-180°, 180°. 3D radiation power pattern (dB); AdB″θ″,ϕ″=-min-AdB″θ″, ϕ″=0°+AdB″θ″=90°, ϕ″,Amax. Maximum directional gain of an antenna element GE,max; 5.3 dBi. NOTE: For UT antenna modelling of handheld devices, optional antenna imbalance can be modelled. If modelled, randomized loss is applied per UT antenna port and randomized loss can be applied independently for the UL and DL directions. No imbalance is modelled by default..

UE antenna placements for handheld UT devices and CPE device in TR38.901

For handheld UT devices, the following additional properties apply: - Four corners of the rectangle and the center of edges of the rectangle that reflect UT form factor are identified as potential locations of the UT antenna and potential locations are enumerated as illustrated in Figure 7.3-2. - Each antenna is assumed to be oriented along the direction determined by the vector connecting the centre of the rectangle to the antenna location. - Reference UT orientation vector of the handheld UT is perpendicular to the plane of the flat UT handheld device, and reference point for near field phase calculation of the UT is assumed to be the center of the plane of the UT handheld. - Device dimensions for UT antenna modelling are (15 cm, 7 cm, 0 cm). Figure 7.3-2: Handheld UT antenna placement candidate locations relative to centre of device (top down view) … For consumer premise equipment (CPE) devices, the following properties apply: - Total 9 candidate antenna locations in 4 corners of the vertical square plane, 4 center of the edges of the vertical square plane, and center of the vertical square plane. The figure below captures the side view on the device. Note that a candidate antenna location (e.g., 9) can be used for multiple antennas. - Device dimensions for UT antenna modelling are (0 cm, 20 cm, 20 cm) Figure 7.3-6: CPE UT antenna placement candidate locations relative to centre of device (side view)

UT polarization direction defined in TR38.901

For cases when a candidate antenna placement location is used for one antenna field pattern (e.g., single polarization): - The polarization direction is indicated by the arrow in Figure 7.3-7, which is parallel with the plane of the handheld UT and perpendicular to the direction from the UT center to the candidate antenna location. Figure 7.3-7: Handheld UT antenna polarization directions for one antenna field pattern (top down view)

Considering one of the major test cases is 8T8R with 2 codewords, which has multiple number of UE antennas, while rCDL-C1 is generated based on omni antenna radiation pattern and co-located cross-polarization antenna modelling for UE side, which may not match the actual situation on the real network, RAN4 should clarify if the UT radiation power pattern, UT antenna placements and UT polarization direction defined in Rel-19 TR38.901 should be considered as the simulation assumption for FR1 SU-MIMO demodulation performance evaluation.

Observation 2: UT radiation power pattern, UT antenna placements and UT polarization direction are introduced in Rel-19 TR38.901, which probably be able to match well with the actual situation on the real network.

Proposal 3: RAN4 to clarify if the UT radiation power pattern, UT antenna placements and UT polarization direction defined in Rel-19 TR38.901 should be considered as the simulation assumptions for FR1 SU-MIMO demodulation performance evaluation.

Proposal 4: If RAN4 decide to include UT radiation power pattern, UT antenna placements and UT polarization direction for deriving CDL-based test requirements, below assumptions could be considered as a starting point:

- For UT radiation power pattern, Table 7.3-2 in 38.901 V19.1.0

- For UT antenna placements, Figure 7.3-2 and Figure 7.3-6 in 38.901 V19.1.0

- For UE polarization direction, Figure 7.3-7 in 38.901 V19.1.0

  - Test scope and test parameters for SCM SU-MIMO requirements

    - General

Antenna Array Virtualization (AAV) assumptions

For the AAV assumption, considering no obvious performance difference between AAV(8,1) and AAV(1,1) assumptions, it is reasonable to only consider AAV(1,1), i.e., Ms=Ns=1.

Proposal 5: For Antenna Array Virtualization (AAV) assumptions, support option 1 (Only consider Ms=Ns=1).

Duplex mode and SCS

Since the evaluation in Rel-19 SI only focus on TDD 30kHz with 40MHz CHBW, we slightly prefer to only cover TDD scenario.

Proposal 6: For duplex mode, slightly prefer to only cover TDD scenario.

    - PDSCH demodulation requirements with SCM

Precoding, MCS and test metric

The agreement for 4T4R 4L test case achieved to use the same test metric as TDL test case. For 8T8R 8L test case, the group agreed to define single SNR for the test, three options as below on how to get the single SNR need further discussion.

For option a, how to select different MCS values for different CW, and these MCS values should be able to guarantee the SNR @ 70% max TP of two codewords are almost the same. It seems whether these MCS values could be found out depends on a coincidence.

We don’t see any essential difference between option b and option c. therefore, we prefer to use option c.

Precoding, MCS and test metric - Agreement and candidate options: - For 4T4R 4L test, it is agreed to use the test metric: SNR @ 70% max TP. - For 8T8R 8L test, it is agreed to use test metric: Define single SNR and further discuss the SNR is based on: - Option a: Based on per CW thput with Different MCS per CW - Option b: Based on total thput with different MCS per CW - Option c: Based on total thput with same MCS per CW - Further discuss: How to define the SNR for option a. How to select the MCS for different CW.

Proposal 7: For 8T8R 8L PDSCH demodulation performance test, define single SNR test metric based on total thput with same MCS per CW.

For the precoding for PDSCH demodulation performance test, keep the same precoding assumption as SI, i.e., random Single Panel Type I and randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook.

Proposal 8: For PDSCH demodulation performance test, keep the same assumption for precoding as SI, i.e., random Single Panel Type I and randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook.

    - PMI reporting requirements with SCM

PMI bias

For rCDL-C1 in TR38.753, the angle values of CDL models are fixed, which is not very suitable for PMI reporting performance evaluation. The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with or even better than closed-loop or reciprocity beamforming since PMI reporting delay.

In addition, the WID description [1] clearly mentioned that the PMI bias issue need to be solved.

Proposal 9: RAN4 shall exploit approaches to address the PMI bias issue.

Currently, rCDL-C1 defined in TR38.901 has fixed angular properties with non-uniform PMI statistics, which result in the PMI bias problem. Similar issue could also be observed for PMI reporting performance evaluation with legacy TDL channel model but without beam steering approach. Beam steering approaches defined in TS38.101-4 B.2.3.2.3 and B.2.3.2.3A are the solutions for legacy TDL channel model with fixed angular properties. Hence, beam steering approach could be a possible solution for rCDL-C1 with PMI bias issue.

Observation 3: Beam steering approaches defined in TS38.101-4 B.2.3.2.3 and B.2.3.2.3A are the solutions for legacy TDL channel model with fixed angular properties.

Proposal 10: Take beam steering approach defined in TS38.101-4 B.2.3.2.3 or B.2.3.2.3A as a possible solution for rCDL-C1 with PMI bias issue.

Whether to define eType II PMI reporting requirements based on SCM

In the study phase for rCDL-C1 in TR38.753, the performance alignment for eType-II codebook is not achieved, the detailed descriptions about simulation results are as following.

#### 8.2.1.1 4 layer, eType-II For PMI testing with 4layer transmission, with rCDL-C1, in eType-II codebook case, three clusters of results can be observed; Cluster 1: include source #3, #7, Cluster 2: include #4, #5, #6, #9. The span of this cluster is < 2.5dB for both 70% and 90% normalized throughput percentiles, and Cluster 3 include source #1, #2, #8. The span of this cluster < 2.5dB for both 70% and 90% normalized throughput percentiles.

Further simulation results alignment should be conducted based on updated rCDL channel model. For results alignment, both TP gain and SNR @ 90% Max TP could be collected to evaluate alignment.

Proposal 11: Further simulation results alignment should be conducted based on updated rCDL channel model. For results alignment, both TP gain and SNR @ 90% Max TP could be collected to evaluate alignment.

Test scope for eType II PMI reporting (if introduced)

Although the WID description [1] mentioned “Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding.”, however we didn’t study the performance of 4T4R case in the Rel-19 WI, and we believe the performance of eType II precoding with 4T4R would be even worse than that with 8T4R. Therefore, we don’t think it make sense to specify requirements PMI performance with 4T4R eType II precoding.

Proposal 12: Update WID to remove 4T4R eType II precoding as “Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eType II precoding.”

Proposal 13: Test scope for eType II PMI reporting if introduced should be only for 8T4R test case.

Rank for eType II PMI reporting (if introduced)

For the rank value used for eType II PMI reporting performance test, considering there are no obvious gain for eType II rank 4 case compared with Type I precoding performance, we prefer to use the same rank value as TDL case.

Proposal 14: Prefer to use rank 2 for eType II PMI reporting test if introduced.

Other test parameters for eType II PMI reporting (if introduced)

For other test parameters for eType II PMI reporting test if introduced, it’s a good idea to reuse the test parameters for the Rel-19 SI, i.e., Table 6.2-1 and Table 6.2-2 in TR38.753.

Proposal 15: For other test parameters for eType II PMI reporting test if introduced, reuse the test parameters for the Rel-19 SI, i.e., Table 6.2-1 and Table 6.2-2 in TR38.753.

Test metric for eType II PMI reporting (if introduced)

For the test metric, if possible, in order to preserve the uniformity of the spec, we would like to reuse the same test metric for the existing eType II requirement based on TDL, i.e., Thput of following PMI (eType II)/ Thput of Random PMI (Type I codebook) with SNR @ 90 % of the maximum throughput for following PMI.

Proposal 16: Reuse the same test metric for the existing eType II requirement based on TDL, i.e., Thput of following PMI (eType II)/ Thput of Random PMI (Type I codebook) with SNR @ 90 % of the maximum throughput for following PMI.

  - Test applicability for SCM SU-MIMO requirements

Test applicability for SCM SU-MIMO requirements

Regarding the test applicability of the new test cases utilizing the rCDL channel model, the fundamental principle is that these test cases are optional. Only UEs for which the manufacturer chooses to verify performance under these specific channel conditions need to be tested.

It is important to note that supporting these tests is not defined as a new UE capability. The sole difference between these new test cases and the existing ones lies in the channel model (rCDL). The UE itself requires no functional updates or specific features to be tested under these conditions. Passing these tests signifies that the UE's performance has been validated against an additional set of standardized channel conditions, which primarily involves the test environment utilizing the rCDL model on the test equipment.

Therefore, our proposal is to simply introduce these new test cases in the applicability rules with a clear declaration that they are optional. There is no need to define any new UE capabilities or feature signaling related to this.

Proposal 17: Introduce one single new declaration for all rCDL based demodulation performance tests, this declaration related with test environment and test resource but not the UE capability. In addition, these tests should be optional.

Release independent for SCM SU-MIMO requirements

In discussing the versioning approach for the new rCDL based test cases, we would like to propose aligning them specifically with Release 20 rather than adopting a release-independent designation. We believe this approach offers several practical benefits for the ecosystem.

It’s true that RAN4 is defining rCDL based requirements for the existing UE features, e.g., 4T4R rank4 requirement applies to Rel-15 UEs and 8T8R rank8 requirement applies to Rel-17 8Rx UEs supporting max 8 layers. While we recognize that the foundational feature work for CDL was initiated in earlier releases (Rel-15/17), the precise channel environments essential for standardized and repeatable demodulation testing have been fully specified and stabilized within the Rel-20 WI timeframe.

Furthermore, we acknowledge that the implementation of these new channel models in test equipment and the subsequent calibration efforts across the industry will require a certain lead time. Tying the applicability of these tests to Rel-20 naturally accommodates this implementation phase, setting realistic expectations for when support will be widely available.

Finally, given that these test cases are defined as optional—serving as valuable additional performance verifications rather than addressing a critical or urgent interoperability issue—we believe the necessity for a release-independent designation is considerably reduced. The standard practice of versioning them with Rel-20 appears to be the most straightforward and logically consistent path forward, minimizing complexity while adequately serving their intended purpose.

In summary, we see strong merits in anchoring these optional rCDL-based tests to Rel-20.

Observation 4: rCDL channel model standardized in Rel-20, related implementation and calibration in TE require time, and rCDL based performance requirements assumed to be optional and non-urgent.

Proposal 18: For new rCDL based performance requirements, prefer not to define release independent.

- Conclusion

In this contribution, we provide analysis and views on FR1 SU-MIMO SCM topic.

The observations and proposals could be summarized as:

Proposal 1: Updated rCDL channel model parameters based on the latest angular scaling procedure as in TR38.901 v19.1.0 and scale factor ‘s’ using desired AS: ASD – 25; ASA – 60; ZSD – 5, ZSA – 15 as below.

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.4 | -117.2 | 95.4 | 101.1 |
| 2 | 76 | -1.2 | -22.8 | 123.5 | 98 | 70.9 |
| 3 | 80 | -5.5 | -34.5 | -140 | 101.6 | 66.9 |
| 4 | 232 | 0 | -7.8 | 166.7 | 99.1 | 77.1 |
| 5 | 240 | -10.4 | 39.6 | 68 | 110 | 61.7 |
| 6 | 290 | -10.1 | -50 | 77.5 | 92 | 54.7 |
| 7 | 300 | -13.7 | 44.2 | -71.8 | 111.7 | 69.5 |
| 8 | 340 | -14.1 | -71.2 | 60.7 | 88.7 | 48.2 |
| 9 | 448 | -8.1 | -44 | 78.9 | 107.3 | 106.9 |
| 10 | 476 | -9.8 | -49.9 | -89.5 | 108.2 | 47.4 |
| 11 | 790 | -11.7 | -59.1 | 90.4 | 87.8 | 49.2 |
| 12 | 988 | -16.2 | 58.9 | 46.8 | 108.2 | 126.6 |
| 13 | 1552 | -16.9 | 56.6 | 6.4 | 91.3 | 51.7 |
| 14 | 1676 | -16.9 | 49.8 | 23.7 | 88 | 60.3 |
| 15 | 2000 | -18.8 | -74.4 | 8.7 | 86.3 | 33.4 |
| 16 | 2042 | -20.1 | 52.1 | 28.8 | 112.8 | 50.8 |
| 17 | 2298 | -19 | 52.8 | 25.3 | 87.8 | 31.5 |
| 18 | 2418 | -18.7 | 61.4 | 21 | 87.6 | 50.6 |
| 19 | 2566 | -24.6 | 69.8 | 1.6 | 110 | 43.4 |
| 20 | 3152 | -25.8 | -88.6 | 49.3 | 114.8 | 41.4 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 2 | 15 | 3 | 7 | 7 |

Observation 1: For evaluation results of the CDL in RAN4#116 meeting, truncation appears to have a considerable effect which is non negligible, especially on 8-layer and PMI test cases.

Proposal 2: RAN4 to clarify whether the truncation for 38.901 CDL model is actually needed or not and clarify the exact value for the number of clusters RAN4 could support, e.g., 12, 16, 20.

Observation 2: UT radiation power pattern, UT antenna placements and UT polarization direction are introduced in Rel-19 TR38.901, which probably be able to match well with the actual situation on the real network.

Proposal 3: RAN4 to clarify if the UT radiation power pattern, UT antenna placements and UT polarization direction defined in Rel-19 TR38.901 should be considered as the simulation assumptions for FR1 SU-MIMO demodulation performance evaluation.

Proposal 4: If RAN4 decide to include UT radiation power pattern, UT antenna placements and UT polarization direction for deriving CDL-based test requirements, below assumptions could be considered as a starting point:

- For UT radiation power pattern, Table 7.3-2 in 38.901 V19.1.0

- For UT antenna placements, Figure 7.3-2 and Figure 7.3-6 in 38.901 V19.1.0

- For UE polarization direction, Figure 7.3-7 in 38.901 V19.1.0

Proposal 5: For Antenna Array Virtualization (AAV) assumptions, support option 1 (Only consider Ms=Ns=1).

Proposal 6: For duplex mode, slightly prefer to only cover TDD scenario.

Proposal 7: For 8T8R 8L PDSCH demodulation performance test, define single SNR test metric based on total thput with same MCS per CW.

Proposal 8: For PDSCH demodulation performance test, keep the same assumption for precoding as SI, i.e., random Single Panel Type I and randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook.

Proposal 9: RAN4 shall exploit approaches to address the PMI bias issue.

Observation 3: Beam steering approaches defined in TS38.101-4 B.2.3.2.3 and B.2.3.2.3A are the solutions for legacy TDL channel model with fixed angular properties.

Proposal 10: Take beam steering approach defined in TS38.101-4 B.2.3.2.3 or B.2.3.2.3A as a possible solution for rCDL-C1 with PMI bias issue.

Proposal 11: Further simulation results alignment should be conducted based on updated rCDL channel model. For results alignment, both TP gain and SNR @ 90% Max TP could be collected to evaluate alignment.

Proposal 12: Update WID to remove 4T4R eType II precoding as “Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eType II precoding.”

Proposal 13: Test scope for eType II PMI reporting if introduced should be only for 8T4R test case.

Proposal 14: Prefer to use rank 2 for eType II PMI reporting test if introduced.

Proposal 15: For other test parameters for eType II PMI reporting test if introduced, reuse the test parameters for the Rel-19 SI, i.e., Table 6.2-1 and Table 6.2-2 in TR38.753.

Proposal 16: Reuse the same test metric for the existing eType II requirement based on TDL, i.e., Thput of following PMI (eType II)/ Thput of Random PMI (Type I codebook) with SNR @ 90 % of the maximum throughput for following PMI.

Proposal 17: Introduce one single new declaration for all rCDL based demodulation performance tests, this declaration related with test environment and test resource but not the UE capability. In addition, these tests should be optional.

Observation 4: rCDL channel model standardized in Rel-20, related implementation and calibration in TE require time, and rCDL based performance requirements assumed to be optional and non-urgent.

Proposal 18: For new rCDL based performance requirements, prefer not to define release independent.

- Reference

[1] RP-252957, WID: NR demodulation performance: Phase 6, RAN#109 meeting, Apple

[2] R4-2509114, 3GPP TR38.753 V1.1.0

[3] R4-2514823, WF on NR_demod_Ph6_Ph5, RAN4#116bis meeting, China Telecom