3GPP TSG-RAN WG4 Meeting #117	R4-2521055

Dallas, US, 17 – 21, November 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0237 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS38.101-5 Revise section 9.7 and 10.8 requirement for NTN Ku band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Clause numbers in Table 9.7.1-1 have some mistakes; - Receiver antenna off-axis performance should apply to “NS_206N”, but currently it is not applicable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Revise clause numbers in Table 9.7.1-1; - Apply receiver antenna off-axis performance to “NS_206N”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mistakes will remain, and requirements are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.7 and 10.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

9.7	Additional regional requirements indicated by NS

9.7.1	General

Additional regional requirements can be signalled by the network. Each group of additional regional requirements is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR NTN frequency band number of the applicable FR1-NTN operating bands above 10 GHz and FR2-NTN operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [8].

Table 9.7.1-1 specifies the additional regional requirements with their associated network signalling values, the applicable satellite orbit scenario(s) and applicable FR1-NTN operating bands above 10 GHz and FR2-NTN operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 9.7.1-2.

Table 9.7.1-1: Additional regional requirements indicated by Network Signalling label

| Network Signalling label | Requirements (clause) | Applicable Satellite orbit scenario | NR satellite Band | Channel bandwidth (MHz) |
| --- | --- | --- | --- | --- |
| NS_200N |  | GSO and LEO | Table 5.2.2-1 for bands above 10 GHz | Table 5.3.5-1 |
|  |  |  | Table 5.2.3-1 | Table 5.3.5-2 |
| NS_201N | Clause 9.2.2.3 Clause 9.5.3.2 Clause 9.5.3.3 Clause 9.6.1.1 Clause 9.6.1.2 Clause 10.8.1 | GSO | n512 | 50, 100, 200, 400 |
| NS_202N | Clause 9.5.3.2 Clause 9.5.3.3 Clause 9.6.1.1 | LEO | n512 | 50, 100, 200, 400 |
| NS_203N | Clause 9.2.2.4 Clause 9.5.2.2.2 Clause 9.6.2.2 | GSO | n509 | 50, 100, 200, 400 |
|  |  |  | n248 | 10, 15, 20, 25, 35, 50, 70, 100 |
| NS_204N | Clause 9.5.2.2.2 | LEO | n509 | 50, 100, 200, 400 |
|  |  |  | n248 | 10, 15, 20, 25, 35, 50, 70, 100 |
| NS_205N | Clause 9.2.2.5 Clause 9.5.3.2 Clause 9.5.3.3 Clause 10.8.2 | GSO | n509 | 50, 100, 200, 400 |
|  |  |  | n248 | 10, 15, 20, 25, 35, 50, 70, 100 |
| NS_206N | Clause 9.5.3.2 Clause 9.5.3.3 Clause 10.8.2 | LEO | n509 | 50, 100, 200, 400 |
|  |  |  | n248 | 10, 15, 20, 25, 35, 50, 70, 100 |
| NS_207N | Clause 9.2.2.6 Clause 9.5.2.2.2 Clause 9.6.2.3 | GSO | n508 | 50, 100, 200 |
|  |  |  | n247 | 10, 15, 20, 25, 35, 50, 70, 100 |
| NS_208N | Clause 9.5.2.2.2 | LEO | n508 | 50, 100, 200 |
|  |  |  | n247 | 10, 15, 20, 25, 35, 50, 70, 100 |

Table 9.7.1-2: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n512 | NS_200N | NS_201N | NS_202N |  |  |  |  | Reserved |
| n511 | NS_200N |  |  |  |  |  |  | Reserved |
| n510 | NS_200N |  |  |  |  |  |  | Reserved |
| n509 | NS_200N | NS_203N | NS_204N | NS_205N | NS_206N |  |  | Reserved |
| n508 | NS_200N | NS_207N | NS_208N |  |  |  |  | Reserved |
| n248 | NS_200N | NS_203N | NS_204N | NS_205N | NS_206N |  |  | Reserved |
| n247 | NS_200N | NS_207N | NS_208N |  |  |  |  | Reserved |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. NOTE 2: For band n511 and n510, only NS_200N can be used to map. |  |  |  |  |  |  |  |  |

==============Next change==============

10.8	Receiver antenna off-axis performance

10.8.1	Minimum requirements for band n512

The following regional requirements are applicable to NTN VSAT operating in band n512 towards geostationary satellite orbit.

The receiver antenna off-axis gain of each co-polarized components in any direction φ degrees from the antenna main beam shall not exceed the levels specified in Table 10.8.1-1.

Table 10.8.1-1: Off-axis Co-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φmin ≤ φ ≤ 48° | 32 – 25log10(φ) |
| 48° ≤ φ ≤ 85° | -10 |
| 85° ≤ φ ≤ 180° | 0 |
| NOTE: φmin = 1o or 100λ/D (degrees) whichever is the greater, for D/λ≥ 50. φmin = 2o or 114(D/ λ)-1.09 (degrees) whichever is the greater, for D/λ 50. where D is the nominal diameter of the antenna |  |

The receiver antenna off-axis gain of each cross-polarized components in any direction φ degrees from the antenna main beam shall not exceed the levels specified in Table 10.8.1-2.

Table 10.8.1-2: Off-axis Cross-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φr ≤ φ ≤ 7° | 23 – 20log10(φ) |
| NOTE: φr = 1o or 100λ/D (degrees) whichever is the greater where D is the nominal diameter of the antenna |  |

10.8.2	Requirement for network signalling value “NS_205N” and “NS_206N”

When "NS_205N" or "NS_206N" is indicated in the cell, the following requirements shall apply to NTN VSAT.

The receiver antenna off-axis gain of each co-polarized components in any direction φ degrees from the antenna main beam shall not exceed the levels specified in Table 10.8.2-1.

Table 10.8.2-1: Off-axis Co-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φmin ≤ φ ≤ 48° | 32 – 25log10(φ) |
| 48° ≤ φ ≤ 85° | -10 |
| 85° ≤ φ ≤ 180° | -2 |
| NOTE: φmin = 1o or 100λ/D (degrees) whichever is the greater, for D/λ≥ 50. φmin = 2o or 114(D/ λ)-1.09 (degrees) whichever is the greater, for D/λ 50. where D is the nominal diameter of the antenna |  |

The receiver antenna off-axis gain of each cross-polarized components in any direction φ degrees from the antenna main beam shall not exceed the levels specified in Table 10.8.2-2.

Table 10.8.2-2: Off-axis Cross-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φr ≤ φ ≤ 7° | 23 – 20log10(φ) |
| 7° < φ ≤ 32.3° | 20.2 – 16.7log10(φ) |
| 32.3° < φ ≤ 75° | -5 |
| 75° < φ ≤ 180° | -2 |
| NOTE: φr = 1o or 100λ/D (degrees) whichever is the greater where D is the nominal diameter of the antenna |  |

==============End of change==============