3GPP TSG-RAN WG4 Meeting #117	R4-2521059

Dallas, US, 17 – 21, November 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for (e)RedCap enh

# Agenda item:	7.4.3.1

# Document for:	Approval

# 1 Introduction

In RAN#109 meeting, a new Work Item [1] was approved on (e)RedCap UE enhancement with the following RAN4 objective. In this contribution, we want to share some further views about (e)RedCap enhancement from UE RF requirements perspective.

- Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD - For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026) [RAN4] - Note 3: No new PDSCH demodulation requirement shall be defined.

# 2 Discussion

Issue 4-1: Whether 3MHz UE CBW should be optional or mandatory in Rel-20 for (e)Redcap FFS whether 3MHz should be optional or mandatory in Rel-20 for (e)Redcap Issue 4-3: UE capability FFS in next meeting what UE capabilities of (e)Redcap need to be updated to support 3MHz and sent LS to RAN2.

In last meeting, we already reached many agreements for the most of issues and there are two issues left to be further discussed as shown in the above. The first remaining issue is whether 3MHz UE CBW should be optional or mandatory in Rel-20 for (e)RedCap. 3MHz was introduced as optional in Rel-18 in less than 5MHz work item. However, after that, 3MHz is mandatory support for band n106 in WID NR 900MHz US in Rel-18 since band n106 only support 3MHz CBW. The following table shows that 3MHz is optional or mandatory for bands n26, n28, n85, n100 and n106.

Table 2-1 Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 7 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| n26 | 15 | 34 | 5 | 74 | 10 | 15 | 20 | 253 | 303 |  |  |
|  | 30 |  |  |  | 10 | 15 | 20 | 253 | 303 |  |  |
| n28 | 15 | 34 | 5 |  | 10 | 15 | 207 | 257 | 307 |  | 404,7 |
|  | 30 |  |  |  | 10 | 15 | 207 | 257 | 307 |  | 404,7 |
| n85 | 15 | 34 | 5 |  | 10 | 15 |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 |  |  |  |  |  |
| n100 | 15 | 34 | 5 |  |  |  |  |  |  |  |  |
| n106 | 15 | 3 |  |  |  |  |  |  |  |  |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

In Rel-20, 3MHz also can be introduced as optional for (e)RedCap. Especially for band n106, it should be mandatory for (e)RedCap.

Proposal 1: In Rel-20, 3MHz can be introduced as optional for (e)RedCap, but mandatory for band n106.

During Rel-18 less than 5MHz discussion, some capabilities were introduced, and all specified that “This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18” . Additionally, in the WID, it states that “Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB)”. Hence, only 15PRB transmission bandwidth configuration related capabilities will be supported in (e)RedCap, which is shown in the following:

support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only (except for the PRACH formats with 1.25kHz SCS). It applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2], this feature shall be indicated for the band.

support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only (except for the PRACH formats with 1.25kHz SCS). It is applicable when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL.

To support less than 5MHz channel bandwidth for (e)RedCap UEs, the following capabilities need to be updated:

support3MHz-ChannelBW-Asymmetric-r18

support3MHz-ChannelBW-Symmetric-r18

Proposal 2: To support less than 5MHz channel bandwidth for (e)RedCap UEs, the following capabilities need to be updated:

support3MHz-ChannelBW-Asymmetric-r18

support3MHz-ChannelBW-Symmetric-r18

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for (e)RedCap UE enhancement and the proposals are made as follows:

Proposal 1: In Rel-20, 3MHz can be introduced as optional for (e)RedCap, but mandatory for band n106.

Proposal 2: To support less than 5MHz channel bandwidth for (e)RedCap UEs, the following capabilities need to be updated:

support3MHz-ChannelBW-Asymmetric-r18

support3MHz-ChannelBW-Symmetric-r18

# 4 Reference

- RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple