3GPP TSG-RAN WG4 Meeting #117														    R4-2521091

Dallas, USA, Nov. 17-21, 2025

Agenda item:			7.3.3

Source:	Samsung

Title:	Discussion on test method for NTN VSAT at Ka and Ku bands

Document for:	Discussion & Approval

1	Introduction

The core part for Rel-20 UE OTA is focused on VSAT test method in Ka and Ku bands [1, RP-252903]

- Define enhanced test methodologies to cover NTN Ka and Ku bands - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority - FR1 OTA test method extension is not precluded, if also applicable - Study whether and how to support circular polarization measurements - Linear polarization is the baseline - Specify the required quiet zone to cover VSAT device type based on industry needs - 60cm QZ as a starting point - Specify preliminary MU assessment for test methodologies

RAN4#116bis meeting was the first meeting, companies have common views that IFF method should be prioritized while have different views on circular polarization. Detailed test methods are supposed to be further discussed in this meeting. In this contribution we will further discuss the open issues including polarization, white box approach and beam steering aspects.

2	Discussion

2.1	polarization

The discussion on polarization in last meeting is related with several aspects. There is testability challenge to support circular polarization, e.g., difficulty to cover large frequency range, and test system complexity to support both CP and LP; the WID says linear polarization is baseline and there is practical GEO satellite in operation with linear polarization; the core requirement does not have clear requirement in terms of polarization.

The agreement of last meeting is as following [2, R4-2514820]:

### Sub-topic 2-6: Polarizaion RAN4 start to work on whether and how to support of CP testing in IFF system. Linear polarization is supported as basis based on the WID scope. The core requirements of NTN will be investigated to define the applicability of polarization on the requirements in the next RAN4 meeting.

Taking all above discussion and agreements into account, a minimum common ground is that the NTN test system for VSAT at Ku and Ka bands should at least support linear polarization. Support of circular polarization need to address the testability issue and requirement ambiguity firstly. And RAN4 should avoid separate test system for different polarizations.

Observation 1:	Support of circular polarization need to address the testability issue and requirement ambiguity firstly.

Proposal 1:	NTN test system for VSAT at Ku and Ka bands should at least support linear polarization.

Proposal 2:	RAN4 should avoid separate test system for different polarizations.

2.2	white-box approach

As discused in our previous paper [3, R4-2513876], the experience in FR2 testability indicates that black-box approach is facing severe limited SNR range issue even with 5cm antenna radiating aperture size assumption. In last meeting it was agreed that the QZ size will be 63cm or higher. This will make black box approach becoming infeasible.

Observation 2:	large radiating aperture diameter and large QZ size for NTN VSAT testing makes the black-box approach becoming in feasible.

Hence, it is proposed to adopt white-box approach. Companies can further discuss how to align the UE positioning with reference to test center.

Proposal 3:	it is proposed to adopt white-box approach for NTN VSAT testing at Ku and Ka bands.

2.3	beam steering

Looking at the MOP requirement for NTN VSAT in TS 38.101-5, taking mobile VSAT as example, the beam peak here means “beam peak directions”, which means all the beam peak directions within the declared supported minimum elevation angle (as shown in Figure 9.2.1.2-1) need to pass the Minimum peak EIRP requirement in Table 9.2.1.2-1. From this sense, VSAT NTN only has spherical coverage requirements, and there is no requirement for a single peak direction. All the requirements within the required spherical coverage are treated equally, which is called as minimum peak EIRP here.

Observation 3:	If comparing FR2 TN mmWave MOP requirements, VSAT NTN only has ‘spherical coverage’ requirements, and there is no requirement for a single peak direction.

With this difference compared with FR2 TN mmWave requirements, in can be concluded that beam peak search is not needed, and thus also no need to specify a dedicated measurement grid for beam peak search.

Proposal 4:	Beam peak search is not needed, and thus also no need to specify a dedicated measurement grid for beam peak search.

#### 9.2.1.2 Minimum requirements for Mobile VSAT The following requirements define the maximum output power radiated by the Mobile VSAT for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are specified in Table 9.2.1.2-1. The requirement should be verified with test metrics of EIRP (Link=Tx beam peak direction, Meas=Link angle). The peak EIRP of Tx beam peak direction should be verified within the declared minimum elevation angle supported for transmitting. The steered beam peak directions can be achieved by mechanical steering and/or electronic steering according to VSAT Type. Where the supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 9.2.1.2-1 below is taken as an example. Figure 9.2.1.2-1 Example measurement grid for min peak EIRP with the declared supported minimum elevation angle Table 9.2.1.2-1: Minimum peak EIRP for Mobile VSAT Operating band; UE Type; Min peak EIRP (dBm). n512, n511; 4; 70 (NOTE 1). 5; 70 (NOTE 1). n509, n508; 4; 70. 5; 70. 6; 60. n248, n247; 4; 70. 5; 70. 6; 60. NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance. The value corresponds to a 100 MHz CBW configuration with 66 RBs and 120 kHz SCS.. The maximum output power values for TRP and EIRP for mobile VSAT are specified in Table 9.2.1.2-2 below. Table 9.2.1.2-2: Maximum output power limits for Mobile VSAT Operating band; UE Type; TRPMAX (dBm); EIRPmax (dBm). n512, n511, n510; 4; 35; 76.21. 5; 43; 76.21. n509, n508; 4; 47; 88.22,4. 5; 47; 88.22,4. 6; 40; 76.62,4. n248, n247; 4; 47; 88.23,4. 5; 47; 88.23,4. 6; 40; 76.63,4. NOTE 1: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. NOTE 2: Maximum EIRP is defined using 6RBs allocation with 120kHz SCS. NOTE 3: Maximum EIRP is defined using 27RBs allocation with 30kHz SCS or using 54RBs allocation with 15kHz SCS. NOTE 4: For land ESIM operating in CEPT regions, the upper limits of EIRPmax is 84.5 dBm..

Note that there are VSAT types with mechanical steering and VSAT types with electrical steering. It is an open issue whether to test beam steering [3]

### Sub-topic 2-10: Beamlock RAN4 further check whether beam steering is needed for VSAT conformance testing.

For VSAT types with mechanical steering, it is noticed that the single probe test system can not perform 3D minimum peak EIRP testing with beam steering because the boresight of the VSAT will be constantly pointed to the unique probe. However, on the other hand, the minimum peak EIRP testing at single direction (boresight direction) may be enough for mechanical steering case.

Observation 4:	For mechanical steering VSAT, the single probe test system can not perform 3D minimum peak EIRP testing with beam steering because the boresight of the VSAT will be constantly pointed to the unique probe.

Observation 5:	For mechanical steering VSAT, the minimum peak EIRP testing at single direction (boresight direction) may be enough.

For VSAT types with electrical steering, or for VSAT types with hybrid mechanical and electrical steering (no matter it is declared as mechanical steering VSAT type or electrical steering VSAT type), beam steering is needed in the 3D minimum peak EIRP testing.

Proposal 5:	For VSAT types with electrical steering, or for VSAT types with hybrid mechanical and electrical steering (no matter it is declared as mechanical steering VSAT type or electrical steering VSAT type), beam steering is needed in the 3D minimum peak EIRP testing.

For VSAT types with hybrid steering, the detailed testing spherical range can be further discussed.

Regarding beam lock, the principle of FR2 testing can be reused.

Proposal 6:	Adopt beam lock according to the same principle as FR2 testing.

3	Conclusion

Observation 1:	Support of circular polarization need to address the testability issue and requirement ambiguity firstly.

Proposal 1:	NTN test system for VSAT at Ku and Ka bands should at least support linear polarization.

Proposal 2:	RAN4 should avoid separate test system for different polarizations.

Observation 2:	large radiating aperture diameter and large QZ size for NTN VSAT testing makes the black-box approach becoming in feasible.

Proposal 3:	it is proposed to adopt white-box approach for NTN VSAT testing at Ku and Ka bands.

Observation 3:	If comparing FR2 TN mmWave MOP requirements, VSAT NTN only has ‘spherical coverage’ requirements, and there is no requirement for a single peak direction.

Proposal 4:	Beam peak search is not needed, and thus also no need to specify a dedicated measurement grid

Observation 4:	For mechanical steering VSAT, the single probe test system can not perform 3D minimum peak EIRP testing with beam steering because the boresight of the VSAT will be constantly pointed to the unique probe.

Observation 5:	For mechanical steering VSAT, the minimum peak EIRP testing at single direction (boresight direction) may be enough.

Proposal 5:	For VSAT types with electrical steering, or for VSAT types with hybrid mechanical and electrical steering (no matter it is declared as mechanical steering VSAT type or electrical steering VSAT type), beam steering is needed in the 3D minimum peak EIRP testing.

Proposal 6:	Adopt beam lock according to the same principle as FR2 testing.

Reference

- RP-252903    WID: Enhancement of UE OTA test method and requirements for NR, RAN#109

- R4-2514820    WF on NR_UE_OTA_Enh, Eutelsat Group, RAN4#116bis

- R4-2513876    Discussion on test method for NTN Ka and Ku bands, Samsung, RAN4#116bis