3GPP TSG-RAN WG4 Meeting #117	R4-2521126

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.1.2

Source:	Ericsson

Title:	Discussion on SDR requirements for 6Rx

Document for:	Discussion

# 1	Introduction

Following the agreements made in the RF session below, the following UE demodulation and CSI reporting requirements will be defined but applied to handheld UE and FWA devices separately.

Issue 2-1-1: Way forward for 6-layers support for handheld UE and FWA devices Way Forward: - RAN4 Chair to take note of the following in the meeting report: - “RAN4 unable to conclude on a common set of simulation assumptions to determine feasibility of 6 layer for handheld UE In Rel-19” - 6-layer performance requirements will be only defined for FWA. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications.

After the discussion in RAN4 #116bis meeting in Prague, companies have reached agreements regarding the test scenarios and all of parameter assumptions.

In this paper, we will provide our simulation results together with our proposals on MCS values for SDR requirements.

# 2	Discussion

## 2.1	SDR requirements

Based on the agreements of the WF [1], companies are encouraged to provide simulation results to decide the MCS value as per the agreed static channel matrix and parameters.

For QPSK/16QAM/64QAM/256QAM with 6 MIMO layers, we have conducted the following simulation cases as in the table below:

| TxRx antenna configuration | Maximum number of PDSCH MIMO layers | Maximum modulation format | MCS index Table for PDSCH | MCS | Scaling factor | SNR (dB) @ 85% max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 8Tx6Rx | 6 | 6 (64QAM) EVM 6% | Table 5.1.3.1-1: MCS index table 1 | 28 | 1 | 22.9 20.8 |
|  |  |  |  | 27 |  | 18.7 |  |
|  |  |  |  | 26 |  | 17.7 |  |
|  |  |  |  | 25 |  | 27.9 |  |
|  | 6 | 8 (256QAM) EVM 3% | Table 5.1.3.1-2: MCS index table 2 | 27 |  | 26 |  |
|  |  |  |  | 26 |  | 26 |  |
|  |  |  |  | 25 |  | 23.9 |  |
|  |  |  |  | 24 |  | 22 |  |
|  |  |  |  | 23 |  | 25.9 |  |

From the simulation results above, we propose to consider MCS 26 for 64QAM and MCS 23 for 256QAM.

Proposal 1: Consider MCS25 for 256QAM

Proposal 2: Consider MC28 for 64QAM

For 1024QAM with 2 MIMO layers, we have also conducted the following simulations:

| TxRx antenna configuration | Maximum number of PDSCH MIMO layers | Maximum modulation format | MCS index Table for PDSCH | MCS | Scaling factor | SNR (dB) @ 85% max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 2Tx6Rx | 2 | 10 (1024QAM) EVM 2.5% | Table 5.1.3.1-4: MCS index table 4 | 24 |  | 25.9 |  |
|  |  |  |  | 23 |  | 23.8 |  |

We can observe that the performance of MCS23 is within a reasonable range, and thus we propose to consider MCS23 as the MCS value for 1024QAM with scaling factor 1.

Proposal 3: Consider MCS24 for 1024QAM with 2 layers

# 3	Summary

In this contribution, we shared our views for the demodulation requirement discussion for 6Rx covering both handheld UEs and FWA. Proposals are summarized as follows:

Proposal 1: Consider MCS25 for 256QAM

Proposal 2: Consider MCS28 for 64QAM

Proposal 3: Consider MCS24 for 1024QAM with 2 layers

# References

[1]	R4-2514788: Way Forward for Topic for [116][215] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon, RAN4 #116bis, Prague, CZ

# Annex

Parameter configurations and example test cases for information:

Table 5.2.5.1.1-2: Test parameters Parameter; Unit; Value. Test case type; Only single carrier test. TDD pattern; 7DS2U, S: 6D+4G+4U. Channel bandwidth / SCS; 40 MHz / 30 kHz. Propagation channel; 4 Layers: Refer to Issue 2-2-1 and 2-2- - TDLA30-10 ULA Low - TDLA30-10 ULA Medium 6 Layers: TDLA30-10 ULA Low. Antenna configuration; 4 Layers: 4T6R 6 Layers: 8T6R. MCS; 4 Layers - ULA Low: Option 1: MCS20 (Table 2); Option 2: MCS17(Table 1) - ULA Medium: MCS13 (Table 1) 6 Layers: MCS17(Table 1). PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PRB bundling type; Static. PRB bundling size; 2. TRS configuration; Symbol#{5, 9}. PT-RS; Not configured. DM-RS configuration; 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: Refer to Issue 2-3-3. Overhead; 0. Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx; Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1. (N1, N2), (O1, O2); 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1). Special slot (S slot) scheduling; Not schedule PDSCH in special slot for 6 layers cases. Number of HARQ Processes; 8. HARQ ACK/NACK bundling; Not configured. Maximum HARQ transmissions; 4. Redundancy version coding sequence; {0,2,3,1}. PDSCH & PDSCH DMRS Precoding configuration; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. Receiver type assumption; MMSE-IRC with joint 6Rx MIMO detector. Tx EVM assumption; - 6% for QPSK/16QAM/64QAM - 3% for 256QAM. Test metric; SNR@70% max TP. Table 5.2.5.1.1-3: Minimum performance for Rank 4 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; xxx; 10 / 15; MCS17; TDLA30-10; 4x6, ULA medium B; 70; xxx. 1-2; xxx; 10 / 15; MCS17; TDLC300-100; 4x6, ULA medium B; 70; xxx. 1-3; xxx; 10 / 15; MCS17; TDLA30-10; 4x6, ULA Low; 70; xxx. Table 5.2.5.1.1-4: Minimum performance for Rank 6 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 2-1; xxx; 10 / 15; MCS13; TDLA30-10; 8x6, ULA Low; 70; xxx.