3GPP TSG-RAN WG4 Meeting #117				                R4-2521142

Dallas, USA, Nov. 17-21, 2025

Source: 	China Telecom

Title: 	Views on R20 4Tx HPUE requirements

Agenda Item:	7.1.4

Document for:	Discussion

Introduction

In last meeting[1], only antenna isolation reached an agreement and there are still many requirements need to be settled before simulation. More importantly, the features related to MOP like ULFPTx should be clear at the beginning of this release.

So in this paper, we bring our views on MOP and ULFPTx for 4Tx for CPE/FWA UE with 4x26dBm PA.

Discussion

- 2.1 4Tx MOP

In Rel-19, we achieved 1dB MPR reduction for single carrier UE by relaxing ACLR and this work took us a lot of time that we spend one release to converge the details. Besides, RAN4 has numerous ways to reduce output power but is difficult to introduce a power increasing feature. So there is no reason to give up 32 dBm total MOP for 4Tx UE with 4x26dBm PA.

On the other hand, we should stick to the WID to specific requirements for 32 dBm total MOP rather than 31, and no need to wait for the simulation results before setting the Maximum output power. Adding new MPR values is not too late if the 32dBm simulation result is negative.

Proposal1: 4Tx total output power should be 32dBm with +2/-3 tolerance. We can define large MPR value for 4Tx if the 32dBm simulation result is negative.

- 2.1 4Tx ULFPTx

UE ULFPTx has three transmission mode 0/1/2. To achieve 32dBm with 4x26dBm PA only supporting mode1 seems enough. However, we should also consider the power fullback when determine the full power mode.

Considering a 4Tx UE with 4x26dBm PA to support 32dBm MOP, it can use mode1 to take advantage of full power transmission for PUSCH. If power fullback occurs due to such as SAR restriction, mode1 may be no longer the best option. For example, if the ΔPPowerClass or P-MPR is 3dB and the UE is at the edge of the cell and requires low-rank transmission to ensure connection, so the MOP is 29dBm. Although ULFPTx mode1 is indeed supported by the UE but mode2 is the optimal choice for UE to support single TX chain up to 29dBm to enhance coverage. In a word, for the wider application of 4x26dBm PA UE, UE should both support ULFPTx mode1 and mode2.

Proposal2: For the wider application of 4x26dBm PA UE, UE should both support ULFPTx mode1 and mode2.

Besides, we have discussed full power mode reporting in Rel-18 when design the ΔPPowerClass report mechanism but had no agreement. 4Tx UE has more UL layers so the set of coherent port will be more complex. Knowing the full power transmission mode corresponding to the current reference power class is helpful for NW. We should discuss this reporting mechanism in Rel-20 under this WI.

Proposal3: Discuss whether and how introducing full power transmission mode reporting corresponding to current PC for 4Tx UE in Rel-20.

Conclusion

Proposal1: 4Tx total output power should be 32dBm with +2/-3 tolerance. We can define large MPR value for 4Tx if the 32dBm simulation result is negative.

Proposal2: For the wider application of 4x26dBm PA UE, UE should both support ULFPTx mode1 and mode2.

Proposal3: Discuss whether and how introducing full power transmission mode reporting corresponding to current PC for 4Tx UE in Rel-20.

References

- R4-2515087 Way Forward for [116bis][316] NR_UE_RF_Ph5, Nokia, Murata, RAN4#116-bis