3GPP TSG-RAN WG4 Meeting #117	R4-2521172

# Dallas, USA, 17th November – 21st November, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test case for EMR based direct SCell activation in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce test case on EMR based fast direct SCell activation requirements in the RRM spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | EMR based fast direct SCell activation requirements are tested for FR1 SCell with different UE capabilities considered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case for EMR based fast direct SCell activation is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.3.X2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

A.6.5.3.X2	EMR based Direct SCell activation at SCell addition of unknown SCell in FR1

A.6.5.3.X2.1	Test Purpose and Environment

The purpose of this test is to verify that the EMR (early measurement reporting) based direct SCell activation delay as defined in clause 8.3.4A, when the SCell in FR1 is unknown by the UE at the time of activation.. The supported test configurations for NR PCell are shown in table A.6.5.3.X2.1-1. The supported test configurations for NR SCell are shown in table A.6.5.3.X2.1-1A. Test configurations for NR PCell and test configurations for NR SCell are chosen independently.

The test scenario comprises one PCell (Cell 1) and one SCell (Cell 2) as outlined in table A.6.5.3.X2.1-2. Cell-specific parameters for PCell and SCell are provided in table A.6.5.3.X2.1-3 and table A.6.5.3.X2.1-4, respectively.

The test consists of two successive time periods with duration T1 and T2, respectively. There are two carriers, each with one cell. Cell 1 (PCell) is on RF channel 1 (PCC), and Cell 2 (SCell) is on RF channel 2 (SCC). Cell 1 and Cell 2 both operate according to one of the configurations in table A.6.5.3.X2.1-1 and table A.6.5.3.X2.1-1A respectively.

Before the test starts the UE is connected to Cell 1 on RF channel 1. The UE is only monitoring RF channel 1 and is not aware of Cell 2 on RF channel 2.

The test consists of 6 successive time periods, with time duration of T1, T2, T3, T4, T5 and T6 respectively.

During T1, the UE is connected to Cell 1 only and shall not have any timing information of Cell 2. UE is configured with early measurement reporting for Cell 2 in

measIdleCarrierListNR-r16 for UE supporting measValidationReportEMR-r18 or

idleInactiveNR-MeasReport-r16 only, or

measReselectionCarrierListNR-r18 for UE supporting measValidationReportReselectionMeasurements-r18.

Beam level reporting for early measurements is configured. The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

During T2 and T3 the UE is in idle mode.

At the beginning of T2, Cell 2 becomes detectable however cell reselection shall not be performed. Signal level of Cell 2 is set to the value given in table A.6.6.9.2.1-3. The time when T331 timer expires defines the ending point of T2.

At the beginning of T3, the signal level of the neighbour cell is set to turned off.  The duration of the T3 equals to measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 depending on the UE capabilities of the UE under test.

The time when TE sends the paging message is defined as the starting point of T4. During T4, in this test the UE shall send measurement report with SSB index of Cell 2 to the PCell.

At the beginning of T5 the UE is configured to measure RF channel 2 in measurement gaps. During T5, the UE detects and measures Cell 2 on RF channel 2, and sends a measurement report containing Cell 2 to the test equipment. After having received a measurement report containing Cell 2, the test equipment deconfigures the measurement gaps and thereafter sends a RRC connection reconfiguration message to the UE by which it configures the SCell (Cell 2) in activated state (sCellState is set to activated).

Time period T6 starts when the UE receives the RRC connection reconfiguration message at the UE antenna connector. The corresponding slot at which the message is received at the UE antenna connector is denoted n. The UE shall complete activation of the SCell no later than in slot n + NdirectNR slot length, as specified in clause 8.3.4A. From slot n+ NdirectNR slot length and onwards the UE shall report valid CSI both for PCell and SCell.

The test equipment verifies the activation time by counting the slots between the RRC connection reconfiguration message is sent and until CSI report with non-zero CQI for both PCell and SCell is received.

Table A.6.5.3.X2.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration,

Table A.6.5.3.X2.1-1A: Supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Table A.6.5.3.X2.1-2: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Inter-frequency neighbor cell (SCell to-be) |  | Cell 2 | Inter-frequency neighbor cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern |  | gp0 | Measurement gap is used during parts of time period T5 for detection of Cell 2. |
| CSI reporting periodicity | ms | 2 | CSI reporting periodicity for periodic reporting of CQI for PCell and, when added, SCell. |
| SCell measurement cycle (measCycleSCell) | ms | 160 | Measurement cycle for SCell does not come into effect in direct activation at SCell addition. |
| Timing offset between Cell 1 and Cell 2 | s | MRTD | The value of maximum timing offset depends upon the carrier aggregation scenario. |
| T1 | s | 10 | During this time UE connects to PCell and early measurement report on the carrier frequency of Cell 2 is configured. |
| T2 | s | 30 | During this time UE is in RRC_IDLE state and Cell 2 becomes detectable. |
| T3 | s | 5 | During this time UE determines the validity of reported idle/inactive and reselection measurements. |
| T4 | s | 1 | During this time UE shall send a measurement report with SSB index of Cell 2 to the PCell. |
| T5 | s | 7 | During this time period the PCell shall be known and Cell 2 shall be detected as an inter-frequency neighbor cell. |
| T6 | s | 1 | During this time period Cell 2 shall be configured and directly activated as SCell. |
| measIdleValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportEMR-r18 is tested in this test. |
| measReselectionValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportReselectionMeasurements is tested in this test. |
| measIdleDuration-r16 | s | 60 | Value for timer T331 for UE performing idle/inactive measurements while in RRC_IDLE or RRC_INACTIVE. |
| A3-offset | dB | -15 |  |
| THARQ | ms | k1×NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting | ms | 2 | the delay uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| k | ms |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.6.5.3.X2.1-3: NR Cell specific test parameters

| Parameter |  |  | Unit | Cell 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 | T6 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1,2 | MHz | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP configuration |  | Initial DL |  | DLBWP.0.1 |  |  |  |  |  |
|  |  | Initial UL |  | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL |  | DLBWP.1.1 |  |  |  |  |  |
|  |  | Dedicated UL |  | ULBWP.1.1 |  |  |  |  |  |
| TCI state |  |  |  | TCI.State.0 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |  |  |
| OCNG Pattern |  |  |  | OP.1 |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| reportConfigType |  |  |  | periodic |  |  |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |  |  |
| CSI reporting periodicity |  | Config 1,2 | slot | 5 |  |  |  |  |  |
|  |  | Config 3 |  | 10 |  |  |  |  |  |
| CSI reporting offset |  | Config 1,2 | slot | 3 |  |  |  |  |  |
|  |  | Config 3 |  | 5 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 |  | dBm/SCS | -104 |  |  |  |  |  |
|  | Config 3 |  |  | -101 |  |  |  |  |  |
| Ês/Iot |  |  | dB | 17 |  |  |  |  |  |
| Ês/Noc |  |  | dB | 17 |  |  |  |  |  |
| SS-RSRP Note3 | Config 1,2 |  | dBm/SCS | -87 |  |  |  |  |  |
|  | Config 3 |  |  | -84 |  |  |  |  |  |
| Io Note3 | Config 1,2 |  | dBm/9.36 MHz | -59.0 |  |  |  |  |  |
|  | Config 3 |  | dBm/38.16 MHz | -52.9 |  |  |  |  |  |
| Propagation condition |  |  |  | AWGN |  |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP, SCH_RP, and Io levels have been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

Table A.6.5.3.X2.1-4: NR Cell specific test parameters for NR Scell

| Parameter |  | Unit | Cell 2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  |  | T4 | T5 |  | T6 |
| Duplex mode | ConfigSCell 1 |  | FDD |  |  |  |  |  |  |  |  |
|  | ConfigSCell 2,3 |  | TDD |  |  |  |  |  |  |  |  |
| TDD configuration | ConfigSCell 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |
| BWchannel | ConfigSCell 1,2 | MHz | 10: NPRB,c = 52 |  |  |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | 40: NPRB,c = 106 |  |  |  |  |  |  |  |  |
| BWP configuration | Initial DL |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |
|  | Initial UL |  | N/A |  |  |  |  |  |  |  |  |
|  | Dedicated DL |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |
|  | Dedicated UL |  | N/A |  |  |  |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |  |  |  |
| CSI-RS configuration for CSI reporting | ConfigSCell 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |  |  |  |
|  | ConfigSCell 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |  |  |  |
| TRS Configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | SR.1.1 FDD |  |  |  |  |  |  |  |  |
|  | ConfigSCell 2 |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | SR.2.1 TDD |  |  |  |  |  |  |  |  |
| Dedicated CORESET parameters | ConfigSCell 1 |  | CCR.1.1 FDD |  |  |  |  |  |  |  |  |
|  | ConfigSCell 2 |  | CCR.1.1 TDD |  |  |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | CCR.2.1 TDD |  |  |  |  |  |  |  |  |
| RMSI CORESET parameters | ConfigSCell 1 |  | N/A |  |  |  |  |  |  |  |  |
|  | ConfigSCell 2 |  |  |  |  |  |  |  |  |  |  |
|  | ConfigSCell 3 |  |  |  |  |  |  |  |  |  |  |
| OCNG Pattern |  |  | OP.1 |  |  |  |  |  |  |  |  |
| SSB Configuration | ConfigSCell 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |  |  |  |
| reportConfigType |  |  | N/A |  |  |  |  |  |  |  |  |
| reportQuantity |  |  | N/A |  |  |  |  |  |  |  |  |
| CSI reporting periodicity | ConfigSCell 1,2 | slot | N/A |  |  |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |  |  |  |  |
| CSI reporting offset | ConfigSCell 1,2 | slot | N/A |  |  |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |  |  |  |  |  |  |  |  |
| Noc Note2 | ConfigSCell 1,2 | dBm/SCS | -98 |  |  |  |  |  |  | -104 |  |
|  | ConfigSCell 3 |  | -95 |  |  |  |  |  |  | -101 |  |
| Ês/Iot |  | dB | -infinity | 7 |  | -2 |  | -2 |  | 17 |  |
| Ês/Noc |  | dB | - infinity | 7 |  | -2 |  | -2 |  | 17 |  |
| SS-RSRP Note3 | ConfigSCell 1,2 | dBm/SCS | -infinity | -91 |  | -100 |  | -100 |  | -87 |  |
|  | ConfigSCell 3 |  | -infinity | -88 |  | -97 | -97 |  |  | -84 |  |
| Io Note3 | ConfigSCell 1,2 | dBm/9.36 MHz | -70.5 | -62.26 |  | -67.92 | -67.92 |  |  | -59.0 |  |
|  | ConfigSCell 3 | dBm/38.16 MHz | -63.96 | -56.17 |  | -61.83 | -61.83 |  |  | -52.9 |  |
| Propagation condition |  |  | AWGN |  |  |  |  |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |  |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP, SCH_RP, and Io levels have been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

A.6.5.3.X2.2	Test Requirements

The UE shall complete the direct activation of the SCell no later than at slot n + NdirectNR slot length.

The UE shall report non-zero CQI for SCell from slot n + NdirectNR slot length and onwards throughout time period T6.

The rate of correct events observed during repeated tests shall be at least 90 %.

==============End of change==============