# 3GPP TSG-RAN WG4 Meeting #117                                    R4-2521175

# Dallas, USA, Nov. 17-21, 2025

# Title: 	Simulation results on UE demodulation performance requirements for 6Rx

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.1.2

# Document for:	Discussion

# 1 Introduction

In RAN #105 meeting, the revised WID of UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 was approved in [1].

In this contribution, we give some initial simulation results on 6Rx demodulation requirements based on the last meeting conclusion [2].

# 2 Discussion

In this section, we mainly evaluate the performance with different candidates for propagation condition for 4 MIMO layers and 6 MIMO layers.

Table 1. Simulation assumption

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Test case type |  |  | Only single carrier test |
| TDD pattern |  |  | 7DS2U, S: 6D+4G+4U |
| Channel bandwidth / SCS |  |  | 40 MHz / 30 kHz |
| Propagation channel |  |  | 4 Layers: TDLA30-10 ULA correlation channel with α = 0 and β = 0.07179 6 Layers: TDLA30-10 ULA Low |
| Antenna configuration |  |  | 4 Layers: 4T6R 6 Layers: 8T6R |
| MCS |  |  | 4 Layers: MCS13 (Table 1) 6 Layers: MCS17(Table 1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| DM-RS configuration |  |  | 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: Rel-15 Type 1 DMRS 2+2: {1000-1003, 1004, 1006} |
| Overhead |  |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| (N1, N2), (O1, O2) |  |  | 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1) |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Receiver type assumption |  |  | MMSE-IRC with joint 6Rx MIMO detector |
| Tx EVM assumption |  |  | 6% |
| Test metric |  |  | SNR@70% max TP |

The simulation results as shown in the following table.

Table 2. Simulation results for 6Rx with 4 layers and 6 layers.

| Test num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 4 | 40/ 30 | MCS 13 | TDLA30-10 | 4x6, ULA correlation channel with α = 0 and β = 0.07179 | 70 | 17.1 |
| 2 | 6 | 40/ 30 | MCS 17 | TDLA30-10 | 8x6, Low | 70 | 19.5 |

# 3 Conclusion

In this contribution, we provided the initial simulation results for 6Rx demodulation performance requirements.

# 4 References

- RP-250780 New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4. Huawei, HiSilicon.

- R4-2514788, Way Forward for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx.