# 3GPP TSG-RAN WG4 Meeting #117                                    R4-2521193

# Dallas, USA, Nov. 17-21, 2025

# Title: 	Discussion on inter-vendor training collaboration and interoperability for two-sided models

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.9.3

# Document for:	Approval

# 1 Introduction

In RAN #108 meeting, the new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are shown in following table.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In the following section, we will provide the initial discussions for inter-vendor training collaboration and interoperability for two-side models.

# 2 Discussion

In the last meeting, RAN4 discussed the technical issues in detail, but still have some open issues [2] as shown in the following table. Next, we would like to address in the following for Rel-20 CSI compression.

- Proposals for assumptions for the dataset including how to cover scalability, channel models, number of samples - Proposals for how to evaluate the dataset including potential metrics to compare different dataset contributions - Proposals on model hyperparameters, considering in particular complexity, considering also quantization size - Proposals and reasoning for scalability solutions - Proposals and reasoning for training parameters - Proposals for performance metrics for evaluating the different potential model structures - Proposals for how to capture the model and the dataset in 3GPP (format, how to store etc.)

In the following, we will discuss the detailed one by one for the remain issues.

Regarding the aspects of model structure scalability, the following alternatives were agreed in RAN1.

- For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters

As mentioned in the above, RAN1 studied the token dimension and feature dimension for different data dimension. In Rel-19, RAN4 assumed that the dimension of training dataset including subband size, Tx port, and channel dimensions. As evaluated in RAN1, the number of subband size and Tx port are also considered as token dimension and feature dimension, which illustrated in the above Alt1, Alt2, and Alt 3. Based on our understanding, our theoretical analysis suggests that there is correlation between CSI in subbands. As the token dimension is like correlation between traditional sentences, features can be better learned. Therefore, we think that the Alt 1, using subband as the token dimension and Tx port as a feature dimension could be considered. Compared with Alt1 and Alt2, the complexity of Alt3 could be a little higher. Therefore, we prefer to consider Alt 1 as candidate approach to study the scalability.

On the other side, considering the scalability of the feature and token dimension, padding at the input is easier and has lower complexity. Zero-padding is a powerful technique that forces a mode to learn essential features by destroying accidental structure in the data. It significantly improves the model’s generalization ability by increasing the position diversity and scale flexibility of the data. Therefore, we prefer to consider zero-padding for scalability study.

Regarding scalability of payload configuration, RAN4 needs more discussion on this issue, which will affect the final generalization performance. As shown in the above candidate options, Alt1, Alt2, and Alt3 also different implementations. Alt1 specifies output layer for each payload configuration, which means that Alt1 needs to specific different output layers. For each different payload mapping to different trained models. Once the model is trained, different payload configurations can choose different output layers to meet the payload configuration. The only thing to note is that it needs to store different modules, which increases complexity. Compared with Alt1, Alt2 only needs to train the maximum payload configuration with output layer. To achieve the scalability, the latent message could be truncated for other different payloads during the training process. The advantage is that only one module needs to be stored, which can reduce the complexity compared to Alt1. Regarding Alt3, which is the different approach compared to Alt1 and Alt2. It needs to consider different quantization parameters, e.g., 2bit, 4bit, etc. From our understanding, the quantization approach only brings different feedback bits, which determines the feedback overhead of the AI based approach compared with legacy method. Fewer bits of quantization means less feedback overhead. However, in AI training, latent message is like a “blueprint”, which determines how to extract features from the original information and compress and store them; while the length of latent space is like the size of the “canvas”, which ultimately determines how much effective information can be accommodated. Thus, Alt3 does not have a significant impact on performance due to the difference in quantization bits. This is the biggest difference between Alt3 and Alt1, Alt2. Therefore, from the scalability point of view, Alt3 could perhaps be deprioritized.

Observation 1. Our theoretical analysis suggests that there is a correlation between the CSI values in the subbands. As the token dimension is like the correlation between traditional sentences, features can be learnt more effectively.

Observation 2. In terms of the scalability of the payload configuration, different lengths of latent messages need to be considered for both Alt. 1 and Alt. 2. The difference is that Alt. 1 requires training for different lengths and storing parameters, whereas Alt. 2 only considers the maximum length of the latent message during training. A truncated approach needs to be considered for different lengths.

Observation 3. Alt 3 only consider quantization approach with different quantization bits during training process, e.g. 2bit, 4bit quantization.

Observation 4. In AI training, latent message is like a “blueprint”, which determines how to extract features from the original information and compress and store them; while the length of latent space is like the size of the “canvas”, which ultimately determines how much effective information can be accommodated.

Proposal 1. Propose to use subband as the token dimension and Tx port as a feature dimension when considering the choice of token dimension and feature dimension.

Proposal 2. Propose to use padding approach for the study of scalability over the feature dimension and token dimension.

Proposal 3. Propose to use truncation of the output of the linear layer output for scalability over payload configurations.

In Rel-19, RAN4 studied the mix dataset approach to evaluation SGCS performance for Option 3 and Option 4. Based on the mix dataset, the simulation among different companies could be aligned well, and the feasibility of two side model approach is verified. Therefore, for Rel-20 WI, we believe that the mix dataset also needs to be considered for test decoder training. During the Rel-19 study process, RAN4 did not consider the performance metrics for validating different companies’ datasets. When RAN4 evaluated the performance among different companies’ datasets, which observed that some companies’ datasets can’t achieve a better performance compared with other companies. Based on this observation from companies, RAN4 removed some companies’ datasets from the mixed dataset.

Currently, the motivation of consider performance metrics for companies’ datasets is very clearly, to avoid the different distributions from companies’ dataset. Considering the performance requirements will be defined in Rel-20, we believe that the performance metric for different companies’ datasets is very necessary. RAN4 at least needs to make sure that the datasets from different companies follow the same distribution. In the last meeting, some candidate options are proposed by some companies, such as PSE (power spectral entropy), Frequency coherence, Spatial coherence, etc.

PSE quantifies the “disorder” or complexity of a signal’s frequency domain distribution be calculating the information entropy of the power spectrum. The closer the entropy values, the more similar the complexity of the frequency domain energy distribution of the two signals. Frequency coherence calculates (e.g., the Pearson correlation coefficient) between the frequency-domain data of two signals (e.g., the amplitude/phase spectra after Fourier transformation), measuring the strength of linear correlation between specific frequency components. The closer the correlation coefficient is to 1, the more synchronized the energy/phase changes at the corresponding frequencies. Spatial correlation measures the distributional association of data in spatial dimensions (e.g., the trend of numerical values at different spatial locations). Spatial autocorrelation coefficients (Moran’s I, Geary’s C) or cross correlation analysis are commonly used. In general, the above candidate options also can be used to define performance metric, how to evaluate the dataset to compare different dataset contributions requires more discussion.

Another candidate option is that using models from Rel-19 to evaluation the different companies’ datasets. From this point of view, we think this approach needs more clarification for Rel-20 study. In Rel-19, the model is trained based on SLS and CDL channel model. However, how to generate the training dataset is under discussion. Because the legacy requirements are based on LLS (e.g., TDL) to evaluate the performance. Therefore, if RAN4 wants to reuse the legacy approach to evaluate the AI based CSI compression, we believe that whether the Rel-19 model trained based on the CDL channel model can be used for the Rel-20 evaluation requires more discussion. If RAN4 wants to use this option to evaluate the companies’ datasets, we believe that this discussion depends on how to generate the training dataset in Rel-20.

Observation 5. PSE, frequency coherence, and spatial coherence approaches also can be used to evaluate the companies’ datasets.

Observation 6. If RAN4 wants to use Rel-19 models to evaluate the companies’ datasets, we believe that this discussion depends on how to generate the training dataset in Rel-20.

As mentioned above, the process of generating the dataset requires further discussion. For example, should it be based on the CDL or TDL channel model? This is particularly relevant for the TDL channel model, as it raises the question of whether SNR levels need to be considered. Firstly, regarding the dataset generation approach, we prefer to consider TDL channel model to generate datasets. Our concern is that whether the dataset generated by the CDL channel model based on system level simulation can be used for the TDL channel model of link level simulation. At least without evaluation, it cannot be considered possible. Thus, we propose to use TDL channel model to generate the datasets. On the other hand, whether SNR levels need to be considered in dataset generation process. Currently, RAN4 needs to find the correspond SNR point of 90% maximum throughput to define the Gama value in legacy requirements. If RAN4 agrees to reuse this test metric to define CSI demodulation requirements, RAN4 needs to firstly find the SNR point of 90% maximum throughput. Thus, RAN4 needs to discuss how to generate the datasets to train the model based on the SNR point or only consider the ideal channel estimation without SNR impact. Meanwhile, if SNR point based approach is agreed, another issue is that whether needs to consider different SNR levels or only consider single SNR levels. For different SNR levels mean that RAN4 needs to create a mixed dataset for different SNR levels and single SNR only needs to consider the correspond SNR point of 90% throughput.

Based on our analysis, two different potential approaches are mentioned in the above for TDL channel model-based datasets. One potential method is to create datasets based on the different SNR levels, e.g., traversing different SNR points for scalability. Single SNR point is another approach which only considers the 90% throughput.

Observation 7. If RAN4 wants to use CDL based channel model to generate datasets, RAN4 needs to evaluate whether CDL based training datasets could be used for link level simulation and TDL channel model.

Observation 8. For datasets based on TDL channel models, two different potential approaches can be considered. One is based on mixed datasets based on a single SNR level , one is to create a mixed dataset based on different SNR level to generate datasets.

Proposal 4. Propose to consider TDL channel model to generate training dataset, and the dataset also needs to consider different SNR levels for scalability.

Proposal 5. PSE, frequency coherence, and spatial coherence approaches also can be used to evaluate the companies’ datasets.

# 3 Conclusion

In this contribution, we give some detailed discussions on testability and interoperability issues for AI based CSI compression, the conclusions are:

Observation 1. Our theoretical analysis suggests that there is a correlation between the CSI values in the subbands. As the token dimension is like the correlation between traditional sentences, features can be learnt more effectively.

Observation 2. In terms of the scalability of the payload configuration, different lengths of latent messages need to be considered for both Alt. 1 and Alt. 2. The difference is that Alt. 1 requires training for different lengths and storing parameters, whereas Alt. 2 only considers the maximum length of the latent message during training. A truncated approach needs to be considered for different lengths.

Observation 3. Alt 3 only consider quantization approach with different quantization bits during training process, e.g. 2bit, 4bit quantization.

Observation 4. In AI training, latent message is like a “blueprint”, which determines how to extract features from the original information and compress and store them; while the length of latent space is like the size of the “canvas”, which ultimately determines how much effective information can be accommodated.

Observation 5. PSE, frequency coherence, and spatial coherence approaches also can be used to evaluate the companies’ datasets.

Observation 6. If RAN4 wants to use Rel-19 models to evaluate the companies’ datasets, we believe that this discussion depends on how to generate the training dataset in Rel-20.

Observation 7. If RAN4 wants to use CDL based channel model to generate datasets, RAN4 needs to evaluate whether CDL based training datasets could be used for link level simulation and TDL channel model.

Observation 8. For datasets based on TDL channel models, two different potential approaches can be considered. One is based on mixed datasets based on a single SNR level, one is to create a mixed dataset based on different SNR level to generate datasets.

Proposal 1. Propose to use subband as the token dimension and Tx port as a feature dimension when considering the choice of token dimension and feature dimension.

Proposal 2. Propose to use padding approach for the study of scalability over the feature dimension and token dimension.

Proposal 3. Propose to use truncation of the output of the linear layer output for scalability over payload configurations.

Proposal 4. Propose to consider TDL channel model to generate training dataset, and the dataset also needs to consider different SNR levels for scalability.

Proposal 5. PSE, frequency coherence, and spatial coherence approaches also can be used to evaluate the companies’ datasets.

# 4 References

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements.

- R4-2514614, WF on R20 NR_AIML_air_Ph2, Ericsson.