3GPP TSG-RAN WG4 Meeting #117	R4-2521197

Dallas, USA, 17th – 21st November 2025

Source:	Nokia

Title:	Discussion on UE RF issues for US 4.9 GHz band

Agenda item:	5.11.4

Document for:	Approval

Introduction

RAN at #108 meeting agreed the WID on Introduction of NR TDD 4.9GHz band for US operation [1]. In RAN4#116, the initial study phase concluded that 3GPP would rely on the band manager described by FCC for the coexistence issues with incumbents and thus no specific 3GPP work would be required for them [2]. RAN4 is now expected to work on the core part of the WI in the following.

Following the completion of the study phase, the core objectives of this WI are:

-	Specify a new NR TDD operating band for the frequency range, 4940-4990 MHz to support the following SCS and channel bandwidths:

| SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
| 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
| 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |

- Specify band numbering, system parameters and RF characteristics for the channel bandwidths defined for the new band.

- Address potential BS and UE co-existence issues, if any.

  - Specify band specific BS emissions requirements if needed

  - Specify UE additional emission limits and A-MPR if needed

- Specify the BS, repeater and IAB RF requirements.

- Specify the UE RF requirements for the new band. Reuse NR band n79 UE hardware assumption if possible.

- Update the related technical specifications to include support for the new band

- CRs shall not be approved until ongoing litigation is resolved to avoid specification rework and potential NBC changes

Note: The new NR band will be introduced in a REL-independent way starting from REL-15.

In this contribution, we discuss the UE RF aspects for the US 4.9GHz band for UE Power Class 3, as RAN considered that HPUE would be handled later in a basket WI [3].

Discussion

We submitted the summary of the existing regulatory requirements for US 4.9GHz band in RAN4#116 [4]. FCC Title 47 CFR §90.1213 specifies Band plan, §90.1215 Power limits and §90.210 Emission masks [5][6].

The center frequency, bandwidth and channel numbers are defined in §90.1213 (a). Aggregation up to 50 MHz bandwidth in any manner is allowed. 25 MHz is not listed in FCC regulations but can be aggregated from 5, 10, 15 or 20 MHz and therefore UE channel bandwidths listed in the WID (shown above) are aligned with regulations.

UE channel bandwidths in WID are consistent with regulations.

Power limits in §90.1215 (a)(1) specify the conducted output power (TRP) limit listed in Table 1.

Table 1: Maximum limits of transmitting power of stations operating in 4940-4990 MHz

| Channel bandwidth (MHz) | Low power maximum conducted output power (dBm) | High power maximum conducted output power (dBm) |
| --- | --- | --- |
| 1 | 7 | 20 |
| 5 | 14 | 27 |
| 10 | 17 | 30 |
| 15 | 18.8 | 31.8 |
| 20 | 20 | 33 |
| 30 | 21.8 | 34.8 |
| 40 | 23 | 36 |
| 50 | 24 | 37 |

In RAN4 #116-bis meeting, we agreed to assume only the high power devices [7]. Therefore, TRP limit per channel bandwidth corresponds to PSD 20 dBm/MHz for all the channel bandwidths listed above.

Only high power devices are considered in this WI.

Furthermore, a peak power spectrum density of 21 dBm/MHz is specified in FCC Power limits for High power devices in §90.1215 (a). This means the peak PSD can be 1 dB higher than the channel average.

As specified in §90.1215 (a)(2), High power devices using channel bandwidths other than those listed above are permitted. Therefore, TRP limit for 25 MHz channel bandwidth can be up to 34.98 dBm. In RAN4 #116-bis meeting, we agreed to scale the TRP limit from the other channel bandwidths, therefore it was agreed to be 34 dBm [7].

PSD limit 20 dBm/MHz for the entire channel bandwidth and 21 dBm/MHz for the peak density are applied to all the channel bandwidths specified in WID.

In the last meeting, there was a proposal to include 1 dB margin to accommodate imperfect UL spectral flatness so that 20 dBm/MHz is applied for all RB allocations. It is unclear how this margin is derived. It may depend on how many samples are simulated. Unless there is any further justification, we propose we leave it to each company who simulates A-MPR.

It is proposed to assume 20 dBm/MHz for the entire channel bandwidth and 21 dBm/MHz for the peak power spectral density in A-MPR simulations. How to handle the margin for spectral flatness or simulation variance is up to each company.

Although RAN considered that HPUE would be handled later in a basket WI [3], there was proposal to consider PC2 as the prototype PC instead of PC3 [8][9]. Full power waveform is not available for HPUE (PC1, PC1.5 or PC2) due to the strict peak PSD limit and FCC mask M requirement. Thus, HPUE will need a different approach than PC3 to specify the power classes which would be beyond the scope of the HPUE basket WI due to its complexity. In our view, the lack of full power waveform would discourage operators and vendors from deploying and developing HPUE. Therefore, we think that PC3 is not a special case, but HPUE is. Hence, we propose we focus on PC3 now and handle HPUE later. If HPUE basket is not a right WI due to the complex mechanism of power backoff, we can consider having another WI.

It is proposed to focus on PC3 in this WI and defer HPUE to the basket WI or another dedicated WI.

AS PSD is limited to 21 dBm/MHz, the achievable TRP is 21+10 log(BW) at best. So for BW<1.585MHz (4.4 PRBs in 30 kHz), TRP will never reach 23 dBm.

Maximum TRP of UE power class cannot be achieved for narrow PRB allocations.

EIRP limit is defined in FCC Power limits. If antenna gain is more than 9 dBi, the excess must be reduced from the conducted power limit. As high gain antenna is not considered for UE Power Class 3, the power reduction due to EIRP limit is not required.

Power reduction due to EIRP limit does not need to be taken into account for UE power class 3.

As WID [1] suggests that we reused NR band n79 UE hardware assumption, if possible, it is proposed that UE Power Class 3 is defined with the maximum output power 23 dBm with +2/-3 dB tolerance, reused from n79.

UE maximum output power for PC3 is 23 dBm with +2/-3 dB tolerance, reused from n79.

Emission mask M is applicable for high power transmitter (greater than 20 dBm) operating in 4940-4990 MHz frequency band. The power spectral density of the emissions must be attenuated below the output power of the transmitter as follows:

(1) On any frequency removed from the assigned frequency between 0-45% of the authorized bandwidth (BW): 0 dB.

(2) On any frequency removed from the assigned frequency between 45-50% of the authorized bandwidth: 568 log (% of (BW)/45) dB.

(3) On any frequency removed from the assigned frequency between 50-55% of the authorized bandwidth: 26 + 145 log (% of BW/50) dB.

(4) On any frequency removed from the assigned frequency between 55-100% of the authorized bandwidth: 32 + 31 log (% of (BW)/55) dB.

(5) On any frequency removed from the assigned frequency between 100-150% of the authorized bandwidth: 40 + 57 log (% of (BW)/100) dB.

(6) On any frequency removed from the assigned frequency between above 150% of the authorized bandwidth: 50 dB or 55 + 10 log (P) dB, whichever is the lesser attenuation.

(7) The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission and a video bandwidth of 30 kHz. The power spectral density is the power measured within the resolution bandwidth of the measurement device divided by the resolution bandwidth of the measurement device. Emission levels are also based on the use of measurement instrumentation employing a resolution bandwidth of at least one percent of the occupied bandwidth.

<Agreement>: Resolution bandwidth

- Adopt a resolution bandwidth for the FCC part 90 mask to 1% of the configured UE channel bandwidth.

  - Spec wording is FFS.

The resolution bandwidth is at least 1% of the bandwidth in FCC which are consistent with 3GPP spectrum emission masks specified in TS 38.101-1 clause 6.5.2.2, except for 50 MHz channel bandwidth. 30 kHz is used for the first 1 MHz in 3GPP for 50 MHz channel bandwidth. In the previous meeting, the following agreement is made.

The resolution bandwidth is agreed 1% for all the channel bandwidth including 50 MHz.

Figure 1 shows the comparison of FCC masks with 3GPP masks (SEM and Spurious (for above BWchannel+5MHz)) when the requirement is scaled in dBm/MHz for 10/20/40/50 MHz channel bandwidths. FCC masks are more stringent than 3GPP spectrum emission masks for out-of-band. Further, the mask is defined also in-band, i.e., within the channel bandwidth.

Figure 1: Comparison of FCC and 3GPP emission masks for PC3

Emission masks in FCC for US 4.9 GHz band are more stringent than 3GPP general UE emissions masks for out-of-band emissions. Further, the FCC mask is also appliable within the channel bandwidth.

The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission and a video bandwidth of 30 kHz.

<Agreement>: Zero dB reference for applicable FCC Emission mask M

- The zero dB reference for FCC Emission mask M is calculated based on UE power class maximum output power and the configured UE channel bandwidth.

  - The zero dB reference is used for mask location and for A-MPR analysis.

In the previous meeting, the following agreement is made.

0 dB reference of the emission mask is defined with TRP 23 dBm/channel bandwidth (MHz) for UE power class 3.

One possible way to handle PSD limits and emissions masks together is to apply A-MPR. The following agreement in the previous meeting is that we incorporate both FCC PSD limits and Emission masks in one Network Signaling mechanism.

<Agreement>: New NS, Impacted frequency range for new NS, Other requirements for new NS

- Specify a new NS for the US 4.9GHz band with FCC part 90 including:

  - The Mask M requirement (note this covers the 5% channel bandwidth edge (inside), the SEM domain and the spurious domain).

  - Maximum output power limitation in Sub-topic 4-1.

It is agreed we incorporate FCC PSD limits and Emission masks together in one Network Signaling mechanism.

Conclusion

The following Proposals were made:

Proposal 1: It is proposed to assume 20 dBm/MHz for the entire channel bandwidth and 21 dBm/MHz for the peak power spectral density in A-MPR simulations. How to handle the margin for spectral flatness or simulation variance is up to each company.

Proposal 2: It is proposed to focus on PC3 in this WI and defer HPUE to the basket WI or another dedicated WI.

Proposal 3: UE maximum output power for PC3 is 23 dBm with +2/-3 dB tolerance, reused from n79.

- References

- RP-252929 Revised WID on Introduction of NR TDD 4.9GHz band for US operation, Fujitsu, RAN#109

- RP-252913 Status report for Introduction of NR TDD 4.9GHz band for US operation, Fujitsu, RAN#109

- Draft meeting report RAN #109

- R4-2509418 On 4940-4990 MHz band regulations for adjacent spectrum protection, Nokia, RAN4#116

- https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-Y

- https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-I

- R4-2515103 Way Forward for [116bis][322] NR_TDD_4.9GHz_band, Fujitsu, RAN4#116-bis

- R4-2514445 Topic summary for [116bis][322] NR_TDD_4.9GHz_band, Fujitsu, RAN4#116-bis

- R4-2515083	Ad-hoc meeting minutes for [116bis][322] NR_TDD_4.9GHz_band, RAN4#116-bis