3GPP TSG-RAN WG4 Meeting # 117	R4-2521208

Dallas, US, 17 – 21 November 2025

Agenda Item:	7.12.4

Source:	Ericsson

Title:	On event prediction for AI/ML aided mobility

Document for:	Approval

# Introduction

The WID for AI/ML aided mobility concerns scenarios where the AI/ML model for RRM event prediction is deployed at UE. For event prediction both indirect event prediction and direct event prediction are within the scope of WI, and the following objectives are identified for RAN4 [1].

For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]:

- Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction

  - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction.

- For direct event prediction, study feasibility of core requirement in Q2 2026.

- Specify LCM-related core requirements to support the LCM functionality management , if any

Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions.

# Discussion

According to the objectives for RAN4, RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect event prediction and to study the feasibility of defining core requirement for direct event prediction in Q2 2026.

RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect event prediction.

RAN4 to study feasibility of defining core requirement for direct event prediction in Q2 2026.

Snippets of events A1∼A6 that are within the scope of the WI are provided in Table 1. For detection of all these events, predicted RRM measurements can be used provided that the predicted RRM measurements are reliable and accurate. This is the scenario that is identified as indirect event prediction in WID. To define the delay requirements for reporting, legacy approach can be considered. Delay requirement for periodic reporting shall be discussed as a part of RRM measurement prediction and delay requirement for event triggered reporting shall be discussed as part of event prediction thread. In this regard RAN4 does not have to spend time on identifying ways to reuse core requirement for RRM measurement prediction for indirect event prediction.

Table 1. Events and corresponding triggering conditions.

| Event | Triggering condition(s) |
| --- | --- |
| A1 | Serving cell becomes better than threshold. |
| A2 | Serving cell becomes worse than threshold. |
| A3 | Neighbor cell becomes offset better than SpCell. |
| A4 | Neighbor cell becomes better than threshold. |
| A5 | SpCell becomes worse than threshold1 and neighbor cell becomes better than threshold2. |
| A6 | Neighbor cell becomes offset better than SCell. |

For detection of A1∼A6 events, predicted RRM measurements can be used provided that the predicted RRM measurements are reliable and accurate.

Requirement for periodic reporting of predicted RRM measurement is suitable to be discussed in the thread for requirements for RRM measurement prediction.

RAN4 to define delay requirement for reporting triggered by indirect event prediction irrespective of whether the core requirement for RRM measurement prediction can be reused or not.

# Summary

In this contribution indirect event prediction for AI/ML aided mobility is discussed. The observations and proposal below summarize the discussion presented in this paper.

- RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect event prediction.

RAN4 to study feasibility of defining core requirement for direct event prediction in Q2 2026.

For detection of A1∼A6 events, predicted RRM measurements can be used provided that the predicted RRM measurements are reliable and accurate.

Requirement for periodic reporting of predicted RRM measurement is suitable to be discussed in the thread for requirements for RRM measurement prediction.

- RAN4 to define delay requirement for reporting triggered by indirect event prediction irrespective of whether the core requirement for RRM measurement prediction can be reused or not.

# References

RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, Interdigital.