3GPP TSG-RAN WG4 Meeting # 117	R4-2521209

Dallas, US, November 17 – 21 2025

Agenda Item:	8.8.1

Source:	Ericsson

Title:	Discussion on general aspects related to AI/ML study for 6G

Document for:	Approval

# Introduction

In this contribution general aspects related to AI/ML study for 6G are discussed. Interoperability issues, issues related to LCM and post deployment testing, and the considerations for channel model for evaluations of AI/ML functionalities are particularly discussed.

# Discussion

## Interoperability

#### Interoperability issues relating to one-sided model deployment

During the 5G-Advanced study item and work item phases, it has been observed that one-sided model deployment is a feasible approach to support use cases such as RRM measurement prediction for mobility, positioning, CSI prediction, and beam management. In these scenarios, the AI/ML model is deployed at the UE, which is configured by the network node to perform inference on measurements and report the results back to the network.

If model performance varies across different UEs and their respective models, the network must be aware of which UE it is interacting with, when and where AI/ML functionality can be configured for that UE, and what level of performance can be expected. Requiring such detailed knowledge for each individual UE undermines interoperability among UEs that support AI/ML for the same feature resulting into increased complexity of deploying AI/ML-based functionality in 6G systems.

For effective deployment of AI/ML based features, performance of UEs from different vendors implementing AI/ML based functionality needs to be consistent with each other. It is important that the AI/ML based operations performed by UEs with different flavors of AI/ML model architecture are uniform. In situations where the network needs to support a heterogeneous mix of UEs, some capable of using AI/ML models for measurement inference and others that are not, it is also crucial to ensure that the performance of both types of UEs are comparable and within the same error threshold. Ensuring consistency in terms of performance among variety of UEs is therefore essential to enable the network node to make uniform decisions regardless of the types of AI/ML model supported by the UE and whether the UE network is interacting with bears AI/ML capability or not.

Interoperability across UEs in our view is fundamental for 6G and 3GPP specification should play a vital role to achieve it. If a decent level of interoperability cannot be guaranteed, the deployment complexity would increase enormously, mandating a need to update or modify specification for every new UE on the market.

In 6G, for the same functionality, network needs to support a UEs with different flavours of AI/ML model implementation.

In 6G, for the same functionality, network needs to support a mix of UEs that can use AI/ML model to infer measurements and UEs that do not support AI/ML based inferencing.

If the AI/ML model performance varies between different UEs and their models, then the network needs to know which UE it is interacting with, and when/where it can configure AI/ML functionality to this UE and the performance that can be expected from this UE. Needing to have such knowledge for each of the UEs will compromise interoperability and increase the complexity of deploying the AI/ML based functionality in 6G.

RAN4 to study interoperability issues arising from the deployment of UE sided AI/ML models for agreed functionalities in 6G.

#### Interoperability issues relating to two-sided model deployment

In contrast to the one-sided model, two-sided model deployment refers to scenarios or use cases where AI/ML models supporting a specific functionality are deployed both at the UE and the network node. In simpler terms, the AI/ML operation is partitioned such that one part is executed by the model hosted at the UE, while the other part is executed by the model hosted at the network node.

For such operations, ensuring interoperability between the AI/ML models deployed at the two different nodes in the cellular network is essential. A representative example of this approach is CSI compression, which is currently being specified as part of the 5G-Advanced Release 20 work item. Insights gained from this work item, along with the strategies developed to address interoperability challenges in two-sided AI/ML model deployment, can potentially be leveraged to resolve similar issues in AI/ML-based 6G features that require two-sided model deployment.

Interoperability for two-side model is being addressed for CSI compression in 5G-A Rel. 20 WI. Learnings from this WI and the approaches identified to handle interoperability issues in two-side AI/ML model deployment can potentially be used to address interoperability issues in AI/ML based 6G features that require two-side model deployment.

## LCM and post deployment testing

To support an AI/ML-based feature, the AI/ML model deployed at the UE undergoes various stages of life cycle management (LCM). LCM procedures such as model training, retraining, and updates are critical to ensuring the reliability of the AI/ML model, but they also introduce complexity into the testing process.

Depending on the deployment strategy, either a trained or an untrained model may be deployed at the UE. When a trained model is deployed, it is assumed that the model’s performance has been validated prior to deployment. In this case, the UE supporting the AI/ML-based feature is tested and its performance verified before being introduced into the field. However, when the model is updated post-deployment, the challenge arises of how to validate the performance of the updated AI/ML model of a UE in field. This is particularly important to ensure that prediction accuracy does not degrade after the model update.

A similar challenge exists when an untrained AI/ML model is deployed at the UE. In such cases, the model must first be trained, potentially through a data collection procedure performed by the UE, before it can be used for inference. Since the training occurs after deployment, the same questions regarding performance validation apply.

The impact of LCM procedures on RAN4 requirements can potentially be addressed once the AI/ML-based features to be studied for 6G are identified. However, discussions around the increased testing complexity introduced by these procedures can already begin within RAN4.

LCM procedures like model training, model retraining, and model update are critical to ensure reliability of the AI/ML model performance.

LCM procedures like model training, model retraining, and model update impose complexity to testing procedures.

RAN4 to study testing aspects related to performance validation of the UE based AI/ML model that is updated after deployment and the model that is trained after deployment.

### Performance monitoring

Performance of an AI/ML model is often constrained to the scenario for which it is trained for. It is therefore essential to monitor the performance of the AI/ML model continuously. For UE-side models, performance monitoring can be conducted either by the UE itself or by the network. In the case of UE-side monitoring, the performance metric used by the UE may not be reported to the network. Conversely, for network-side monitoring, the UE must report the relevant monitoring metric to the network node. Based on the reported metric, if the network determines that the AI/ML model’s performance has degraded to a point where it is no longer reliable or useful, it may take corrective actions. These actions could include configuring the UE to fall back to a non-AI/ML procedure (thus avoiding AI/ML-based measurement inferencing), retraining the model, or updating the AI/ML model to maintain consistent performance. For network-side monitoring, the accuracy and quality of the reported monitoring metric are critical. If the metric lacks sufficient accuracy or quality, the network node cannot reliably configure the UE to take appropriate corrective actions.

Monitoring requirements become even more significant when AI/ML model implementation is UE implementation specific. This leads to scenarios where different UE vendors deploy different flavors of AI/ML models to support the same feature. Consistency in the quality and accuracy of monitoring metrics reported across different AI/ML implementations is needed to ensure that the network can take uniform actions based on the reported metrics, regardless of the UE model variation. While the specific monitoring metric may vary depending on the AI/ML-based use case, the fundamental question of what such a metric should represent and the KPI for performance monitoring needs to be studied by RAN4.

For functionalities to be supported by UE-sided model, choice of the AI/ML model for deployment can be up to UE implementation.

For functionalities to be supported by UE-sided model, different flavors of AI/ML model may be deployed by different UE vendors for the same functionality.

RAN4 to study and identify monitoring metric relevant for UE sided model monitoring. Details of monitoring metric can be studied on a per use case basis.

## Channel model for Evaluation

As we proposed for previous meeting [1], there could be different test setups for AI based receiver, such as legacy setup by test signal and channel emulator, or data driven test framework. If the legacy setup is applied, the variable CDL-based channel model could be used to have more richness and coverage of the propagation environments. Otherwise, the test dataset could be considered if data driven test framework is agreed for AI-based performance tests.

We also propose to consider following aspects when using CDL models for AI receiver demodulation requirements.

Model variants depending on scenarios covered by AI feature

Model variants depending on time, frequency and spatial drifting of the channel and UE capabilities

Balance between channel model richness and test complexity.

How to avoid test signals being used for AI model training.

Currently in 3GPP AI studies, most AI models are trained by system simulation data and corresponding channel characters are based on RAN1 CDL model definitions. Although the implemented AI model might be trained by real field data, it could be possible to take simulation channel for the standard performance tests since the channel model in system simulation also has high richness and variable during the simulation. In that case, a potential method is that getting channel filter coefficients dataset based on simulations and take it as database for channel emulator usage. This method is like a mixed way between channel model driven and data driven tests. The benefit is that the channel coefficient data set might be independent from the Tx signal and could be applied for wider test cases.

However, the limitations of TE, such as storage space, number of channel filter taps and coefficient precision etc., should be studied first. If TE have such limitations, it means we might need to convert CDL models from system simulations to a “reduced” version model and store them in TE finally. It needs more studies to check how the AI model performance change when they are tested by these “reduced” version model.

Alignment among companies would focus on the RAN1 channel model implementation and the channel coefficients distribution. The experience from 2-sided CSI comparison could be referred as the starting point.

The randomness would lead to performance offset at a certain SNR or throughput point. In that case, how to define performance metric should be studied accordingly.

If the test framework for AI based receiver is using test signal and channel emulator, following aspects should be considered for channel model application:

- Model variants depending on scenarios covered by AI feature

- Model variants depending on time, frequency and spatial drifting of the channel and UE capabilities

- Balance between channel model richness and test complexity.

- How to avoid test signals being used for AI model training.

It might be possible to take channel coefficients from system level simulations to create data set for standardization tests.

The alignment method and the performance metrics could be different from legacy tests.

Study the feasibility and methodology of taking channel coefficient from system/link level simulations as data set for further tests.

The TE limitations, such as storage space, number of channel filter taps, coefficient precision etc., should be studied and improved for AI if possible.

Study feasible performance metrics based on channel model with rich randomness.

# Summary

In this contribution, interoperability issues, issues related to LCM and post deployment testing, and the considerations for channel model for evaluations of AI/ML functionalities for 6G are discussed.

The observations and proposals below summarize the discussion presented in this contribution.

Interoperability

- In 6G, for the same functionality, network needs to support a UEs with different flavours of AI/ML model implementation.

In 6G, for the same functionality, network needs to support a mix of UEs that can use AI/ML model to infer measurements and UEs that do not support AI/ML based inferencing.

If the AI/ML model performance varies between different UEs and their models, then the network needs to know which UE it is interacting with, and when/where it can configure AI/ML functionality to this UE and the performance that can be expected from this UE. Needing to have such knowledge for each of the UEs will compromise interoperability and increase the complexity of deploying the AI/ML based functionality in 6G.

- RAN4 to study interoperability issues arising from the deployment of UE sided AI/ML models for agreed functionalities in 6G.

Interoperability for two-side model is being addressed for CSI compression in 5G-A Rel. 20 WI. Learnings from this WI and the approaches identified to handle interoperability issues in two-side AI/ML model deployment can potentially be used to address interoperability issues in AI/ML based 6G features that require two-side model deployment.

LCM and post deployment testing

LCM procedures like model training, model retraining, and model update are critical to ensure reliability of the AI/ML model performance.

LCM procedures like model training, model retraining, and model update impose complexity to testing procedures.

RAN4 to study testing aspects related to performance validation of the UE based AI/ML model that is updated after deployment and the model that is trained after deployment.

Performance monitoring

For functionalities to be supported by UE-sided model, choice of the AI/ML model for deployment can be up to UE implementation.

For functionalities to be supported by UE-sided model, different flavors of AI/ML model may be deployed by different UE vendors for the same functionality.

RAN4 to study and identify monitoring metric relevant for UE sided model monitoring. Details of monitoring metric can be studied on a per use case basis.

Channel model for Evaluation

If the test framework for AI based receiver is using test signal and channel emulator, following aspects should be considered for channel model application:

- Model variants depending on scenarios covered by AI feature

- Model variants depending on time, frequency and spatial drifting of the channel and UE capabilities

- Balance between channel model richness and test complexity.

- How to avoid test signals being used for AI model training.

It might be possible to take channel coefficients from system level simulations to create data set for standardization tests.

The alignment method and the performance metrics could be different from legacy tests.

Study the feasibility and methodology of taking channel coefficient from system/link level simulations as data set for further tests.

The TE limitations, such as storage space, number of channel filter taps, coefficient precision etc., should be studied and improved for AI if possible.

Study feasible performance metrics based on channel model with rich randomness.

# References

R4-2513060, RAN4 study on 6G AI/ML, Ericsson.