3GPP TSG-RAN WG4 Meeting #117	R4-2521216

Dallas, Tx, USA, 17th – 21st November 2025

Source: 	Ericsson

Title:  	Discussion on performance requirements for NTN Ku band

Agenda Item:	5.10.2

Document for:	Discussion

# Introduction

This contribution outlines our point of view on on the issues outlined in WF [1].

# Discussion

Issue 1-2-4: Test cases for Ku bands in FR1-NTN

WayForward • Further discuss the following options • Option 1: reuse exiting test cases defined for SAN FR1 o Add a table clarifying test applicability for the corresponding test cases for VSAT UEs. o Add OTA related parameters Option 2: reuse existing test cases defined for FR2-NTN, unless otherwise specified o Introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

In our opinion, the issue can be addressed by adding the clauses for VSAT UE test cases under the corresponding clauses for SAN FR1. The content can either refer to existing test cases defined for FR2-NTN or introduce new configurations as needed.

Proposal 1: Adding the clauses for VSAT UE test cases under the corresponding clauses for SAN FR1. The content can either refer to existing test cases defined for FR2-NTN or introduce new configurations as needed.

#### Issue 1-2-5: Test applicability and principle

Agreement - The testing related to satellite access defined in A.3.36 with FR2 can be a reference. - If mobile VSAT UE supports both GSO and NGSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. - RAN4 to discuss the test applicability for VSAT supporting Ku bands in both FR1-NTN and FR2-NTN, if such VSAT exists. - Option 1: it is at least to pass two test configurations: one with FR1 numerology and one with FR2 numerology. - Other options are not precluded.

The possibility of choosing only one of the two tests arises when one test is clearly easier to pass than the other.

However, the tests with FR1 numerology and those with FR2 numerology do not show a significant difference in the demand on UE.

It may be possible to design the test cases for FR1 and FR2 numerology with different parameters to better balance coverage and workload, but this is not currently reflected in test applicability.

Proposal 2: Support Option 1, VSAT UE supporting both GSO and NGSO shall pass two test configurations: one with FR1 numerology and one with FR2 numerology.

# Conclusion

Proposal 1: Adding the clauses for VSAT UE test cases under the corresponding clauses for SAN FR1. The content can either refer to existing test cases defined for FR2-NTN or introduce new configurations as needed.

Proposal 2: Support Option 1, VSAT UE supporting both GSO and NGSO shall pass two test configurations: one with FR1 numerology and one with FR2 numerology.

# References

- R4-2515055, WF [116bis][310] R19 RRMperf_Part2_NTN_V2